3GPP TSG-RAN WG4 Meeting #115	R4-2507934

Malta, MT, 19th-23rd May 2025

Title: 	           WF on HPUE RF requirements

Agenda Item:	7.1.2.1.2

Source: 	Qualcomm, Skyworks

Document for:	Approval

# 1. PC1.5 for intra-band contiguous UL CA

## 1.1 MPR values for intra-band contiguous ULCA with non-contiguous RB allocation

Agreement:

  - The following MPR values have been adopted for intra-band contiguous ULCA with non-contiguous RB allocations

| Modulation |  | PC1.5 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | FWA |  |  |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | PI/2 BPSK | 5 | 9.5 | 16.5 | 4.5 | 9 | 16 |
|  | QPSK | 5 | 9.5 | 16.5 | 4.5 | 9 | 16 |
|  | 16QAM | 5 | 9.5 | 16.5 | 4.5 | 9 | 16 |
|  | 64QAM | 7 | 9.5 | 16.5 | 6.5 | 9 | 16 |
|  | 256QAM | 8.5 | 9.5 | 16.5 | 8 | 9 | 16 |
| CP-OFDM | QPSK | 5.5 | 10 | 17 | 5 | 9.5 | 16.5 |
|  | 16QAM | 5.5 | 10 | 17 | 5 | 9.5 | 16.5 |
|  | 64QAM | 7 | 10 | 17 | 6.5 | 9.5 | 16.5 |
|  | 256QAM | 9.5 | 10 | 17 | 9 | 9.5 | 16.5 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3 UE indicating TxD supported |  |  |  |  |  |  |  |

## 1.3 MPR values for intra-band non-contiguous ULCA (with dualPA)

Agreement:

- The following MPR values have been agreed:

MPR= MA+2 to meet -30dBm/MHz Where MA is defined in clause 6.2A.2.2.1.2 for Handheld UE

MPR= MA+2 to meet -13dBm/MHz Where MA is defined in clause 6.2A.2.2.2.2 for Handheld UE

## 1.4 A-MPR applicability for intra-band contiguous ULCA with contiguous RB allocation

Online agreement:

- PC1.5 A-MPR for CA_NS_04 for contiguous CC/contiguous RBs section 6.2A.3.1.1.1 of TS38.101-1 [2] should be revised as follows:

## 1.5 CA_NS_04 A-MPR for intra-band contiguous ULCA with non-contiguous RB allocation

Comment:

Companies are encouraged to investigate the following proposal and the accompanying modification to the A-MPR algorithm captured in R4-2507906:

In section 6.2A.3.1.1.1.2 for non-contiguous allocations make the following correction:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

IF (Ffilter,low < FIM3,low_block,high) OR  (2490.5 MHz> FIM3,low_block,low)

-  A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3

Else

-	if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR

-	if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3, Table 6.2A.2.1-4, Table 6.2A.2.1-5 for PC3 for bandwidth classes B and C, PC2 for bandwidth classes B and C and PC1.5 bandwidth class C respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-  FIM3,low_block,low = (2 * Flow_alloc,low_edge ) – Fhigh_alloc,high_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.1.1

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.1.1

6.2A.3.1.1.1.3	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA = 	13; 	     0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;     3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;     16.56 ≤ B < 21.96

6; 	 21.96 ≤ B

AMPRIM3=MA, Where MA is defined as follows for 2 Tx PC1.5

MA = 	16.5; 	0 ≤ B < 1.44

15.5; 	1.44 ≤ B < 2.88

14.5;     2.88 ≤ B < 5.76

12.5; 	5.76 ≤ B < 10.8

10.5; 	10.8 ≤ B <23.04

10;       23.04 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 (MHz), where SCS1 and SCS2 are expressed in kHz.

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency

Online agreement:

  - Discuss the requirements and assumption for PC1.5

## 1.6 CA_NS_04 A-MPR values for intra-band non-contiguous ULCA (with dualPA)

Agreement:

- The following A-MPR values have been agreed:

A-MPRIM3= MA+2 to meet -25dBm/MHz, where MA is defined in clause 6.2A.3.1.2.1.3 for Handheld UE

A-MPRIM3= MA+2 to meet -13dBm/MHz, where MA is defined in clause 6.2A.3.1.2.1.4 for Handheld UE

## 1.7 CA_NS_04 A-MPR values for intra-band contiguous ULCA with non-contiguous RB allocation

Comment:

Companies are encouraged to investigate the PC2 AMPRIM3 requirement for 1Tx and 2Tx for intra-band contiguous n41C. A set of example values that can be used as a starting point are given below

AMPRIM3=MA, Where MA is defined as follows

MA = [16]; 	0 ≤ B < 1.44

[15]; 	1.44 ≤ B < 2.88

[14];      2.88 ≤ B < 5.76

[12.5]; 	5.76 ≤ B < 10.8

[10.5]; 	10.8 ≤ B < 23.04

[9];      23.04 ≤ B

Online agreement:

  - The missing requirements for PC2 needs to be added by Maintenance since Rel-17, while technical discussion can be organized in this WI

  - Discuss the requirements and assumption for PC1.5

1.8 PCMAX/PCMAX,C for intra-band non-contiguous ULCA (with dual PA)

Online agreement:

- PCmax equation are not modified to account for a PSD difference threshold of 0dB or 6dB.