3GPP TSG-RAN WG4 Meeting #115													R4-2507939

Malta, MT, 19th – 23rd May, 2025

Source:	Nokia

Title:	TP for FS_NR_FR1_DL_Frag_Carrier RRM aspect

Agenda item:	7.5.2

Document for:	Discussion

TP to TR 38.755 on RF aspects

--- > Start of TP < ---

## 5.4	General aspects of fragmented CA

### 5.4.1	SCell control for fragmented CA

If a change in HW configuration between UE using 1 Rx chain or 2 Rx chains for reception in a fragmented carrier aggregation deployment does impact the UE performance, system performance or the network, there is a need to ensure that whatever ‘mode’ the UE is operating in, this is aligned between the UE and the network. Hence, it needs to be well aligned between UE and network whether the UE is applying 1Rx chain or 2 Rx chain.

In many scenarios, where the UE can change between using 1Rx or 2Rx chains without performance impact, such UE autonomous switching could be possible without specification impact. However, if the UE change between using 1Rx or 2Rx in a Fragmented CA setup, does have system impact, it needs to be clear how and when such HW reconfiguration is allowed and can be performed.

If RAN4 explores solutions which operates differently than the current network controlled SCell operations, RAN4 may involve other working groups if needed.

A fragmented carrier configuration can consist of PCell + SCell and SCell + SCell.

--- > End of TP < ---