3GPP TSG RAN WG4 Meeting #115		 R4-2507947

St Julian’s, Malta  19th – 23th May, 2025

Agenda Item:	7.5.3

Source: 	MediaTek Inc., Xiaomi, Huawei

Title: 	TP for 8.3.2 Triggering condition proposal for option 2

Document for:	Approval

1. Introduction

UE architecture assumption and initial evaluation based on existing test configurations were agreed and captured in the WF [12]. In the previous meeting, RAN4 continue discussion on signalling aspects and general aspect were agreed and captured in WF[22]. In this contribution, we continue discussion and provide our proposals on the open issues.

TP for TR38.755

8	Study of means for a UE to inform the network of appropriate CA configuration it can support

<Start of TP>

8.3	Triggering condition to enable ”One Rx RF chain”

8.3.2 Triggering condition proposal for option 2: UE measurement to determine and indicate to the network

Triggering condition to enable ”One Rx RF chain” with UE assisted in-gap interference measurement and indication

When the UE enables one Rx RF chain to receive two non-contiguous carriers under the network, it was assumed another adjacent channel downlink carrier, from collocated second operator, in the gap may also exists at the same time. From companies’ simulation, when the power difference between the in-gap blocker and wanted carriers are higher than the image rejection ratio, i.e. 25dBc, it may seriously degrade the receiving signal quality of wanted carriers. A new measurement mechanism can be needed to judge the in-gap blocker power level and compare with the wanted signal power levels. Referring [23], an example UE states and example flow of state change are proposed as following:

State 1: UE is configured for CC1 on the band and meets the requirements accordingly

State 2: UE is configured for non-contiguous CC1 and CC2 on the band and uses two Rx chains to receive them and meets existing DL NCCA requirements accordingly

State 3: UE is configured for non-contiguous CC1 and CC2 on the band and uses one Rx RF chain to receive them and meets possible new requirements accordingly

Stete 4: UE is configured same as state 1 (CC1, Pcell) and has been checked no more available Rx RF chain to receive intra-band non-contiguous CC2 using second Rx RF chain

Figure 8.3.2-1: Description of different UE states for fragmented carriers

Figure 8.3.2-2: State diagram for switching between the states

When UE has more available Rx RF chain, UE can just apply legacy DL NCCA architecture to maintain best performance on receiving each CC as illustrated in Figure 8.3.2-2. UE can also be configured for measurement by network to check whether the large interference exists and whether UE is capable to receive one more CC when needed while UE is in state 2.

When UE don’t have more available Rx RF chain (state 4), UE can still be configured for measurement by network doing same check. UE can just report to network capable or not-capable for the one Rx RF chain as UE assisted information.

When UE is configured by network to do measurement on both wanted CCs and in-gap interference, UE can simply using existing RSSI measurement approach for in-gap interference measurement. Network can configure the measurement bandwidth and the virtual RB location to the UE, wherein the RB location to be measured in the in-gap region can be referring to PCC or one of the CCs. The details of measurement flow can be further discussed during WI phase if there is work item in the future.

The benefit if UE can measure in-gap interference can be observed from the state chart is ping-pong between state 2 and state 3 or between state 4 and state 3 can be reduced if large in-gap interference can be discovered.

Figure 8.3.2-3: Spectrum plot of DL fragmented carriers and in-gap interference

It would be possible to measure noise level of both wanted CCs and in-gap range if total BW is within the 100MHz under one Rx RF chain mode. One possible method to measure in-gap interference level is to referring existing RSSI measurement procedure with virtual RB ranking in the in-gap region or refer to measuring neighbour cell procedure. The measurement would be applicable in both one Rx RF chain mode and legacy (partial-separated Rx RF chain) mode.

The ping-pong between state 2/4 and state 3 can be reduced if UE can measure in-gap interference.

An alternative example to avoid frequent interruption due to UE measurement with self-assessment, single shot measurement/interruption can be applied before UE indication to Network.  The workflow can be refereed to below figure.

Figure 8.3.2-4: Work flow on UE switching between normal mode and single Rx mode

In-gap inter-operator interference may not be the only factor that has impact on DL receiving performance when one Rx RF chain mode is configured. During Rx performance evaluation and discussion between companies, it was found self-band Tx leakage also plays an important role. It would be more clear to understand these impacts via evaluating the two factors with the help from system level simulation. The simulation assumptions are listed in the Table 8.3.2-1 below.

Table 8.3.2-1: Assumptions for system level simulation

| Parameter | NR |
| --- | --- |
| Carrier | 2G Hz |
| DL BW | 20MHz |
| UL BW | 20MHz |
| Antenna pattern | TR 38.901 Table 7.3-1 |
| Network layout | 19-sites (57 sectors) with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA |
| System loading and activity | Full buffer 100% |
| Network layout | 19-sites [57 sectors] with wrap-around |
| DL power control | No |
| UL power control | TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB |
| NR UE dropping | NR UE: 100% outdoor UE number: DL active UE: 10 UE per cell |
| Pathloss model | TR 38.901 UMA |
| O2I penetration loss | High penetration loss as in TR 38.901 |

It is assumed co-located inter-operators, taking 19-sites with 3 sectors in each site. 10 UEs in every sector per operator. These are illustrated below:

Figure 8.3.2-5: Network layout model for system level simulation

For the UE parameter assumptions, they are listed in the Table 8.3.2-2. One Rx RF chain ACS performance is evaluated with values of 15 and 26 dB. 33dB is assumed as performance reference of partial separated Rx RF chain configuration.

Table 8.3.2-2: UE assumptions for system level simulation

|  |  | NR |
| --- | --- | --- |
| UE | Tx ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | Duplexer isolation between T/R | 50dB |
|  | Rx baseband analog filter Tx leakage rejection | 5dB (one Rx RF chain) 15dB (partial separated Rx RF chain) |
|  | Rx ACS | 15, 26dB (one Rx RF chain) 33dB (partial separated Rx RF chain) |

Simulation result is illustrated in below figures.

Figure 8.3.2-6: CDF of DL signal and interference distribution w w/o ACI of Tx leakage

From the simulation results, some important aspects can be observed:

a.) In-gap interference and Tx leakage both have impact on DL receiving SINR performance. Which one occupies larger portion may depend on UE implementation and the distance/channel condition between UE and gNB.

b.) UE self-judgement mechanism would be needed with considering both in-gap interference and Tx leakage. As long as UE can guarantee pass the adjusted minimum requirements, if specified. How to judge whether UE is capable for one Rx RF chain mode can be left to UE implementation.

c.) Even though there are some degradations on the DL receiving SINR, there are certain percentage of UEs can still benefit from enabling one Rx RF chain when higher order CA combo is configured, with acceptable degradation.

On how to decide to enable ”one RF Rx chain“ or fall-back to normal mode, this can be left to UE implemention as long as UE can guranteen the Rx performance.  Once UE detect ”single Rx chain operation” is not suitable, then UE can indiate to NW and NW can reconfigure UE with single CC or normal CA mode.

There are several ways to enable triggering condition, those options were summarized in below table.

Table 8.3.2-3: Options for triggering condition to enable ”One Rx RF chain”

| Options | Spec impact | Signaling overhead | UE implementation complexity | Potential performance impact |
| --- | --- | --- | --- | --- |
| Option 1-1: Existing channel quality reporting and procedures without any change | Minimal | No additional signaling | No additional reporting | Not possible to monitor Tx leakage and in gap interference before UE switched to single RX mode. Ping-pong effect between different states with frequently switching on/off when the RF performance degradation caused by “One Rx RF chain” is very large |
| Option 1-2: Reuse the existing channel quality reporting procedure with “One Rx RF chain”share Rx architecture measurement. | Small | Update to measurement configuration is needed to request UE performs measurement in “One Rx RF chain” | No additional reporting | NW is fully aware of the UE’s performance difference between “One Rx RF chain” state and legacy state Measurement gap and interruption required before UE enter into Single RX mode for reporting |
| Option 2a: UE measurement reporting for in gap interference | Big impact with NW configuration and UE reporting (RAN1, RAN2 also need to be involved) | Signaling on measurement configuration and reporting | UE needs to follow NW configuration for measurement and reporting | NW can manage interference Measurement gap and interruption required before UE enter into Single RX mode for reporting Measurement reliability and/or accuracy has not been analyzed |
| Option 2b: UE self-assessment with 1bit indication to network | Small impact with 1bits indication FFS whether measurement gap interruption required for monitoring interference before UE enter into CA mode | Limited signaling overhead on indication | UE self-judgment, no measurement reporting required | NW can aware whether UE state is suitable for single Rx mode Measurement gap and interruption required before UE enter into Single RX mode for reporting The UE indication may imply UE can pass RF minimum requirements NW doesn’t have accurate expectation of how good UE could behave with the “One Rx RF chain” |

The UE could evaluate how good it could behave with the “One Rx RF chain” based on the measurements it needs, such as the self-interference, the strength and location of the in-gap interference, etc. Then the UE indicate the evaluation result to the network, either a simple indication of “yes” or “no”, or in some finer granularity.

With the discussion above, option 1 Existing channel quality reporting and procedures or option 2b UE self-assessment with 1bit indication to network may be more preferable

There are ways to limit the potential impacts on the DL performance for PCell, e.g., 1) TDD CA configuration or FDD CA configurations that have large duplex gap would experience no or little self-interference issue. 2) In case of self-interference for FDD CA configurations, PCC SCC Swapping can be used. 3) The use of 1 Rx RF chain can be enabled when the UE is not located at cell edge

<End of TP>

4. Reference for the contribution:

To be consistent with TR38.755, the reference contributions are based on TR38.755 v0.2.0

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”.

[3] 	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4] 	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5] 	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[6] 	R4-2413339 Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, RAN4#112, Nokia

[7] 	R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112bis, Nokia

[8] 	RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

[9] 	R4-2411691 On general aspects of fragmented carriers, RAN4#112, Huawei

[10] 	RP-241269, On Fragmented Carriers in Rel-19, Apple

[11] 	RP-241361, Recommended WF on Fragmented Carriers, MediaTek

[12] 	R4-2417201 WF on NR FR1 DL fragmented carriers study, RAN4#112bis

[13] 	R4-2413031 On architecture options for fragmented spectrum reception, RAN4#112, Skyworks

[14] 	R4-2416138 Discussion on UE Rx chain architecture and network assumption, RAN4#112bis, MediaTek

[15] 	R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

[16] 	R4-2415804 On methods for reducing the number of UE Rx chain, RAN4#112bis, Huawei

[17]	R4-2503479 TP to TR 38.755 on Clause 5, RAN4#114bis, CATT

[18]	R4-2503558 TP for TR38.755: Company C evaluation results, RAN4#114bis, Qualcomm

[19]	R4-2504452 TP on UE RF requirements impact for fragmented carriers, RAN4#114bis, Apple

[20]	R4-2504214 Further discussion of impacts on UE RF requirements and DL performance impacts, RAN4#114bis, MediaTek

[21]	R4-2504129 General aspects of Fragmented CA, RAN4#114bis, Nokia

[22]	R4-2505109 WF on NR FR1 DL fragmented carriers study, RAN4#114bis, MediaTek, Apple, Qualcomm

[23]	R4-2503556 Procedures for fragmented carriers, RAN4#114bis, Qualcomm

[24]	R4-2505784 Methods for reducing the number of UE Rx chains, RAN4#115, Xiaomi

[25]	R4-2507476 TP for TR 38.755 on triggering condition, RAN4#115, Huawei