3GPP TSG-RAN WG4 Meeting #115	R4-2507967

# St Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | Draft | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-1 to introduce blocking characteristic, spurious response for ATG UE CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add the blocking characteristic spurious response, for ATG UE CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add the blocking characteristic spurious response, for ATG UE CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are incomplete for ATG UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6J, 7.7J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change 1>>

## 7.6J	Blocking characteristics for ATG

### 7.6J.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

### 7.6J.2	In-band blocking for ATG

For ATG UE, the in-band blocking requirement defined in clause 7.6.2 applies.

### 7.6J.2A	In-band blocking for ATG CA

#### 7.6J.2A.1	In-band blocking for ATG Intra-band contiguous CA

For ATG UE with intra-band contiguous CA operation, the in-band blocking requirement defined in clause 7.6A.2.1 applies.

#### 7.6J.2A.2	In-band blocking for ATG Inter-band CA

### For ATG UE with inter-band CA operation, the in-band blocking requirement defined in clause 7.6A.2.3 applies.

### 7.6J.3	Out-of-band blocking for ATG

For ATG UE, the out-of-band blocking requirement defined in clause 7.6.3 applies.

### 7.6J.3A	Out-of-band blocking for ATG CA

#### 7.6J.3A.1	Out-of-band blocking for ATG Intra-band contiguous CA

For ATG UE with intra-band contiguous CA operation, the out-of-band blocking requirement defined in clause 7.6A.3.1 applies.

#### 7.6J.3A.2	Out-band blocking for ATG Inter-band CA

### For ATG UE with inter-band CA operation, the out-of-band blocking requirement defined in clause 7.6A.3.3 applies.

NOTE:	In 3GPP, the ATG UE out-of-band blocking specification is defined to ensure the telecommunication link and there may be other sources of interference and regulatory issues that need to be considered when designing ATG UE, i.e. avionic equipment.

## <<End of Change 1>>

## <<Start of Change 2>>

## 7.7J	Spurious response for ATG

For ATG UE, the spurious response defined in clause 7.7 applies.

### 7.7J.1A	Spurious response for ATG CA

#### 7.7J.1A.1	Spurious response for ATG Intra-band contiguous CA

For ATG UE with intra-band contiguous CA operation, the spurious response defined in clause 7.7A.1 applies.

#### 7.7J.1A.2	Spurious response for ATG Inter-band CA

For ATG UE with inter-band CA operation, the spurious response defined in clause 7.7A.3 applies.

## <<End of Change 2>>