3GPP TSG-RAN4 Meeting #115	                                                                     R4-2507970

Malta, MT, 19th – 23rd May 2025

Agenda item:			7.12.1

Source:	CMCC

Title:	WF on NR ATG enhancement

Document for:	Approval

Introduction

This document captures the agreements on Rel-18 and Rel-19 ATG UE RF requirements in RAN4#115.

Topic #1: R18 UE RF requirements maintenance

Issue 1-1: Tx requirements definition for two types of ATG UE antenna

Agreement:

- All transmit signal quality requirements should be specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antennas(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array.

Topic #2: R19 ATG UE RF for inter-band CA and UL MIMO

Issue 2-1: UL configuration for MSD analysis for CA_n3A-n39A with n3 UL.

Agreement:

- Use the following tables to incorporate MSD requirements for different power levels into the spec

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1722.5 | 25 | 15 | 50 (RBstart=83) | 1900 | 30 | MSD Table Y | >ACLR2 |

Issue 2-2: MSD for CA_n3A-n39A with n3 UL

Agreement:

| Output power | MSD (dB) |  |  |  |  | MSD liner average |
| --- | --- | --- | --- | --- | --- | --- |
|  | Huawei | LGE | CMCC | QC | ZTE |  |
| 23dBm | 0.16 | 0.2 | 2.4 | 1.0 | 0.74 | 1.0 |
| 24dBm | 0.19 | 0.4 | 2.5 | 1.3 | 1.0 | 1.2 |
| 25dBm | 0.24 | 0.6 | 2.7 | 1.6 | 1.29 | 1.4 |
| 26dBm | 0.29 | 0.9 | 2.8 | 1.9 | 1.62 | 1.6 |
| 27dBm | 0.36 | 1.2 | 2.9 | 2.3 | 2 | 1.8 |
| 28dBm | 0.44 | 1.5 | 3 | 2.7 | 2.43 | 2.1 |
| 29dBm | 0.55 | 1.9 | 3 | 3.2 | 2.91 | 2.4 |
| 30dBm | 0.68 | 2.3 | 3.1 | 3.6 | 3.46 | 2.7 |
| 31dBm | 0.84 | 2.8 | 3.1 | 4.1 | 4.05 | 3.1 |
| 32dBm | 1.04 | 3.4 | 3.2 | 4.5 | 4.71 | 3.5 |
| 33dBm | 1.28 | 4.0 | 3.2 | 5.0 | 5.41 | 4.0 |
| 34dBm | 1.58 | 4.6 | 3.3 | 5.4 | 6.16 | 4.5 |
| 35dBm | 1.94 | 5.3 | 3.3 | 6.0 | 6.96 | 5.0 |
| 36dBm | 2.36 | 6.1 | 3.4 | 6.6 | 7.79 | 5.7 |
| 37dBm | 2.87 | 6.9 | 3.6 | 7.3 | 8.66 | 6.4 |
| 38dBm | 3.46 | 7.7 | 3.8 | 8.1 | 9.56 | 7.2 |
| 39dBm | 4.16 | 8.5 | 4.2 | 9.0 | 10.48 | 8.0 |
| 40 dBm | 4.95 | 9.4 | 4.7 | 10.2 | 11.43 | 8.9 |

Issue 2-3: ATG UE feature of maximum output power.

Agreement:

- There is no need for further clarification since ATG only support DL CA.