3GPP TSG-RAN WG4 Meeting #115	R4-2507973

St Julian’s, Malta, 19th - 23th May, 2025

Source:	KDDI, Murata, Samsung

Title:	TP to TR 38.719-02-01 for adding impact of harmonic mixing to CA_n18-n77 with UL n77

Agenda item:			6.3.3

Document for:	Approval

- Introduction

The impacts of uplink (UL) and downlink (DL) harmonic mixing associated with REFSENS, delta Tib, delta Rib, and other factors for CA_n18-n77 have been analyzed and incorporated into TR 38.717-02-01-h00, specifically in Section 6.41 of 3GPP TR 38.717-02-01 V17.0.0.

During the standardization phase for Release 17, studies on REFSENS for operator-specific band combinations were permitted. Band n18 is exclusively allocated in Japan (for KDDI), and for CA_n18-n77, the band n77 is constrained to specific frequency ranges: 3520-3560 MHz, 3700-3800 MHz, and 4000-4100 MHz. Consequently, the fourth-order harmonic mixing of n18 DL falling within 3440-3500 MHz of n77 was deemed insignificant. Thus, throughout Release 17 and early stage of Release 18, it was widely considered unnecessary to evaluate the impact of fourth-order harmonic mixing of n18 into n77 UL for CA_n18-n77.

However, in the later stages of Release 18, RAN4 decided to assess intermodulation distortion (IMD) and mixing spurious distortion (MSD) across the entire global band of n77. As a result, notes such as Note 8 [2] in Table 7.3A.5-1 were introduced, specifying that certain test points could be omitted for Japan.

This Technical Proposal (TP) addresses the impact of fourth-order harmonic mixing of n18 into n77 UL for CA_n18-n77, considering the full global band range of n77 (3300-4200 MHz).

In the latest specification 3GPP TS 38.101-1-i90 (2025-03), the influence of fourth-order harmonic mixing of n18 into n77 UL for CA_n18-n77 is not explicitly included due to historical considerations. In contrast, the harmonic mixing effects for CA_n5-n77 and CA_n26-n77 are documented in Table 7.3A.4-4 and summarized in Table 1 below.

Table 1 (abstracted from Table 7.3A.4-4): Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n5 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 0.8 | NOTE 8 | UL1/DL4 |
| n77 | n26 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4 |

We propose reusing the CA_n26-n77 setup for cases where DL BW = 5 MHz, since band n18 falls within band n26, while 815-824 MHz lies outside the range of band n5. For DL BW = 15 MHz, the MSD value has been calculated accordingly. Furthermore, REFSENS differences between band 26 and band n18 are not considered, following Nokia’s comment on R4-2412850.

- References

[1] 3GPP TR 38.717-02-01 V17.0.0 (2022-06)

[2] 3GPP TS 38-101-1-i90(2025-03)

[3] R4-2412850, TP for TR 38719-02-01 CA_n18A-n41A

Text Proposal

<Start of Text Proposal>

5.x	CA_n18-n77

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Unaffected parts are omitted.

5.x.1.3	UE co-existence studies for 1 band UL

Unaffected parts are omitted.

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ nX-nY.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n18 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 815 | 1630 | 2445 | 3260 | 4075 |  |
| n77 | fLow | fHigh | 830 | 1660 | 2490 | 3320 | 4150 |  |
| DL1 | 3300 | 4200 | N/A |  |  | D | D | UL harmonic |
| DL23 | 6600 | 8400 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 |  |  | N/A |  | N/A |  |
| DL4 | 13200 | 16800 |  | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | The two direct hits from the band n18 to the band n77: the 4th and the 5th harmonic to the band n77, have been specified in Rel-17 and hence can be omitted. |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n18 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 860 | 875 | N/A |  |  |  |  | UL harmonic |
| DL23 | 1720 | 1750 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 | 2580 | 2625 |  |  | N/A |  | N/A |  |
| DL4 | 3440 | 3500 | D | N/A | N/A | N/A | N/A |  |
| DL54 | 4300 | 4375 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is a harmonic mixing direct hit case, i.e., UL4/DL1. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

5.x.1.4	∆TIB,c and ∆RIB,c values

TIB,c and RIB,c values have already been defined so this section is omitted.

5.x.1.5	REFSENS requirements

Reference sensitivity exceptions due to the two direct harmonic hits from UL n18 to DL n77 have been specified in Rel-17 and hence are omitted.

Reference sensitivity exceptions due to harmonic mixing from UL n77 for DL CA_n18-n77 is shown in Table 5.x.1.5.

Table 5.x.1.5 (for Table 7.3A.4-4): Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n18 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4X |
| n77 | n18 | 10 | 15 | 25 | 15 | 2.5 | NOTE 8 | UL1/DL4X |
| Unaffected part (NOTE 8) is omitted. NOTE X: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 – 4100 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

5.x.1.6	OOB blocking exception requirements

OOB exception for this CA combination has already been defined in Rel-17, so this section is omitted.

.

<End of Text Proposal>