3GPP TSG RAN WG4 Meeting #115	R4-2508084

St. Julian’s, Malta 19th ‒ 23rd May, 2025

Agenda item:			7.22.1

Source:	Huawei, Hisilicon, OPPO, CATT, Vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Mediatek, APPLE, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Title:	Simulation assumptions on AI/ML CSI compression

Document for:	Approval

- Introduction

RAN4#115 WF on AI/ML part 2 [1] Option 3 … Further simulations (not agreement): In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-xxxxxx is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures. - Performance assuming training and testing with the Samsung frozen decoder - Performance achieved with a jointly trained low complexity encoder and matching decoder - Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity) - Consider transformer, CNN, MLP The aim of this investigation is to determine whether the low complexity encoders are compatible with the frozen Samsung decoder (do not experience any greater performance loss than if paired with a low complexity decoder), and/or could be enhanced by training the frozen decoder with multiple different complexity encoders.

According to Moderator’s summary, ad-hoc tentative observations and agreements, interested companies are encouraged to bring further simulation results with lower-complexity encoder structures and with other encoder backbones. It is observed that the agreed encoder is impractical due to its extremely high complexity, and different UE vendors may employ encoders with different backbones and with different complexity levels. Verifying whether it is feasible to achieve a common decoder, which is compatible with encoders having different backbones and complexity levels, can make the feasibility verification more substantial. The simulation results will also provide guidance for the design of the encoder/decoder in the WI stage (if AI-CSI turns to WI).

- Verification procedure for lower-complexity encoder training

When there is no backbone mismatch between the encoder and the decoder, with the assumption of a common encoder structure both for the frozen decoder training and the encoder retraining, the following observations are achieved.

RAN4#115 Ad hoc minutes on AI/ML air interface study [1] When training “own” encoder with the frozen decoder, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results Around - 2% to +1.4% variation in SGCS from the average

The intention here is to further check the feasibility of achieving a common decoder that is compatible to encoders with different backbones and with different complexities.

  - Verification procedure in Option 3

Figure 1 Procedures to derive SGCS-1/2/3/4

Note: Agreed encoder/decoder means that the model structure is the agreed one, while parameters are based on companies’ own training. Model structures of Encoder-1 and agreed decoder refer to the structures we used for deriving Samsung’s frozen decoder.

The detailed training procedure of SGCS-4 Step-1 is described as follows.

- For each loop, use the Loss to update parameters of the decoder, in addition,

  - update parameters of Encoder-1

  - update parameters of Encoder-2

  - update parameters of Encoder-3

  - update parameters of Encoder-4

Note: In each loop, decoder parameters are updated for one time based on the total loss, each Encoder (e.g., Encoder-1,2,3,4) is also updated for one time based on the backpropagation results from the decoder.

- One of the candidate methods to calculate the Loss:

Loss=n1* Loss1+ n2* Loss2,  n1+n2=1

Note:  n1> n2, using  n1=0.7,  n2=0.3 as a starting point

Loss1=average (lossV,V1+lossV,V2+lossV,V3+lossV,V4)

Loss2=average (lossV1, V2+lossV1, V3+lossV1, V4+lossV2, V3+lossV2, V4+lossV3, V4)

- Note: Introducing Loss2 is to ensure that comparable SGCSs can be achieved for each encoder. Otherwise, after N-to-1 joint training, SGCS of some encoders (SGCS-4 in Step-1) may be much lower than that of others.

- Note: Each lossX,Y can be the reconstruction loss function used in training of SGCS-0/1/2/3, e.g. SGCS loss.

- Note: Other methods of calculating the Loss are not precluded. Companies are encouraged to report other methods if they use.

By comparing SGCS-1 and SGCS-2,

- if SGCS-2 is similar to SGCS-1, we can conclude the feasibility of achieving a common decoder that is compatible to encoders with different backbones and with different complexities.

- if SGCS-2 is much lower than SGCS-1,

  - SGCS-3 is similar to SGCS-0 or SGCS-4 (Step-2) is similar to SGCS-1, we can conclude the feasibility but low-complexity encoder with different backbones should be used for deriving the test decoder.

  - SGCS-3 is much lower than SGCS-0 and SGCS-4 (Step-2) is much lower than SGCS-1, it may result from backbones mismatch or the encoder’s complexity is not high enough. Reasonable backbone and structure for test decoder is needed to ensure the compatibility with different reasonable encoders.

Note: Observations observed from Option 3 also apply to Option 4.

- Model structure assumptions for encoder with lower complexity (for information)

Although the model structures are just for feasibility study, it would be helpful for simulation results alignment to use common model structures for the lower-complexity encoder. In practice, UE vendors may use CNN, Transformer or MLP as the backbone. According to the simulation results from RAN4 #114bis, SGCS may drop with a MLP encoder if the frozen decoder is trained without considering MLP. Companies are encouraged to provide simulation results for different backbones, including MLP, transformer and CNN, with different complexity levels.

## 3.1 Transformer based lower-complexity encoder (Encoder 2)

Table 1. Description of Transformer-based low-complexity encoder

| Model type | Transformer |
| --- | --- |
| FLOPs | 3.63M |
| Model size | 1.6Mbyte |
| d_model | 64 |
| d_feedforward | 256 |
| N_head | 8 |
| d_head | 64 |
| N_TF | 4 |

## 3.2 CNN based lower-complexity encoder (Encoder 3)

Table 2. Description of CNN-based low-complexity encoder

| Model type | CNN (Resnet like design) |
| --- | --- |
| FLOPs | 4.14M |
| Model size | 92Kbyte |
| Number of Resnet blocks | 2 |
| Layer after Resnet blocks | 2 layers (one FC layer, one Conv layer) |
| Layer before Resnet blocks | 1 layer (one Conv layer) |
| Layer within a resnet block | 2 Conv layer and direct combination |
| CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
| Kernel/Channel size within a resnet block | (3,3,16) |
| Size of message (feature map) passed across layers | (13,32) |

## 3.3 MLP based lower-complexity encoder (Encoder 4)

Table 3. Description of MLP-based low-complexity encoder

| Model type | MLP |
| --- | --- |
| FLOPs | 0.22M |
| K | 64 |

- Format of simulation results to submit in next meeting

| Option 3 low-complexity encoders with different backbones |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | （1-to-1）Joint training | New encoder training with (Samsung’s) frozen decoder | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| Encoder-1 | Companies' name | SGCS-0 | SGCS-1 | SGCS-4 (step-1) | SGCS-4 (step-2) |
| Encoder-2 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
| Encoder-3 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
| Encoder-4 | Companies' name | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |

- References

- R4-2508051, WF on AI/ML part 2, RAN4#115

- R4-2507944, Collection of simulation results for AI/ML CSI compression, RAN4#115