3GPP TSG-RAN WG4 Meeting #115									                         R4-2508258

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.4.1

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NonCol_intraB_ENDC_NR_CA_Ph2

Document for:	Information

Topic #1:	RRM requirements

Sub-topic 1: RRM requirements impact

(Online) Issue 1-1: Whether to define RRM requirements when maxMIMO-layer＞4 (it is collocated)

Background

In R18, BS signalling nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC is only present for a UE configured with maxMIMO-layers ≤2. For the case UE configured with maxMIMO-layers =4, it is collocated, however no any related description in R18 RAN2 and RAN4 specification.

R18 RAN2 spec nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15].. nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.. R18 RAN4 RRM spec (take BFD requirements as an example) - 8.5.5 Requirements for SSB based candidate beam detection - 8.5.5.1 Introduction The requirements in this clause apply for each SSB resource in the set q1 configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.5.5.2 unless the UE supports bwpOperationMeasWithoutInterrupt-r18, provided that the SSB is within the configured UE-specific CBW. The requirements in this clause apply when UE is required to perform candidate beam detection on no more than 1 serving cell per band unless otherwise specified. For UE supporting intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided for the configured FR1 intra-band non-contiguous CA, the requirements in this clause apply when UE is required to perform candidate beam detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers, and no more than 1 serving cell per band otherwise..

- Proposals

  - Option 1 (Huawei): When defining requirements for R19 non-collocated WI, the same principle as in R18 is followed, that is, there is no description on UE behaviour when maxMIMO-layer＞4 (it is collocated)

- Recommended WF:

Moderator: we can directly go to the CR.

(Online) Issue 1-2: RRM requirements when maxMIMO-layer = 4

Background

Based on the approved LS R4-2505161 and agreements in previous meeting, the BS signallings are summarized as below.

BS signalling - No new NW signaling needed from Type 4 to single carrier fallback. - When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed - When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1. - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2. - When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, and the details are up to RAN2 NOTE: maxMIMO-layers intends for the legacy BS signaling

For maxMIMO-layers=2 case, UE follows Rel-18 specification. No new BS signalling is needed, and R18 requirements can be reused.

For maxMIMO-layers=4 case, new BS signalling(s) is needed and is left RAN2 to decide.

Agreement:

  - For R19 non-collocated WI, RRM requirements are to be defined when maxMIMO-layers =4.