3GPP TSG-RAN WG4 Meeting #115	R4-2508267

Malta, Malta, May 19th – May 23rd, 2025

Agenda item:			7.23.1

Source:	Nokia

Title:	Adhoc session minutes for FS_NR_AIML_Mob

Document for:	Approval

# Introduction

This document contains the ad-hoc meeting minutes for the discussion on AI/ML mobility SI. The ad-hoc meeting discussed some of the topics from the moderator’s summaries [1][2].

# Discussion.

## Topic#2: RAN4 requirements for RRM Measurement prediction

### Issue 2-5: Impacts on beam/cell level measurements

- Background:

The following agreement was achieved in RAN4#112bis meeting:

Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Furthermore, following agreement was reached in RAN4#114bis meeting:

Agreement: - For Rel. 19 study: - RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

The figure below from TS 38.300 (Figure 9.2.4-1) should be considered as a reference for this discussion.

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

  - Proposal 2 (vivo): For the following conclusion made in RAN4#114bis, RAN4 to update the wording as:

    - For Rel-19 study, RAN4 to not consider L1 beam-level (Point A1) measurement prediction for UE sided model when L1 beam-level (Point A1) predicted results as the report content

  - Proposal 3 (ZTE): RAN4 should study the impacts on requirements for L3 cell-level prediction first and the prediction accuracy for L3 beam-level prediction can use the conclusion of L3 cell level as baseline.

  - Proposal 4 (Tejas): RAN4 should study the mobility enhancements based on L3 cell-level measurement predictions in conjunction with the relaxation of L1 beam-level measurements.

  - Proposal 5 (CATT): RAN4 to deprioritize the study on impact of L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

  - Proposal 6 (Apple): RAN4 to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E)

- Candidate Options:

  - L3 beam-level (Point E) measurement:

    - Option 1: Do not consider L3 beam-level (Point E)

    - Option 2: Consider L3 beam-level (Point E) measurement prediction as second priority

    - Option 3: RAN4 should consider studying the impacts on requirements for L3 beam-level (Point E)

  - L1 beam-level (Point A1):

    - Option 1: Update the agreement reached in RAN4#114bis meeting as:

      - For Rel-19 study, RAN4 to not consider L1 beam-level (Point A1) measurement prediction for UE sided model when L1 beam-level (Point A1) predicted results as the report content

- Recommended WF:

  - Discuss above listed options for L3 beam-level (Point E) and L1 beam-level (Point A1) separately.

  - L3 beam-level (Point E) measurement:

    - Option 1: Do not consider L3 beam-level (Point E)

    - Option 2: Consider L3 beam-level (Point E) measurement prediction as second priority

    - Option 3: RAN4 should consider studying the impacts on requirements for L3 beam-level (Point E)

Discussion:

E//: We prefer Option 2.

ZTE: We prefer Option 3 however we can compromise for Option 2.

Huawei: For all sub use cases, we consider point C. Therefore, we prefer Option 2.

Nokia: We also support Option 2.

MediaTek: RAN2 is discussing L3 beam level measurements. We can consider it in the work item phase.

CATT: We have similar view as ZTE. We should treat beam level measurement as second priority in work item phase, depending upon the progress in RAN2 and as well on the work item scope.

Vivo: RAN2 is studying beam level measurements.

Agreement:

Consider L3 beam-level (Point E) measurement prediction depending upon the progress in RAN2.

Note: RAN4 didn’t study impact of L3 beam level measurements. However, it doesn’t preclude the discussion on L3 beam level measurements in the work item phase.

### Issue 2-1: Relative RSRP accuracy definition

- Background:

Following agreement was reached in RAN4#114bis meeting:

Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

- Proposals and Observations:

  - Proposal 1 (Nokia): For intra-frequency L3-RSRP relative accuracy definition, RAN4 to cover comparison between two RSRP levels from the same cell or different cells.

  - Proposal 2 (Nokia): When defining the relative accuracy for intra-frequency predicted L3-RSRP, RAN4 to consider both of the following cases: Comparison between two predicted values, and comparison between one predicted value and one measured value..

  - Proposal 3 (Nokia): : RAN4 consider the following definition of accuracy of relative in intra-frequency; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP level 1 – reported RSRP level 2) – (ground truth of RSRP level 1 – ground truth of RSRP of level 2), where level 1 and level 2 may belong to the same or different cells on the same frequency. [Note: Reported RSRP can be either measured or predicted]

  - Proposal 4 (Nokia): RAN4 to extend the definition of inter-frequency predicted relative L3-RSRP accuracy to include comparisons between a predicted value and a measured value, regardless of whether one is used as the input for the other.

  - Proposal 5 (Nokia): For inter-frequency scenarios, RAN4 consider the following definition relative accuracy; Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported  RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2. [Note: Reported RSRP can be either measured or predicted]

  - Proposal 6 (vivo): For intra-frequency L3-RSRP, regarding the definition- ‘Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.’, If interpreted in the following manner, the definitions under both approaches can be considered.

    - When the beam here refers to L1 beams, it corresponds to the scenario of predicting L3 (at point A or A1) from L1

    - When the beam here refers to L3 beams, it corresponds to the scenario of predicting RSRP at point E

  - Proposal 7 (vivo): For inter-frequency L3-RSRP, regarding the definition

‘accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].’,

      - For the case that the reported RSRP of Cell2 is predicted, it needs to firstly clarify which scenario this actually corresponds to

      - For the cases that the reported RSRP of Cell2 is measured and is a combination of both predicted and measured samples, both of these two cases can be considered

  - Proposal 8 (Huawei): For inter-frequency L3-RSRP,

    - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

    - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2,

      - if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, or

      - if cell 1 is the target cell based on intra-cell prediction in temporal domain

  - For intra-frequency L3-RSRP,

    - The below definition of beam-level relative predicted accuracy of beam-level L3 RSRP is needed if sub-use cases of L3 filtered beam-level measurement predication are considered in RAN4 in R19, otherwise the definition can be deprioritized.

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

  - Proposal 9 (Ericsson): Reformulate the term “Accuracy of predicted relative L3-RSRP” in the agreements reached in RAN4#114bis to “Relative accuracy of  predicted L3-RSRP measurements”.

  - Proposal 10 (Ericsson): Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), if cell 2 is measured cell to infer measurement for cell 1 (target cell) in the same carrier frequency.

  - Proposal 11 (Ericsson): Relative accuracy of predicted L3-RSRP measurements = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different carrier frequency than cell 2 but in the same FR as cell 2 and cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2.

  - Proposal 12 (Ericsson): Definition for relative accuracy of predicted L3-RSRP measurements in intra-frequency scenarios where beam 1 and beam 2 are from the same cell is not needed.

  - Proposal 13 (Ericsson): Relative accuracy of predicted L3-RSRP measurements for temporal prediction is defined as:

    - Relative accuracy of predicted L3-RSRP measurements = (reported predicted RSRP1 of a cell/beam at time T1 – reported RSRP2 of the same cell/beam at time T2) – (ground truth of RSRP1 of a cell/beam at time T1 – ground truth of RSRP2 of the same cell/beam at time T2).

    - Reported RSRP2 can be predicted or measured.

    - T1 and T2 refer to time instances for temporal domain prediction.

  - Proposal 14 (CMCC): for accuracy of predicted relative L3-RSRP for inter-frequency, it is proposed that the reported RSRP of cell2 can be predicted or measured

  - Proposal 15 (Qualcomm): RAN4 to update the definition of relative L3-RSRP agreed in RAN4#114bis as follows:

    - For intra-frequency L3-RSRP,

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

        - Whether and how to define the requirements on the accuracy of predicted relative beam level L3-RSRP, when the two beams are from the same cell, should be discussed further during the work item phase.

        - Note that whether it becomes the same as the accuracy of predicted relative beam level L3-RSRP would depend on how the definition of the ground truth is established.

    - For inter-frequency L3-RSRP,

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2 or another cell on a different frequency than cell 2 but in the same FR as cell 2.

        - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition.

        - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples.

    - The applicability of the relative RSRP accuracy definition should be confined within the same frequency range

  - Proposal 16 (NTT DOCOMO): Predicted samples are applied for the reported RSRP of cell 2 to keep relative RSRP accuracy meaningful.

  - Proposal 17 (NTT DOCOMO): No need to consider the combination of predicted and measured samples from RAN4 accuracy requirement perspective.

  - Proposal 18 (ZTE): For temproal domain cell level prediction, no need to consider predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell.

  - Proposal 19 (ZTE): If RAN4 consider the relative accuracy for spatial domian or L3 beam level prediction, predicted relative L3-RSRP accuracy when beam1 and beam2 are from the same cell is needed.

  - Proposal 20 (ZTE): For the case that cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, the reported RSRP of cell2 should be measured results.

  - Proposal 21 (OPPO): For the definition of relative accuracy for intra-frequency measurement, not to consider RSRP difference between different beams from the same cell.

  - Proposal 22 (OPPO): For the definition of relative accuracy for inter-frequency measurement, not to consider RSRP difference between predicted results and measured results, i.e., only consider the difference between two predicted results

  - Proposal 23 (LG Electronics): The beam-level relative L3-RSRP accuracy requirement is not necessary for intra-frequency L3-RSRP.

  - Proposal 24 (LG Electronics): Regarding inter-frequency L3-RSRP, for the predicted relative RSRP accuracy requirement, the reported RSRP of cell2 should be predicted value.

  - Proposal 25 (Tejas): RAN4 to clarify the difference between ground truth RSRP and reported RSRP

  - Proposal 26 (Tejas): RAN4 should explore potential enhancements in inter-frequency measurement prediction by combining with temporal domain prediction.

  - Proposal 27 (Xiaomi): For intra-frequency, the definition of the accuracy of predicted relative L3-RSRP shall be based on different UE capabilities:

    - If UE support to predict for multiple cells:

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency

    - If UE doesn’t support to predict for multiple cells:

      - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported measured RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency

  - Proposal 28 (Xiaomi): Accuracy of predicted relative L3-RSRP for different beams of the same cell is not needed.

  - Proposal 29 (Xiaomi): For inter-frequency, if UE support to predict for multiple cells, the relative RSRP can be compared between two different predicted RSRPs. Otherwise, the relative RSRP can be compared between one predicted RSRP and one measured RSRP from two different cells.

  - Proposal 30 (CATT): RAN4 to capture the relative RSRP prediction definition for intra-frequency beam L3-RSRP in the TR as one of study outcomes.

  - Proposal 31 (Apple): Define Relative RSRP Accuracy Requirements (Conditional)

    - If RAN4/RAN2 standardizes multi-cell output prediction scenarios, RAN4 will:

      - Develop relative RSRP accuracy metrics, such as:

        - Rank order accuracy (correctly ranking cells by predicted RSRP).

        - Differential error thresholds (e.g., ±1 dB tolerance for relative differences between top-K cells).

      - Align test methodologies with multi-cell prediction dynamics (e.g., beam/cell correlation, interference coupling).

- Candidate options:

For intra-frequency case, Relative accuracy of predicted L3-RSRP =:

  - Option 1a:   (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

  - Option 1b:  (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

    - Whether and how to define the requirements on the accuracy of predicted relative beam level L3-RSRP, when the two beams are from the same cell, should be discussed further during the work item phase.

    - Note that whether it becomes the same as the accuracy of predicted relative beam level L3-RSRP would depend on how the definition of the ground truth is established.

  - Option 2: (reported predicted L3-RSRP level 1 – reported RSRP level 2) – (ground truth of RSRP level 1 – ground truth of RSRP of level 2), where level 1 and level 2 may belong to the same or different cells on the same frequency. [Note: Reported RSRP can be either measured or predicted]

  - Option 3: (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.’, If interpreted in the following manner, the definitions under both approaches can be considered

    - When the beam here refers to L1 beams, it corresponds to the scenario of predicting L3 (at point A or A1) from L1

    - When the beam here refers to L3 beams, it corresponds to the scenario of predicting RSRP at point E

  - Option 4: (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), if cell 2 is measured cell to infer measurement for cell 1 (target cell) in the same carrier frequency.

  - Option 5a (If UE supports to predict for multiple cells): (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency

  - Option 5b (If UE doesn’t support to predict for multiple cells): (reported predicted L3-RSRP of cell 1 – reported measured RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency

- For inter-frequency case, Relative accuracy of predicted L3-RSRP =:

  - Option 1a: = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference.

  - Option 1b: Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2 or another cell on a different frequency than cell 2 but in the same FR as cell 2.

    - if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2, or

    - if cell 1 is the target cell based on intra-cell prediction in temporal domain

    - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition.

    - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples.

  - Option 2: (reported predicted L3-RSRP of cell 1 – reported  RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2. [Note: Reported RSRP can be either measured or predicted]

  - Option 3: reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].

    - For the case that the reported RSRP of Cell2 is predicted, it needs to firstly clarify which scenario this actually corresponds to

    - For the cases that the reported RSRP of Cell2 is measured and is a combination of both predicted and measured samples, both of these two cases can be considered

  - Option 4: (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different carrier frequency than cell 2 but in the same FR as cell 2 and cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2

  - Option 5: For inter-frequency, if UE support to predict for multiple cells, the relative RSRP can be compared between two different predicted RSRPs. Otherwise, the relative RSRP can be compared between one predicted RSRP and one measured RSRP from two different cells.

- For temporal prediction case, Relative accuracy of predicted L3-RSRP =:

Option 1: Relative accuracy of predicted L3-RSRP measurements for temporal prediction is defined as:

- Relative accuracy of predicted L3-RSRP measurements = (reported predicted RSRP1 of a cell/beam at time T1 – reported RSRP2 of the same cell/beam at time T2) – (ground truth of RSRP1 of a cell/beam at time T1 – ground truth of RSRP2 of the same cell/beam at time T2).

    - Reported RSRP2 can be predicted or measured.

    - T1 and T2 refer to time instances for temporal domain prediction.

- Recommended WF

- Discuss above listed Candidate Options separately for intra-freq and inter-freq cases.

Discussion:

- For intra-frequency L3-RSRP,

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or

  - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell.

    - FFS if this definition will be needed.

Xiaomi: It is not clear yet if UE can predict for multiple cells. It may depend on UE capability.

OPPO: First bullet, UE may predict one cell in one time instance, and the other measurement in another time instance. Therefore, we still need the first bullet. We don’t need second bullet since it is spatial domain definition and not temporal domain definition.

Apple: For second bullet, we think this definition is not needed. AI/ML metric should be aligned with legacy measurement metric.

Qualcomm: This definition should be considered keeping 6G framework in mind. Therefore, second bullet should be considered.

Huawei: Regarding second bullet, if beam level measurement is supported in the study item then we are open to consider the second bullet.

E//: If we replace predicted by measured for cell2, will it address Xiaomi’s concern?

Xiaomi: Yes, it will address our concern.

E//: We don’t know the framework for 6G, therefore, we should be based on 5G framework. It means that we should not consider second bullet.

Nokia: For the first bullet, we want to change the wording. Replace cell with level in the first bullet. Level may mean cell or bell.

Qualcomm:

Apple: Level in legacy are used to preclude some of the RSRP margins during testing. That has nothing to do with beam measurements. (reference tdoc for beam level difference: R4-1814657)

NTT Docomo: If Ground truth is the measured RSRP then the definition of relative RSRP accuracy may need to be reconsidered if cell 2 is measured RSRP

Tentative Agreement:

- For intra-frequency L3-RSRP,

  - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency

    - Cell2 RSRP can be measured or predicted.

    - If Cell 2 RSRP is measured. FFS on how to consider time difference between predicted and measured RSRPs

  - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress and if the AI/ML mobility framework allows to do so.

### Issue 2-7: Inter-frequency prediction scenario

- Background:

The following agreement was reached in RAN4#114bis meeting:

Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

- Proposals and Observations:

  - Proposal 1 (Nokia): For inter-frequency prediction scenario, RAN2 has no sufficient clarity on the definition of correlation coefficient; therefore, RAN4 to wait for the progress in RAN2.

  - Proposal 2 (Nokia): For inter-frequency prediction scenario, in a cluster approach, RRM measurement prediction performance can be enhanced when measurement from a group of cells in one carrier frequency is used by the UE as an input. Therefore, RAN4 should consider the impact of cluster approach.

  - Proposal 3 (vivo): As the key element affecting RSRP prediction accuracy for inter-frequency prediction, correlation coefficient need to consider it when defining metrics. Regarding EPRE difference, as well as frequency distance and frequency separation, hese are coupled with the correlation coefficient and no need to splitting them into more detailed factors for discussion

  - Proposal 4 (vivo): For the cluster-based approach, the criteria for selecting cells can influence inter-frequency prediction accuracy. This factor could also be considered as a potential aspect

  - Proposal 5 (Huawei): For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy is to be specified under certain conditions,

    - channel correlation coefficient

    - FFS on cluster/single cell approach, depending on whether explicit indication and corresponding UE capability are specified in RAN2.

  - Proposal 6 (Huawei): cluster approach or single cell approach would impact on the value of prediction accuracy.

  - Proposal 7 (CMCC): for cluster approach, it is proposed to follow RAN2 agreements that model takes measurements from more than one cell as inputs.

  - Proposal 8 (Qualcomm): RAN4 to not consider EPRE difference as one of the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario.

  - Proposal 9 (OPPO): RAN4 to discuss the measurement period requirements for inter-frequency measurement prediction considering the impact of skipped measurement frequencies or skipped measurement instances which may not need to be considered in the CSSF calculation.

  - Proposal 10 (Tejas): It is proposed that RAN4 not study the impact of cluster-based and non-cluster-based inter-frequency prediction scenarios.

  - Proposal 11 (Tejas): No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario.

  - Proposal 12 (Xiaomi): For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

  - Proposal 13 (MediaTek): For inter-frequency prediction scenario, it is not feasible to study the impact of correlation coefficient unless RAN4 define a common dataset.

  - Proposal 14 (Apple):RAN2’s assumption of collocated cells for frequency domain prediction may represent an unrealistic scenario. RAN4 to investigate frequency domain prediction methodologies for non-collocated cells, ensuring alignment with real-world deployment conditions. (scenario #3)

  - Proposal 15 (Apple): RAN4 to clarify the exact meaning of the correlation coefficient between measured and predicted frequency layers, and how it relates to multi-frequency channel generation (7.6.5 in TR 38.901)

- Candidate Options:

- Regarding EPRE difference

  - Option 1: RAN4 to not consider EPRE difference as one of the factors

  - Option 2: RAN4 to consider EPRE difference as one of the factors

- Regarding Correlation Coefficient

  - Option 1: RAN2 has no sufficient clarity on the definition of correlation coefficient; therefore, RAN4 to wait for the progress in RAN2

  - Option 2: RAN4 to consider channel correlation coefficient for the definition of performance metric for inter-frequency scenarios

  - Option 3: It is not feasible to study the impact of correlation coefficient unless RAN4 define a common dataset.

  - Option 4: To consider the framework of multi-frequency simulation in section 7.6.5 in TR 38.901 for inter frequency evaluations and drop the correlation coefficient

- Regarding Cluster Approach

  - Option 1: RAN4 to consider the impact of cluster approach

  - Option 2: RAN4 to consider the impact of cluster approach

  - Option 3: RAN4 to consider the impact of cluster approach depending on whether explicit indication and corresponding UE capability are specified in RAN2

  - Option 4: Extend the clustering approach to support multi-output cell prediction per frequency layer, enabling the possibility of skipping measurement gaps

- Impact on CSSF:

  - RAN4 to consider the impact on CSSF calculation

  - RAN4 to not consider the impact on CSSF calculation

- Non-collocated Scenario:

  - RAN4 to consider non-collocated scenarios for inter-frequency inter-cell prediction

- Recommended WF:

Discuss above listed candidate options.

Discussion:

- Regarding EPRE difference

  - Option 1: RAN4 to not consider EPRE difference as one of the factors

  - Option 2: RAN4 to consider EPRE difference as one of the factors

Discussion:

Vivo: How we should consider EPRE difference?

Qualcomm: EPRE should be already provided to the UE.

Apple: We understand Qualcomm’s point. It is hard to say to consider or not consider

Xiaomi: EPRE difference should be consistent between training and inference.

MediaTek: How the EPRE of the neigboring cell will be known? No signaling exists at this stage.

- Regarding Correlation Coefficient

  - Option 1: RAN2 has no sufficient clarity on the definition of correlation coefficient; therefore, RAN4 to wait for the progress in RAN2

  - Option 2: RAN4 to consider channel correlation coefficient for the definition of performance metric for inter-frequency scenarios

  - Option 3: It is not feasible to study the impact of correlation coefficient unless RAN4 define a common dataset.

  - Option 4: To consider the framework of multi-frequency simulation in section 7.6.5 in TR 38.901 for inter frequency evaluations and drop the correlation coefficient

MediaTek: We don’t know how to consider correlation coefficient. They just used it as a metric. Different correlation coefficient is reported from different companies.

Qualcomm: We share the same view as MediaTek:

OPPO: It is based on different datasets for different frequencies in RAN2. RAN2 may not have further clarification on it. We can just say that it is the relationship between two frequencies.

Vivo: It can be a system information. It can be discussed during evaluation in RAN4

Xiaomi: We have similar view as vivo.

E//: We shouldn’t rely on RAN2 understanding.

Agreement:

RAN4 to study whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

## Topic#3: RAN4 requirements for measurement event prediction

### Issue 3-1: Performance metric for event prediction use case

- Background:

The following agreement was reached inRAN4#114bis meeting:

Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

- Proposals and Observations:

  - Proposal 1 (Nokia):

    - For event-based measurement reporting based on indirect event prediction, RAN4 to consider a similar accuracy definition as measurement prediction.

    - RAN4 to study a simpler metric for A3 measurement event prediction

      - For the time domain case A measurement event prediction, RAN4 to consider prediction of A3 event early enough a head of ground truth (actual event) as performance metric for true event prediction.

    - For the time domain case B measurement event prediction, RAN4 may consider a similar performance metric definition as that of temporal domain case A.

  - Proposal 2 (Ericsson):

    - RAN4 to consider same accuracy requirement for temporal prediction of RRM measurement and indirect event prediction use case.

    - Performance requirement for direct event prediction needs to be defined for event prediction provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility

  - Proposal 3 (Xiaomi): RAN4 to only consider precision and not consider recall in event prediction accuracy KPI.

    - For RSRP accuracy requirement for event triggered reporting, if there are multiple predicted RSRPs in one report, we propose to evaluate RSRP accuracies continuously over multiple time instances to enhance the performance of event reporting.

    - For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances.

  - Proposal 4 (OPPO): For indirect prediction,

    - If the report includes the predicted RSRPs, the requirements for measurement prediction can be reused.

    - The following can be considered as possible KPI to evaluate the performance:

      - Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.)

      - Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges)

      - Alt 3: The handover failure rate per UE under certain time durations

  - Proposal 5 (Huawei):

    - In indirect event prediction, if the MR includes the intermediate predicted RSRP corresponding to the predicted event occurrence time, the intermediate RSRP shall satisfy the measurement prediction accuracy.

    - RAN4 to discuss the following options for defining accuracy for direct event prediction:

      - Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that F1 score is consistently above a value.

      - Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1].

    - RAN4 to discuss the following options for indirect event prediction

      - Option 1: Define accuracy for indirect event prediction

      - Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD (time difference between the ground truth event and the predict event), under the condition that F1 score consistently is above a value.

      - Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score, under the conditions of certain fixed ETD. More specifically, the qualified UE shall satisfy F1 score consistently is above a value, under the condition that event is predicted within certain ETD.

      - Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

  - Proposal 6 (CATT): Successful prediction rate can be used as a test metric for AI/ML model performance monitoring during a time period for both direct and indirect measurement event prediction.

    - RAN4 to define accuracy requirement for indirect case and reuse the accuracy requirement for RRM measurement prediction, when the report includes the predicted RSRPs. No accuracy requirements need to be defined for each final result report.

    - RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction, based on RAN2 agreement.

  - Proposal 7 (Qualcomm):

    - RAN4 to consider the following as one of the performance metrics for AI/ML-based measurement event prediction:

      - One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%).

      - The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP.

      - The above two metrics may jointly serve as a performance assessment metric for event prediction, or one can be down-selected. The choice of the metric and the details could further depend on the following aspects to be discussed and decided by RAN2:

        - Whether the AI model is for indirect or direct event prediction.

        - What metrics are to be reported to the serving cell.

  - Proposal 8 (CMCC):

    - For indirect measurement event prediction, if the report includes the predicted RSRPs, it is proposed to define requirements for the predicted RSRPs, and the conclusion on RRM measurement prediction can be reused.

  - Proposal 9 (Apple):

    - Investigate the translation of RAN2 HO KPIs into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures.

    - Different cells may operate under distinct network conditions (e.g., transmit (TX) codebook configurations, beamwidths, TX implementation variations). RAN4 to investigate whether an AI/ML model trained on data from one cell (e.g., neighbor cell 1) can generalize effectively for inference on another cell. Critical conditions (e.g., beam alignment parameters, signal propagation characteristics, SINR) must be defined to ensure consistency between training and inference phases for robust generalization.

    - RAN4 should consider the following performance metrics for AI/ML-based measurement event prediction:

      - Baseline Metrics from L3-RSRP Prediction

        - Retain one or multiple existing performance metrics defined for AI/ML-based L3-RSRP prediction

      - Event-Specific Temporal Accuracy

        - Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP.

        - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2.

      - Combined or Down-Selected Metrics

        - The above metrics (L3-RSRP prediction accuracy + temporal gap) may be:

        - Jointly used for comprehensive performance assessment, or

        - Independently selected depending on use-case priorities.

  - Proposal 10 (vivo):

    - F1 score under certain fixed ETD conditions can be served as the performance metric for evaluating the accuracy of measurement event prediction.

    - If the report includes the predicted RSRPs corresponding to the predicted event occurrence time, RAN4 will define requirement for the predicted RSRPs

- Candidate options:

- For indirect case:

  - Option 1: RAN4 to study the applicability of existing measurement event criteria and reporting requirements to event prediction use case

  - Option 2: RAN4 to define a new performance metric for event prediction

    - Option 2-1: RAN4 to study a simpler metric for A3 measurement event prediction

    - Option 2-2: If the MR includes the intermediate predicted RSRP corresponding to the predicted event occurrence time, the intermediate RSRP shall satisfy the measurement prediction accuracy

    - Option 2-3: define successful prediction rate as a test metric for AI/ML model performance monitoring

    - Option 2-4: RAN4 to consider the following as one of the performance metrics for AI/ML-based measurement event prediction:

      - One or multiple of the performance metrics to be defined for AI/ML-based L3-RSRP prediction in terms of predicted absolute L3-RSRP accuracy, predicted relative L3-RSRP accuracy, and others such as Top-K(%) and Top-K/1(%).

      - The gap in the time domain from the predicted time instance when the event is predicted to occur to the time instance when the event occurs, based on the to-be-defined ground truth L3-RSRP.

    - Option 2-5: Investigate the translation of RAN2 HO KPIs into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study

    - Option 2-6: Use F1 score under certain fixed ETD conditions as a performance metric for evaluating the accuracy of measurement event prediction

    - Option 2-7: RAN4 should consider the following performance metrics for AI/ML-based measurement event prediction:

      - Baseline Metrics from L3-RSRP Prediction

        - Retain one or multiple existing performance metrics defined for AI/ML-based L3-RSRP prediction

      - Event-Specific Temporal Accuracy

        - Define a time-domain gap metric: The difference between the predicted time instance when the event is forecasted to occur (e.g., entering condition met at T2) and the actual time instance when the event occurs, based on ground truth L3-RSRP.

        - Example: If an event is predicted at T2 but occurs at T3 (ground truth), the gap is ΔT = T3 – T2.

      - Combined or Down-Selected Metrics

        - The above metrics (L3-RSRP prediction accuracy + temporal gap) may be:

        - Jointly used for comprehensive performance assessment, or

        - Independently selected depending on use-case priorities.

    - Option 2-8: RAN4 to only consider precision and not consider recall in event prediction accuracy KPI.

      - For RSRP accuracy requirement for event triggered reporting, if there are multiple predicted RSRPs in one report, evaluate RSRP accuracies continuously over multiple time instances to enhance the performance of event reporting.

      - For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances.

    - Option 2-9: RAN4 to investigate the generalization capability of AI/ML models across cells operating under distinct network conditions, and define critical parameters needed to ensure consistency between training and inference phases

    - Option 3: RAN4 to consider same accuracy requirement for temporal prediction of RRM measurement and indirect event prediction use case

- For direct case:

  - Option 1: Performance requirement for direct event prediction needs to be defined for event prediction provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility.

- Recommended WF

  - Discuss above listed Candidate Options.

Discussion:

Agreement from RAN4 # 114bis meeting:

- Potential RRM requirement impact for event prediction use case - for indirect case:

- RAN4 to study the requirement for the predicted event triggered reporting

  - RAN4 to study the requirement for [delay and accuracy]

  - RAN4 to discuss the metric for the event prediction

- Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time],

  - Candidate options:

    - Option 1: RAN4 will define requirement for the predicted RSRPs

    - Option 2: RAN4 will not define requirement for the predicted RSRPs

OPPO: First we need to define the definition of the delay requirement.

- First interpretation can be the delay for the event occurring

- Second interpretation can be the inference delay

Regarding predicted RSRPs, we don’t have a strong view . However, Option 1 is more straightforward.

Moderator: delay considered is the delay for event occurrence.

Vivo: We should firstly discuss, what should be the starting point of event prediction.

Qualcomm: We think the same as vivo. We need to define the timeline, for the delay of event prediction.

Apple: We want to clarify that what is the requirement for predicted RSRP?

Moderator: it is rather RSRP accuracy.

Xiaomi: There are two cases. One to predict event in advance. Second, to reduce measurements. Both cases should be considered.

E//: Accuracy requirements defined for RRM prediction use case should be reused.

Huawei: Regarding the accuracy, we should use only for indirect case

Docomo: We should reuse RRM prediction requirement and define performance metric for event prediction on top of rrm prediction metric.

Agreement:

In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

Discussion:

Event prediction accuracy metric:

  - Option 1: RAN4 to study the applicability of existing measurement event criteria and reporting requirements to event prediction use case

  - Option 2-6: Use F1 score under certain fixed ETD conditions as a performance metric for evaluating the accuracy of measurement event prediction

  - Option 3: A new event prediction accuracy metric will be needed in RAN4.

Apple: RAN2 has multiple criteria.

Qualcomm: Option 3

E//: We should wait for RAN2’s further progress.

Vivo: Change the title

Huawei: Here, we can’t follow the existing legacy criteria. Since, in legacy, it depends on UE capability. Option 2 and Option 3 are difference

Docomo: We need to discuss the delay requirement separately from the event prediction accuracy requirement.

# References

[1] R4-2505564, “Topic summary for [115][215] FS_NR_AIML_Mob_Part1”, Moderator(Nokia), RAN4#115

[2] R4-2505565, “Topic summary for [115][216] FS_NR_AIML_Mob_Part2”, Moderator(OPPO), RAN4#115