# 3GPP TSG-RAN WG4 Meeting #115	R4-2508277

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.27.1

Source:	Ericsson

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part1

Document for:	Approval

# Topic #1: On-demand SSB(OD-SSB) requirements (AI 7.27.3.1)

### Sub-topic 1-1: OD-SSB based deactivated SCell L3 measurement (Case 1)

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 1: Lower bound for measurement in FMW

Agreement:

Introduce UE capability on whether a lower bound is needed, i.e.,

- No signalling is needed if UE doesn’t need the lower bound (default value), or,

- The lower bound for measurement/cell identification period in FMW is derived based on 10ms periodicity in case 1 and case 2

Note: RAN4 requests RAN2 to implement the UE’s capability based on the above agreement.

Sub-issue 2: OD-SSB in FMW collided with MG

- Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

- Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 1: The start point of T2

Agreement:

- The time point of OD-SSB deactivation.

Sub-issue 2: The minimum interval(T2) between two FMWs

Agreement:

- For FR1, 5*max (measCycleSCell, DRX cycles)

- For FR2, 4s for power class 1/5 and 3s for power class 2/3/4

Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2

Agreement:

- The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2.

Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs

The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion.

- Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2.

- Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2.

- Other option is not precluded.

### Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-2-1: Requirement for RAN1 Alt Time-C1

Agreement

Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation.

Issue 1-2-2: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity.

- Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if

  - The periodicity of AO-SSB and OD-SSB are same

- It doesn’t impact the WI’s completion.

Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies)

Sub issue 1: RAN2 LS for scenario 3

Agreement:

- RAN4 to confirm RAN2’s WS to introduce a new servingCellMO.

Sub issue 2: Requirement for scenario 3

Agreement:

RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion.

Issue 1-2-5: Scheduling restriction for L1 measurement after SCell activation

Agreement

- In Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

- The scheduling restriction shall be applied to both AO-SSB and OD-SSB if the requirement is defined based on both of them.

### Sub-topic 1-3: OD-SSB based SCell activation

Issue 1-3-1: The known cell condition in Case 2

Agreement

- The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination.

  - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

- FFS whether apply to Alt Time-C2, if the requirement is defined.

- FFS whether apply to scenario 3, if the requirement is defined.

- FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

Issue 1-3-2: The known cell condition – period of known condition

Agreement

- Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

### Sub-topic 1-4: General issues

Issue 1-4-1: OD-SSB based multiple SCells activation

Agreement

- RAN4 to define the requirements in the case of multiple OD-SSB SCells.

  - It doesn’t impact the WI’s completion.

  - The scope can be limited as follows:

    - In deactivated SCell stage, the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping.

    - [For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells.

        - OB-SSB is assumed to be activated on all the multiple Scells to be activated before Scell activation command.]

Issue 1-4-3: Additional Processing time for OD-SSB activation

Agreement

- RAN4 should introduce UE capability with values of 2 ms and 5 ms.

Issue 1-4-4: Additional Processing time for OD-SSB parameter update

Agreement

- Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

  - The additional processing time is 5ms or 2ms based on UE capability reporting.

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

- Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE.

  - The additional processing time is 5ms

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

  - Note 2: No capability is needed

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline

Agreement

- RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS.

- Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation.

  - Option 1: No UE additional processing time for OD-SSB deactivation

  - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

Issue 1-4-7: L1 measurement in scenario 3B

Agreement

  - When MAC-CE indicates OD-SSB updating in scenario 3B,

    - UE performs the L1-RSRP measurement based on updated OD-SSB periodicity from Tfirst_SSB.

    - When DRX is configured, follow legacy principle to define requirements

  - Tfirst_SSB is the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and the addition processing time defined in RAN4.

Issue 1-4-8: Neighbor cell measurement in the carrier of OD-SSB serving cell before OD-SSB activation in Case 1

Agreement

- UE to perform neighbour cell measurement based on legacy requirement.

Issue 1-4-10: OD-SSB interruption requirements

RAN4 to discuss the following proposals for interruption.

# Topic #2: General (AI 7.27.2)

### Sub-topic 2-1: UE capability

Agreement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. Netw_Energy_NR_enh | 52-1 | Lower bound of measurement periodicity of 10ms for the deactivated measurement requirement in fast measurement window on OD-SSB SCell | Indicates a lower bound of measurement periodicity as 10ms. The lower bound is applied to the deactivated SCell measurement requirement in fast measurement window on OD-SSB SCell | At least one among RAN1 feature of 61-1, 61-2, 61-2a, 61-3, 61-4, 61-4a | Yes | n/a | UE performs on-demand SSB based deactivated SCell measurement without a lower bound within fast measurement window | Per BC | n/a | n/a | n/a | UE indicates whether the lower bound of 10ms is needed as the minimum supported measurement periodicity. | Optional with capability signaling |

# Topic #3: NES enh. performance requirements (AI 7.27.4)

### Sub-topic 3-1: OD-SSB performance part

Sub-issue 1: Test scope

Issue 3-1-1: Scenario

RAN4 to discuss whether the following test scenarios can be agreed.

- RAN4 to define the OD-SSB based test case for both Case 1 and Case 2.

- RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a.

- For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B.

- For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

- RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

Sub-issue 2: Test case list

Issue 3-1-3: Test case list

RAN4 to discuss whether the following OD-SSB test cases can be agreed.

- SCell activation in Case 1

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- Fast SCell Activation with OD-SSB;

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement(deactivated SCell) in Case 1

- Intra-frequency measurement(deactivated SCell) in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

Sub-issue 3: Test configuration

Issue 3-1-4: General principle

RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed.

  - Both FR1 and FR2

  - At least test OD-SSB periodicities with 5ms, 20ms. FFS 10ms

  - Only test non-DRX mode

  - The test without SSB time index detection

  - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

  - SMTC and measCycleSCell as 40ms/160ms

### Sub-topic 3-2: OD-SIB1 performance part

Issue 3-2-1: Test case

- Proposals

  - Option 1: RAN4 not to define OD-SIB1 RRM performance test case.

  - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

### Sub-topic 3-3: SSB adaptation performance part

Issue 3-3-1: Test case list

RAN4 to discuss whether the following SSB adaptation test case list can be agreed.

- L1-RSRP

- L1-SINR

- CBD

- L3 measurement

- Deactivated SCell measurement

- normal SCell activation

- direct SCell activation

- PUCCH SCell activation

Issue 3-3-2: General principle

RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed.

- Both FR1 and FR2

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement

- Only test non-DRX mode

- without SSB time index detection

Issue 3-3-3: SMTC and SSB configuration

- Proposals

  - Option 1:

    - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation.

  - Option 2:

    - SSB periodicity: 40 ms to 20 ms

    - SMTC periodicity: 40ms

  - Option 3:

    - SSB periodicity from 80ms to 20ms

    - SMTC periodicity from 80ms to 20ms