3GPP TSG-RAN WG4 Meeting #115	                                                	      																			     R4-2508282

Malta, MT, 19 – 23 May, 2025

Agenda item:			7.29.1

Source:	Apple

Title:	WF on RRM requirements for NR_Mob_Ph4_Part1

Document for:	approval

# Topic #1: General aspects and work plan

#### Issue 1-1: RAN4 UE feature list for event triggered L1 reporting

Candidate options:

- Option 1: not needed (vivo)

#### Issue 1-2: RAN4 UE feature list for CSI-RS based L1 measurement

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_Mob_Ph4 | 56-1 | Number of CSI-RS resources for L1-RSRP measurement within a slot | - The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot [across candidate cells] for L1-RSRP measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the number of CSI-RS resources for L1 measurement within a slot. | Per BC | No | No | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} [Note: FFS It is also counted in FG 2-24] | Optional with capability signaling |
|  | 56-2 | Number of total CSI-RS resources to be measured | - The max number of total CSI-RS resources of serving cells and [candidate cells] [across all CCs] for L1 measurement. Both LTM candidate cell(s) and serving cells for CSI-RS based L1-RSRP measurement are counted, regardless of whether CSI-RS based L1-RSRP measurement on serving cell is configured in [LTM-CSI-ResourceConfig-r18] or not. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the total number of CSI-RS resources of serving cells and neighbouring cells across all CCs for L1 measurement. | Per BC | No | No | N/A | Candidate values: [{2,4,8,12,16,32,64}] Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
|  | 56-3 | Skip SSB based L1-RSRP measurement for [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement | - Indicates support for skipping SSB-based L1-RSRP during [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | N/A | SSB based L1-RSRP cannot skipped while performing CSI-RS based L1-RSRP | Per UE | no | FR2-1 only | N/A |  | Optional with capability signalling |
|  | [56-4] | [FFS: Number of CSI-RS resources for L1-RSRP measurement per serving carrier] | - The max number of CSI-RS resources UE can measure for L1-RSRP per serving carrier | 63-1 or 63-2 | Yes | No | There is no limitation on the number of CSI-RS resources per serving carrier for L1 measurement. | Per BC | No | No | N/A | Candidate value: {1,2,3,4,5,6,7,8} | Optional with capability signalling |

# Topic #2: event triggered L1 measurement reporting

#### Issue 2-1: applicability of L1-RSRP reporting delay

Candidate options:

- Option 1: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements: (vivo, Xiaomi)

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

- Option 2: Seems no need to capture assumptions listed under option-1 into core requirements. (Apple)

- Option 2a: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases. (HW)

#### Issue 2-2: Event triggered reporting delay for L1 reporting

Agreement:

- The event triggered measurement reporting delay shall be no larger than the maximum of L1-RSRP measurement periods on the cells corresponding to the event.

  - If serving cell is involved in event evaluation, L1-RSRP measurement period of serving cell refers to TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS respectively for SSB based and CSI-RS based L1-RSRP measurement, as defined in 9.5, assuming TReport = 0

  - If neighbor cell is involved in event evaluation:

    - L1-RSRP measurement period of neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_intra  for SSB based intra-frequency measurement, as defined in 9.14, assuming TReport = 0.

    - L1-RSRP measurement period of neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_inter  for SSB based inter-frequency measurement, as defined in 9.15, assuming TReport = 0.

    - L1-RSRP measurement period of neighbor cell refers to [TL1-RSRP_Measurement_Period_CSI_intra] for CSI-RS based intra-frequency measurement, as defined in [9.x], assuming TReport = 0.

#### Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options:

- Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///)

  - Details:

    - Option 1a:(vivo)

      - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability.

      - Reporting criteria for LTM event triggered reporting is 4 per frequency layer.

      - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured.

    - Option 1b: E///

      - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

      - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS

- Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO)

- Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

#### Issue 2-4: Impact on other LTM requirements

Agreement in RAN4#114bis:

- For FR1:

  - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

    - Case 1: leaving condition of the event is configured and corresponding condition is not met,

      - the related cell switch delay and TCI state activation requirements are applicable

    - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

Candidate options:

- Proposal 1: considering it is invalid that only event entering condition is configured and leaving condition is not configured, update previous agreement to: (Huawei)

  - For FR1:

    - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met

    - The related cell switch delay and TCI state activation requirements are applicable

  - The above optimization is not applicable to FR2

- Proposal 2: Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable. (Nokia)

- Proposal 3: Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report.

- Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia)

- Proposal 5: add the following condition on agreements under issue 2-7 in RAN4#114bis: (Apple)

  - Number of active TCI does not exceed that UE can support.

- Proposal 6: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases. (MTK)

- Proposal 7: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable. (ZTE)

- Proposal 8: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following: (E///)

  - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured

# Topic #3: CSI-RS based L1 measurement

#### Issue 3-1: definition on CSI-RS based intra-frequency L1 measurement

Agreement:

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and

  - It is applied for SCS = 60KHz

- at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP.

Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement.

- RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19.

#### Issue 3-2: whether step 2 is skipped for serving cell?

Candidate options:

- Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW)

- Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO)

- Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC).

- Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

#### Issue 3-3: Sharing factor for resource collision between serving and neighbour cell

Candidate options:

- For FR2 CSI-RS L1 measurement requirement,

  - if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain,.

    - Option 1: sharing factor shall be introduced. (HW, QC, E///)

    - Option 2: introduce measurement restriction as legacy. (MTK, vivo)

#### Issue 3-4: Scaling factor for Rx beam sweeping

Agreement:

- For neighbor cell CSI-RS based L1-RSRP measurement in FR2, if SSB based measurement is skipped, the measurement period for CSI-RS based L1-RSRP will be scaled by 8.

#### Issue 3-5: UE measurement behaviour if CSI-RS configuration exceeds UE measurement capability

Agreement:

- RAN4 requirements apply assuming network will not configure CSI-RS based L1 RSRP measurement for LTM candidate cell exceeding UE measurement capability.

  - FFS on whether to capture this agreement in RAN4 spec.

#### Issue 3-6: Measurement restriction

Candidate options:

- Option 1: Apple, ZTE, Ericsson

  - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

- Option 2: vivo

  - When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

#### Issue 3-7: Measurement with repetition ON/OFF

Candidate options:

- Option 1: Nokia

  - For both FR1 and FR2. If the repetition RRC object is not configured the requirements treat this as the repetition would be “off”.  In other words, UE ignores the repetition parameter from RAN4 requirement point of view.

#### Issue 3-8: Requirement applicability

Agreement:

- For intra-f CSI-RS based L1-RSRP measurement on neighbor cell (when RTD<CP) in FR1, reuse legacy measurement period, measurement restriction and scheduling restriction requirements as baseline.

- For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define scheduling restriction.

#### Issue 3-9: early CSI acquisition

Candidate options:

- Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///)

- Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC)

- Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

# Topic #4: performance

## Event-triggered L1 reporting

#### Issue 4-1-1: test case list for event-triggered L1 reporting

Candidate options:

- Option 1: Nokia

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

  - At least 1 test case for event-triggered periodic reporting.

- Option 2: Apple

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2

- Option 3: CATT

- The new test cases for event triggered L1 measurement reporting need to defined in the clauses defined for R18 LTM.

  - TC for LTM Intra-frequency event triggered SSB based L1 measurement reporting tests without gap under non-DRX

  - TC for LTM Inter-frequency event triggered SSB based L1 measurement reporting tests with gaps under non-DRX

  - TC for LTM Intra-frequency event triggered SSB based L1 measurement reporting tests without gap under non-DRX with SSB index reading

  - For event triggered CSI-RS based L1 measurement reporting, only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined.

- Option 4: cover both SSB and CSI-RS based, in FR1 and FR2, repsectively (vivo)

- Option 5: follow aspects should be considered (ZTE)

  - With gap &without gap, intra-f & inter-f

  - non-DRX & DRX

  - With SSB index reading & without SSB index reading

  - Other aspects are not precluded.

- Option 6: CMCC

  - Both FR1 and FR2-1 are considered.

  - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered.

  - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered.

- Option 7: RAN4 to consider the list of test cases agreed in Release 18 to perform event-triggered L1-RSRP testing, thereby defining the same list of test cases for event-triggered L1-RSRP. Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting. (E///)

  - Intra-f L1-RSRP measurement for LTM

    - Intra-f L1-RSRP measurement in FR1

    - Intra-f L1-RSRP measurement in FR2

      - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated

    - Intra-f L1-RSRP measurement in FR2

      - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated

  - Inter-f L1-RSRP measurement with MG

    - Inter-f L1-RSRP measurement with MG in FR1

    - Inter-f L1-RSRP measurement with MG in FR2

  - Inter-f L1-RSRP measurement without gap for LTM

    - Inter-f L1-RSRP measurement without gap in FR1

    - Inter-f L1-RSRP measurement without gap in FR2

      - 2 neighbor cells, none of neighbor cells’ TCI state activated

- Option 8: HW

  - LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

  - LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement)

  - LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement)

  - LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

  - FFS on LTM Inter-frequency event triggered reporting tests for CSI-RS based L1 measurement

#### Issue 4-1-2: test configuration for event-triggered L1 reporting

Candidate options:

- Option 1: On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met. (Nokia)

#### Issue 4-1-3: others

Candidate options:

- Option 1: it is proposed to define test cases for LTM event 2/3/4/5 (CMCC)

- Option 2: At least Event LTM3 and Event LTM4 be included in the RRM testcase. (E///)

## CSI-RS based L1 measurement

#### Issue 4-2-1: CSI-RS based L1 measurement accuracy

Candidate options:

- Option 1: CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell. (Apple, Nokia, CATT, vivo, MTK, ZTE, CMCC, E///)

- Option 1a: reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3. (vivo)

- Option 1b: (HW)

  - The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are:

    - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

    - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

    - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

  - The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified:

    - Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

    - Absolute and relative CSI-RS based inter-frequency L1-RSRP accuracy requirements for FR1 and FR2

  - Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

- Option 1c: (CATT)

  - RAN4 to define new clauses for CSI-RS based L1-RSRP requirements for neighbor cell in clauses 10.1.19D, 10.1.19E, 10.1.20A and 10.1.20B as R18 SSB based L1-RSRP requirements for neighbor cell.

Recommended WF:

- Reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the following conditions:

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

- The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified:

  - Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

  - Absolute and relative CSI-RS based inter-frequency L1-RSRP accuracy requirements for FR1 and FR2

- Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

#### Issue 4-2-2: test case list for CSI-RS based L1 measurement

Candidate options

- Option 1: Nokia

  - FR1 intra-frequency without gaps test case.

  - FR2 intra-frequency without gaps test case with CSI-RS beam sweeping and without FR2 SSB measurements.

  - FR2 intra-frequency without gaps test case without CSI-RS sweeping and with FR2 SSB measurements.

- Option 2: Apple

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

- Option 3: CATT

  - CSI-RS based Intra-frequency L1-RSRP measurement in FR1. (CATT)

- Option 4: cover both delay and accuracy (vivo)

- Option 5: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested. (CMCC)

- Option 6: E///

  - Intra-f L1-RSRP measurement in FR1

  - Intra-f L1-RSRP measurement in FR2

  - Inter-f L1-RSRP measurement without gap in FR1

  - Inter-f L1-RSRP measurement without gap in FR2

#### Issue 4-2-3: test configuration for CSI-RS based L1 measurement - BW

Candidate options:

- Option 1: contain only 48 PRBs within the active BWP. (Nokia)

#### Issue 4-2-4: test configuration for CSI-RS based L1 measurement - resource type

Candidate options:

- Option 1: Define at least one test case with semi-persistent CSI-RS resource. Test cases not defined as semi-persistent have periodic CSI-RS resource configuration. Discuss whether the semi-persistent test case is an independent test case. (Nokia)

#### Issue 4-2-5: others

Candidate options:

- Option 1: Define test case(s) to verify early CSI acquisition if core requirements are specified. (vivo)

- Option 2: As the CSI-RS based L1-RSRP measurement procedure has been verified by event triggered reporting test case for CSI-RS L1-RSRP, it is no need to specify specific test for CSI-RS based L1-RSRP measurement. (HW)

- Option 3: In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. (HW)

## Conditional LTM

#### Issue 4-3-1: CLTM test scope – frequency range

Candidate options:

- Option 1: (Nokia, Apple, vivo, ZTE)

  - FR1 to FR1

  - FR2 to FR2

- Option 2: (CMCC)

  - PCell switch to a neighboring LTM candidate cell

    - FR1 to FR1

    - FR1 to FR2

    - FR2 to FR2

    - FR2 to FR1

  - PCell switch to an LTM candidate cell that is a serving SCell in MCG

    - FR1 cell to FR1 cell

    - FR2 cell to FR2 cell

- Option 3: E///

  - FR1 Pcell switch to a FR2 neighboring LTM candidate cell,

  - FR2 Pcell switch to a neighboring FR1 LTM candidate cell,

  - Pcell FR1 switch to an LTM candidate cell that is a FR1 serving SCell in MCG,

  - FR2 PCell switch to an LTM candidate cell that is a FR2 serving SCell in MCG.

#### Issue 4-3-2: CLTM test scope - intra/inter-frequency

Candidate options:

- Option 1: cover both intra and inter-frequency CLTM. (Nokia, Apple, vivo, ZTE, HW)

#### Issue 4-3-3: CLTM test scope – L1/L3 based CLTM

Candidate options:

- Option 1: cover both L1 and L3 based CLTM. (Nokia, CATT, ZTE)

- Option 2: RAN4 to discuss how to verify L1 and L3 measurement based candidate evaluation for CLTM such as whether to reuse partial LTM test procedure. (E///)

#### Issue 4-3-4: CLTM test scope – RACH-less and RACH-based CLTM

Candidate options:

- Option 1: cover both RACH-less and RACH based CLTM. (Nokia, CATT, vivo, HW)

- Option 2: RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure. (E///)

#### Issue 4-3-5: CLTM test scope – early DL sync

Candidate options:

- Option 1: cover both CLTM with and without early DL sync. (Nokia)

#### Issue 4-3-6: CLTM test scope – PCell/PSCell

Candidate options:

- Option 1: cover both PCell swith and PSCell switch. (ZTE)

#### Issue 4-3-7: subsequent CLTM test scope

Candidate options:

- Option 1: Apple

  - Intra-frequency subsequent CLTM from FR1 to FR1

  - Inter-frequency subsequent CLTM from FR2 to FR2

- Option 2: HW

  - RACH based Intra-frequency subsequent conditional PCell swith

  - RACH based Inter-frequency subsequent conditional PCell swith

  - RACH less Intra-frequency subsequent conditional PCell swith

  - RACH less Inter-frequency subsequent conditional PCell swith

#### Issue 4-3-8: others

Candidate options:

- Proposal 1: On high level, CLTM test cases to have serving cell and candidate cell, and the UE shall trigger cell switch based on signal level of one of these cells changing so that the cell switch condition becomes met. (Nokia)

- Proposal 2: Ensure in the test cases with early UL and DL synchronization that the UE has valid DL synchronization for the target TCI state and a valid TA at the time of condition becoming met. (Nokia)

- Proposal 3: for UE which can pass subsequent CLTM, test case for CLTM can be skipped. (Apple)

- Proposal 4: There is no need to define the new test cases for PDCCH ordered RACH for (subsequent) CLTM. (CATT, ZTE)

- Proposal 5: The legacy test configurations and parameters used in R18 LTM test cases can be a baseline for conditional LTM test cases. (CATT)