3GPP TSG-RAN WG4 Meeting #115      				                                          	R4-2508283

Malta, MT, 19th - 23rd May 2025

Agenda item:			7.29.1

Source:	China Telecom

Title:	WF on RRM requirements for NR_Mob_Ph4_Part2

Document for:	Approval

Notes

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other proposals can be proposed.

Topic #1: RRM discussion

General aspects and early synchronization

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement

RAN4#114bis < Way forward > FFS the following options: Conclude this issue in the next meeting. - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Option 3: Not define requirement for any of them.

< Agreement >

  - Not define requirements for L1 measurement on deactivated Scell.

  - Not define requirements for UE based TA measurement.

Conditional LTM delay

Issue 1-2-1: TRRC

< Agreement >

  - TRRC is the RRC procedure delay.

Issue 1-2-2: Tmeasure

< Agreement >

In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE starts TCLTM-RRC-processing to a target cell.

For SSB based L1-RSRP measurement:

- Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed.

- Measurement latency for both serving and candidate cells need to be considered, and follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item.

For SSB based L3-RSRP measurement:

- Reuse Tmeasure in CHO.

For CSI-RS based L1-RSRP measurement:

- For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period.

- Further discuss the conditions for TCSI-RS_SFN_intra = 0.

- Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS.

For CSI-RS based L3-RSRP measurement:

- RAN4 does not define CLTM requirements for the case where CSI-RS based L3-RSRP measurements are used in the execution condition evaluation.

Issue 1-2-3: TLTM-RRC-processing /TCLTM_execution

< Agreement >

  - TCLTM-RRC-processing is used in C-LTM delay, discuss the description of TCLTM-RRC-processing in draftCR directly.

Issue 1-2-4: General on Tfirst-RS and TRS-proc

< Agreement >

  - [Tfirst-RS and TRS-proc] are included in Tinterrupt.

Issue 1-2-5: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

< Agreement > ad hoc

  - Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list.

- Note: the limitation of 160/480 ms from Rel-18 still applies.

Issue 1-2-6: TLTM-processing

< Agreement >

  - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

    - Reuse Rel-18 LTM values for TLTM-processing in the CLTM cell switch delay.

Issue 1-2-7: TLTM-IU

< Agreement >

- TLTM-IU is the interruption uncertainty during LTM cell switch.

    - For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

    - For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

Subsequent Conditional LTM

Issue 1-3-1: General aspects of Subsequent Conditional LTM delay

< Agreement > ad hoc

  - Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

    - TRRC will not be considered in subsequent conditional LTM delay.

    - TEvent_DU needs to be updated

Issue 1-3-2: The components of subsequent conditional LTM delay

- Since issue 1-3-1 has been agreed, this issue is closed.

Issue 1-3-3: Other

< Way forward > FFS the following proposal: - Proposal 1: Define initial and subsequent CLTM cell switch delay requirements in the same section under section 6.3. - Proposal 2: RAN4 specifies the initial CLTM and subsequent CLTM delay separately.

< Agreement >

- RAN4 specifies the initial CLTM and subsequent CLTM delay separately.