3GPP TSG-RAN WG4 Meeting # 115											R4-2508285

Malta, MT, May 19th – May 23rd, 2025

Agenda item:			7.30.1

Source:	Moderator (Nokia)

Title:	WF on RRM requirements for NR_XR_Ph3

Document for:	Approval

# Topic 1 Scenarios for measurement skipping

## Issue 1-1: UE power class

<Way forward>

- Option 1: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

# Topic 2 Measurements with gaps

## Issue 2-1: Type 2 gaps

<Agreement>

- MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules.

- The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules.

- DCI skipping cannot be applied if any gap is configured with gapAssociationPRS

- FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms

## Issue 2-2: NCSG

<Agreement>

- MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17.

## Issue 2-3: Preconfigured measurement gaps

<Agreement>

- MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17.

## Issue 2-4: Rel-18 concurrent gaps

<Agreement>

- MG cancelling DCI doesn’t apply to Rel-18 concurrent gaps.

# Topic 3 XR skipping requirements impact

## Issue 3-1: Formulas for measurement delay extension without DRX

<Way forward>

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

  - Option 2: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

  - Option 3:

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

    - where Tskip = Nskip ´ Max(MGRP , SMTC period)

  - Other options are not precluded

  - where Nskip is the number of  skipped MG occasions in the measurement period.

  - FFS: CSSFinter

## Issue 3-2: Formulas for measurement delay extension with DRX

<Way forward>:

- Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

  - DRX cycle > 320 ms

    - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

- Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

  - DRX cycle ≤ 320 ms

    - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5)  Max(MGRP, SMTC period, DRX cycle))  CSSFinter

  - DRX cycle > 320 ms

    - Ceil(8+XPSS/SSS,gaps)  DRX cycle  CSSFinter

- Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

  - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms

  - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms

  - Where

    - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle)

    - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle

- Other options are not precluded

## Issue 3-3: Formulas for measurement delay extension inter-RAT

<Way forward>:

- Discuss how to extend the formulas in 3-1 and 3-2 for inter-RAT

## Issue 3-4: Formulas for measurement delay extension with concurrent gaps

<Way forward>:

- Discuss how to extend the formulas in 3-1 and 3-2 with Rel-17 concurrent gaps

## Issue 3-5: Conditions for measurement skipping

<Way forward>:

- Option 1: No conditions are defined

- Option 2: Define the maximum number of gaps skipped during the measurement period.

- Option 3: Define a maximum separation between two non-skipped gaps for measurement

## Issue 3-6: Impact on different frequency layers

<For information>:

- the original proposals of this issue can be found in R4-2505575 and Chairman notes.

<Agreement>:

- Not pursue Proposal 1 and 2 in Rel-19.

## Issue 3-7: UL XR impact

<For information>:

- the original proposals of this issue can be found in R4-2505575 and Chairman notes.

<Agreement>:

- This issue is closed.

# Topic 4 UE assistance information

## Issue 4-1: Additional aspects for UAI

<Agreement>:

<Agreement>: Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting. - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

- The UE may provide the UAI related information, for the UAI in the agreement captured above, per MG configuration.

  - per-FR gap if gap is configured with gapConfig with per-FR gaps

  - per gap configuration if gap is configured with gapConfig-r17

# Topic 5 Additional signalling aspects

## Issue 5-1: Semi-static schemes

<Agreement>:

- From RAN4 perspective, no consensus to support semi-static solutions in Rel-19.

## Issue 5-2: Skipping indication on frequency domain

<Agreement>:

- It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs.

- Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

## Issue 5-3: Partial measurement skipping

<For information>:

- the original proposals of this issue can be found in R4-2505575 and Chairman notes.

<Agreement>:

  - This issue is closed.

# Topic 6 CR handling

## Issue 6-1: Naming convention

<Agreement>:

- Use “Measurement gap occasion cancelation” to refer to measurement gaps that were skipped or cancelled by DCI.

- Use Lcancel for number of skipping occasions

- Indication of which formula/table to use

  - For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met (new)

- Use RAN2 pseudo code approach for new clause 9.1.y

# Topic 7 UE capabilities

## Issue 7-1: UE capabilities for XR – general proposals

<Agreement>:

- Define optional UE feature for support of UAI with per UE capability, [and no FR1/FR2 differentiation].

## Issue 7-2: UE capabilities for XR - tables

<Way forward>:

Start discussion based on this

| - Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57.NR_XR_Ph3 | 57-1 | Recommended ratio of gap occasions for cancelation report. | Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation. | 64-1 | yes | n/a | The UE does not support reporting recommended ratio for gap cancelation | Per UE | no | no |  |  | Optional with capability signalling |