3GPP TSG-RAN WG4 Meeting # 115	             R4-2508287

# Malta , MT, 19th – 23rd May, 2025

Agenda Item:	7.31.1

Source: 	Moderator (CATT)

Title: 	WF on RRM requirements for NR_NTN_Ph3_Part1

Document for:	Approval

0 Notes:

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other options can be proposed.

- < Tentative Agreement > will be changed to < Agreement > if no comments are received.

Topic #1: RRM core requirements

    - Sub-topic 1-1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

Issue 1-1-1: SDT requirements

< Agreement >:

- For the conclusion on how to define CG-SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2.

  - The agreement reached in RAN4#114 meeting only focus on RAN4’s requirements.

Issue 1-1-2: MDT requirements

- Background

The agreement in the previous meeting:

RAN4#114: #### Issue 1-1-3: MDT requirements Agreement: - For MDT requirements in RRC_IDLE and RRC_INACTIVE state, - Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.

<Way Forward>:

  - Proposal 1: Not to define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap with NTN.

  - Proposal 2: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN. The legacy requirement from NTN can be reused.

  - Proposal 3: RAN4 to discuss whether to send LS to RAN2 / RAN3 informing them about the RAN4 agreement on specifying MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT.

  - Proposal 4:

    - RAN4 shall consider the necessity of defining MDT requirements and then decide whether to define relevant MDT requirements.

    - For measurement requirements, the legacy measurement requirements shall be reused for 1Rx and 2Rx RedCap UEs in NTN.

    - For accuracy relative time stamp requirements, RAN4 shall reuse the same drift of relative time stamp for 1Rx and 2Rx RedCap UEs in NTN.

    - Sub-topic 1-2 SMTCs and 'in parallel' related features

Issue 1-2-1: The proximity distance between two SMTCs

Ad-hoc agreement

< Agreement >:

- When 1Rx and 2Rx (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

    - the two SMTC occasions are fully or partially overlapping in time domain, or

    - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4ms+ x= Yms.

      - For a UE capable of Feature 25-2 (parallelMeasurementWithoutRestriction-r17):

        - If UE is configured to measure 1 satellite in each of the SMTC occasions, x = 0

        - Otherwise, x = 1

      - For a UE incapable of Feature 25-2 (parallelMeasurementWithoutRestriction-r17):

        - x = 1

Issue 1-2-2: SMTC adjustment in idle/inactive

<Way Forward>:

- Proposal 1: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN.

- Proposal 2: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN.

  - Proposal 2a: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode.

- Proposal 3: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

Issue 1-2-3: Further clarification on Feature 25-1(parallelSMTC)

- Background

The agreements for Feature 25-1(parallelSMTC) in the last meeting is copied as below:

##### RAN4#114-bis: ##### Issue 1-2-1-1: Feature 25-1(parallelSMTC) Agreement: - For Feature 25-1 (parallelSMTC), not differentiate 1Rx and 2Rx RedCap UEs for this UE capability. - RedCap UE is mandatory to support parallel measurements on 2 SMTC-s for a single frequency carrier - ‘Parallel’ means the SMTCs are TDM with proximity distance.

Ad-hoc agreement

< Agreement >:

- RedCap UEs with NTN still optional support parallel measurements on 4 SMTC-s for a single frequency carrier, and the UE capability of Feature 25-1(parallelSMTC) can be reused.

    - Sub-topic 1-3 The impact of HD-FDD

Issue 1-3-1: General HD-FDD impact

- Background

The agreement in the last meeting:

RAN4#114-bis: #### Issue 3-1-1: General HD-FDD related impact Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements

<Way Forward>:

- Proposal 1 (ZTE): The legacy requirements and applicable conditions can be reused for defining requirements of RedCap UE for HD-FDD in NTN scenario:

    - Paging reception requirements in Idle/Inactive state;

    - Handover interruption time;

    - Random access;

    - RRC connection release with redirection to NR;

    - Scheduling availability of UE performing intra-/inter- measurement.

- Recommended WF

No need to further discuss, we have agreed to reuse the legacy principle for the above requirements in the last meeting. The details can be discussed in CR directly.

Issue 1-3-2: The impact of HD-FDD on minimum requirement for L1 indication for RLM and LR

online agreement

< Agreement >:

- For the impact of HD-FDD on minimum requirement for L1 indication for RLM in clause 8.1B.6 and LR in clause 8.5B.4, the legacy requirements in TN RedCap and applicable conditions can be reused.

  - For each RS configuration, at least one RS sample must fall with DL occasion within an indication period.

- If the above condition is not met, no requirement shall be applied.

  - Not capture it in the spec.

Issue 1-3-3: The impact of HD-FDD on scheduling availability of UE related to CSI-RS and PRS

online agreement

< Agreement >:

- When NW indicates UL overriding DL for collision handling,

  - RAN4 to clarify that the measurement requirements for CSI-RS and PRS apply provided that no collision between CSI-RS & PRS and UL transmission.

- Otherwise,

  - CSI-RS based L1 measurement and PRS based measurement is prioritized over the UL transmission.

  - The scheduling restriction requirements for PUCCH/PUSCH/SRS will apply.

Issue 1-3-4: The impact of HD-FDD on intra/inter measurements

online agreement

Agreement:

- For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, regardless of the network indication.

Issue 1-3-5: The impact on NR Handover due to HD-FDD

<Way Forward>:

- Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted.

  - Proposal 1a: Regarding Issue 3-2-1 (The impact on NR Handover due to HD-FDD), adopt Option 1 with 'availability' defined as 'no collision with UL transmission at the UE'.

- Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

    - Sub-topic 1-4 UE capability

Issue 1-4-1: General principle for designing the UE capabilities for (e)RedCap UE with NTN

<Way Forward>:

- Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC)

  - FFS: The Rel-19 requirement can be release independent from Rel-17.

- Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung)

- Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

Topic #2: RRM perf requirements

    - Sub-topic 2-1: (e)RedCap UE with FR1-NTN

Issue 2-1-1: Measurement accuracy requirement

- Background

The agreement in the previous meeting:

RAN4#114-bis: #### Issue 2-2-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Agreement: - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB. RAN4#114: #### Issue 2-3-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Agreement: - For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused. - For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred, and the same relaxation (as the measurement accuracy requirements defined for 1Rx TN-RedCap ) for accuracy should be reused.

<Way Forward>:

- Proposal 1 (CMCC):

  - For 2Rx RedCap UE, reuse the measurement accuracy requirement of NTN.

  - For 1Rx RedCap UE, define new measurement accuracy requirement on NTN operating bands with reusing the RSRP accuracy and side conditions from legacy 1Rx RedCap UE.

- Proposal 2 (vivo): For the performance requirement in NTN for NR Phase 3, measurement accuracy requirements need to be defined for (e)Redcap UEs with FR1-NTN.

- Proposal 3 (HW): RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

  - Intra- and inter-frequency RSRP

  - Intra- and inter-frequency RSRQ

  - Intra- and inter-frequency SINR

  - L1-RSRP

  - Rx-Tx for NW verified location

Issue 2-1-2: Test configuration and parameters for (e)RedCap UE with FR1-NTN

<Way Forward>:

  - Proposal 1 (CATT):

    - Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN.

    - 15 kHz SCS and 10MHz bandwidth can be configured in test cases.

    - Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN.

    - Proposal 1a (ZTE): RAN4 shall consider the following general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

  - Proposal 2 (HW): Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values

  - Proposal 3 (CMCC): Take the NTN test setups as the reference, and use the ‘refer to’ method when drafting the spec to avoid duplicated contents.

  - Proposal 4 (ZTE): RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs.

Issue 2-1-3: How and where to capture the test cases for 1Rx and 2Rx (e)RedCap UE with NTN,

<Way Forward>:

- Proposal 1(ZTE): The existing NTN test cases can be used as a baseline for RedCap UEs in NTN scenario.

  - Proposal 1a (HW): Re-use existing test cases for NTN in A.14 for RedCap in NTN

    - Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values

- Proposal 2 (ZTE): For RedCap UEs in NTN scenario, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Issue 2-1-4: Test scope

Issue 2-1-4-1: Whether to define separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN,

<Way Forward>:

- Proposal 1 (CATT, CMCC): RAN4 to define the separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN.

- Proposal 2 (HW): Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs

- Proposal 3 (ZTE): Discuss case by case.

Issue 2-1-4-2: General test scope

<Way Forward>:

The following Table is based on the input from companies, and it can be a baseline.

| Test category |  | Company comments |
| --- | --- | --- |
| RRC_IDLE state mobility | Cell reselection | Support: CATT, Ericsson, vivo, ZTE, CMCC, HW |
| RRC_IDLE state mobility | Configured Grant based Small Data Transmissions (CG-SDT) | Support: CATT, Ericsson, vivo |
| RRC_CONNECTED state mobility | Handover - RACH-based - RACH-less | Support: CATT, vivo, ZTE, CMCC, HW |
|  | CHO - time-based - location-based | Support: CATT, Ericsson, vivo, ZTE, CMCC, HW |
|  | satellite switching with re-synchronization | Support: CATT, vivo, ZTE, CMCC, HW |
|  | RRC Connection Mobility Control | Support: CATT, Ericsson, vivo, ZTE, CMCC, HW |
| Timing | UE transmit timing | Support: CATT, Ericsson, vivo, CMCC, HW |
|  | UE timer accuracy | Support: vivo |
|  | Timing Advance | Support: CATT, Ericsson, vivo, CMCC, HW |
| Signalling characteristics | Radio Link Monitoring | Support: CATT, Ericsson, vivo, ZTE, CMCC, HW |
|  | Beam Failure Detection and Link recovery | Support: CATT, Ericsson, vivo, ZTE, CMCC, HW |
|  | Active BWP switch delay | Support: CATT, Ericsson, vivo, CMCC, HW |
|  | Active TCI state switching delay | Support: vivo |
|  | UE-specific CBW change | Support: CATT, Ericsson, vivo, CMCC, HW |
|  | Pathloss reference signal switching delay | Support: CATT, Ericsson, vivo, HW |
| Measurement procedure | NR intra-frequency measurements | Support: CATT, Ericsson, vivo, ZTE, CMCC, HW |
|  | NR inter-frequency measurements | Support: CATT, Ericsson, vivo, ZTE, CMCC, HW |
|  | L1-RSRP measurements for Reporting | Support: CATT, Ericsson, vivo, ZTE, CMCC, HW |
|  | NR measurements for positioning | Support: CATT, Ericsson |
| Measurement Performance | SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance | Support: Ericsson, CMCC, HW |
|  | L1-RSRP measurement Performance for beam reporting | Support: Ericsson, CMCC, HW |

- RAN4 to further discuss the specific test case list with configurations.

    - Sub-topic 2-2 Other parts for NR NTN

Issue 2-2-1: Test scope for downlink coverage enhancement

<Way Forward>:

- Proposal 1 (ZTE): For downlink coverage enhancement, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

- Proposal 2 (Ericsson, CATT, HW, ZTE): RAN4 to consider adding a new test case for RRC Re-establishment for the UE capable of coverage enhancement.

  - Proposal 2a (HW): Define test case for RRC Re-establishment with 160ms SSB periodicity and without serving cell based on existing test case in A.6.3.2.1.3.

- Proposal 3 (CATT, HW): Other test cases need to wait for the progress in core part.

  - Proposal 3a (HW): Wait for conclusion on potential SMTC enhancement before discussing whether to define new test cases for cell reselection and L3 measurement.

- Proposal 4 (ZTE): For RRC connection release with redirection to NR, handover, satellite switch with re-sync and cell re-selection, no new test cases shall be defined.

Issue 2-2-2: Test configuration for downlink coverage enhancement

<Way Forward>:

- Proposal 1: RAN4 need to discuss whether to introduced new SSB patterns for DL enhancement NTN UE.