3GPP TSG-RAN WG4 Meeting #115	                               R4-2508391

Malta, MT, 19th – 23rd May, 2025

Title:	LS on event-triggered L1-RSRP reporting

Response to:

Release:	R19

Work Item:	NR_MIMO_Ph5-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Chenchen Zhang

zhang.chenchen@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN4 is discussing measurement requirements for event-triggered L1-RSRP reporting. It is RAN4 common understanding that NW can configure “timeRestrictionForChannelMeasurement” for event-triggered L1-RSRP reporting. For Event-2, RAN4 agreed to define separate requirements for measurement period when the UE is configured or not configured with“timeRestrictionForChannelMeasurement”, similar to existing measurement period requirements. Specifically, for SSB-based and periodic and semi-persistent CSI-RS based L1-RSRP measurement, if timeRestrictionForChannelMeasurement is configured, the measurement period is based on M = 1, and event evaluation should be based on the most recent result from both new beam and current beam; If timeRestrictionForChannelMeasurement is not configured, the measurement period for current and new beam is based on M = 3. For aperiodic CSI-RS based L1-RSRP, M is always equal to 1.

2	Actions

To RAN1

ACTION: 	RAN4 respectfully asks RAN1 to take above information into account and provide feedback if there is any issue with the above RAN4 agreement.

3	Dates of next TSG RAN WG4 meetings

TSG-RAN WG4 Meeting #116			August 25 to 29, 2025	Bengaluru, IN

TSG-RAN WG4 Meeting #116-bis			October 13 to 17, 2025      Prague, CZ