3GPP TSG-RAN4 Meeting # 115	R4-2508403

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on MRTD for asymmetric DL sTRP&UL mTRP deployment of R19 MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Two scenarios are allowed in the asymmetric DL sTRP/UL mTRP deployment in R19 MIMO: Scenario #1: UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing; Scenario #2: UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs. For Scenario #1, no need to define MRTD. For Scenario #2, the requriements of MRTD is needed and we capture the exact requirements in Clause 7.6.8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the relevant requirements of MRTD for Scenario #2 of the asymmetric DL sTRP/UL mTRP deployment in R19 MIMO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of MRTD for Scenario #2 of the asymmetric DL sTRP/UL mTRP deployment is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.1, 7.6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2507297 |  |  |  |  |  |  |  |  |

<Start of Change #1>

7.6	Maximum Receive Timing Difference

7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of an E-UTRA cell belonging to the MCG and the closest slot timing boundary of a cell belonging to SCG to be aggregated for EN-DC operation.

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of an E-UTRA cell belonging to the SCG to be aggregated for NE-DC operation and the closest slot timing boundary of a cell belonging to MCG.

A UE shall be capable of handling a relative receive timing difference between slot timing boundary of a cell belonging to MCG in FR1 or FR2-1 and the closest slot timing boundary of a cell belonging to the SCG FR1 or FR2-1 to be aggregated for NR-DC operation.

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of a cell belonging to MCG in FR1 and the closest subframe timing boundary of a cell belonging to the SCG in FR2-2 to be aggregated for NR-DC operation.

A UE shall be capable of handling a relative receive timing difference among the closest slot timing boundaries of different carriers in FR1 and/or FR2-1 to be aggregated in NR carrier aggregation.

A UE shall be capable of handling a relative receive timing difference among the closest subframe timing boundaries of different carriers to be aggregated in FR1 and FR2-2 NR inter-band carrier aggregation.

An FR2-1 PC6 UE supporting simultaneousReceptionTwoQCL-r18 shall be capable of handling a relative receive timing difference between the subframe boundaries of signals on the same CC received using two different Rx chains simultaneously in HST FR2 bidirectional deployment.

A UE supporting [FG 40-2-1 or FG 40-2-2] shall be capable of handling a relative receive timing difference between the slot boundaries of signals on the same CC received from two TRPs.

A UE supporting [FG 30-1 or 30-2] shall be capable of handling a relative receive timing difference among the subframe timing boundary of different TCI states indicated for simultaneous reception on a single carrier.

The requirements defined in clause 7.6 are also applicable when UE is configured to receive multiple PDSCH transmission occasions from one or more QCL sources on any one of the aggregated NR carriers.

[The requirements defined in clause 7.6 are also applicable for the UE capable of [singleDCI multi TRP with 2 TA], provided the UE is configured with 2 TAs and not configured with PL offset in joint/UL TCI state(s). In this scenario, when UE is configured with SSB-MTC-additionalPCI, the UE may expect to receive SSB from UL TRP(s).]

<End of Change #1>

<Start of Change #2>

7.6.8	Minimum requirements for Multi-TRPs

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in table 7.6.8-1 below, if the UE does not indicate support of rxTimingDiff-r18.

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in table 7.6.8-2 below, provided that the UE indicates that it is capable of rxTimingDiff-r18.

[A UE supporting [singleDCI multi TRP with 2 TA] shall be capable of handling at least a relative receive timing difference between slot timing of different TRPs on the same carrier at the UE receiver as shown in table 7.6.8-4 below, if the UE does not indicate support of [TimingDiffwithSSBRxfromULTRP-R19], and in Table 7.6.8-5, if the UE indicates that it is capable of [TimingDiffwithSSBRxfromULTRP-R19].]

Table 7.6.8-1: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of rxTimingDiff-r18

| Frequency Range | Maximum receive timing difference |
| --- | --- |
| FR1 | CP lengthnote 1 |
| FR2-1 | CP lengthnote 1 |
| NOTE 1: CP length of the maximum SCS on the carrier. |  |

Table 7.6.8-2: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of rxTimingDiff-r18

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 |

A UE supporting [FG 30-1 or fastBeamSweepingMultiRx-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different TCI states on the same carrier at the UE receiver as shown in table 7.6.8-3.

Table 7.6.8-3: Maximum receive timing difference requirement for UE supporting multi-RX

| Frequency Range | Maximum receive timing difference |
| --- | --- |
| FR2-1 | CP length |

Table 7.6.8-4: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE not capable of [TimingDiffwithSSBRxfromULTRP-R19]

| Frequency Range | Maximum receive timing difference |
| --- | --- |
| FR1 | CP lengthnote 1 |
| FR2-1 | CP lengthnote 1 |
| NOTE 1: CP length of the maximum SCS on the carrier. |  |

Table 7.6.8-5: Maximum receive timing difference requirement for multi-TRPs with two TAs for UE capable of [TimingDiffwithSSBRxfromULTRP-R19]

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 |

<End of Change #2>