3GPP TSG-RAN4 Meeting #115	R4-2508445

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to 38.133 on LTM CSI-RS applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR implements the CSI-RS applicability section |  |  |  |  |  |  |  |  |
| Summary of change: |  | 9.14a.2 Requirements applicability for CSI-RS based L1-RSRP measurements for LTM candidate cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirement support for the feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The CR is implemented according to the following agreements: RAN4#112bis <Agreement>: - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells <Agreement>: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition RAN4#114 <Agreement>: - Confirm the RAN4#114 tentative agreement “Use measurable and specify the side condition for RS being measurable” RAN4#114bis <Agreement>: - Use legacy side condition of SINR=-3dB. <Agreement>: - Define RRM requirement for CSI-RS based L1 measurement requirement for periodic CSI-RS and semi-persistent CSI-RS. <Agreement>: - RRM requirements (delay and accuracy) apply under the following condition: - at least 48 RBs of the CSI-RS configured for measurement is within the active BWP. <Agreement>: Only consider CSI-RS with repetition OFF in RAN4 requirements |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

### 9.14a.2	Requirements applicability

The requirements in the clause 9.14a are applicable to FR1 and FR2-1 for LTM.

The requirements in clause 9.14a apply for CSI-RS L1-RSRP measurements for configured LTM candidate cell, provided the following conditions are met:

-	The cell is known,

-	The CSI-RS L1-RSRP measurement is configured as periodic CSI-RS or semi-persistent CSI-RS measurement, and

-	at least 48 RBs of the CSI-RS configured for measurement is within the active BWP, and

-	repetition is set to “OFF” [or not configured].

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The SSB from the target cell configured for L3 measurement remains detectable according to the cell identification requirements in clause 9.2, and

-	The CSI-RS from the target cell configured for L1 measurement remains measurable

Otherwise, the cell is unknown.

A CSI-RS resource configured for L1-RSRP for LTM candidate cell shall be considered measurable when for each relevant CSI-RS the following conditions are met:

-	LTM CSI-RS L1-RSRP related side conditions given in clause [10.x.x] for FR1 and [10.x.x] for FR2-1 are met respectively, for a corresponding band.

<End of change 1>