3GPP TSG-RAN WG4 Meeting #115                                                               R4-2508631

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.25.1

Source:	Samsung

Title:	Ad-hoc meeting minutes for [115][306] NR_duplex_evo_General

Document for:	Discussion

# Introduction

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved [RP-234035], with WID further revised in the follow-up RAN plenary [RP-240789]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”.

Over RAN4#114bis, there was extensive discussion on SBFD general aspects and potential new requirements for SBFD operation for FR1 and FR2-1, which results in the progress captured in the approved WF [R4-2504751].

This document is provided for the moderator summary of thread [306] on Rel-19 work item on evolution of NR duplex operation (SBFD) for general aspect and potentially new requirements for SBFD, in which the following highlighted agenda items are supposed to be covered specifically:

- Evolution of NR duplex operation: Sub-band full duplex (SBFD) [NR_duplex_evo] - Moderator summary and conclusions [NR_duplex_evo] - General aspects (including RAN4 aspects for SBFD system parameters) [NR_duplex_evo-Core] - BS RF and conformance requirements [NR_duplex_evo-Core] - Potentially new requirements for SBFD operation for FR1 and FR2-1 including BS conformance [NR_duplex_evo-Core/Perf] - Modification of existing Tx requirements for FR1 and FR2-1 including BS conformance [NR_duplex_evo-Core/Perf] - Modification of existing Rx requirements for FR1 and FR2-1 including BS conformance [NR_duplex_evo-Core/Perf] - RRM core requirements [NR_duplex_evo-Core] - RRM requirements for UE-to-UE CLI handling [NR_duplex_evo-Core] - RRM impacts for SBFD operation [NR_duplex_evo-Core] - RRM performance requirements [NR_duplex_evo-Perf] - UE and BS Demodulation performance requirements [NR_duplex_evo-Perf]

# Topic #1: General system parameters for SBFD operation

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505589 | Huawei, HiSilicon | Proposal 1: Use 38dBm conducted output power as the power level threshold for FR1 MR SBFD capable BS. Proposal 2: No RAN4 spec impact regarding DL subband size configuration is expected when the SSB symbol is overlapped with SBFD symbol. |
| R4-2506006 | CATT | Proposal 1: About declaration and flexibility for SBFD operation, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm. Proposal 2: About restriction on the number of UL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm. Proposal 3: About restriction on the number of DL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm. Proposal 4: The declaration of guardband size should ensure the benefits from SBFD operation, and it can be “The benefits from SBFD operation are achievable with the declared guardband size.” in the RAN4 specification. Proposal 5: For each SBFD subband configuration, there may be the following three options for guardband position. All these positions should be part of the declaration, i.e., declare a single fixed guardband position for each configuration, or only one position common for all configurations. Option1: All guardband is confined in DL subband Option2: All guardband is confined in UL subband Option3: Half guardband is confined in DL subband, and the other half is confined in UL subband Proposal 6: DL subband should include the whole DL initial BWP if the SSB symbol is overlapped with SBFD symbol. |
| R4-2506372 | CMCC | Proposal 1: Considering the transmission power of MR BS in current networks, use 47 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. |
| R4-2506530 | Qualcomm Germany | Proposal 1: RAN4 to consider the MR SBFD-capable power threshold value (X dBm) as a compromise between MR maximum BS rated carrier TRP output power limit given in Table 9.3.1-1, TS 38.104 (i.e., 43 dBm) and representative values adopted in Section 9.3, TR 38.858. Proposal 2: RAN4 to confirm Table 2 in this document regarding Rel-19 SBFD capable DL/UL subband configurations. Proposal 3: For FR2-1 bands, restriction on channel bandwidth for DUD configuration should be handled by the network, given that RAN1 has agreed that 1) SSB symbols configured with SBFD subbands are SBFD symbols, 2) only DL receptions within DL usable PRBs are allowed for SBFD aware UEs, and 3) SSB block is assumed to be within DL subband. |
| R4-2507481 | Samsung | RAN4 aspects for SBFD system parameters Proposal 1: For FR2 BS, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. Proposal 2: For FR1 MR BS, use X = 38 dBm (either as TRP (for BS Type 1-O) or total conducted power (for BS Type 1-H)) as the power level threshold, to differentiate all declared configurations shall be tested or not. • Note: As agreed, the power level threshold is established for FR1 MR BS: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested Proposal 3: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not. • Note: As agreed, the power level threshold is established for FR1 MR BS: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable: Proposal 4: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether the restriction on the number of DL subband sizes supported is applied or not. • Note: As agreed, the power level threshold is established for FR1 MR BS: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. Proposal 5: If the SSB symbol is overlapped with SBFD symbol, there is neither restriction required nor RAN4 requirement impact identified. |
| R4-2507482 | Samsung | draftCR to TS38.104 on general clauses for SBFD-capable BS |
| R4-2507518 | ZTE Corporation, Sanechips | Proposal 1: for FR1 MR SBFD BS, propose to use the 33dBm as TRP or total conducted power as the power threshold. Proposal 2: to add the clarification note in the specification, the frequency range of DL sub-band configuration of SBFD BS should cover both SSB and CORESET#0 if there are any overlapping. |
| R4-2507636 | Nokia | Observation 1: The guardband plays a big role in terms of avoiding inter-subband interference. Smaller guardband does not always translate into better performance. Proposal 1: RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work. Proposal 2: Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. Proposal 3: Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to UL subband sizes supported. Proposal 4: Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to DL subband sizes supported. Proposal 5: If it is decided that a clause is introduced to the RAN4 specification related to guardband size, the text should say: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead.” |
| R4-2507724 | Ericsson | Observation 1 Regarding the introduction of power level threshold for FR1 MR, what matters is the total amount of power that the RX needs to handle, which is related to the total number of transmitters and carriers in the system. Observation 2 9.9 OTA spatial emission has already been introduced in the latest release (V19.0.0). The clause numbers need to be moved backward accordingly. Observation 3 The test model for SBFD DL subbband needs to be scalable. Observation 4 New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband. Observation 5 Signal statistics may change for SBFD test models and need to be investigated. Impact of DL subband on signal statistics needs to be analyzed. Observation 6 Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands. Proposal 1 Do not define a value per TX chain as the power level threshold for FR1 MR. Proposal 2 Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. Proposal 3 Use the following updated clause structure and Requirement set applicability for SBFD-capable BS. |
| R4-2507682 (treated in thread [307]) | Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom, Deutsche Telekom | Proposal 1: At least for FR1 macro BS, the deployment scenario of SBFD-SBFD co-existence in adjacent channels should be considered for defining RF requirements for inter-operator co-existence. |
| R4-2507714 | Samsung | Draft BigCR to TS38.104 for SBFD BS RF requirement |

## Open issues summary

### Sub-topic 1-1: Declaration and flexibility for SBFD operation

- [Moderator] In previous RAN4 meetings, the following agreements are achieved on declaration and flexibility for SBFD operation:

Agreement in RAN4#113: - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications Agreement in RAN4#114: - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - This restriction shall not be applied to FR1 LA BS - FFS FR1 MR BS Agreement in RAN4#114bis: - For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

##### Issue 1-1-1: Declaration and flexibility for SBFD operation

- Proposals on FR2 BS:

  - Proposal 1 (Samsung): For FR2 BS, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

- Proposals on FR1 MR BS for the threshold:

  - Option 1 (CATT, ZTE, Nokia, Ericsson): 33dBm

  - Option 2 (Huawei, Samsung): X=38dBm

  - Option 3 (CMCC): X=47dBm

  - Other proposals:

    - Proposal 2 (Ericsson): Do not define a value per TX chain as the power level threshold for FR1 MR. (Regarding the introduction of power level threshold for FR1 MR, what matters is the total amount of power that the RX needs to handle, which is related to the total number of transmitters and carriers in the system.)

    - Proposal 3 (Qualcomm): RAN4 to consider the MR SBFD-capable power threshold value (X dBm) as a compromise between MR maximum BS rated carrier TRP output power limit given in Table 9.3.1-1, TS 38.104 (i.e., 43 dBm) and representative values adopted in Section 9.3, TR 38.858.

- Moderator Recommendation:

  - For FR2 BS, adopt Proposal 1.

  - For FR1 MR BS, discuss different options of X values.

- Ad-hoc discussion:

  - Ericsson: For total power 33dBm, analog filter may be needed. For 38 dBm, the analog filter shall be needed.

  - CMCC: support option 3, based on our deployment 47dBm is used for MR.

    - Ericsson: if so, meaningless to use this threshold. The threshold is not introduced to limit MR deployment. It is related to testing.

  - Qualcomm: 33dBm is something not used in SI. Should we use the existing study.

  - ZTE: 33dBm is proposed based on our experience.

  - Huawei: 5dBm offset is proposed.

- Ad-hoc agreement:

  - For FR2 BS, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

  - Proposals on FR1 MR BS for the threshold:

    - Decide the value from the range of [33dBm, 38dBm] during this meeting.

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

- [Moderator] Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. In RAN4#113, the following agreement is provided:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

In which option 2b is given in RAN4#112:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

##### Issue 1-2-1: Restriction on the number of UL subband sizes supported

- [Moderator] For the restriction-2 (i.e., restriction on the number of UL subband sizes supported), the following agreement is reached in RAN4#114bis:

Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

- Proposal on the way to determine the threshold for FR1 MR BS:

  - Proposal 1: Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

- Moderator Recommendation: Adopt the following bullets:

  - “For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

    - Maximum 3 subband configurations allowed for FR1 WA BS

      - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS.

      - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations.

    - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable.

      - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.”

- Ad-hoc agreement:

  - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

    - Maximum 3 subband configurations allowed for FR1 WA BS

      - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS.

      - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations.

    - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable.

      - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

##### Issue 1-2-2: Restriction on the number of DL subband sizes supported

- [Moderator] For the restriction-3 (i.e., Restriction on the number of DL subband sizes supported), the following agreement is achieved in RAN4#114bis:

Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

- Proposal on the way to determine the threshold for FR1 MR BS:

  - Proposal 1: Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

- Moderator Recommendation: Adopt the following bullets:

  - “For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

    - This restriction shall be applied to FR1 WA BS

    - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

      - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.”

- Ad-Hoc meeting agreement:

  - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

    - This restriction shall be applied to FR1 WA BS

    - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

      - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

##### Issue 1-2-3: Restriction on guardband size for SBFD

- [Moderator] Based on RAN4#114bis agreement:

Agreement: - Restriction on guardband size for SBFD: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guardband. - The declaration of guardband size should ensure the benefits from SBFD operation. - In the RAN4 specification, the clause “It is expected that guardband size should be minimized to ensure the benefits from SBFD operation.” could be captured, and the detailed wording can be further refined.

- Proposal on the detailed wording mentioned in RAN4#114bis agreement:

  - Proposal 1 (CATT): The declaration of guardband size should ensure the benefits from SBFD operation, and it can be “The benefits from SBFD operation are achievable with the declared guardband size.” in the RAN4 specification.

  - Proposal 2 (Nokia): If it is decided that a clause is introduced to the RAN4 specification related to guardband size, the text should say: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead.”

- Proposal on restriction on the location of guardband (not discussed in previous meetings):

  - Proposal 3 (CATT): For each SBFD subband configuration, there may be the following three options for guardband position. All these positions should be part of the declaration, i.e., declare a single fixed guardband position for each configuration, or only one position common for all configurations.

    - Option1: All guardband is confined in DL subband

    - Option2: All guardband is confined in UL subband

    - Option3: Half guardband is confined in DL subband, and the other half is confined in UL subband

- Moderator Recommendation:

  - For proposal 1&2, the group can further study.

  - For proposal 3, based on moderator’s understanding, the guardband is implicitly given by frequency configuration based on RAN1 agreement, so guradband should be outside of uplink and downlink subbands.

- Ad-hoc discussion:

  - Nokia: it is last meeting’s decision, and we need to discuss more on how to implement that. For the new proposed one from CATT, still have concern on that.

##### Issue 1-2-4: Restriction on DL subband size

- [Moderator] Based on RAN4#114bis agreement:

Agreement: - If the SSB symbol is not overlapped with SBFD symbol, - There is no restriction on DL subband size. - If the SSB symbol is overlapped with SBFD symbol, - FFS the impact from RAN4 perspective.

It should also be noted that there is a relevant working assumption from RAN1:

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

- Proposals:

  - If the SSB symbol is overlapped with SBFD symbol,

    - Option 1 (Huawei, Samsung): there is no RAN4 impact expected regarding DL subband size configuration

      - Option 1a (Samsung): there is neither restriction required nor RAN4 requirement impact identified for DL subband size configuration.

    - Option 2 (CATT, ZTE): restriction is needed:

      - Option 2a (CATT): DL subband should cover the whole DL initial BWP

      - Option 2b (ZTE): DL subband should cover both SSB and CORESET#0

- Moderator Recommendation:

  - Based on the working assumption from RAN1, seems that it is expect that the SSB can still be received for both SBFD-aware and legacy UEs, therefore there is no restriction needed. Based on that group can further discuss.

- Ad-hoc discussion:

  - ZTE: Option 2a and 2b are reasonable proposed, but not necessarily to have RAN4 impact at least on RAN4 core part, maybe FFS during performance part on the test configuration.

- Ad-hoc agreement:

  - If the SSB symbol is overlapped with SBFD symbol,

    - there is no RAN4 impact at least on RAN4 core requirement for expected regarding DL subband size configuration

    - FFS the test configuration during performance part discussion.

### Sub-topic 1-3: Requirement applicability and BS RF specification structure for SBFD

##### Issue 1-3-1: Missed clause for EEIRP introduced in Rel-19

- [Moderator] It has been identified that clause “9.9 OTA spatial emission” has already been introduced in the latest release (V19.0.0)”, which should be considered for SBFD requirement.

- Proposal from Ericsson:

  - Proposal 1: Use the following updated clause structure and Requirement set applicability for SBFD-capable BS.

12	Requirements for SBFD-capable BS	Error! Bookmark not defined.

12.1	General	Error! Bookmark not defined.

12.2	Conducted transmitter characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.2.1	General	Error! Bookmark not defined.

12.2.2	Base station output power for SBFD	Error! Bookmark not defined.

12.2.3	Output power dynamics for SBFD	Error! Bookmark not defined.

12.2.4	Transmit ON/OFF power for SBFD	Error! Bookmark not defined.

12.2.5	Transmitted signal quality for SBFD	Error! Bookmark not defined.

12.2.6	Unwanted emissions for SBFD	Error! Bookmark not defined.

12.2.7	Transmitter intermodulation for SBFD	Error! Bookmark not defined.

12.2.8	Transient period between SBFD and non-SBFD	Error! Bookmark not defined.

12.2.9	In-channel adjacent subband leakage for SBFD	Error! Bookmark not defined.

12.3	Conducted receiver characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.3.1	General	Error! Bookmark not defined.

12.3.2	Reference sensitivity level for SBFD	Error! Bookmark not defined.

12.3.3	Dynamic range for SBFD	Error! Bookmark not defined.

12.3.4	In-band selectivity and blocking for SBFD	Error! Bookmark not defined.

12.3.5	Out-of-band blocking for SBFD	Error! Bookmark not defined.

12.3.6	Receiver spurious emissions for SBFD	Error! Bookmark not defined.

12.3.7	Receiver intermodulation for SBFD	Error! Bookmark not defined.

12.3.8	In-channel selectivity for SBFD	Error! Bookmark not defined.

12.3.9	In-channel adjacent subband blocking for SBFD	Error! Bookmark not defined.

12.4          Conducted performance requirements for SBFD-capable BS

12.5	Radiated transmitter characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.5.1	General	Error! Bookmark not defined.

12.5.2	Radiated transmit power for SBFD	Error! Bookmark not defined.

12.5.3	OTA base station output power for SBFD	Error! Bookmark not defined.

12.5.4	OTA output power dynamics for SBFD	Error! Bookmark not defined.

12.5.5	OTA transmit ON/OFF power for SBFD	Error! Bookmark not defined.

12.5.5.1	General	Error! Bookmark not defined.

12.5.5.2	OTA transmitter OFF power for SBFD	Error! Bookmark not defined.

12.5.5.3	OTA transient period for SBFD	Error! Bookmark not defined.

12.5.6	OTA transmitted signal quality for SBFD	Error! Bookmark not defined.

12.5.7	OTA unwanted emissions for SBFD	Error! Bookmark not defined.

12.5.8	OTA transmitter intermodulation for SBFD	Error! Bookmark not defined.

12.5.9	OTA spatial emission for SBFD	Error! Bookmark not defined.

12.5.10	OTA transient period between SBFD and non-SBFD	Error! Bookmark not defined.

12.5.11	OTA in-channel adjacent subband leakage for SBFD	Error! Bookmark not defined.

12.6	Radiated receiver characteristics for SBFD-capable BS	Error! Bookmark not defined.

12.6.1	General	Error! Bookmark not defined.

12.6.2	OTA sensitivity for SBFD	Error! Bookmark not defined.

12.6.3	OTA reference sensitivity level for SBFD	Error! Bookmark not defined.

12.6.4	OTA dynamic range for SBFD	Error! Bookmark not defined.

12.6.5	OTA in-band selectivity and blocking for SBFD	Error! Bookmark not defined.

12.6.6	OTA out-of-band blocking for SBFD	Error! Bookmark not defined.

12.6.7	OTA receiver spurious emissions for SBFD	Error! Bookmark not defined.

12.6.8	OTA receiver intermodulation for SBFD	Error! Bookmark not defined.

12.6.9	OTA in-channel selectivity for SBFD	Error! Bookmark not defined.

12.6.10	OTA in-channel adjacent subband blocking for SBFD	Error! Bookmark not defined.

12.7	   Radiated performance requirements for SBFD-capable BS

- Table 12.1-1: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | 12.2.2 |  |  |
| Output power dynamics | 6.3 |  |  | 12.2.3 |  |  |
| Transmit ON/OFF power | 6.4 |  |  | 12.2.4 |  |  |
| Transmitted signal quality | 6.5 |  |  | 12.2.5 |  |  |
| Occupied bandwidth | 6.6.2 |  |  | 12.2.6.2 |  |  |
| ACLR | 6.6.3 |  |  | 12.2.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | 12.2.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | 12.2.6.5 |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | 12.2.7 | NA | NA |
| Transient period between SBFD and non-SBFD | NA |  |  | 12.2.8 |  |  |
| In-channel adjacent subband leakage | NA |  |  | 12.2.9 |  |  |
| Reference sensitivity level | 7.2 |  |  | 12.3.2 |  |  |
| Dynamic range | 7.3 |  |  | 12.3.3 |  |  |
| In-band selectivity and blocking | 7.4 |  |  | 12.3.4 |  |  |
| Out-of-band blocking | 7.5 |  |  | 12.3.5 |  |  |
| Receiver spurious emissions | 7.6 |  |  | 12.3.6 |  |  |
| Receiver intermodulation | 7.7 |  |  | 12.3.7 |  |  |
| In-channel selectivity | 7.8 |  |  | 12.3.8 |  |  |
| In-channel adjacent subband blocking | NA |  |  | 12.3.9 |  |  |
| Performance requirements | 8 |  |  | 12.4 |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | 12.5.2 | 12.5.2 | 12.5.2 |
| OTA base station output power |  | 9.3 | 9.3 |  | 12.5.3 | 12.5.3 |
| OTA output power dynamics |  | 9.4 | 9.4 |  | 12.5.4 | 12.5.4 |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | 12.5.5 | 12.5.5 |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | 12.5.6 | 12.5.6 |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | 12.5.7.2 | 12.5.7.2 |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | 12.5.7.3 | 12.5.7.3 |
| OTA operating band unwanted emission |  | 9.7.4 | 9.7.4 |  | 12.5.7.4 | 12.5.7.4 |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | 12.5.7.5 | 12.5.7.5 |
| OTA transmitter intermodulation |  | 9.8 | NA |  | 12.5.8 | NA |
| OTA spatial emission |  | 9.9 | 9.9 |  | 12.5.9 | 12.5.9 |
| OTA transient period between SBFD and non-SBFD |  | NA | NA |  | 12.5.10 | 12.5.10 |
| OTA in-channel adjacent subband leakage |  | NA | NA |  | 12.5.11 | 12.5.11 |
| OTA sensitivity | 10.2 | 10.2 | NA | 12.6.2 | 12.6.2 | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | 12.6.3 | 12.6.3 |
| OTA dynamic range |  | 10.4 | NA |  | 12.6.4 | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | 12.6.5 | 12.6.5 |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | 12.6.6 | 12.6.6 |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | 12.6.7 | 12.6.7 |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | 12.6.8 | 12.6.8 |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | 12.6.9 | 12.6.9 |
| OTA in-channel adjacent subband blocking |  | NA | NA |  | 12.6.10 | 12.6.10 |
| Radiated performance requirements |  | 11 | 11 |  | 12.7 | 12.7 |

- Moderator Recommendation:

  - RAN4 adopt the above proposal for the updates given by Ericsson.

- Ad-hoc agreement:

  - RAN4 adopt the above proposal for the updates given by Ericsson.

    - Need to update the agreement and also the work splitting plan.

### Sub-topic 1-4: CLI handling impact on RF requirement

##### Issue 1-4-1: CLI handling impact on RF requirement

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved. Based on previous meeting’s discussion, there is no agreement to either revisit or confirming no revisit the “FFS” bullet.

<From RAN4#112> - Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Further proposals are provided:

  - Proposal 1 (Nokia): RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Moderator Recommendation:

  - It has been proposed and discussed for several meetings, but not easily to be concluded.

### Sub-topic 1-5: draftCR to TS 38.104

- [Moderator] As the worksplitting planned for dCR drafting, the following dCR is prepared which are related to general parts for SBFD operation.

  - R4-2507482, draftCR to TS38.104 on general clauses for SBFD-capable BS, Samsung

    - Moderator Recommendation:

      - View collection before and during the meeting.

  - R4-2507714, Draft BigCR to TS38.104 for SBFD BS RF requirement, Samsung

    - Moderator Recommendation:

      - The Tdoc is reserved for post-meeting endorsed dCR merging if needed.

# Topic #2: SBFD-aware UE related issues

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505589 | Huawei, HiSilicon | Observation 1: Might be no harm to further update the spec as e.g. “uplink symbols and SBFD symbols transmitted in a certain evaluation period” with the intention of better accommodating SBFD-aware UE max duty cycle evaluation. Observation 2: For UL resource muting, there is no transmit quality or MPR requirement impact can be expected for PUSCH at least for lower modulation order as comb-2 pattern is already supported since LTE. Proposal 3: If MPR performance degradation can be identified, scheduling restriction can be considered rather than introducing RAN4 minimum requirements impact. |
| R4-2507518 | ZTE Corporation, Sanechips | Proposal 3: for FR1 UL 256QAM, the MPR impacts due to the CLI muting pattern is less than 0.48dB for CP-OFDM and less than 0.39dB for DFT-s-OFDM. Proposal 4: in order to simplify the spec change, propose to add the following two sentence in the general section 6.1. For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. |
| R4-2507650 | Qualcomm Incorporated | Observation 1: Resource muting for the studied QPSK waveforms at the left edge of the channel have less than 0.5 dB impact on MPR. Proposal 1: Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with allocations at the edges of the channel have minimal impact on the transmit waveform quality. Observation 2: Resource muting for the studied QPSK waveforms with full allocation has negligible impact on MPR. Proposal 2: Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with full allocations have no impact on the transmit waveform quality. Proposal 3: Resource muting should be studied for the remainder of QPSK allocations along with other modulations to gauge its impact on transmit waveform quality. |
| R4-2507397 | Ericsson | Observation 1 Any UL symbols transmitted—whether TDD UL symbols or SBFD symbols—are considered as UL symbols for the purpose of UL duty cycle evaluation. Observation 2 MPR remains unaffected by the introduction of UL resource muting. Proposal 1 The SBFD symbol does not affect the UL duty cycle evaluation in the RAN4 UE specification, and no further clarification is needed. |
| R4-2506875 | vivo | Observation 1: The term ‘uplink symbols transmitted’ as a whole, not ‘uplink symbols’ standalone, means ‘uplink time’, ‘uplink transmission time’ or ‘uplink transmission’ during the evaluation period for HPUE SAR. Proposal 1: No change for HPUE duty cycle SAR part in RAN4 specification. |
| R4-2506047 | CATT | Observation 1: there are two alternatives to make changes on RAN4 specs in order to ensure the correct uplink duty cycle evaluation for SBFD-aware UEs: one for generalization, the other for patching. Observation 2: The generalization way is future-proof but requiring to update the same term in too many places throughout the spec, while the patching way is simple but is less user-friendly. Proposal 1: RAN4 to take Alternative #2 (Patching) for the updates on uplink duty cycle evaluation. Proposal 2: RAN4 to add a clarification sentence under sub-clause 6.1 as “For an SBFD-aware UE, uplink transmissions in both non-SBFD and SBFD symbols shall be taken into account as uplink symbols transmitted for uplink duty cycle evaluation”. |
| R4-2506048 | CATT | draftCR to TS 38.101-1 on updating duty cycle evaluation with the introduction of SBFD |
| R4-2507481 | Samsung | Proposal 6: RAN4 consider the following two options to guarantee the common understanding for SBFD-aware UE for HPUE duty cycle evaluation: - Option 1: RAN4 shall introduce CR from Rel-15 to explain that “uplink symbols” should be interpreted as any symbols possible for UE transmission. - Option 2: After listing TS 38.101 series as impacted specification in SBFD WID, RAN4 shall introduce CR to explicitly explain that “uplink symbols” should be interpreted as any symbols possible for UE transmission “including SBFD symbols”. |
| R4-2506530 | Qualcomm Germany | Observation 1: The HPUE duty cycle evaluation is based on UL transmissions carried by the UE and not only on the symbol assignment by the network. Observation 2: Duty cycle evaluation considers UL transmissions by a (SBFD-aware or non SBFD-aware) UE independent if such UL transmissions were based on UL symbols, flexible symbols assigned to UL by the network, or UL subband of an SBFD symbol. Proposal 4: No additional considerations for SBFD-aware UE is expected to the HPUE duty cycle evaluation given in TS 38.101-1. Observation 3: Any modifications to TS 38.101-1 would require an update of the Rel-19 SBFD WI to include it in the list of impacted existing TSs. |

## Open issues summary

### Sub-topic 2-1: RF requirement impact for PUSCH with resource muting

##### Issue 2-1-1: RF requirement impact for PUSCH with resource muting

- [Moderator] Triggered by incoming LSs from RAN1 [R1-2410889/R4-2500006] and [R1-2503089/R4-2505306], RAN4 is supposed to check impact on transmit signal quality/MPR. The following way forward is agreed in RAN4#114bis:

Way forward: - RAN4 has discussed this issue and will try to draft a reply LS to RAN1 based on existing results in next meeting. - Evaluation on remaining scope can be continued, and the issues can be raised if found in future.

- Following proposal/observations provided:

  - PAPR increasement from new waveform:

    - Ericsson: MPR remains unaffected.

  - Resource muted DFT-s-OFDM and CP-OFDM waveforms with allocations at the edges of the channel: SEM limited

    - Qualcomm: less than 0.5 dB impact on MPR for QPSK

    - Apple (in last meeting Tdoc R4-2504455): no impact observed for pi/2 BPSK, QPSK, 16QAM and 64QAM

    - Huawei: no MPR performance degradation

    - ZTE: less than 0.48dB for CP-OFDM and less than 0.39dB for DFT-s-OFDM for QPSK, 16QAM, 64QAM and 256QAM

  - Resource muted DFT-s-OFDM and CP-OFDM waveforms with full allocations: ACLR limited

    - Qualcomm: negligible impact on MPR for QPSK

    - Apple (in last meeting Tdoc R4-2504455): No degradation for pi/2 BPSK, QPSK, 16QAM and 64QAM

    - ZTE: less than 0.48dB for CP-OFDM and less than 0.39dB for DFT-s-OFDM for QPSK, 16QAM, 64QAM and 256QAM

- To be evaluated:

  - Potential scope mentioned by Qualcomm: 256QAM to be added.

- Other proposals on the general consideration for scheduling restriction:

  - Proposal 1 (Huawei): If MPR performance degradation can be identified, scheduling restriction can be considered rather than introducing RAN4 minimum requirements impact.

  - Proposal 2 (Qualcomm): Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with allocations at the edges of the channel have minimal impact on the transmit waveform quality.

- Moderator Recommendation:

  - Based on existing results, the RAN4 evaluation on the signal quality impact from the new waveform shall be concluded in this meeting:

    - LS to RAN1 can be drafted based on existing results.

    - Evaluation on remaining scope can be continued, and the issues can be raised if found in future.

- Ad-hoc discussion:

  - Huawei: need to be careful about the content of LS. 256QAM needs FFS.

  - Qualcomm: will share the drafted LS and QC has the updated results.

  - ZTE: how about FR2? No results at least from ZTE perspective.

### Sub-topic 2-2: Update for HPUE duty cycle evaluation

##### Issue 2-2-1: The necessity to update for HPUE duty cycle evaluation

- [Moderator] The necessity to update the wording for HPUE duty cycle evaluation due to introduction of SBFD has been discussed. The following way forward is achieved in RAN4#114bis:

Way forward: - RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

- Proposals:

  - Option 1 (ZTE, CATT, Huawei): Further explanation/revision is required for the existing wording “uplink symbols transmitted”, (and “Patching” method will be used, i.e., keep the wording “uplink symbols transmitted” used for uplink duty cycle evaluation unchanged, however, in some general sub-clause add explanation).

    - Note: Samsung and Qualcomm mentioned the necessity of updating WID to include TS38.101 series as impacted existing spec.

  - Option 2 (Ericsson, vivo, Qualcomm, Nokia): No further clarification/change is needed.

    - Observation: The HPUE duty cycle evaluation is based on UL transmissions carried by the UE and not only on the symbol assignment by the network.

- Moderator Recommendation:

  - The group can discuss this issue further by considering capturing some explanatory description in RAN4 agreement as compromise.

- Ad-hoc meeting discussion:

  - Nokia: support Option 2.

  - ZTE: just want to clarify it is not only UL symbols.

  - vivo: from HPUE evaluation perspective, UL symbols means the symbols with UL transmissions. No need to change the spec.

  - Charter: agree with vivo.

  - Moderator: How about “capturing some explanatory description in RAN4 agreement as compromise.”

  - ZTE: we still need a note in the specification which product team really care.

  - Ericsson: no clarification is needed.

##### Issue 2-2-2: If the update is needed, how to introduce in RAN4 spec

- [Moderator] If the necessity of the update to TS38.101 is confirmed (from last issue), then:

  - RAN4 consider the following two options to guarantee the common understanding for SBFD-aware UE for HPUE duty cycle evaluation:

    - Option a (Samsung): RAN4 shall introduce CR from Rel-15 to explain that “uplink symbols” should be interpreted as any symbols possible for UE transmission.

    - Option b (ZTE, Samsung, CATT): After listing TS 38.101 series as impacted specification in SBFD WID, RAN4 shall introduce CR from Rel-19:

      - Text proposal-1 (ZTE):

        - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

        - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission.

      - Text proposal-2 (CATT):

        - For an SBFD-aware UE, uplink transmissions in both non-SBFD and SBFD symbols shall be taken into account as uplink symbols transmitted for uplink duty cycle evaluation

- Moderator Recommendation:

  - If the necessity of update is confirmed, discuss how to introduce it in RAN4 spec.

# Topic #3: SBFD adjacent channel coexistence

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505459 | Charter Communications, Inc | Proposal 1: Reduce pMax of the SBFD UE power by XdB. Proposal 2: Send LS from RAN4 to RAN1 asking to approve our agreement Proposal 3: A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%). |

### Sub-topic 3-1: SBFD-SBFD co-existence in adjacent channel

##### Issue 3-1-1: SBFD-SBFD co-existence scenario within Rel-19 scope or not

- [Moderator] After coordination with moderator for [307], this topic will be treated in [307] to avoid double efforts.

### Sub-topic 3-2: SBFD-TDD co-existence in adjacent channel

##### Issue 3-2-1: Fair coexistence between SBFD and TDD networks

- [Moderator] In RAN4#114, for limiting maximum UE transmit power in SBFD symbols/slots, the following way forward is achieved:

##### Issue 1-4-1: Limiting Maximum UE Transmit power in SBFD symbols/slots. Way Forward: - No further RAN4 discussion is expected until RAN1 confirm the necessity of introducing the mechanism of limiting Maximum UE Transmit power in SBFD symbols/slots

In RAN4#114bis, the following way forward is achieved:

##### Issue 3-2-1: Fair coexistence between SBFD and TDD networks Way forward: - RAN4 can further wait RAN1 conclusion on fair co-existence between SBFD and TDD networks.

In this meeting, further proposals are given from Charter’s contribution.

- Proposals from Charter:

  - Proposal 1: Reduce pMax of the SBFD UE power by XdB.

  - Proposal 2: Send LS from RAN4 to RAN1 asking to approve our agreement

  - Proposal 3: A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%).

- Moderator Recommendation:

  - Further discussion on this issue and especially the necessity/possibility to have a RAN4 agreement on the three proposals.

- Ad-hoc discussion:

  - Charter: based on Rel-18 discussion, UE-UE CLI is indeed a problem as identified by companies. We also recognize that the lower pMax can’t be applied to all UE, which will reduce the coverage. From RAN1, there are two options: (1) scheduling restriction; (2) lower power cap for some UE. Our suggestion: from RAN4, we can suggest the value (maybe in square bracket) for how much offset we need for some particular UE with CLI impact to others.

  - Qualcomm: similar discussion in Athens meeting, and at that time, we decide to wait for RAN1 decision.

    - Charter: we can’t have the ping-pong between RAN1 and RAN4.

  - Vivo: it can depend on BS scheduling.

  - Qualcomm: During SI, we assume the worst case, i.e., the flat ACLR model to the adjacent channel. In reality, we will have lower interference compared to the one assumed in SI.

    - Charter: we can discuss how much we need in next meeting based on simulation.

  - Nokia: need a solution for that so we know the intention of it. Need to consider the TU.

  - Samsung: question to Charter, how about other solutions to solve this issue. P-max can be one solution, and how we make the final decision on how to choose the solution.

# Topic #4: New BS RF requirements for SBFD

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505590 | Huawei, HiSilicon | Observation 1: Assuming 45dB ACLR and 53dBm BS output power over 100MHz, roughly -12dBm/MHz level residual noise level in the adjacent channel can be expected. However, it will get higher for U6G since the ACLR requirement is relaxed to 38dB. Observation 2: Seemingly no need to introduce assumption into the core part spec if existing requirement can be reused. Observation 3: Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques. Proposal 1: Suggest to reuse ACLR absolute basic limit given in Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2-1, for the new in-channel adjacent subband leakage. Proposal 2: For in-channel adjacent subband blocking, the source interference only consists of the inter-site interference and the co-site inter-sector interference. The self-interference shall be precluded. Proposal 3: Specify the in-channel adjacent subband blocking as -27dBm/100MHz. |
| R4-2506009 | CATT | Proposal 1: For absolute limit for in-channel adjacent subband leakage, the table 6.6.3.2-2 in TS 38.104 for FR1 and table 9.7.3.3-2 for FR2 in TS 38.104 can be reused. Proposal 2: The in-channel adjacent sub-band blocking requirement should consider the interference from inter-sectors BSs and inter-site BSs. Proposal 3: For in-channel adjacent sub-band blocking requirement, -32dBm can be used for MR BS and -34dBm can be used for LA BS. Proposal 4: The isolation between two sectors can be assumed to be 70dB. |
| R4-2506930 | Qualcomm Germany | Observation 1: UE’s knowledge of the location of the transient period for Case B and C is important to ensure proper transition between SBFD and non-SBFD symbols. Proposal 1: RAN4 to discuss if the UE’s knowledge of location of the transient periods between SBFD and non-SBFD is relevant to Rel-19 WI discussions. Observation 2: The in-channel subband sub-band Tx leakage and blocking depend on the self-interference, inter-site and inter-sector interference considerations; thus, they are implementation dependent. Proposal 2: RAN4 to agree on reusing ACLR absolute basic limit given in Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2-1, for the absolute limit for in-channel adjacent subband leakage. |
| R4-2507483 | Samsung | Proposal 1: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 SBFD DU/UD case can be specified by separating the different BS categories and bands: BS category / BS class; In-channel adjacent subband leakage absolute basic limit for bands other than n104; In-channel adjacent subband leakage absolute basic limit for bands n104. Category A Wide Area BS; -8 dBm/MHz; -1 dBm/MHz. Category B Wide Area BS; -10 dBm/MHz; -3 dBm/MHz. Medium Range BS; -20 dBm/MHz; -13 dBm/MHz. Local Area BS; -27 dBm/MHz; -20 dBm/MHz. Proposal 2: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 SBFD DUD case can be defined with the same limit given in Proposal 1. Proposal 3: For FR1 WA BS even without gNB-to-gNB CLI handling into account, -35 dBm can be viewed as a reasonable level to define the SBFD in-channel adjacent subband blocking requirement. |
| R4-2507484 | Samsung | draftCR to TS38.104 on transient period requirements for SBFD-capable BS |
| R4-2507519 | ZTE Corporation, Sanechips | Proposal 1: for FR1 local SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed; Proposal 2: for FR1 WA and MR SBFD BS, up to 7.4dB relaxation could be allowed. Observation 1: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the in-band blocking level. Observation 2: in-channel blocking requirement for FR1 SBFD BS are summarize as following: Without CLI; With CLI coordination; In-band blocking. FR1 WA SBFD BS; -51.28dBm; -63.8dBm; -43. FR1 MR SBFD BS; -51.656dBm; N/A; -38. FR1 LA SBFD BS; -45.24dBm; N/A; -35dBm. Proposal 3: for the feasibility of in-channel blocking requirement, analog filter could be assumed for FR1 WA SBFD and FR1 MR BS and no analog filter could be assumed for FR1 LA SBFD and FR2 SBFD BS. |
| R4-2507637 | Nokia | Proposal 1: Use the existing ACLR absolute limits as the new in-channel adjacent subband leakage requirements. Proposal 2: Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes. Proposal 3: Introduce the in-channel adjacent subband blocking level of -18dBm. |
| R4-2507725 | Ericsson | Observation 1 Operators cannot deploy SBFD selectively at just a few locations where UL coverage enhancement is desired, while other sites deploy regular TDD. Observation 2 Operators cannot deploy SBFD with varying subband configurations across sites. Observation 3 In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and has not been studied so far. Observation 4 CLI handling schemes for the case of inter-site interference could suppress a portion of the interference, but they cannot completely eliminate it, because it is not always feasible, and RX digital beamforming cannot be considered. Proposal 1 Add a note in TS 38.104 where appropriate that “Operators shall deploy SBFD uniformly across all sites with exactly the same subband configuration.” Proposal 2 Specify -13 dBm/MHz as the requirement level for the SBFD in-channel adjacent sub-band leakage. Proposal 3 Define in-channel adjacent subband blocking requirement without considering CLI handling schemes. Proposal 4 For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors. Proposal 5 Terminology “non-SBFD” need to be introduced. Proposal 6 Use Figure 1 for the illustration of Transient period between SBFD and non-SBFD in TS 38.104. |

## Open issues summary

### Sub-topic 4-1: Requirements for Transient period between SBFD and non-SBFD

- [Moderator] In RAN4#114 and 114bis, the following WF is achieved for “Location of transient period between SBFD and non-SBFD”:

Agreement in RAN4#114: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below: Agreement in RAN4#114bis: - For Case-B, guard symbol(s) should be reserved before UL symbols - The transient period can be reserved only in the DL part(s) of SBFD symbol(s) or in the whole channel bandwidth, up to gNB implementation. - For the location of transient periods Case-C, - Both Option 1 (guard symbols reserved in non-SBFD DL symbol(s)) and Option 2 (blanked SBFD symbols in the beginning of SBFD UL transmission) shall be allowed, up to gNB implementation. - For the location of transient period between SBFD and non-SBFD symbols, - No need to be specified in RAN4 core requirement; - For conformance testing for relevant requirements, - FFS test configurations.

##### Issue 4-1-1: Remaining issue for the location of transient period

- Proposals/Observations on remaining issue for the location of transient period:

  - Observation 1 (Qualcomm): UE’s knowledge of the location of the transient period for Case B and C is important to ensure proper transition between SBFD and non-SBFD symbols.

  - Proposal 1 (Qualcomm): RAN4 to discuss if the UE’s knowledge of location of the transient periods between SBFD and non-SBFD is relevant to Rel-19 WI discussions.

- Moderator Recommendation:

  - Given the information contained in LS to RAN1, we expect RAN1 will determine how the BS implementation can be interpreted by UE, or if there is some further restriction given by RAN1.

  - Accordingly, we expect no further RAN4 discussion is needed on the location of transient period. Further collect views if needed.

##### Issue 4-1-2: Text proposal related to transient period between SBFD and non-SBFD

- Proposals on text proposal related to transient period between SBFD and non-SBFD:

  - Proposal 1 (Ericsson): Terminology “non-SBFD” need to be introduced.

  - Proposal 2 (Ericsson): Use Figure 1 for the illustration of Transient period between SBFD and non-SBFD in TS 38.104.

Transmitter output power in UL subband/DL subband(s)

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

NOTE: The figure illustrates the transition between SBFD (either DUD or DU/UD patterns) and non-SBFD. The switching between downlink and uplink depends on the switching subband, i.e., UL subband for Case A or Case C, DL subband(s) for Case B or Case D.

- Moderator Recommendation:

  - Two proposals are agreed and then discuss how to capture in the corresponding dCRs (dCR for general part and dCR for transient period).

- Ad-hoc meeting agreement:

  - Terminology “non-SBFD” need to be introduced.

  - Use Figure 1 for the illustration of Transient period between SBFD and non-SBFD in TS 38.104.

### Sub-topic 4-2: In-channel adjacent subband leakage

- [Moderator] In RAN4#113, RAN4 agree to introduce new requirement for in-channel adjacent subband leakage ratio, and in RAN4#114 and 114bis, the following agreement is achieved:

Agreement in RAN4#114: - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit. Way forward in RAN4#114bis: - FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage. - Absolute limit for in-channel adjacent subband leakage: - FFS by considering the following exemplary cases by performing analysis including measurement or simulation: - FR1: - DU/UD: 80MHz (DL) – 20MHz (UL) - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL) - Other cases are not precluded - FR2-1: - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL) - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL) - Other cases are not precluded - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit.

##### Issue 4-2-1: Assumption for in-channel adjacent subband leakage requirement

- Proposals on whether/how the assumption when define the TX leakage requirement:

  - Proposal 1 (Ericsson): Add a note in TS 38.104 where appropriate that “Operators shall deploy SBFD uniformly across all sites with exactly the same subband configuration.

  - Proposal 2 (Huawei): no need to introduce assumption into the core part spec if existing requirement can be reused.

- Moderator Recommendation:

  - Discuss the way to refine the wording can be acceptable to all companies.

##### Issue 4-2-2: Absolute limit for in-channel adjacent subband leakage

- Absolute limit for in-channel adjacent subband leakage

  - Option 1 (Qualcomm, CATT, Ericsson, Nokia, Huawei): legacy ACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104)

    - Huawei mentioned a higher value for band n104.

  - Option 2 (Samsung): 5dB relaxation on ACLR absolute basic limit for FR1:

| BS category / BS class | In-channel adjacent subband leakage absolute basic limit for bands other than n104 | In-channel adjacent subband leakage absolute basic limit for bands n104 |
| --- | --- | --- |
| Category A Wide Area BS | -8 dBm/MHz | -1 dBm/MHz |
| Category B Wide Area BS | -10 dBm/MHz | -3 dBm/MHz |
| Medium Range BS | -20 dBm/MHz | -13 dBm/MHz |
| Local Area BS | -27 dBm/MHz | -20 dBm/MHz |

  - Option 3 (ZTE):

    - for FR1 local SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed;

    - for FR1 WA and MR SBFD BS, up to 7.4dB relaxation could be allowed.

- Moderator Recommendation:

  - See the ways to differentiate for different BS classes are possible.

### Sub-topic 4-3: In-channel adjacent subband blocking

- [Moderator] In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband blocking requirement, while no need to introduce new requirement for in-channel adjacent subband selectivity. There is some further way forward achieved in RAN4#114bis:

Agreement from RAN4#113: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration. Way forward in RAN4#114bis: - New requirement for in-channel adjacent subband blocking: - The requirement can be derived by considering co-site inter-sector case and/or inter-site case - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification. - The range of [-35dBm to -27dBm] can be used as the starting point for further study.

##### Issue 4-3-1: In-channel adjacent subband blocking requirement

- Proposals on the level of in-channel adjacent subband blocking requirement:

  - For FR1 WA BS:

    - Option 1 (Samsung): -35 dBm.

    - Option 2 (Ericsson, Huawei): -27 dBm.

    - Option 3 (Nokia): -18 dBm.

  - For FR1 MR BS:

    - Option 1 (CATT): -32 dBm.

  - For FR1 LA BS:

    - Option 1 (CATT): -34 dBm.

  - Relevant proposals:

    - Proposal 1 (CATT): The in-channel adjacent sub-band blocking requirement should consider the interference from inter-sectors BSs and inter-site BSs.

    - Proposal 2 (CATT): The isolation between two sectors can be assumed to be 70dB.

    - Proposal 3 (Huawei): For in-channel adjacent subband blocking, the source interference only consists of the inter-site interference and the co-site inter-sector interference. The self-interference shall be precluded.

    - Proposal 4 (ZTE): For the feasibility of in-channel blocking requirement, analog filter could be assumed for FR1 WA SBFD and FR1 MR BS and no analog filter could be assumed for FR1 LA SBFD and FR2 SBFD BS.

- Moderator Recommendation:

  - Discuss on the above proposals/options firstly.

##### Issue 4-3-2: Assumption of gNB-to-gNB CLI handling

- Proposals/Observations:

  - Option 1 (Ericsson, Nokia): Define in-channel adjacent subband blocking requirement without considering CLI handling schemes.

    - Observation 1 (Ericsson): In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and has not been studied so far.

    - Observation 2 (Ericsson): CLI handling schemes for the case of inter-site interference could suppress a portion of the interference, but they cannot completely eliminate it, because it is not always feasible, and RX digital beamforming cannot be considered.

  - Option 2 (Huawei, ZTE, Samsung): Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

    - Observation 3 (ZTE):

|  | Without CLI | With CLI coordination | In-band blocking |
| --- | --- | --- | --- |
| FR1 WA SBFD BS | -51.28dBm | -63.8dBm | -43 |
| FR1 MR SBFD BS | -51.656dBm | N/A | -38 |
| FR1 LA SBFD BS | -45.24dBm | N/A | -35dBm |

- Moderator Recommendation:

  - The discussion on assumptions will closely related to Issue 4-3-1.

### Sub-topic 4-4: draftCR to TS 38.104

- [Moderator] As the worksplitting planned for dCR drafting, the following dCR is prepared which are related to new BS requirements for SBFD-capable BS.

  - R4-2507484, draftCR to TS38.104 on transient period requirements for SBFD-capable BS, Samsung

    - Moderator Recommendation:

      - View collection before and during the meeting.

# Topic #5: BS RF conformance requirements for SBFD

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505591 | Huawei, HiSilicon | Observation 1: The existing UL FRC can be reused for SBFD testing. Observation 2: How to accommodate SBFD time domain pattern with the test model need to be further studied. Observation 3: The interference signal generation for specific requirement shall be aligned with the assumptions for the corresponding requirement. |
| R4-2507487 | Samsung, Huawei | Workplan for Evolution of NR Duplex Operation (RAN4 Performance Part) |
| R4-2507481 | Samsung | Proposal 7: RAN4 shall begin the BS RF conformance testing discussion by considering the following key general issues: - Issue-1: Additional manufacturer declaration(s) is needed for SBFD-capable BS. o It should be noted that at least the supported SBFD configuration for FR1 WA BS and some FR1 MR BS should be included. o FFS other declaration is required - Issue-2: Additional test configuration(s) particular for SBFD requirement. - Issue-3: RF channels to be tested for SBFD operation o FFS RF channels to be tested for SBFD operation, e.g,, to guarantee SBFD carrier is configured at the left/right-most of the operating band. - Issue-4: Additional test models are needed for SBFD-capable BS. o FFS new test models are required to contain SBFD configurations for UL/DL subband and guardband. |

### Sub-topic 5-1: Workplan for performance part

##### Issue 5-1-1: Workplan for performance part

- Workplan for performance part is provided in [R4-2507487], as below:

Table 1: WI plan for Evolution of NR duplex operation (RAN4 Performance Part)

| Schedule and TU | Work plan |
| --- | --- |
| May, 2025 (0.5 RF + 0.25 RD TU for Perf. part) | RAN4 (performance) - Begin discussion on - BS RF conformance requirements to support SBFD operation at gNB - Discussion on RF conformance requirement scope. - RRM performance requirements to support SBFD operation - Discussion on RRM performance requirement scope. - SBFD-capable BS and/or SBFD-aware UE demodulation performance requirements to support SBFD operation - Discussion on demodulation performance requirement scope. |
| August, 2025 RAN4#116 (0.5 RF + 0.25 RD TU for Perf. part) | RAN4 (performance) - Continue discussion on - BS RF conformance requirements to support SBFD operation at gNB - Confirm the scope for BS RF conformance requirements. - Identify and resolve key test issues for BS RF conformance testing. - CR drafting work-splitting confirmed for the conformance testing general requirements and test cases for SBFD-capable BS - RRM performance requirements to support SBFD operation - Confirm the scope for RRM performance requirements considering the UE-to-UE CLI handling and impacted RRM core requirements. - Identify and resolve key issues for RRM performance testing. - CR drafting work-splitting on RRM performance testing general requirements and test cases - SBFD-capable BS and/or SBFD-aware UE demodulation performance requirements to support SBFD operation - SBFD-capable BS and/or SBFD-aware UE demodulation performance requirements to support SBFD operation. - Identify and resolve key issues for demodulation performance testing. - CR drafting work-splitting on demodulation performance testing general requirements and test cases |
| October, 2025 RAN4#116bis (0.5 RF + 1 RD TU for Perf. part) | RAN4 (performance) - Continue discussion and prepare draft CR(s) on - BS RF conformance requirements to support SBFD operation at gNB - RRM performance requirements to support SBFD operation - SBFD-capable BS and/or SBFD-aware UE demodulation performance requirements to support SBFD operation |
| November, 2025 RAN4#117 (0.5 RF + 1 RD TU for Perf. part) | RAN4 (performance) - Finalize discussion and endorse draft CR(s) for - BS RF conformance requirements to support SBFD operation at gNB - RRM performance requirements to support SBFD operation - SBFD-capable BS and/or SBFD-aware UE demodulation performance requirements to support SBFD operation |
| February, 2026 RAN4#118 (0.5 RF + 1 RD TU for Perf. part) | RAN4 (performance) - Resolve remaining issues and CR updates on introduced performance requirements |

- Moderator Recommendation:

  - Approve the workplan in BDaT session.

    - For RRM and Demod part of workplan, can be discussed in other threads.

### Sub-topic 5-2: General observations/proposals for BS RF conformance testing

##### Issue 5-2-1: General observations/proposals for BS RF conformance testing

- Proposals are summarized as:

  - Observations from Huawei:

    - Observation 1: The existing UL FRC can be reused for SBFD testing.

    - Observation 2: How to accommodate SBFD time domain pattern with the test model need to be further studied.

    - Observation 3: The interference signal generation for specific requirement shall be aligned with the assumptions for the corresponding requirement.

  - Proposals from Samsung: RAN4 shall begin the BS RF conformance testing discussion by considering the following key general issues:

    - Issue-1: Additional manufacturer declaration(s) is needed for SBFD-capable BS.

      - It should be noted that at least the supported SBFD configuration for FR1 WA BS and some FR1 MR BS should be included.

      - FFS other declaration is required

    - Issue-2: Additional test configuration(s) particular for SBFD requirement.

    - Issue-3: RF channels to be tested for SBFD operation

      - FFS RF channels to be tested for SBFD operation, e.g,, to guarantee SBFD carrier is configured at the left/right-most of the operating band.

    - Issue-4: Additional test models are needed for SBFD-capable BS.

      - FFS new test models are required to contain SBFD configurations for UL/DL subband and guardband.

  - Observations from Ericsson:

    - Observation 1: The test model for SBFD DL subbband needs to be scalable.

    - Observation 2: New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

    - Observation 3: Signal statistics may change for SBFD test models and need to be investigated. Impact of DL subband on signal statistics needs to be analyzed.

    - Observation 4: Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands.

- Moderator Recommendation:

  - Discuss the proposals and observations provided here.