3GPP TSG-RAN WG4 Meeting #115														R4-2508649

Malta, MT, 19th – 23rd May 2025

Agenda item:				7.11.1

Source:						Moderator (Intelsat)

Title:							Way Forward for [115][313] NR_NTN_Ku_Band_Coexistence…

Document for:			Information

# Introduction

This document captures the discussion on the Ku Band Co-existence simulations and proposes a way forward for further work on this topic.

# Way Forward

Following discussion during online and at ad-hoc sessions the following were agreed as a way forward on the Ku Band Co-existence topic

  - Practical considerations

Note:  For the purpose of writing the TR, at the time of this 3GPP co-existence study, there is no TN band defined near the simulated frequencies.

2.2 On the Throughput Performance

Use 5% percentile for the first preference and if no data is available then use average throughput loss results

Note: in real satellite networks, the cell size is not well defined, and beams are overlapping

2.3 On the guard band between the terrestrial and the non-terrestrial Ku band in simulations:

Guard band was not considered, and it is not necessary for the co-existence analysis

2.4 On the Calibration Data

Additional calibration data for the phase array would be needed as the current data is spread-out

Calibration Assumptions:

For the calibration matrix, to use the assumptions from TR 38.863 Annex A.1 Table A.1-1 and Annex B.1 Table B.1-1 with further modifications based on the current parameters.

- NTN assumptions

  - Re-use from Annex A.1 Table A.1-1 from TR 38.863 with BW 100 MHz

  - #UE – 1 DL/ 10 UL

  - Elevation angle – 35 degrees for both GEO and LEO

  - Urban scenario

  - VSAT phased array 45x45 assumptions

    - VSAT NF of 6 dB

- TN AAS assumptions

  - TN antenna parameters for 11 GHz and 14 GHz

  - #UE – 1 DL/ 3 UL

  - Urban scenario

  - 100% outdoor

  - ISD is 0.45 km

Calibration Metrics:

UL - SINR and Coupling Loss distributions at 5%, 50% and 95%iles on the CDF.
DL - SINR, coupling loss and transmit power distributions at 5%, 50% and 95%iles on the CDF.

Prioritize the simulation results for the parabolic and then tackle the calibration for the phase array case

Note: For the parabolic antenna, the actual simulation results presented are also spread out and would need additional data

2.5 On the FCC and CEPT Mask Compliance

On the FCC and CEPT mask for the co-existence study the consensus is to not to use the masks

2.6 On the Preliminary Simulation Results

The following Table 1 illustrates summary of the results submitted to date for the Ku Band Co-existence simulations. The results illustrate the simulated ACIR of various interference scenarios.

Table 1 Summary of Ku Band Co-existence results

| ACIR（worst case btw 35/90 deg） |  |  | Xiaomi R4-25057620 | Qualcomm R4-2505747 | ZTE R4-2506258 | Ericsson R4-2507672 |
| --- | --- | --- | --- | --- | --- | --- |
| scenario 1 NTN UL aggressor TN UL victim VSAT ACLR to be defined | GEO | Phase array | 3.6 | 25/25 | 15 |  |
|  |  | Parabolic | 9.6 | 20/15 | 21 | 3 |
|  | LEO | Phase array | 0 | 25/0 | 0 |  |
|  |  | Parabolic | 2.1 | 20/0 | 0 | 8 |
| scenario 4 TN DL aggressor NTN UL victim SAN ACS to be defined | GEO | Phase array | 10.3 | 20/15 | 28.21 |  |
|  |  | Parabolic | 9.9 | 20/15 | 28.7 | 16 |
|  | LEO | Phase array | 39.5 | 25/25 | 38.46 |  |
|  |  | Parabolic | 37.7 | 30/25 | 38.46 | 22 |
| scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined | GEO | Phase array | 18.3 | 45/45 | 18.37 |  |
|  |  | Parabolic | 25 | 40/40 | 38.58 | 35 |
|  | LEO | Phase array | 19.3 | 45/45 | 24.36/>40 |  |
|  |  | Parabolic | 26.2 | 40/40 | 41.32 | 35 |
| scenario 6 NTN DL aggressor TN DL victim SAN ACLR to be defined | GEO | Phase array | 0 | 0 | 0 |  |
|  |  | Parabolic | 0 | 0 | 0 | 0 |
|  | LEO | Phase array | 0 | 0 | 0 |  |
|  |  | Parabolic | 0 | 0 | 0 | 0 |

Initial observations are

1- The results provided in the Table 1 above are not well calibrated. Companies are invited to perform further calibration for the next RAN4 meeting #116

2- Follow the Ka Band procedure for ACIR averaging, please see TR 38.863 section 6.3.2

3- Preliminary simulation results for Scenarios 4 and 5 produced the highest ACIR values.

a.	Scenario 4 is related to the SAN ACS

b.	Scenario 5 is related to VSAT ACS

4- For Scenario 4, the LEO case, produced high ACIR values

5- For Scenario 5 all scenarios produced high ACIR values

2.7 For Future RAN4 Meeting

- Companies are invited to provide additional calibration data for the phase array VSATs

- Companies are invited to provide co-existence simulation results based on the agreed simulation assumptions.

- Companies are invited to provide mitigation techniques to provide better protection to NTN based on the VSAT ACS and SAN ACS co-existence results.

- Companies are invited to provide analysis with considerations of NTN SAN/ VSAT ACLR and ACS values in order to ensure the coexistence between NTN and TN.

- For the 78x78 phased array antenna configurations, companies are invited to prioritize scenario 1 for ACLR value and scenario 5 for the ACS value in the co-existence simulations.