3GPP TSG-RAN WG4 Meeting #115	                                                                     R4-2507046

Malta, MT, 19th-23rd May, 2025

Title: 	Draft TP introduction of comparison of spatial channel models for SU-MIMO cases

Source: 	Huawei, HiSilicon

Agenda item:       7.16.2

Document for:	Approval

Background

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.

- Draft TP

# 6  Comparison of spatial channel Models

- 6.1  PDSCH performance under SU-MIMO scenario

This section provides comparison for different spatial channel models with common simulation assumptions captured in Table 6.1-1.

Table 6.1-1: Common Simulation assumptions

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Rank |  |  | 8 |
| Antenna configuration |  |  | 8T8R |
| MCS |  |  | 13 (64 QAM table) |
| Channel model |  |  | 4 TDL-C channel instances and 4 clusters (Table B-2) 1 TDL-C channel instance and 4 clusters (Table B-3) 2 TDL-C channel instances and 4 clusters (Table 5-3) TDLC300-10 Low TDLC300-10 Medium,XP CDL channel model |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | For CDL channel: Single Panel Type I;(i1,1=i1,2=i2)=(4,0,0) For TDL channel: Single Panel Type I; (i1,1=i1,2=i2)=(0,0,0) |
|  |  | Codebook configuration | (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | Rank 4: SNR@70% of maximum throughput Rank8: SNR(dB) @ 30% and 70% of max throughput for each codeword |

Simulation assumptions for CDL channel specifically are captured in Table 6.1-2:

Table 6.1-2: Simulation assumptions for CDL channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank8 |  |
| FR / Carrier frequency |  | FR1,3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

Simulation results are captured in [R4-2507823]

Observations: TBD

- Conclusion

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.