3GPP TSG-RAN WG4 Meeting #115	R4-2508755

St. Juian’s, Malta, 19th ‒ 23rd May 2025

Title: 	Way Forward for [115][333] NR_NTN_Ph3_demod

Agenda Item:	7.31.1

Source: 	Ericsson

Document for:	Approval

# Workplan for NR_NTN_Ph3 demodulation performance part

Agreement:

|  | SAN demodulation | UE demodulation |
| --- | --- | --- |
| RAN4#115 (2025 May) | Discuss the scope of SAN demodulation requirements. Agree with the initial simulation assumption for SAN demodulation. | Discuss the scope of UE demodulation requirements. Agree with the initial simulation assumption for UE demodulation. |
| RAN4#116 (2025 August) | Agree with the scope of SAN demodulation. Update the simulation assumption for SAN demodulation. Collect the initial simulation results for alignment. | Agree with the scope of UE demodulation. Update the simulation assumption for UE demodulation. Collect the initial simulation results for alignment. |
| RAN4#116bis (2025 October) | Finalize the simulation assumption. Collect the simulation results. Decide the CR work split. | Finalize the simulation assumption. Collect the simulation results. Decide the CR work split. |
| RAN4#117 (2025 November) | Collect the simulation results. Review the draft CRs for TS38.108/181. | Collect the simulation results with impairments. Review the draft CRs for TS38.101-5. |
| RAN4#118 (2026 February) | Collect the simulation results. Agree with the CR for TS38.108/181. | Collect the simulation results with impairments. Agree with the CR for TS38.101-5. |

# SAN/UE demodulation requirements

Issue 1-1-1: Broadcast service

Agreement:

- No demodulation performance impact for broadcast service

Issue 1-1-2: Regenerative payload

Agreement:

- No demodulation performance impact for regenerative payload

# SAN demodulation requirements

## Sub-topic 2-1	RedCap/eRedCap

Issue 2-1-1: SAN demodulation requirements for RedCap/eRedCap

Agreement:

- No SAN demodulation requirements for the support of (e)RedCap UEs.

## Sub-topic 2-2	Uplink capacity/cell throughput enhancements

Issue 2-2-1: PUSCH requirements for UL coverage enhancements

Agreement:

- Define SAN PUSCH demodulation requirements for DFT-s-OFDM with orthogonal cover codes.

Issue 2-2-2: OCC parameters

Agreement:

- Apply inter-slot OCC length 2.

- Apply PUSCH type A repetitions

Way forward:

- FFS whether to define PUSCH requirements with inter-slot OCC length 4.

Issue 2-2-3: Detailed PUSCH parameters

Way forward:

- Assume to set the following parameters:

  - FDD SCS=15KHz with 5MHz CBW and FDD SCS=30KHz with 10MHz CBW

  - Both 1Rx and 2Rx, with test applicability rule for different Rx number

  - Same MCS for two UEs for PUSCH with OCC

  - Same channel model for two UEs for PUSCH with OCC

  - Frequency offset between two UEs

    - Baseline assumption: same frequency offset for two UEs in one OCC group. FFS what frequency offset value is assumed.

    - FFS whether to consider different frequency offsets values for two UEs additionally.

- FFS for other parameters.

# UE demodulation requirements

## Sub-topic 3-1	Downlink coverage enhancements

Issue 3-1-1: PDCCH with repetition for CSS

Agreement:

- Not define requirements for PDCCH with repetition.

Issue 3-1-2: PDSCH with repetition for Msg4/SIB1

Agreement:

- Not define requirements for PDSCH with repetition for Msg4/SIB1.

Issue 3-1-3: PBCH periodicity extension

Agreement:

- Not define requirements for PBCH periodicity extension.

## Sub-topic 3-2	RedCap/eRedCap

Issue 3-2-1: PDSCH demodulation requirements for RedCap

Agreement:

For 2Rx UEs,

- Reuse Rel-17 NTN PDSCH demodulation requirements.

  - Companies can check whether all the Rel-17 NTN features can be applicable for RedCap.

Way forward:

For 1Rx UE,

- FFS whether to add new PDSCH requirements with NTN channel or not.

Issue 3-2-2: PDSCH demodulation requirements for eRedCap

Way forward:

For 2Rx UEs,

- Option 1:

  - Add new PDSCH requirements with BW reduction (25RB) with NTN channel.

- Option 2:

  - Reuse Rel-17 NTN PDSCH demodulation requirements without BW reduction.

    - Applicable test cases (TBS ≤ 10,000 bits): TS 38.101-5 Table 8.2.1.2.2.1.1-3 Tests 1-1, 1-3, and 1-4.

For 1Rx UE,

- FFS whether to add new PDSCH requirements with NTN channel or not.

Issue 3-2-3: PDSCH scheduling for HD-FDD

Way forward:

- Option 1: Define new reference channel for HD-FDD (e)RedCap with the same performance requirements as for FDD.

Issue 3-2-4: Test case of PDSCH for RedCap 1Rx (if defined)

Way forward:

- Option 1:

  - MCS4, NTN-TDLA100-200, 1x1, Disabled HARQ

  - MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes

  - MCS4, NTN-TDLA100-200, 1x1, 32 HARQ processes

- Option 2:

  - MCS4, NTN-TDLA100-200, 1x1, 52RBs

  - MCS13, NTN-TDLC5-200, 1x1, 52RBs

- Option 3: Similar test configurations as the existing PDSCH cases

Issue 3-2-6: Test case of PDSCH for eRedCap 1Rx (if defined)

Way forward:

- Option 1:

  - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ

  - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

  - MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes

  - MCS13, NTN-TDLA100-200, 1x1, without reduced BB BW (40RBs), 16 HARQ processes

- Option 2:

  - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RBs)

  - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RBs)

- Option 3: Similar test configurations as the existing PDSCH cases

Issue 3-2-7: Test case of PDSCH for eRedCap 2Rx (if defined)

Way forward:

- Option 1:

  - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RB), Disabled HARQ

  - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 16 HARQ processes

  - MCS4, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 32 HARQ processes

  - MCS13, NTN-TDLA100-200, 1x2, without reduced BB BW (40RBs), 16 HARQ processes

- Option 2:

  - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RBs)

  - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RBs)

- Option 3: Check if the existing PDSCH requirements for 2Rx can be reused for eRedCap without reduced baseband

Issue 3-2-8: Applicability of time-varying Doppler shift and propagation delay model for NGSO

Way forward:

- Option 1: Apply the time-varying Doppler shift and propagation delay model for NGSO to (e)RedCap demodulation requirements.

- Option 2: Not to apply the time-varying Doppler shift and propagation delay model for NGSO to (e)RedCap demodulation requirements in Rel-19.

Issue 3-2-9: PDCCH/PBCH/SDR/CSI requirements

Agreement:

- Not define PDCCH, PBCH, SDR, and CSI reporting requirements for RedCap/eRedCap UEs over NR-NTN.

  - All the existing requirements including PDCCH, PBCH, SDR and CSI reporting requirements in TS38.101-4 can be applicable for Rel-19 NTN access capable RedCap/eRedCap UE according to the UE capability.