3GPP TSG-RAN WG4 Meeting # 115											R4-2505566

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.24.1

Source:	Moderator (Samsung)

Title:	Topic summary for [114][217] NR_MIMO_Ph5_Part1

Document for:	Information

Introduction

This topic summary covers the contributions submitted under the following AI for first part of Rel-19 RRM for NR MIMO Phase 5:

- 7.2	NR MIMO Phase 5

- 7.24.4 RRM core requirements

  - 7.24.4.1 Enhancement for UE-initiated/event-driven beam management

- 7.24.5 RRM performance requirements

Recommended issues to online discussion:

Issue 1-1

Issue 1-2

Issue 1-4

Issue 1-5

Issue 1-3

Issue 1-7

Issue 1-6

Issue 1-8

Issue 1-9

Topic #1: RRM core requirements for UE-initiated/event-driven beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505350 | Qualcomm Technologies Ireland | Proposal 1: For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam. Proposal 2: RAN4 should define delay requirements for the measurement period of UE-initiated/event-driven beam management taking both possible values of M into account. Proposal 3: For the event triggered measurement reporting delay TL1_measurement period, Option 2 in Issue 1-2 should be adopted. The measurement reporting delay should be defined based on the beam that detects the event. Observation 1: In discussions in RAN4 #114bis the opinion was shared that including Tfirst UL channel in the reporting delay means that after the event has been detected, no additional measurements are conducted even if SSB or CSI-RS fall into the time interval Tfirst UL channel. Proposal 4: It should be explicitly captured in an agreement that the UE is not required to continue evaluating the event in the time interval Tfirst UL channel once the event has already been detected. Proposal 5: RAN4 should discuss whether an LS should be sent to RAN1 summarizing the RAN4 agreements for the measurement periods and asking for further clarity of the RAN1 agreement. Observation 2: In case no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested. Proposal 6: RAN4 should not define requirements for the window-based approach whether the event has occurred M times. Proposal 7: RAN4 could reuse the same requirement structure as in section 9.5 for CSI-RS L1-RSRP for inter-cell beam management. Section 9.13 of TS 38.133 is not impacted. Observation 3: RAN1 proposal 5.1’ demands the UE to treat the new beam the same way as the beams in the active TCI state list. Observation 4: Since it is not known to the UE when the network triggers a beam switch, the UE must potentially store the QCL properties for a very long time. Observation 5: Existing UE capabilities for the size of active TCI state list should be sufficient to avoid additional switching delays. Proposal 8: The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management. Observation 6: The number of parallel measurements a UE can support is limited and for example given by the number of CSI-RS resources a UE can support. The evaluation of the various events adds additional processing load. Proposal 9: First the definition of the core requirements and UE capabilities should be finalized and then RAN1/RAN4 working groups can investigate potential limitations of the number of event triggering and reporting criteria. |
| R4-2505388 | China Telecom | Proposal 1: If TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. Proposal 2: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19. Proposal 3: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements. |
| R4-2505729 | Nokia | Observation 1: For UEI beam report, UE needs to measure a new beam and compare it with a current beam to determine if the event triggering condition is met. Observation 2: During event evaluation, same set of conditions should apply for the current beam and the new beam. Proposal 1: When the timeRestrictionForChannelMeasurement is not configured, the UE shall use the same number of samples (i.e. M=3) for the current beam as well as the new beam. Proposal 2: The event triggered measurement delay shall be defined based on the measurement period of the RS which determines the triggering of the UEI event. Proposal 3: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement period_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement period_newbeam. Proposal 4: Define reporting delay requirement also for the window/timer based approach, where the delay takes into account that the UE shall send the report after M event occasions have occurred within the window/timer. Proposal 5: If the indicated TCI state changes during the L1 measurement time, the evaluation of the event trigger determination shall start after the TCI state switch is completed. However, there is no need to capture this scenario in RAN4 requirements. Proposal 6: If UE initiated DL synchronization based on UEIBM report is supported (as discussed by RAN1), RAN4 to discuss the time point at which the UE can be expected to have performed DL synchronization: before sending the UEI beam report or after the report is sent. Observation 2: No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management. Proposal 7: RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not defined a clear use case. Observation 3: RAN1 has agreed to reuse the same design as event-2 for all other supported events Proposal 8: Requirements defined for non-window based event determination shall be applicable to event-2 and any other supported event which reuses the same design as event-2. Proposal 9: Introduce the number of supported UEIBM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4. Proposal 10: Reuse the agreement under NR_Mob_Ph4 regarding coordination between mobility and MIMO events: RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19. |
| R4-2505770 | Xiaomi | Proposal 1：For M in L1 measurement periods, reuse the same values of legacy, measurement period is based on 3 sample. Proposal 2: If TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. Proposal 3: RAN4 to Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19. Proposal 4: Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress. Proposal 5: If indicated TCI states is changed during the L1 measurement time, the L1 evaluation time should restart. |
| R4-2505813 | Samsung | Proposal 1: If timeRestrictionForChannelMeasurement is not configured, reuse the same values of legacy, measurement period is based on 3 sample for SSB based and for periodic and semi-persistent CSI-RS resources. Proposal 2: The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel, if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. Proposal 3: RAN4 to specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19. Proposal 4: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements. Proposal 5: For both MAC-CE based DL/UL TCI state switch delay and DCI based DL/UL TCI state switching delay, The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management. Proposal 6: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. Proposal 7: The principle of Event-2 can be used for event-1 and event-7 easily. For event-7, maximum L1-RSRP measurement period are accounted of all active TCI states. Proposal 8: For event trigger L1 reporting, do not define such capability in parallel per category like L3. Proposal 9: If P8 is agreed, no need to consider such capability compatibility with LTM. |
| R4-2505908 | Apple | Reporting Delay Requirements Observation 1: There has been no RAN1 discussion on applicability of timeRestrictionForChannelMeasurement for UEIBR. Proposal #1: Wait for RAN1 discussion on applicability of timeRestrictionForChannelMeasurement for UEIBR. Proposal #2: RAN4 monitor progress in RAN1#121 and if necessary, send LS to RAN1 seeking clarification on applicability of timeRestrictionForChannelMeasurement for UEIBR in RAN4#115. Proposal #3: Using the existing definition of sharing factor ‘P’, the sharing factors for new beam and current beam are PN and PC respectively. Proposal #4: The measurement period for current beam and new beam each with periodicity TRS for FR1 and non-overlapping and non-adjacent in FR2 are defined as - TL1-RSRP_currBeam = M*N*PC*TRS TL1-RSRP_newBeam = M*N*PN*TRS Proposal #5: The measurement period for current beam and new beam each with periodicity TRS for overlapping or adjacent in FR2 are defined as - TL1-RSRP_currBeam = M*N*2*PC*TRS TL1-RSRP_newBeam = M*N*2*PN*TRS Observation 2: The agreement from last meeting - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH is not definable. Observation 3: The RS that triggered the event is only known at the UE. Observation 4: The minimum requirements should allow measurement period for new beam and current beam. Observation 5: Tfirst UL channel should be defined as time to first PUCCH after L1 measurement period for UEIBR. Proposal #6: The L1 measurement period for UEIBR is the maximum of L1 measurement periods to measure both current beam and new beams. Proposal #7: The Tfirst UL channel is defined as time to first PUCCH after L1 measurement period for UEIBR. Observation 6: It is not feasible to define core requirement for reporting delay with time window-based triggering Observation 7: The purpose of defining requirements based on evaluation window/ counter is unknown. Proposal #8: Measurement reporting delay requirement for UE initiated beam report is defined for eventInstanceCount-r19 =1. Proposal #9: To extend the requirements for case when eventInstanceCount-r19 >1 is configured based on UE capability, add note longer delay is expected if eventInstanceCount-r19 >1 is configured. CSI-RS Based measurements Observation 8: CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell. Proposal #10: For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. Proposal #11: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirement. TCI State Switching Requirements Proposal #12: For DCI based DL/UL TCI state switch, RAN4 shall assume the current requirements are applicable unless there is any agreement in RAN1 on update to beam application delay. Observation 9: UEIBR doesn’t guarantee that RS Is tracked for PDSCH reception. Observation 10: In case the time between the UEIBR and TCI state switch is > 1280 ms, the TCI state would be unknown. Proposal #13: There is no need to specify new unified TCI state switching delay requirements unless the TCI state switch is only defined for known TCI state, and TCI state in active TCI state list. Applicability Proposal #14: Measurement reporting delay requirement applicability - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period Proposal #15: RAN4 prioritize requirements for Event 2 and discuss Event 1/7 requirements later. Capabilities Proposal #16: For event trigger L1 reporting, do not define such capability in parallel per category like L3. Proposal #17: Discuss later in the WI on compatibility of capabilities with LTM. |
| R4-2506297 | LG Electronics Inc. | Proposal 1: If timeRestrictionForChannelMeasurement is not configured, - Reuse the same values of legacy, measurement period is based on 3 sample without restriction Proposal 2: For TL1_measurement period, Option1 (maximum of the L1 measurement periods) is preferred Proposal 3: Specify only no-time window based requirement Proposal 4: For unified TCI switching delay requirements, RRM impact is expected when RAN1 agrees to introduce reduced TCI state switch delay |
| R4-2506586 | CMCC | Proposal 1: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. Proposal 2: it is proposed to specify requirements to cover the case of no time window and the case with time window with configurable M. Proposal 3: for CSI-RS based L1-RSRP for inter-cell BM, it is proposed to reuse the same requirements structure as that in Rel-17 9.5. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, same set of requirements are applied. Proposal 4: if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is proposed to define a separate capability for L1 measurement, not prefer to mix L1 measurement reporting capability with legacy L3 measurement reporting capability. Proposal 5: if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is not preferred to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO. |
| R4-2506711 | Huawei, HiSilicon | Proposal 1: Only define RRM requirements when timeRestrictionForChannelMeasurement is configured. Observation 1: For different cases (i.e. RSRP change of current beam and new beam), and different UE implementations, it is not clear which measurement will trigger the event. Observation 2: To define minimum requirements, the measurement delay shall be defined that there is at least one measurement for new beam and one measurement for current beam after the conditions are met over the air. Based on observation 1 and 2, we proposed to define the delay requirements as option 1. Proposal 2: If TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. Observation 3: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading. Proposal 3: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering. Proposal 4: When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, where C is configured counter. Proposal 5: For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance. Observation 4: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell. Proposal 6: CSI-RS based L1-RSRP measurement requirements for UEIBR are defined which applies to both intra-cell and inter-cell without restriction/limitation. Proposal 7: For UEIBR, No RRM impacts on TCI state switching requirements. Observation 5: Based on current RAN1 feature list, UE can explicitly indicate the supported event (event 1/2/7) and the supported number of new beams for each event. Proposal 8: Do not define RAN4 capabilities on support of Event Triggering and Reporting Criteria. |
| R4-2507011 | Ericsson | Proposal 1: For UE initiated beam reporting M value is 1. RAN4 to revisit the M value when the timeRestrictionForChannelMeasurement is introduced in RAN1. Proposal 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement period_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement period_newbeam. Proposal 3: RAN4 to define UEI BM requirements when the measurement window/counter is configured. Proposal 4: RAN4 to agree on following as the method to specify the requirement when the time window is configured: after the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window. Proposal 5: RAN4 to agree TFirst UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time, where Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1. Proposal 6: Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS. Proposal 7: RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2. Proposal 8: RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. Proposal 9: For MAC CE based TCI state switching, existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching. Proposal 10: For DCI based TCI state switching, wait for RAN1 progress. Proposal 11: When the indicated TCI state is changed, counter resetting mechanism is defined in RAN1 and RAN4 not to define additional applicability rules when the indicated TCI state change. Proposal 12: Do not see a need for a separate capability like the L3 event evaluation category in the RAN4 specification. If a capability for the number of events to supported is needed, it shall be defined by RAN1. |
| R4-2507288 | ZTE Corporation, Sanechips | Proposal 1: When timeRestrictionForChannelMeasurement is not configured, reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implimentation. Proposal 2: Starting from the definition of the starting point of the event triggered measurement reporting delay, Option 2 is more accurate than Option 1, i.e. if the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam. Proposal 3: For Event-1, the requirements could be derived from Event-2 with only current beam been involved in. Proposal 4: For Event-7, the requirements could be derived from Event-2 with more candidate beams been involved in. Proposal 5: For Event-2, both the basic Case 1 and the optional Case 2 should be considered when defining the evaluation period. For the optional Case 2, the sample number M and timer X should be considered. Proposal 6: Based on RAN1 progress, for the TCI state switching triggered by UEIBR(may or may not receive TCI state switching command), UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored. Proposal 7: RAN4 needs to define requirements for CSI-RS based measurements including both intra-cell and inter-cell cases. - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements. |
| R4-2507816 | MediaTek inc. | Proposal 1: Legacy number of samples (M) for L1-RSRP measurement can be reused. If timeRestrictionForChannelMeasurement is not configured, measurement period is based on 3 sample. Proposal 2: RAN4 to consider the legacy sharing factor (P) for L1-RSRP measurement as baseline: - P value is same for intra-cell SSB resources included both current and new beam - P value is different for inter-cell SSB resources included both current and new beam - P value is different for CSI-RS resources included both current and new beam Proposal 3: RAN4 not to discuss the scenario of different periodicities for the current and new beam since RAN1 has no further agreement whether to support it or not. - Consider the maximum periodicity regardless of same or different periodicities of current and new beam: - Max {TSSB_current, TSSB_new} for TL1_measurement period_SSB - Max {TCSI-RS_current, TCSI-RS_new} for TL1_measurement period_CSI-RS Proposal 4: RAN4 to consider delay requirement of event 2 for both “no time window” and “with time window with configurable M”: Proposal 5: For the case of no time window: - Measurement reporting delay is TL1_measurement period + Tfirst UL channel where, - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. Proposal 6: RAN4 to discuss how to define the delay requirement when only M out of N instances meet event 2 condition within the configured time window. Proposal 7: For the case of time window with configurable M (event instances threshold): - Option 1: Measurement reporting delay can re-use the same principle as TTT for the configurable time window. - Option 2: Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where, - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW. - Ttime_window: time window is configured by NW. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. Proposal 8: Do not consider Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management. Proposal 9: RAN4 to consider skipping the fine timing sync (additional SSB reception) for known TCI state if RAN1 has further agreement on the reduction of additional SSB reception for TCI state of UE-initiated/event-driven beam. Otherwise, reuse the existing unified TCI state delay requirement. Proposal 10: No requirement is applied if indicated TCI states is changed during the L1 measurement time. Proposal 11: Define UE reported capabilities for numbers of event refer to legacy L3 (like legacy L3 in section 9.1.4). - For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9]. |

Open issues summary

Sub-topic 1-1

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Background: in RAN4#114bis meeting: - For M in L1_measurement periods: - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam - If timeRestrictionForChannelMeasurement is not configured, - Further discuss on: - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used - Option 2: No requirements even the NW can configure it. - Option 3: FFS

For M in L1_measurement periods related to timeRestrictionForChannelMeasurement

- Proposals

  - Option 1: (Qualcomm, Nokia, Xiaomi, Samsung, LGE, ZTE, MTK)

    - Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used

    - Option 1a: (Nokia)

      - Use the same number (i.e. M=3) for both current beam and new beam

    - Option 1b: (Samsung)

      - Reuse the same values of legacy, measurement period is based on 3 sample for SSB based and for periodic and semi-persistent CSI-RS resources.

  - Option 2: (Huawei)

    - Only define RRM requirements when timeRestrictionForChannelMeasurement is configured.

  - Option 3: (Apple, Ericsson)

    - Wait for RAN1 discussion on applicability of timeRestrictionForChannelMeasurement for UEIBR. If no further progress, send LS to RAN1 to ask clarification

- Recommended WF

  - Check to agree option 1. If no consensus in this meeting, send LS to RAN1.

For L1_measurement periods

  - Option 1: (Qualcomm)

    - For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

  - Option 2: (Apple)

    - Using the existing definition of sharing factor ‘P’, the sharing factors for new beam and current beam are PN and PC respectively.

    - Option 2a: (Apple)

      - The measurement period for current beam and new beam each with periodicity TRS for FR1 and non-overlapping and non-adjacent in FR2 are defined as - 
TL1-RSRP_currBeam = M*N*PC*TRS 
TL1-RSRP_newBeam = M*N*PN*TRS

      - The measurement period for current beam and new beam each with periodicity TRS for overlapping or adjacent in FR2 are defined as -
TL1-RSRP_currBeam = M*N*2*PC*TRS 
TL1-RSRP_newBeam = M*N*2*PN*TRS

  - Option 3: (MTK)

    - RAN4 to consider the legacy sharing factor (P) for L1-RSRP measurement as baseline:

      - P value is same for intra-cell SSB resources included both current and new beam

      - P value is different for inter-cell SSB resources included both current and new beam

      - P value is different for CSI-RS resources included both current and new beam

    - RAN4 not to discuss the scenario of different periodicities for the current and new beam since RAN1 has no further agreement whether to support it or not

[Background]: It is agreed in last meeting: - For P (sharing factor) in L1_measurement periods: - For SSB based or CSI-RS based, - The definition is the same as legacy - The value can be the same or different for the current beam and new beam

- Recommended WF

  - Decide from two options:

    - Option 1:

      - The measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

    - Option 2:

      - The measurement period can be defined based on different Pc and Pn as TL1-RSRP_currBeam , TL1-RSRP_newBeam

        - As reuse the same definition in legacy {N*Pc*M*TRS } and { N* Pn *M*TSSB }

  - Further discuss whether addition Scaling factor of 2 is needed

  - No need to further discuss different periodicities of current and new beam since no further support in RAN1

Issue 1-2: Components of Event triggered measurement reporting delay?

Agreement in last meeting WF and Chair’s report: Agreement: - The event triggered measurement reporting delay includes the Tfirst UL channel - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH Further discuss the two options and select one option in the next meeting: - The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK) - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

- Proposals

  - Option 1: (China Telecom, Xiaomi, Samsung, Apple, LGE, Huawei)

    - if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

    - Option 1a: (Apple)

      - Revert the definition of Tfirst UL channel : Defined as time to first PUCCH after L1 measurement period for UEIBR

  - Option 2: (Qualcomm, Nokia, CMCC, Ericsson, ZTE)

    - If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam..

    - Option 2a: (Ericsson)

      - Based on agreement in last meeting, add additional component: TFirst UL Channel as the first occasion of the first UL channel after the Tnew_UCI_prep time, where Tnew_UCI_prep time is the time to prepare new UCI, which may be specified in RAN1

  - Option 3: (Qualcomm)

    - It should be explicitly captured in an agreement that the UE is not required to continue evaluating the event in the time interval Tfirst UL channel once the event has already been detected.

- Recommended WF

  - First decide from two options:

    - Option 1: China Telecom, Xiaomi, Samsung, Apple, LGE, Huawei

      - if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

    - Option 2: (Qualcomm, Nokia, CMCC, Ericsson, ZTE)

      - If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.

  - Then further discuss more clarification on

    - Update the definition of the TFirst UL Channel based

    - It should be explicitly captured in an agreement that the UE is not required to continue evaluating the event in the time interval Tfirst UL channel once the event has already been detected.

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

- Proposals

  - Option 1: (Qualcomm, Apple, LGE)

    - Do not define requirements for the window-based approach whether the event has occurred M times.

    - Option 1a: (Qualcomm)

      - RAN4 to discuss whether to send LS to RAN1 summarizing the RAN4 agreements for the measurement periods and asking for further clarity of the RAN1 agreement

    - Option 1b: (Apple)

      - Add note longer delay is expected if eventInstanceCount-r19 >1 is configured

  - Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK)

    - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

    - Option 2a: (Nokia)

      - The delay takes into account that the UE shall send the report after M event occasions have occurred within the window/timer

    - Option 2b: (Huawei)

      - When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, where C is configured counter

      - For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance.

    - Option 2c: (Ericsson)

      - After the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window

    - Option 2d: (MTK)

      - Alt1: Measurement reporting delay can re-use the same principle as TTT for the configurable time window.

      - Alt2: Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

        - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

        - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW.

        - Ttime_window: time window is configured by NW.

        - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

- Recommended WF

  - First decide from two options:

    - Option 1: No requirements for with time window based with eventInstanceCount-r19>1

    - Option 2: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

Issue 1-4: CSI-RS based L1-RSRP for inter-cell BM

- Proposals

  - Option 1: (Qualcomm, China Telecom, Samsung, Apple, CMCC, Huawei, ZTE)

    - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

  - Option 2: (Nokia, MediaTek)

    - Not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not defined a clear use case.

  - Option 3: (Ericsson)

    - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2.

    - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Recommended WF

  - Check to agree option 1

Issue 1-5: Whether to specify new unified TCI switching delay requirements?

- Proposals

  - Option 1: (Qualcomm, Samsung, Huawei, Ericsson (MAC-CE based))

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2: (Nokia)

    - If UE initiated DL synchronization based on UEIBM report is supported (as discussed by RAN1), RAN4 to discuss the time point at which the UE can be expected to have performed DL synchronization: before sending the UEI beam report or after the report is sent.

  - Option 3: (Xiaomi, LGE, Ericsson (DCI based))

    - RAN4 shall wait for more RAN1 progress

  - Option 4: (Apple)

    - For DCI based DL/UL TCI state switch, RAN4 shall assume the current requirements are applicable unless there is any agreement in RAN1 on update to beam application delay.

    - There is no need to specify new unified TCI state switching delay requirements unless the TCI state switch is only defined for known TCI state, and TCI state in active TCI state list.

  - Option 5: (ZTE, MTK (If RAN1 has further agreement))

    - UE is assumed to store the QCL properties of the SSB associated with new beam, the legacy TCI state switching components of TL1-RSRP and Tfirst-SSB+TSSB-proc are ignored.

- Recommended WF

[Background]

Moderator’s observation: all analysis for potential impacts is coming from RAN1#119 meeting as below,

There is no further agreement in RAN1#120 and RAN1#120bis meeting. Even these Alts disappeared in the discussion of RAN1#121 meeting.

Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception. - Other alternatives are not precluded.

- Recommended WF

  - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management if no further RAN1 agreement on reduce beam application latency

Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

- Proposals

  - Option 1: (Nokia, Ericsson, MTK)

    - No need to capture this scenario in RAN4 requirements

  - Option 2: (Xiaomi, Samsung)

    - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

  - Option 3: (Apple)

    - Add the applicability rule of event triggered L1 measurement:

      - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

      - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

- Recommended WF

  - TBA

Issue 1-7: Requirements for event-1 and event-7?

- Proposals

  - Option 1: (Nokia)

    - For event-1 and even-7, reuse the same design as the requirements defined Event-2

  - Option 2: (Samsung)

    - The principle of Event-2 can be used for event-1 and event-7 easily. For event-7, maximum L1-RSRP measurement period are accounted of all active TCI states.

  - Option 3: (Apple)

    - RAN4 prioritize requirements for Event 2 and discuss Event 1/7 requirements later.

  - Option 4: (ZTE)

    - For Event-1, the requirements could be derived from Event-2 with only current beam been involved in.

    - For Event-7, the requirements could be derived from Event-2 with more candidate beams been involved in.

- Recommended WF

  - Agree option 1 in high level

  - For details requirements in CR

    - Company in option 2 think if maximum is used in Issue 1-2 is agreed, Event-1/event-7 can reuse the principle

    - Company in option 4 think if option 2 in Issue 1-2 is agreed, Event-1/event-7 can reuse the principle like more candidate beams been involed

Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

Agreement in last meeting Chair’s report: Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities. - Option 1: - For event trigger L1 reporting, do not define such capability in parallel per category like L3. - Option 2: - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3.

RAN1 agreement: Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.

- Proposals

  - Option 1: (Samsung, Apple, Huawei, Ericsson)

    - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

  - Option 2: (Nokia, MTK)

    - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4.

    - Option 2a: (MTK)

      - For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

  - Option 3: (Qualcomm)

    - First the definition of the core requirements and UE capabilities should be finalized and then RAN1/RAN4 working groups can investigate potential limitations of the number of event triggering and reporting criteria.

  - Option 4: (CMCC)

    - If it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is proposed to define a separate capability for L1 measurement, not prefer to mix L1 measurement reporting capability with legacy L3 measurement reporting capability.

- Recommended WF

  - According to guidance from Chair in last meeting, make decision from option 1(No) and option 2 (Yes)

Issue 1-9: Compatibility of capabilities with LTM

- Proposals

  - Option 1: (Nokia)

    - RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

  - Option 2: (Samsung)

    - If Option 1 in Issue 1-8 is agreed, no need to consider such capability compatibility with LTM

  - Option 3: (Apple)

    - Discuss later

  - Option 4: (CMCC)

    - If it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO.

- Recommended WF

  - Firstly, check whether this capability is needed or not in Issue 1-8

  - Confirm with LTM WI whether to have common capability or not

Sub-topic 1-2: Draft CRs

| T-doc number | Company |  |
| --- | --- | --- |
| R4-2505815 | Samsung | Draft CR on event trigger Reporting based on L1-RSRP measurements for serving cell |
| R4-2505909 | Apple | Draft CR on Event triggered L1 reporting - SSB from different PCI |
| R4-2505773 | Xiaomi | Draft CR on TCI state activation for UEIBM |

Topic #2: RRM performance requirements for Rel-19 MIMO

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505352 | Qualcomm Technologies Ireland | Proposal 1: RAN4 should consider these proposals for the definition of the performance tests. - UE-initiated/event-triggered beam management - Event-2 and event-7 measurement reporting delay: - Current and new beam in list of active beams - Current beam is active, new beam is not active - Event-1 measurement reporting delay - Beam switching delay for known and unknown TCI state for all three events - Event-triggering for intra-cell beam management for SSB and CSI-RS - Event-triggering for inter-cell beam management for SSB only - UE reporting enhancements for CJT deployments - Focus should be TDD 3.5 GHz, SCS = 30 kHz - The test should define absolute phase, frequency and delay offset accuracy requirements subject to the number of samples - For example, 19 ns for delay offset and 3 Hz for frequency offset reporting - The SNR and the number of samples should be defined accordingly to achieve the absolute accuracy requirement. - Asymmetric DL sTRP / UL mTRP - Focus is on FR1 and FR2 scenarios - For FR2, tests should only cover scenario #2 (UL TRP transmits SSB) - For FR1, both scenario #1 (UL TRP does not transmit SSB) and scenario #2 (UL TRP transmits SSB) should be tested - Uplink TCI switching both for Rel-17 and Rel-18 unified TCI state. - Separate UL TCI separate UL TCI - Joint TCI separate UL TCI and separate UL TCI separate UL TCI |
| R4-2505730 | Nokia | Proposal 1: RAN4 to consider the following test case list for UEIBM: a. Reporting based on single measurement: i. Intra-cell SSB-based event-triggered reporting delay ii. Inter-cell SSB-based event-triggered reporting delay iii. Intra-cell CSI-RS based event-triggered reporting delay b. FFS: Inter-cell CSI-RS based event-triggered reporting delay c. FFS: Window-based reporting Proposal 2: UE to be tested with reporting Mode A, if the UE only supports that. FFS whether to test the UE also with Mode B if the UE supports both modes. Proposal 3: If Mode B tests are to be included, introduce reporting Mode A and Mode B as optional test configurations under each UEIBM test. No need to define separate test cases for Mode A and Mode B. Proposal 4: Include UL timing accuracy test cases with 2 TAs for UL-only TRP with 2 TAs for a. Scenario#1, UL-TRP does not transmit RSs. b. Scenario#2, UL-TRP transmits RSs. Proposal 5: RAN4 to define test cases for measurement delay of CJT reporting. |
| R4-2505772 | Xiaomi | Observation 1: In legacy RAN4, there will be performance test to make sure that UE correctly detect the channel variation and report within required delay. Precision = 90% is used. Observation 2: RAN1 introduce a new reporting quantity: the Condition Met Indicator (CMI). Proposal 1: RAN4 to define requirement to verify the performance of condition met indicator(CMI) in the report. Observation 3: With the introduction of CMI, it is necessary to reconsider how the correct rate for event-triggered L1 reporting is defined. Proposal 2: RAN4 to discuss how to define performance metric for event triggered reporting, whether to include CMI together or independently. |
| R4-2505817 | Samsung | Observation 1: The performance requirements focus on other 3 objectives including UEIBM, CJTC reporting, enhancement for asymmetric DL sTRP/UL mTRP scenarios. Observation 2: The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements. Proposal 1: RAN4 to add new test cases to cover: - sTRP serving cell SSB based L1-RSRP - sTRP serving cell CSI-RS based L1-RSRP - sTRP additional PCI cell SSB based L1-RSRP Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1. No TCs are added for event-7. TCs are defined to cover both Mode A and Mode B. FFS on whether to define TC if eventInstanceCount-r19 > 1. We support to define such TC no matter there is core requirements or not. Proposal 2: RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P. Proposal 3: RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F. Proposal 4: RAN4 to add TCs to test measurement period and accuracy in the same test. The TCs covers CJTC-Dd and CJTC-F. Proposal 5: RAN4 to add TCs for two TA core requirements verification. No need to add TCs for TCI state switching. |
| R4-2505911 | Apple | Proposal 1: Introduce test case to verify UE initiated beam reporting delay in FR2. Proposal 2: RAN4 to define mapping tables for time offset, frequency offset, phase offset reporting based on the respective of range and resolution for CJT calibration reporting. Observation 1: Based on the results collected for FO/ TO accuracy the results are very diverse. Proposal 3: RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part. Proposal 4: RAN4 to discuss defining test cases for timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP for scenario#1 and scenario#2. |
| R4-2506588 | CMCC | Proposal 1: For accuracy requirements for enhancement for UE-initiated/event-driven beam management, it is proposed to follow previous agreements that the existing accuracy requirements for L1-RSRP measurements are applied. Proposal 2: On UE-initiated/event-driven beam reporting, it is proposed to define test cases for SSB based L1 measurement period and CSI-RS based measurement period. Proposal 3: it is proposed to define test cases to cover Event-2, Event-1 and Event-7. Proposal 4: if it is agreed to specify requirements to cover both no time window based and with time window based, it is proposed to define test cases for the two cases. Proposal 5: it is proposed to define test cases for L1 measurement reporting delay, since Tfirst UL channel is newly introduced as part of L1 measurement reporting delay. Proposal 6: for CJT calibration, it is proposed to define test cases for delay and frequency offset reporting for - measurement period - scheduling restriction (if core parts impact are identified) Proposal 7: for UE reporting enhancement for CJT calibration, it is proposed to define report mapping for delay offset reporting, frequency offset reporting and phase offset reporting. Proposal 8: for asymmetric DL sTRP/UL mTRP scenarios, it is proposed to define test cases for two TAs enhancements for scenario #1 and scenario #2. |
| R4-2506801 | Huawei, HiSilicon | Proposal 1: For UEIBR, define TC as follows: - SSB-based UEIBR for Event-2. - CSI-RS based UEIBR for Event-2. Note 1: At least define TC when timeRestrictionForChannelMeasurement is configured. Note 2: At least define TC for intra-cell. Proposal 2: RAN4 to discuss whether to define TC for inter-cell UEIBR. Proposal 3: RAN4 to discuss whether to define TC for other Events (i.e. Event 1 and Event 7) Proposal 4: Wait to finalise the core part requirement, including sample size and requirement definition. Proposal 5: For updated 2AT requirements for Scenario #1, the legacy timing accuracy test case can be reused without any new updates. |
| R4-2507014 | Ericsson | Proposal 1: For defining measurement accuracy for delay and frequency offset, we suggest a 50° total phase error budget for CJT coherent combining, with 30° allocated to UE measurement error and the remainder to other contributing factors. Proposal 2: Develop accuracy requirements for delay, frequency and phase offset UE measurement reports for: - Different number of samples - High and low SNR - Normal and extreme conditions Proposal 3: Develop test cases for delay, frequency and phase offset UE measurement reports to test core and accuracy requirement. Proposal 4: RAN4 to define the test cases for Event-2 and Event-7 Proposal 5: RAN4 to consider at least Mode-A in the test case design. Proposal 6: Define test cases with time window-based and non-time window-based configurations. Proposal 7: For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state). Proposal 8: For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state). Proposal 9: RAN4 to specify the periodic first PUCCH resource in the test configuration (at least the periodicityAndOffset). Proposal 10: No MAC CE-based TCI state switching test case is needed with event-driven reporting. Proposal 11: For DCI-based TCI state switch test cases, wait for further RAN1 progress. |

Open issues summary

Sub-topic 2-1: Performance requirements for UE-initiated/event-driven beam management

Issue 2-1-1: Measurement accuracy

- Proposals

  - Option 1: (CMCC)

    - Confirm previous agreement in RAN4#114

- Agreement - The existing accuracy requirements for L1-RSRP measurements can be applied. Not to define new accuracy requirements.

- Recommended WF

  - Confirm previous agreement in RAN4#114. No further discuss is needed.

Issue 2-1-2: Test scope

- Proposals

  - Option 1: (Qualcomm)

    - Event-2 and event-7 measurement reporting delay:

      - Current and new beam in list of active beams

      - Current beam is active, new beam is not active

    - Event-1 measurement reporting delay

    - Beam switching delay for known and unknown TCI state for all three events

    - Event-triggering for intra-cell beam management for SSB and CSI-RS

    - Event-triggering for inter-cell beam management for SSB only

  - Option 2: (Nokia)

    - Reporting based on single measurement

    - Intra-cell SSB-based event-triggered reporting delay

    - Inter-cell SSB-based event-triggered reporting delay

    - Intra-cell CSI-RS based event-triggered reporting delay

    - FFS: Inter-cell CSI-RS based event-triggered reporting delay

    - Tests based on mode A. FFS whether to test the UE also with Mode B if the UE supports both modes. No need to define separate test cases for Mode A and Mode B

  - Option 3: (Samsung)

    - sTRP serving cell SSB based L1-RSRP

    - sTRP serving cell CSI-RS based L1-RSRP

    - sTRP additional PCI cell SSB based L1-RSRP

    - Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1. No TCs are added for event-7.

    - TCs are defined to cover both Mode A and Mode B.

    - FFS on whether to define TC if eventInstanceCount-r19 > 1. We support to define such TC no matter there is core requirements or not.

  - Option 4: (Apple)

    - Introduce test case to verify UE initiated beam reporting delay in FR2

  - Option 5: (CMCC)

    - it is proposed to define test cases for SSB based L1 measurement period and CSI-RS based measurement period.

    - define test cases to cover Event-2, Event-1 and Event-7.

    - cover both no time window based and with time window based, it is proposed to define test cases for the two cases.

    - it is proposed to define test cases for L1 measurement reporting delay, since Tfirst UL channel  is newly introduced as part of L1 measurement reporting delay.

  - Option 6: (Huawei)

    - Define TC as follows:

      - SSB-based UEIBR for Event-2.

      - CSI-RS based UEIBR for Event-2.

    - RAN4 to discuss whether to define TC for inter-cell UEIBR.

    - RAN4 to discuss whether to define TC for other Events (i.e. Event 1 and Event 7)

  - Option 7: (Ericsson)

    - RAN4 to define the test cases for Event-2 and Event-7

    - RAN4 to consider at least Mode-A in the test case design.

    - Define test cases with time window-based and non-time window-based configurations.

    - For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state).

    - For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state).

    - RAN4 to specify the periodic first PUCCH resource in the test configuration (at least the periodicityAndOffset)

    - No MAC CE-based TCI state switching test case is needed with event-driven reporting.

    - For DCI-based TCI state switch test cases, wait for further RAN1 progress.

- Recommended WF

  - Further discuss based on the table below:

|  | QC | Nokia | Samsung | Apple | CMCC | Huawei | Ericsson |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Y |  | Event-2 1 TC for Event-2 No for Event-7 |  | Y | Event-2 FFS on others | Event-2&7 |
| Whether to cover Mode A and Mode B? |  | Mode A FFS on Mode B | Y |  |  |  | Mode A at least |
| Event-trigger for Intra-cell BM for SSB | Y | Y | Y |  |  | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y |  |  | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y |  |  | FFS | ? |
| Event-trigger for Inter-cell BM for CSI-RS |  | FFS |  |  |  |  |  |
| Whether to cover no time window and eventInstanceCount-r19 > 1? |  |  | Y |  | Y |  | Y |
| FR2? |  |  |  | Y |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI |  |  |  |  |  | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |

  - Further discuss on test configuration

    - Reporting based on single measurement

    - Specify the periodic first PUCCH resource in the test configuration (at least the periodicityAndOffset)

Issue 2-1-3: Whether to add requirement to verify the performance of CMI?

- Proposals

  - Option 1: (Xiaomi)

    - RAN4 to define requirement to verify the performance of condition met indicator(CMI) in the report.

    - RAN4 to discuss how to define performance metric for event triggered reporting

- Recommended WF

  - TBA

Sub-topic 2-2: Performance requirements for CJTC reporting

Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

- Proposals

  - Option 1: (Samsung, Apple, CMCC)

    - RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

- Recommended WF

  - Agree option 1

Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

- Proposals

  - Option 1: (Samsung)

    - RAN4 to add accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F.

  - Option 2: (Apple)

    - RAN4 to discuss feasibility of defining accuracy requirements for time offset, frequency offset reporting based on number of samples agreed in core part.

  - Option 3: (Ericsson)

    - Develop accuracy requirements for delay, frequency and phase offset UE measurement reports for:

      - Different number of samples

      - High and low SNR

      - Normal and extreme conditions

- Recommended WF

  - TBA

Issue 2-2-3: Test scope

- Proposals

  - Option 1: (Qualcomm, Ericsson)

    - Absolute phase, frequency and delay offset accuracy requirements

  - Option 2: (Nokia)

    - Add TC for measurement delay

  - Option 3: (Samsung)

    - RAN4 to add TCs to test measurement period and accuracy in the same test. The TCs covers CJTC-Dd and CJTC-F.

  - Option 4: (CMCC)

    - Define test cases for delay and frequency offset reporting for

      - measurement period

      - scheduling restriction (if core parts impact are identified)

  - Option 5: (Huawei)

    - FFS, wait to finalize core part

- Recommended WF

  - Further discuss based on the table below:

| High level: | QC | Nokia | Samsung | CMCC | Huawei | Ericsson |
| --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify CJTC-Dd, measurement period | Y | Y | Y | Y | FFS | Y |
| Whether to define TCs to verify CJTC-f，measurement period | Y | Y | Y | Y | FFS | Y |
| Whether to define TCs to verify CJTC-P， (To be verify?) | Y |  |  |  | FFS | Y |
| Whether to define TCs to verify scheduling restriction |  |  |  | Y | FFS |  |

Sub-topic 2-3: Performance requirements for enhancement for Asymmetric DL sTRP / UL mTRP

Issue 2-3-1: Test scope

- Proposals

  - Option 1: (Qualcomm)

    - Test cases cover FR1 and FR2.

    - For FR2, only cover scenario#2

    - For FR1, cover both scenario#1

    - Uplink TCI switching both for Rel-17 and Rel-18 unified TCI state.

      - Separate UL TCI  separate UL TCI

      - Joint TCI  separate UL TCI and separate UL TCI  separate UL TCI

  - Option 2: (Nokia, Apple)

    - Include UL timing accuracy test cases with 2 TAs for UL-only TRP with 2 TAs for

      - Scenario#1, UL-TRP does not transmit RSs.

      - Scenario#2, UL-TRP transmits RSs.

  - Option 3: (Samsung)

    - RAN4 to add TCs for two TA core requirements verification. No need to add TCs for TCI state switching.

  - Option 4: (CMCC)

    - Define test cases for two TAs enhancements for scenario #1 and scenario #2.

  - Option 5: (Huawei)

    - For updated 2AT requirements for Scenario #1, the legacy timing accuracy test case can be reused without any new updates.

- Recommended WF

  - Further discuss based on the table below:

| High level: | QC | Nokia | Apple | Samsung | CMCC | Huawei |
| --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify timing requirements? |  | Y | Y | Y |  | Y |
| Whether to define TCs to verify TCI state switching requirements? | Y |  |  | N |  |  |
| FR1 and/or FR2? | both |  |  |  |  |  |
| Whether to cover Scenario#1 and Scenario#2? | both | both | both |  | both | Scenarios#1 |