| 3GPP TR 37.862 V0.2.0 (2025-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Rel-19 adding channel bandwidth support to existing NR bands (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	Request management	7

4.2	TR maintenance	7

5	Channel bandwidth(s) added to existing NR bands	8

5.1	Band n48 – 50, 60, 70, 80, 90 and 100 MHz channel BWs.	8

5.1.1	System parameters	8

5.1.2	UE A-MPR	8

5.1.3	UE REFSENS	23

5.1.4	Other	23

5.2	Band n80 – 35, 45, 50 MHz channel BWs	23

5.2.1	System parameters	23

5.2.2	UE A-MPR	23

5.2.3	UE REFSENS	23

5.2.4	Other	23

5.3	Band n84 – 45 MHz channel BW	23

5.3.1	System parameters	23

5.3.2	UE A-MPR	23

5.3.3	UE REFSENS	23

5.3.4	Other	23

Annex A (informative): Change history	25

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report on adding channel bandwidth in existing NR bands under Rel-19 time frame. The purpose is to gather the relevant background information and studies in order to address the addition of channel bandwidth support in existing NR bands as listed in Table 1-1.

Table 1-1: Added channel bandwidth(s) in NR bands during Rel-19 time frame

| Band | Channel bandwidth and SCS |
| --- | --- |
| n5 | 3 MHz (15 kHz SCS) |
| n12 | 3 MHz (15 kHz SCS) |
| n48 | 50 MHz (15, 30 and 60kHz SCS) 60, 70, 80, 90, 100 MHz (30 and 60 kHz SCS) |
| n80 | 35, 45, 50 MHz (15, 30 and 60kHz SCS) |
| n84 | 45 MHz (15, 30 and 60kHz SCS) |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-241684, New WID: Basket WID on adding channel bandwidth support to existing NR bands

[3]		R4-2415525, “Simulation results for NS_27,” Apple, RAN4 #112-bis, Hefei, China, Oct. 2024.

[4]		R4-2417746, “n48: Adding channels of 50MHz and beyond,” Apple, RAN4 #113, Orlando, USA, Nov. 2024.

[5]		R4-2500306, “n48: Adding channels of 50MHz and beyond,” Apple, RAN4 #114, Athens, Greece, Feb. 2025.

[6]		R4-2503563, “On adding n48 NS_27 A-MPR for CBW 50MHz and higher,” Qualcomm, RAN4 #114-bis, Wuhan, China, April 2025.

[7]		R4-2506167, “n48 AMPR for wider BWs,” Qualcomm, RAN4 #115, St. Julians, Malta, May 2025.

[8]		R4-2416214, “n48 NS_27 A-MPR simulations beyond 40 MHz CBW,” Nokia, RAN4 #112-bis, Hefei, China, Oct. 2024.

[9]		R4-2507706, “Beyond 40MHz CBW measurements for Band n48 NS_27,” Skyworks Solutions, Inc., RAN4 #115, St. Julians, Malta, May 2025.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A-MPR	Additional Maximum Power Reduction

BS	Base Station

BW	Bandwidth

GSCN	Global Synchronization Channel Number

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

REFSENS	Reference Sensitivity

# 4	Background

## 4.1	Request management

The procedure to handle new request adding channel bandwidth(s) in existing NR bands is described in the WI ([2]).

## 4.2	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	Channel bandwidth(s) added to existing NR bands

## 5.1	Band n48 – 50, 60, 70, 80, 90 and 100 MHz channel BWs.

### 5.1.1	System parameters

The 50 MHz (15, 30 and 60kHz SCS), 60, 70, 80, 90, and 100 MHz (30 and 60kHz SCS) channel bandwidths are proposed for band n48.

### 5.1.2	UE A-MPR

The RB regions are listed in Table 5.1.2-1. The UE A-MPR from NS_27 are listed in Table 5.1.2-2.

Table 5.1.2-1: RB regions for channel bandwidth

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR |
| 50 MHz | 3575 ≤ FC < 3605 | <12.6 MHz |  |  | A7 |
|  |  | ≥12.6 MHz ≤31.0 MHz |  | ≥22.32 MHz | A2 |
|  |  |  |  | <22.32 MHz | A1 |
|  |  | ≥38.52 MHz |  | ≤3.6 MHz | A7 |
|  | 3645 < FC ≤ 3675 |  | >36.0 MHz |  | A7 |
|  |  |  | ≤36.0 MHz | ≥22.32 MHz | A2 |
|  |  |  | ≥17.6 MHz | <22.32 MHz | A1 |
|  |  |  | ≤10.08 MHz | ≤3.6 MHz | A7 |
|  | 3605 ≤ FC ≤ 3645 | ≤6.12 MHz |  | ≤3.6 MHz | A8 |
| 60 MHz | 3580 ≤ FC < 3620 | <15.84 MHz |  |  | A7 |
|  |  | ≥15.84 MHz ≤31.0 MHz |  | ≥27.36 MHz | A2 |
|  |  |  |  | <27.36 MHz | A1 |
|  |  | ≥44.64 MHz |  | ≤3.6 MHz | A7 |
|  | 3630 < FC ≤ 3670 |  | >42.48 MHz |  | A7 |
|  |  |  | ≤42.48 MHz | ≥27.36 MHz | A2 |
|  |  |  | ≥27.32 MHz | <27.36 MHz | A1 |
|  |  |  | ≤13.68 MHz | ≤3.6 MHz | A7 |
|  | 3620 ≤ FC ≤ 3630 | ≤6.12 MHz |  | ≤3.6 MHz | A8 |
|  |  |  | ≥52.2 MHz | ≤3.6 MHz | A8 |
| 70 MHz | 3585 ≤ FC ≤ 3665 | ≤22.32 MHz |  | <25.56 MHz | A7 |
|  |  |  | ≥ 45.72 MHz | <25.56 MHz | A7 |
|  |  |  |  | ≥25.56 MHz | A7 |
| 80 MHz | 3590 ≤ FC ≤ 3660 | ≤22.32 MHz |  | <25.56 MHz | A7 |
|  |  |  | ≥55.8 MHz | <25.56 MHz | A7 |
|  |  |  |  | ≥25.56 MHz | A7 |
| 90 MHz | 3595 ≤ FC ≤ 3655 | ≤25.92 MHz |  | <28.8 MHz | A7 |
|  |  |  | ≥62.28 MHz | <28.8 MHz | A7 |
|  |  |  |  | ≥28.8 MHz | A7 |
| 100 MHz | 3600 ≤ FC ≤ 3650 | ≤29.16 MHz |  | <32.4 MHz | A7 |
|  |  |  | ≥69.12 MHz | <32.4 MHz | A7 |
|  |  |  |  | ≥32.4 MHz | A7 |

Table 5.1.2-2: A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | QPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
|  | 16 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 64 QAM | 4.5 | 6 | 5 | 4 | 5 | 4 | 11 | 4 |
|  | 256 QAM |  | 6 |  |  |  |  | 11 |  |
| CP-OFDM | QPSK | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 16 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 64 QAM | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
|  | 256 QAM |  | 7 |  |  |  |  | 11.5 |  |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |  |

Apple [3]-[5], Qualcomm [6]-[7], and Nokia [8] simulated the A-MPR. Skyworks measured power class 3 (PC3) power amplifier (PA) to evaluate the Band n48 50 MHz and 100 MHz CBW [9].

The following assumptions and requirements for the simulations are applied in the Apple simulation [3]:

- Power Class 3;

- NR Waveform;

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20 MHz, 100RB0;

- APT model for improved accuracy at high back-off;

- Carrier Leakage: 28dBc;

- Image: 28dBc;

- CIM3: 60dBc;

- CIM5: 70dBc;

- Requirements from NS_27.

The Apple results are listed below:

- The Apple results for channel bandwidth 50 MHz are shown in Figures 1-3 [3].

Figure 1: 50 MHz CBW at lower band edge with QPSK modulation

Figure 2: 50 MHz CBW at 3605 MHz with QPSK modulation

Figure 3: 50 MHz CBW at 3625 MHz with QPSK modulation

- The Apple results for channel bandwidth 60 MHz are shown in Figures 4-5 [3].

Figure 4: 60 MHz CBW at lower band edge with QPSK modulation

Figure 5: 60 MHz CBW at 3620 MHz with QPSK modulation

- The Apple results for channel bandwidth 70 MHz are shown in Figures 6-7 [4].

Figure 6: 70 MHz CBW at lower band edge with QPSK modulation

Figure 7: 70 MHz CBW at 3625 MHz with QPSK modulation

- The Apple results for channel bandwidth 80 MHz are shown in Figures 8-9 [4].

Figure 8: 80 MHz CBW at lower band edge with QPSK modulation

Figure 9: 80 MHz CBW at 3620 MHz with QPSK modulation

- The Apple results for channel bandwidth 90 MHz are shown in Figures 10-11 [5].

Figure 10: 90 MHz CBW at lower band edge with QPSK modulation

Figure 11: 90 MHz CBW at 3625 MHz with QPSK modulation

- The Apple results for channel bandwidth 100 MHz are shown in Figures 12-13 [5].

Figure 12: 100 MHz CBW at lower band edge with QPSK modulation

Figure 13: 100 MHz CBW at 3620 MHz with QPSK modulation

- The Apple proposed A-MPR RB regions presented in Table 3 [5].

Table 3: Apple simulated A-MPR RB regions

| CBW | CBW at lower band edge with QPSK modulation | CBW shifted inwards |
| --- | --- | --- |
| 40MHz |  |  |
| 50MHz |  |  |
| 60MHz |  |  |
| 70MHz |  | N/A |
| 80MHz |  | N/A |
| 90MHz |  | N/A |
| 100MHz |  | N/A |

The Qualcomm results [6]-[7] are listed below:

- The Qualcomm results for channel bandwidth 50 MHz are shown in Figures 14-17.

Figure 14. NS_27 A-MPR results for channel BW 50 MHz at Fc = 3575 MHz

Figure 15. NS_27 A-MPR results for channel BW 50 MHz at Fc = 3605 MHz

Figure 16. NS_27 A-MPR results for channel BW 50 MHz at Fc = 3645 MHz

Figure 17. NS_27 A-MPR results for channel BW 50MHz at Fc = 3675 MHz

- The Qualcomm results for channel bandwidth 60 MHz are shown in Figures 18-21.

Figure 18. NS_27 A-MPR results for channel BW 60 MHz at Fc = 3580 MHz

Figure 19. NS_27 A-MPR results for channel BW 60 MHz at Fc = 3620 MHz

Figure 20. NS_27 A-MPR results for channel BW 60 MHz at Fc = 3630 MHz

Figure 21. NS_27 A-MPR results for channel BW 60MHz at Fc = 3670 MHz

- The Qualcomm results for channel bandwidth 70 MHz are shown in Figures 22-23.

Figure 22. NS_27 A-MPR results for channel BW 70 MHz at Fc = 3585 MHz

Figure 23. NS_27 A-MPR results for channel BW 70 MHz at Fc = 3665 MHz

- The Qualcomm results for channel bandwidth 80 MHz are shown in Figures 24-25.

Figure 24. NS_27 A-MPR results for channel BW 80 MHz at Fc = 3590 MHz

Figure 25. NS_27 A-MPR results for channel BW 80 MHz at Fc = 3660 MHz

- The Qualcomm results for channel bandwidth 90 MHz are shown in Figure 26.

Figure 26. NS_27 A-MPR results for channel BW 90 MHz at Fc = 3595 MHz

- The Qualcomm results for channel bandwidth 100 MHz are shown in Figure 27.

Figure 27. NS_27 A-MPR results for channel BW 100 MHz at Fc = 3600 MHz and Fc = 3650 MHz

The following assumptions and requirements for the simulations are applied in the Nokia simulation [8]:

- Operating band n48 (UL 3550 MHz – 3700 MHz);

- Channel bandwidths 50, 60, 70, 80, 90, 100 MHz;

- Subcarrier spacing: 15 kHz for 50 MHz, 30 kHz for channel bandwidths ≥ 50 MHz;

- Power class PC3;

- WOLA processing;

- I/Q image -28 dBc;

- Carrier leakage -28 dBc;

- CIM3 -60 dBc, CIM5 -70 dBc;

- NS_27 SEM from TS 38.101-1.

Some selected Nokia results are listed in Figures 28-30 [8], more detailed simulation results are presented in [8].

Figure 28: Simulated back-off for QPSK in a 70 MHz channel at the center of n48 UL

Figure 29: Simulated back-off for 64-QAM in a 100 MHz channel at the lower edge of n48 UL

Figure 30: Simulated back-off for 64-QAM in a 100 MHz channel at the center of n48 UL; emissions reach the additional spurious mask on both sides of the band

Skyworks’ measurements for 50 MHz and 100 MHz A-MPR region were presented in [9]. For PA transmitter noise emission measurements in the receiver band, the following parameters are assumed:

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, and 100 RB at lower channel edge with 1 dB maximum power reduction (MPR);

- PC3 operation and 4 dB post-PA loss;

- Local Oscillator (LO) leakage: –28 dBc; IQ image rejection: –28 dB; and

- C-IM3: –60 dBc, C-IM5: –70 dBc.

Table 4 summarizes the measured raw output back-off (OBO) required to meet the NS_27 protection requirements for QPSK waveforms and 50 MHz CBW SCS 15 kHz, 100 MHz CBW SCS 30 kHz.

Table 4: Measured raw OBO vs proposed A-MPR for 50 MHz and 100 MHz CBW.

| QPSK | Fc (MHz) | A-MPR Region | A-MPR (dB) | Raw OBO (dB) | OBO margin (dB) |
| --- | --- | --- | --- | --- | --- |
| DFT_50M_15kHz_1(RBstart=0) | 3575 | A7 | 10.5 | 9.8 | 0.7 |
| DFT_50M_15kHz_1(RBstart=50) | 3575 | A7 | 10.5 | 9.6 | 0.9 |
| DFT_50M_15kHz_1(RBstart=70) | 3575 | A1 | 4.5 | 1.8 | 2.7 |
| DFT_50M_15kHz_1(RBstart=84) | 3575 | A1 | 4.5 | 0 | 4.5 |
| DFT_50M_15kHz_1(RBstart=219) | 3575 | A7 | 10.5 | 0 | 10.5 |
| DFT_50M_15kHz_1(RBstart=269) | 3575 | A7 | 10.5 | 0 | 10.5 |
| DFT_50M_15kHz_200(RBstart=70) | 3575 | A2 | 6 | 5.3 | 0.7 |
| DFT_50M_15kHz_225(RBstart=45) | 3575 | A7 | 10.5 | 8.4 | 2.1 |
| DFT_50M_15kHz_270(RBstart=0) | 3575 | A7 | 10.5 | 9.2 | 1.3 |
| CP_50M_15kHz_1(RBstart=0) | 3575 | A7 | 11.5 | 11.3 | 0.2 |
| CP_50M_15kHz_1(RBstart=50) | 3575 | A7 | 11.5 |  |  |
| CP_50M_15kHz_1(RBstart=70) | 3575 | A1 | 5.5 |  |  |
| CP_50M_15kHz_1(RBstart=84) | 3575 | A1 | 5.5 |  |  |
| CP_50M_15kHz_1(RBstart=219) | 3575 | A7 | 11.5 |  |  |
| CP_50M_15kHz_1(RBstart=269) | 3575 | A7 | 11.5 |  |  |
| CP_50M_15kHz_200(RBstart=70) | 3575 | A2 | 7 | 6.9 | 0.1 |
| CP_50M_15kHz_220(RBstart=50) | 3575 | A7 | 11.5 |  |  |
| CP_50M_15kHz_270(RBstart=0) | 3575 | A7 | 11.5 | 10.9 | 0.6 |
| DFT_100M_30kHz_1(RBstart=0) | 3575 | A7 | 10.5 | 10.1 | 0.4 |
| DFT_100M_30kHz_270(RBstart=0) | 3575 | A7 | 10.5 | 9.9 | 0.6 |

We highlight in red the configurations for which the OBO margin to the proposed A-MPR requirements is small (<1dB) or very small (<0.5dB). Critical regions are A7 and A2 for some corner RB allocations, in particular small RB allocations at 100 MHz CBW. Even though the concerns on Region A2 may be addressed by changing the contours of the border between region A7 and region A2, we do not propose any change, to these regions nor to the A7 A-MPR requirements. It is observed that measurements indicate small margins for: 1) Region A7 for fully allocated and for small RB allocations, e.g., Lcrb=1(RBstart=0) and 2) corner RB allocation of region A2.

### 5.1.3	UE REFSENS

No need to specify UE REFSENS because the new channel BWs in band n48 are all for UL.

### 5.1.4	Other

None

## 5.2	Band n80 – 35, 45, 50 MHz channel BWs

### 5.2.1	System parameters

The 35, 45 and 50 MHz (15, 30 and 60kHz SCS) channel bandwidths are proposed for SUL band n80.

### 5.2.2	UE A-MPR

There is no bandwidth-specific AMPR requirement for SUL band n80.

### 5.2.3	UE REFSENS

No need to specify UE REFSENS for single SUL band.

### 5.2.4	Other

None

## 5.3	Band n84 – 45 MHz channel BW

### 5.3.1	System parameters

The 45 MHz (15, 30 and 60kHz SCS) channel bandwidth is proposed for SUL band n84.

### 5.3.2	UE A-MPR

The bandwidth-specific AMPR requirements for band n1 can be reused for SUL band n84.

### 5.3.3	UE REFSENS

No need to specify UE REFSENS for single SUL band.

### 5.3.4	Other

None

# 5.4		Band n5 – 3 MHz channel BW.

### 5.4.1	System parameters

The 3 MHz (15 kHz SCS) channel bandwidth is proposed for FDD band n5.

### 5.4.2	UE A-MPR

There is no bandwidth-specific AMPR requirements for FDD band n5.

### 5.4.3	UE REFSENS

The UE REFSENS for 3 MHz channel BW in FDD band n5 is -100.2 dBm, which is 2.2 dB lower than that for 5 MHz channel BW.

### 5.4.4	Other

None

# 5.5		Band n12 – 3 MHz channel BW.

### 5.5.1	System parameters

The 3 MHz (15 kHz SCS) channel bandwidth is proposed for FDD band n12.

### 5.5.2	UE A-MPR

The bandwidth-specific AMPR requirements for band n12 can be reused for 3 MHz channel BW.

### 5.5.3	UE REFSENS

The UE REFSENS for 3 MHz channel BW FDD band n12 is -99.2 dBm, which is 2.2 dB lower than that for 5 MHz channel BW.

### 5.5.4	Other

None

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-07 | RAN4#112 | R4-2411198 |  |  |  | Initial TR skeleton | 0.0.1 |
| 2025-02 | RAN4#114 | R4-2501372 |  |  |  | Addition of: - 35, 45 and 50Mhz channel BWs for band n80 - 45 MHz channel BW for band n84 | 0.1.0 |
| 2025-05 | RAN4#115 | R4-2506157 |  |  |  | Addition of: - 3MHz channel BW for bands n5 and n12 - 50, 60, 70, 80, 90, 100 MHz channel BWs for band n48 | 0.2.0 |