| 3GPP TR 38.774 V0.4.0 (2025-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Radio Frequency (RF) requirements for Low-Power Wake-up Signal and Receiver (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background	7

5	System parameters	7

5.1	General	7

5.2	Operating bands	8

5.3	Channel raster	8

6	Simulation assumptions and evaluation	8

6.1	Performance metric	8

6.2	Simulation assumption	8

6.2.1	General	8

6.2.2	Simulation parameters	8

6.3	Simulation evaluation results	9

7	RF requirements	12

7.1	UE RF	12

7.1.1	General	12

7.1.2	Rx SNR evaluations	13

7.1.3	Architecture and NF considerations	13

7.1.4	REFSENS requirements	13

7.1.5	ASCS requirements	14

7.1.6	ACS requirements	14

7.1.7	IBB requirements	14

7.1.8	OBB requirements	14

7.1.9	other Rx requirements	14

7.2	BS RF	14

7.2.1	General	14

7.2.2	LP-WUS power boosting	14

7.2.3	Regulation relevant spectrum requirements	15

7.2.4	Transmitted signal quality	15

7.2.4	other RF requirements	15

8	Testability	15

8.1	Testability for UE Performance verification	15

8.1.1	General Framework	15

8.1.2	other	15

Annex A (informative): Change history	17

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a Technical Report for Rel-19 Radio Frequency (RF) requirements for Low-Power Wake-up Signal and Receiver (LP-WUS/WUR). The purpose is to gather the relevant background information and analysis to address RF requirements for LP-WUS/WUR.

This TR contains discussions and agreements for LP-WUS/WUR RF. The actual RF requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.869: "Study on low-power Wake-up Signal and Receiver for NR".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[5]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Main radio (MR): the Tx/Rx module operating for NR signals/channels apart from signals/channel related to low-power wake-up.

LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

ACS	Adjacent Channel Selectivity

ADC	Analog to Digital Converter

ASCS	Adjacent Subcarrier selectivity

BB	Base Band

BLER	Block Error Rate

BPF	Band Pass Filter

BS	Base Station

CFO	Center frequency offset

FAR	False Alarm Rate

FR1	Frequency range 1

FR2	Frequency range 2

ICS	In-channel Selectivity

IF	Intermediate Frequency

LP-WUS	Low Power-Wake Up Signal

LP-WUR	Low Power-Wake Up Receiver

LP-SS	Low Power- Synchronization Signal

LO	Local Oscillator

LNA 	Low Noise Amplifier

LPF 	Low Pass Filter

LR	LP-WUR

MDR	Miss Detection Rate

MR	Main Radio

NF	Noise Figure

OOB	Out-of-band

OOK	On-Off keying

OFDM	Orthogonal Frequency Division Multiplexing

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

SINR	Signal to Interference plus Noise Ratio

SNR	Signal to Noise Ratio

UE	User Equipment

Tx	Transmitter

4	Background

<Editor’s note: general background and considerations, details TBD. >

The present document is a technical report for Rel-19 Radio Frequency (RF) requirements for Low-Power Wake-up Signal and Receiver (LP-WUS/WUR). It covers RF requirements analysis for both UE and BS side. Some initial analysis of RF aspects for LP-WUS/WUR were captured in TR 38.869 [2].

5	System parameters

5.1	General

The channel arrangements presented in this clause for LP-WUS are based on the operating bands and channel raster defined in sub-clause 5.2 and 5.3 below.

5.2	Operating bands

LP-WUS is designed to operate in the operating bands defined in Table 5.2-1 of TS 38.101-1 [3] and TS 38.101-2 [4], excluding bands n46, n47, n96, n102 and SDL bands.

5.3	Channel raster

The PRB grid between LR and MR is aligned and the LP-WUS RBs can be flexibly allocated within the wider NR carrier. How to make sure the RB alignment between LP-WUS RB and MR RB grid is aligned is UE implementation specific.

6	Simulation assumptions and evaluation

6.1	Performance metric

<Editor’s note: performance metric to specify RF requirements. >

For RF requirements and conformance testing, the performance metric is 1% MDR. Meanwhile, the FAR assumption for MDR evaluation is </=1%.

6.2	Simulation assumption

6.2.1	General

To evaluate the RF performance of LP-WUS/WUR, the link level simulation is performed in this WI. This clause captures the simulation assumption for LP-WUS UE RF and BS RF analysis.

6.2.2	Simulation parameters

For RF performance analysis, the following simulation assumptions are considered.

Table 6.2.2-1 General Link level simulation assumption for FR1 LP-WUR

| Attributes | Assumptions |
| --- | --- |
| Waveform | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |
| Channel structure | 5 bits |
| RM coding | 8/16/32bit for OOK |
| CRC | NO |
| Chip rate | M=4 |
| overlaid OFDM sequence | 4 candidates overlaid sequences for M=4 |
| WUS duration | OOK：RM coding length*2/M OFDM：ceil(5/log2(4)/(M/2)) |
| Coding | 1/2 rate Manchester coding |
| Time error | 0/0.9 us |
| Residual Frequency error | 0/10/20 ppm |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |
| Guard RB | For ASCS: 0 or 1RB on each side of LP-WUS bandwidth For ACS: 1/2/3/4 RB |
| Filter | 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS as baseline, different wanted power level can be also considered |
| Sampling rate | 7.68MHz |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |
| Phase noise |  | Reciprocal mixing can be optionally considered for ASCS/ACS. Note that actual implementation choice of phase noise is driven by jammer requirement and power consumption target. |  |
| Non-linearities | Not modelled |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |
| Channel Model | AWGN |
| Performance metric | - 1% MDR - false alarm rate 1% as side condition |
| Note: The detailed Receiver algorithm is UE implementation . |  |

6.3	Simulation evaluation results

<Editor’s note: detailed simulations from each company. FFS detailed structure for each RF requirement>

### 6.3.1	General

To evaluate some RF performance, link level simulations are performed to collect analysis input.

### 6.3.2	SNR simulations

In this sub-clause, SNR simulation results from different companies are collected for analysis of target SNR for LP-WUR.

Table 6.3.2-1 SNR simulation summary for FR1 Envelop-detection LP-WUR

| Companies Input | SNR |  | RM coding |  |  | ADC |  | Filter order |  | Timing error |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observations and Summary of SNR simulation outcome for envelop-detection receiver: TBA

Table 6.3.2-2 SNR simulation summary for FR1 OFDM-based LP-WUR

| Companies Input | SNR | RM coding | ADC | Filter order | Timing error | Note |
| --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |
| B |  |  |  |  |  |  |
| C |  |  |  |  |  |  |

Observations and Summary of SNR simulation outcome for OFDM-based receiver: TBA

### 6.3.3	ASCS simulations

In this sub-clause, ASCS simulation results from different companies are collected for analysis of in-band interference performance of LP-WUS. This is for information only, because actual implementation choice of filtering is driven by jammer requirements.

Table 6.3.3-1 ASCS simulation summary for FR1 Envelop-detection LP-WUR

| Companies Input | SNR degradation | Guard RB | RM coding |  | ADC |  | Filter order |  | Timing error |  | Phase noise |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |  |  |  |  |  |  |

Summary of ASCS simulation outcome: TBA

Table 6.3.3-2 ASCS simulation summary for FR1 OFDM-based LP-WUR

| Companies Input | SNR degradation | Guard RB | RM coding | ADC | Filter order | Timing error | Phase noise | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A |  |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |  |

### 6.3.4	ACS simulations

In this sub-clause, ACS simulation results from different companies are collected for analysis of ACS performance of LP-WUR. It was agreed that the interference level of LP-WUR ACS is the same as MR, for both type1 and type 2 LP-WUR. This is for information purpose only, because ACS requirements are driven by agreed coexistence requirements applied to the LR.

Table 6.3.4-1 ACS simulation summary for FR1 Type-1 LP-WUR

| Companies Input | Detection | ACS |  | Guard RB |  | Interference level |  | Wanted signal level |  | Filter order |  | Timing error |  | Phase noise |  |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Envelop detection |  |  |  |  |  |  |  |  |
|  | OFDM |  |  |  |  |  |  |  |  |
| B | Envelop detection |  |  |  |  |  |  |  |  |
|  | OFDM |  |  |  |  |  |  |  |  |
| C | Envelop detection |  |  |  |  |  |  |  |  |
|  | OFDM |  |  |  |  |  |  |  |  |

Summary of ACS simulation outcome: TBA

### 6.3.5	FR2 simulations

### 6.3.6	Other aspects

7	RF requirements

7.1	UE RF

<Editor’s note: discussions and analysis for UE RF requirements related. Mainly focus on key aspects and conclusions, rather than detailed simulations from each company>

7.1.1	General

The minimum requirements will be determined assuming there is only one receiver. A 1% MDR criterion will be used for verification of all core RF requirements. Following LP-WUS parameters will be used

Table 7.1.1-1. Common reference channel parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 15/30kHz |
| RM coding | Bits | 16 |
| CRC |  | No CRC |
| Chip rate |  | M=4 (4 chips in an OFDM symbol) |
| Overlaid OFDM sequence |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 4 |
| WUS duration for OOK |  | 8 OFDM symbols |
| WUS duration for OFDM |  | 2 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits | Bits | 5 |

In the specification, tables will be defined corresponding to 15 KHz and 30 KHz SCS.

7.1.2	Rx SNR evaluations

To derive SNR performance of LP-WUS, it was agreed to select OOK-4 M=4 under AWGN channel model with 1% MDR without repetition as worst case.

7.1.3	Architecture and NF considerations

The basic architecture considered for the LP_WUR is a zero-IF architecture for both envelope and sequence based detectors. This assumption is common for both FR1 and FR2. This is just an assumption made to derive parameters which dictate the requirements. This assumption does not preclude any other RF implementations.

In case of FR1, only single RX is assumed. For FR2 requirement derivation, the baseline assumption is an OFDM based receiver with two receiver chains with mutually orthogonally polarized antennas.

In RAN4, there had been no consensus on what IM includes, and no agreement was achieved on individual values of NF and IM, so a joint number was agreed. There are two sets of NF + IM values agreed, and these will be used for REFSENS calculations. The values are,

-	Set 1: 18 dB,

-	Set 2: 13.5 dB.

The above mentioned numbers are for FR1.

7.1.4	REFSENS requirements

It was agreed to derive the REFSENS for LP-WUR in the typical RF manner with AWGN being the channel for which SNR is derived. Thus, the REFSENS for LR is defined as

REFSENSLR (dBm) = -174 + 10log10(BWLR)+ (NFLR + IMLR) + SNRLR.

Here, BWLR corresponds to the bandwidth of the LP-WUS signal in hertz. RAN1 has decided to have 11 RBs for LP-WUS irrespective of SCS, thus the LP-WUS bandwidth will be 1.98 MHz and 3.96 MHz, for 15 and 30 KHz SCS, respectively. Further, there are two set of values for NFLR + IMLR, i.e. 18.0dB and 13.5dB for Type 1 and Type UEs, respectively. There is only single receiver so there is no diversity gain included in the REFSENS.

In the REFSENS test, only the RBs allocated for LP-WUS are populated with the LP-WUS and the rest of the RBs for a given channel bandwidth are left empty. This configuration, however, does not correspond to the real-life case where NR signals in other RBs are generally present and thus can create a coverage shortfall. Separately, the ASCS requirement allows a maximum degradation of 0.5 dB and the side conditions incorporate populated adjacent RBs.

Given the small value of the ASCS degradation factor (0.5 dB), it was agreed not to include this factor in the REFSENS.

For FR1, a SNR value of -4.5 dB for both Type 1 and Type 2 UEs was selected by RAN4 to be used for the REFSENS equation.

7.1.5	ASCS requirements

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent in-band NR signal(s). The in-band LP-WUS and NR signal should be same PSD and be separated by a given frequency offset (guard RB) between LP-WUS and NR, and the NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs.

7.1.6	ACS requirements

7.1.7	IBB requirements

7.1.8	OBB requirements

7.1.9	other Rx requirements

7.2	BS RF

<Editor’s note: discussions and analysis for BS RF requirements related. Mainly focus on key aspects and conclusions, rather than detailed simulations from each company.>

7.2.1	General

7.2.2	LP-WUS power boosting

Two approaches to define the LP-WUS power boosting are discussed, e.g., legacy dynamic range similar to NB-IoT power boosting and EPRE ratio. The definitions for these two approaches are:

For legacy dynamic range similar to NB-IoT power boosting, the LP-WUS power boosting is defined as the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

For EPRE ratio, the LP-WUS power boosting is defined as the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over NR REs (the NR carrier excluding the LP-WUS REs).

The above two approaches are mathematically related, and the specific analysis can be found in R4-2407547 and R4-2419482.

RAN4 decides to adopt legacy dynamic range similar to NB-IoT power boosting. The declaration on the support of LP-WUS and support of LP-WUS power boosting should be separate. The minimum power boosting level should be included in the BS manufacturer declaration table, but not defined as a minimum requirement.

7.2.3	Regulation relevant spectrum requirements

For transmitted signal with LP-WUS and NR in the same carrier, existing NR regulation relevant spectrum requirements, e.g. spectrum emission mask, spurious emission, are applied.

7.2.4	Transmitted signal quality

7.2.4	other RF requirements

8	Testability

<Editor’s note: discussions and analysis for testability solutions>

8.1	Testability for UE Performance verification

8.1.1	General Framework

It has been agreed that same LP-WUS RF requirements will apply to all RRC states, but only one RRC state needs to be tested. It is agreed to verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods). The ultimate test method including the potential test mode will be decided by RAN5 based on RAN4 input.

For demodulation requirements FAR will also be used.

8.1.2	other

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-10 | RAN4#112bis | R4-2415778 |  |  |  | draft TR skeleton | 0.0.0 |
| 2024-11 | RAN4#113 | R4-2418215 |  |  |  | TR skeleton Updated structure | 0.0.1 |
| 2024-11 | RAN4#113 | R4-2418234 |  |  |  | R4-2420392 TP to TR 38.774 on Clause 5 (System parameters) R4-2420393 TP to TR 38.774 on LP-WUS | 0.1.0 |
| 2025-02 | RAN4#114 | R4-2500753 |  |  |  | R4-2501381 TP for TR 38.774 on LP-WUS R4-2503027 TP to TR 38.774 on LP-WUS R4-2500556 TP to TR 38.774 on regulation relevant spectrum requirements (Clause 7.2.3) | 0.2.0 |
| 2025-04 | RAN4#114bis | R4-2503830 |  |  |  | R4-2505236 TP for TR 38.774 on LP-WUS R4-2505148 TP to TR 38.774 on Clause 5 (System parameters) R4-2505149 TP to TR 38.774 on LP-WUS R4-2504743 TP to TR 38.774 on LP-WUS power boosting (Clause 7.2.2) | 0.3.0 |
| 2025-05 | RAN4#115 | R4-2508089 |  |  |  | R4-2508089 TP for TR 38.774 on LP-WUS R4-2508090 TP to TR 38.774 on testability of LP-WUR R4-2508117 TP to TR 38.774 on LP-WUS UE RF Simulations | 0.4.0 |