| 3GPP TR 38.796 V0.5.0 (2025-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power UE (power class 2) for NR FR1 FDD single band for handheld/FWA UEs and high power UE operation (power class 1) for FWVM (fixed-wireless/vehicle-mounted) use case in a single NR FDD band; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

4.1	TR Maintenance	7

5	High Power (Power Class 2) UE for FDD single band	7

6	High Power (Power Class 1) FWVM for FDD single band	11

Annex <A> (informative): Change history	12

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for release 19 basket WI Rel-19 High power UE (power class 1.5 and 2) for NR FR1 TDD/FDD single band, and high power UE operation (power class 1) for fixed-wireless/vehicle-mounted use cases in a single NR band. The purpose is to complete the band specific requirements for the newly requested and uncompleted Rel-18 requests bands for FDD band with PC2 for the different device types and PC1 only for FWVM.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-241680: “New WID: Rel-19 High power UE (power class 1.5 and 2) for NR FR1 TDD/FDD single band, and high power UE operation (power class 1) for fixed-wireless/vehicle-mounted use cases in a single NR band”.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

A-MPR				Additional Maximum Power Reduction

FDD				Frequency Division Duplex

FWA                    		Fixed Wireless Access

FWVM                		Fixed-Wireless/Vehicle-Mounted

HPUE				High Power User Equipment

MSD				Maximum Sensitivity Degradation

REFSENS				Reference Sensitivity power level

UE				User Equipment

4	Background

At 3GPP RAN#104 meeting, a basket Work Item on “Rel-19 High power UE (power class 1.5 and 2) for NR FR1 TDD/FDD single band, and high power UE operation (power class 1) for fixed-wireless/vehicle-mounted use cases in a single NR band” [2] was approved for Rel-19. The present document is a technical report for this basket Work Item, covering the high power on FDD aspects.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

5	High Power (Power Class 2) UE for FDD single band

## 5.1	NR band n26

### 5.1.1	UE maximum output power

Table 5.1.1-1: UE output power for PC2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n26 |  |  |  |  | 26 | +2/-31 | 23 | ±21 |
| NOTE 1: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |  |  |  |  |  |  |

### 5.1.2	A-MPR requirements

A-MPR for n26 PC2 was worked on already in rel-18, which majority of the results captured in TR 38.896.

In rel-19 3 MHz A-MPR results were added and minor adjustments were made to based on companies inputs. There were in total 4 NS-values evaluated, with additional requirements captured in Tables 5.1.2-1 to 5.1.2-4. For each of these the requirements apply also closer than FOOB.

Table 5.1.2-1: Additional requirements "NS_12"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 814 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Table 5.1.2-2: Additional requirements "NS_13"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 817 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Table 5.1.2-3: Additional requirements "NS_14"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz, 15 MHz, 20MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for NR carriers with lower channel edge at or above 824 MHz NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Table 5.1.2-4: Additional requirements "NS_15"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

The final A-MPR definition for n26 PC2 is captured in Tables 5.1.2-5 to Table 5.1.2-12.

Table 5.1.2-5: A-MPR regions for NS_12

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A3 |
|  | 817.5 ≤ Fc < 818.5 | ≤0.18 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
|  |  | >1.8 | >1.08 | A2 |
| 10MHz |  | ≤3.6 | >0 | A1 |
|  |  | >3.6, ≤5.04 |  | A2 |

Table 5.1.2-6: A-MPR for NS_12

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.5 | 3 | ≤ 5.5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 | 4 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 | 4.5 | ≤ 6.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 6.0 | ≤ 6.5 |
| CP-OFDM QPSK | ≤ 9 | 5.0 | ≤ 7.5 |
| CP-OFDM 16 QAM | ≤ 9 | 5.0 | ≤ 7.5 |
| CP-OFDM 64 QAM | ≤ 9 | 5.0 | ≤ 7.5 |
| CP-OFDM 256 QAM | ≤ 11.5 | 7.0 | ≤ 7.5 |

Table 5.1.2-7: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A1 |
|  |  | <0.54, ≤ 1.08 | ≥1.08 | A4 |
|  | 820 ≤ Fc < 821 | ≤0.18 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
|  |  | >1.44 | ≥1.8 | A3 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
|  |  |  | <3.24, ≥2.52 | A4 |

Table 5.1.2-8: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 7 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 7.5 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 8 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 8 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7.5 | ≤ 8 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 10 | ≤ 10 | ≤ 7 |  |

Table 5.1.2-9: A-MPR regions for NS_14

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10MHz | ≤0.18 | <1.08 | A1 |
|  | ≥0 | ≥9 | A2 |
|  | ≥0 | ≥8.1, <9 | A4 |
| 15MHz | ≤1.8 | <1.8 | A1 |
|  | ≥0 | ≥9 | A2 |
|  | ≤1.8 | ≥1.8, <3.6 | A4 |
|  | ≥0 | ≥8.1, <9 | A4 |
| 20MHz | ≤3.42 | <1.8 | A3 |
|  | ≥0 | ≥9 | A2 |
|  | ≤3.42 | ≥1.8, <3.6 | A4 |
|  | ≥0 | ≥8.1, <9 | A4 |

Table 5.1.2-10: A-MPR for NS_14

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 3.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 4 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 4.5 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 6.5 | ≤ 6 | ≤ 8 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 5.5 | ≤ 7 | ≤ 4 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 5.5 | ≤ 7 | ≤ 4 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 5.5 | ≤ 7 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 7.5 | ≤ 8 | ≤ 6.5 |

Table 5.1.2-11: A-MPR regions for NS_15

Table 6.2.3.24-3: A-MPR regions for NS_15 (Power Class 2)

| Channel BW | Carrier Frequency, Fc MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A2 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
|  | 843.0 < Fc ≤ 844.5 |  | ≥ 1.8 | A5 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
|  |  | <2.53, ≥1.8 | ≥1.44 | A5 |
|  |  | <1.08 | >0.36, ≤0.72 | A5 |
|  | 837.5 < Fc ≤ 840.5 |  | ≥ 3.0 | A5 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  |  | <4.14, ≥1.8 | ≥2.52 | A5 |
|  |  | <2.52 | >0.36, ≤0.72 | A5 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
|  |  | <7.2, ≥5.22 | <4.32, ≥3.24 | A5 |
|  |  | <1.44 | >0.36, ≤0.72 | A5 |
|  | 829 < Fc ≤ 835 |  | ≥ 5.4 | A7 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  |  | <4.68, ≥1.8 | ≥2.52 | A5 |
|  |  | <3.96 | >0.36, ≤1.08 | A5 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  |  | <2.7 | >0.36, ≤0.72 | A5 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
|  |  | <1.08 | >0.36, ≤0.72 | A5 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  |  | < 8.46, ≥1.8 | ≥3.6 | A5 |
|  |  | <6.12 | >0.36, ≤1.08 | A5 |
|  |  | <12.24, ≥8.46 | < 5.4, ≥3.6 | A5 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |
|  |  | <4.68 | >0.36, ≤1.08 | A5 |

Table 5.1.2-12: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4.0 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3.5 | ≤ 4.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 4 | ≤ 5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 5 | ≤ 6 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 5 | ≤ 6 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 | ≤ 7 |

### 5.1.3	Reference sensitivity requirements

Table 5.1.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n26 | 0.5 | 0.5 | 0.7 | 0.8 | 2.2 | 2.6 | 2.8 |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |

Table 5.1.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n26 | 1.1 | 1.1 | 1.2 | 1.4 | 5.2 | 6.1 | 6.5 |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |

6	High Power (Power Class 1) FWVM for FDD single band

Annex <A> (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-2411280 |  |  |  | TR skeleton | 0.0.1 |
| 2024-08 | RAN4#112 | R4-2411285 |  |  |  | Withdrawn | 0.1.0 |
| 2024-10 | RAN4#112-bis | R4-2415258 |  |  |  | Withdrawn | 0.2.0 |
| 2024-11 | RAN4#113 | R4-2417574 |  |  |  | Withdrawn | 0.3.0 |
| 2025-04 | RAN4#114-bis | R4-2504095 |  |  |  | Withdrawn | 0.4.0 |
| 2025-05 | RAN4#115 | R4-2505359 |  |  |  | Included TPs/pCRs: R4-2508053 TP to TR 38.796: adding n26 PC2 | 0.5.0 |