3GPP TSG-RAN WG4 Meeting #115	R4- 2508706

St. Julian’s, Malta, May 2025

Title:	Reply to RAN1 LS on impact on transmit signal quality/MPR requirement

Response to:	R1-2410889

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:

Name : Pushkar Kulkarni

Email : pushkark@qti.qualcomm.com

# 1	Description

On UL resource muting, RAN1 has made the following agreements

Agreement If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Note that RAN1 is still discussing the boosting in symbols containing PT-RS. RAN1 would like to ask RAN4 to check impact on transmit signal quality/MPR, if any.

# 2	Reply

RAN4 would like to thank RAN1 for their LS in R1-2410889 and provide the following responses to RAN1 question on transmit signal quality/MPR impact, if any due to RE muting on PUSCH.

RAN4 has studied UL RE muting in FR1 with the suggested muting scheme. Different sets of waveforms are analyzed which are either ACLR, EVM or IBE limited. Observations and conclusions from companies are summarized below.

1)  No Impact on transmit waveform quality observed by three companies

- For UL resource muting, no transmit quality impact can be expected for PUSCH at least for lower modulation order as comb-2 pattern is already supported since LTE.

- MPR remains unaffected by the introduction of UL resource muting.

- Fully allocated UL resource muted DFT-s-OFDM and CP-OFDM waveforms have no impact on transmit waveform quality.

2) Impact on transmit waveform quality observed by two companies

- MPR impact due to the CLI muting pattern is less than 0.5 dB for CP-OFDM and less than 0.4 dB for DFT-s-OFDM.

- UL resource muted waveforms along the edge of the channel have less than 0.5 dB impact on MPR.

# 3	Actions

To RAN1:

Action : 	RAN4 respectfully asks RAN1 to take the above observations into consideration in their future work.

# 4	Dates of next TSG RAN WG 4 meetings

3GPP RAN4#116       		        25th – 29th August, 2025		 Bengaluru, India

3GPP RAN4#116bis       		        13th – 17th October, 2025        		 Prague, Czechia