3GPP TSG-RAN WG4 #116	R4-2509032

Bengaluru, India, August 25-29, 2025

Agenda item:	7.20.4.2		Other RRM Requirements

Source:	Qualcomm Incorporated

Title:						Additional other RRM core requirements for NR MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #112, the work on Rel-19 MIMO Phase 5 was started. Several agreements were made to identify impacts on RRM requirements, and several open issues were outlined in [1].

In this paper we provide our views for the open issues identified in the way forward in [1] in RAN4 #115.

# Discussion

## UE Reporting Enhancements for CJT Deployments

In RAN4 #115 several topics remained open for further discussion in RAN4 #116. It was decided that accuracy requirements for delay and frequency offset reporting will be defined based on one sample, but it remained open whether additional requirements should be defined for three and five samples.

Issue 1-1: How to define measurement period requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

- RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting.

- RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

In our understanding the number of samples in Issue 1-1 refers to the number of NZP-CSI-RS resource sets being configured for TRS. In FR1, one NZP-CSI-RS resource set for tracking consists of four NZP-CSI-RS resources in two consecutive slots, i.e., one sample in Issue 1-1 are four TRS signals in two consecutive slots. Hence, multiple samples in Issue 1-1 refer to multiple NZP-CSI-RS resource sets with a typical spacing in time of domain of 20 ms as seen in field deployments. The device cannot store the channel impulse response of the TRS across multiples of 20 ms. Hence, an averaging across multiple samples in Issue 1-1 is an averaging of estimated delay and frequency offsets. Such an averaging can also be done at network side in case the device reports the frequency and delay offset just based on one sample. This might even be more useful in case the gNB can make use of additional side information in the averaging of e.g., the frequency offset.

Observation 1: The device cannot store the channel impulse response of the TRS across multiples of 20 ms. Hence, an averaging across multiple samples in Issue 1-1 is an averaging of estimated delay and frequency offsets.

Proposal 1: RAN4 should only define accuracy requirements for delay and frequency offset reporting for one sample, i.e., for one NZP-CSI-RS resource set.

The next open issue is Issue 1-2 whether scheduling restriction for delay and frequency offset reporting should be defined for CJT deployments. In our understanding scheduling restrictions mean that the device is not expected to receive or transmit in slots and/or symbols with scheduling restrictions. Since the network does not know in detail how the device will treat those slots and symbols, the network most like will not use slots and symbols with scheduling restrictions at all which causes a loss in throughput. Hence, it should be carefully decided whether scheduling restrictions are defined in the specification or whether it can be left to the network how to handle scheduling in those slots and symbols. Scheduling restrictions should only be defined if the device is not able to handle those slots symbols properly.

Issue 1-2: How to define scheduling restriction requirements for delay and frequency offset reporting for CJT calibration?

Way forward:

- Option 1: The scheduling restriction is specified as:

  - For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting.

- Option 2: No scheduling restrictions needed to be applied for CJT reporting.

For delay offset reporting, the dynamic range of the delay offset reporting is configurable by the network. Supported values are AD  {0.5 CP, CP} as the maximal values. Since the range is configurable, the network must have information what delay offsets between the two non-collocated TRPs are expected. In our view, it can be left to the network to avoid scheduling around the CSI-RS resource if this is really needed. In many scenarios, the symbols around the CSI-RS are not impacted and UE is able to report delay offset accurately. Hence, we do not think that it is required to define scheduling restrictions in the specification.

Observation 2: It can be left to the network to avoid scheduling around the CSI-RS resource if this is really needed. In many scenarios, the symbols around the CSI-RS are not impacted and UE is able to report delay offset accurately.

Proposal 2: No scheduling restrictions should be defined for CJT reporting.

## Asymmetric DL sTRP / UL mTRP

In RAN4 #115 good progress was made for Scenario #2 when the UE expects to receive SSB from the UL TRP(s). for Scenario #1, however, still several questions remained open in RAN4 #115.

In Issue 2-1 it was discussed whether the RACH timing requirements for Scenario #1 in case of 2TA shall be modified to address asynchronous deployments.

Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Way forward: FFS

For Scenario #1 only one TRP transmits downlink reference signals. This downlink reference signal serves as the reference timing for the PRACH. The reference timing of the PRACH is the same for both TRPs.

Proposal 3: For Scenario #1, the reference timing is the first detected path (in time) of the downlink reference signal(s) of the TRP that transmits downlink reference signals. The reference timing of the PRACH is the same for both TRPs

We understand that in case both TRPs are asynchronous, the PRACH sent to the UL-only TRP may be received too early causing interference or causing even a missed PRACH. However, we still believe that this issue can be solved at network side by suitable MAC scheduling and RACH configurations, e.g., RACH formats with repetition and consecutive RACH slots within one frame. Hence, we do not see a need for adding an additional offset to the PRACH timing to mitigate impacts of asynchronous TRPs.

Observation 3: The issue of asynchronous TRPs can be solved at network side by suitable MAC scheduling and RACH configurations, e.g., RACH formats with repetition and consecutive RACH slots within one frame.

Proposal 4: No additional time offset to the PRACH timing should be introduced to mitigate impacts of asynchronous TRPs.

Another discussion that could not be resolved is Issue 2-2 how to apply gradual timing for Scenario #1 when two TAGs are applied.

The gradual timing adjusts the uplink relative to the downlink timing adjustments due to time tracking loop to ensure that the reference timing is (NTA + NTA offset) x Tc before the downlink timing of the reference cell.

Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Way forward:

- Option 1: Gradual timing adjustment applies for each TAG and shall be met for each TAG.

- Option 2: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

Since in Scenario #1 both uplink channels have the same downlink timing reference, the gradual timing adjustment should be applied to both TAGs. Since only one downlink tracking loop is running in this scenario, the only possibility is to apply the same value of the gradual timing to both TAGs. No tapplying the gradual timing adjustment to the UL only TRP means that the error between downlink timing reference and the UL signal drifts away and the error becomes larger than Tc. Hence, we believe that the same gradual timing adjustment should be applied with the same value for both TAGs.

Proposal 5: For Scenario #1, gradual timing adjustment should be applied with the same value for both TAGs.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #115. Our observations and proposals are summarized as follows:

UE reporting enhancement for CJT deployments

Observation 1: The device cannot store the channel impulse response of the TRS across multiples of 20 ms. Hence, an averaging across multiple samples in Issue 1-1 is an averaging of estimated delay and frequency offsets.

Proposal 1: RAN4 should only define accuracy requirements for delay and frequency offset reporting for one sample, i.e., for one NZP-CSI-RS resource set.

Observation 2: It can be left to the network to avoid scheduling around the CSI-RS resource if this is really needed. In many scenarios, the symbols around the CSI-RS are not impacted and UE is able to report delay offset accurately.

Proposal 2: No scheduling restrictions should be defined for CJT reporting.

Asymmetric DL sTRP/UL mTRP

Proposal 3: For Scenario #1, the reference timing is the first detected path (in time) of the downlink reference signal(s) of the TRP that transmits downlink reference signals. The reference timing of the PRACH is the same for both TRPs

Observation 3: The issue of asynchronous TRPs can be solved at network side by suitable MAC scheduling and RACH configurations, e.g., RACH formats with repetition and consecutive RACH slots within one frame.

Proposal 4: No additional time offset to the PRACH timing should be introduced to mitigate impacts of asynchronous TRPs.

Proposal 5: For Scenario #1, gradual timing adjustment should be applied with the same value for both TAGs.

# References

- R4-2508272, WF on RRM requirements for NR_MIMO_Ph5_Part2, RAN4 #115, May 2025

- R4-2504955, Collection of simulation assumptions for CJT calibration in NR MIMO Ph5 Part2, RAN4 #115, May 2025

- R4-2502667, LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP / UL mTRP, RAN4 #114, February 2025

- R1-2503091, Draft Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP / UL mTRP, RAN1 #120bis, April 2025