3GPP TSG-RAN WG4 Meeting #116												R4-2509036

Bengaluru, India, 25 – 29 August, 2025

Agenda item:			7.22.5

Source:	Huawei, HiSilicon

Title:	on ambient device OTA tests

Document for:	approval

# Introduction

During RAN4 #114bis [2] and RN4 #115 [1], framework for ambient OTA tests has been agreed. This contribution outlines test procedures for backscattering performance and sensitivity. Associated CR for test procedures, coordinate system and MU have been submitted to this meeting.

# Discussion

The aim of the test procedures is to minimise modifications to existing systems for TRP and TRS tests with the following two options. The associated test procedure CR is for option 1.

Option 1: CW and Reader use the same antenna with orthogonal polarisations

Option 2: CW and Reader use different antennas with orthogonal polarisations

The main differences between option 1 and 2 are:

- achievable isolation between CW and Reader to ensure Reader can decode D2R signals

- amount of modification if any in existing labs

Below is a list of isolation mechanisms between CW and Reader for both options:

- orthogonal linear polarisation

- circulator

- side band filtering of D2R signal

- interference cancellation at Reader

- spatial isolation (option 2 only)

Backscattering power and device sensitivity are the performance metrics to be measured. It would be easier to only measure backscattering power at one direction declared by device manufacturers to avoid spherical scan. Besides a single direction measurement can be implemented by all existing OTA test systems with minimum modification.

Proposal 1: measure backscattering power in a single direction declared by manufacturers

However, it would be prudent to use a value for the device sensitivity measurement over an agreed solid angle, for example, hemisphere.

Proposal 2: measure device sensitivity over a solid angle, e.g. hemisphere or partial sphere.

D2R signal bandwidth has various values depending on chip duration and SFS repetition rate in signal design [3, 4]. It would be time consuming to test the full range. Namely it is more efficient to test a single bandwidth.

Proposal 3: select a single bandwidth for OTA tests.

Although the test procedures below are for backscattered power and sensitivity measurement, the test procedures can be easily adapted to other measurements.

Proposal 4: The OTA tests for other requirements can be conducted based on the baseline methodology for sensitivity and backscattering. The detailed updating/simplification for specific requirements can be discussed in conformance requirements.

Following the agreed test procedures in [2], the test procedures below are for option 1 and can be modified for option 2 if option 1 is deemed infeasible.

During tests, device is placed on a platform with either combined axis or distributed axis at the origin of a Cartesian coordinate. Test antenna with two linear orthogonal polarizations support both CW and Reader, namely CW and Reader share the same antenna with CW occupying one polarization and Reader using both polarizations.

Polarization orientation of CW may need to be chosen in such a way to avoid complete polarization mismatch between CW and device as both use linear polarization.

Backscattered power

Backscattered power is only measured at the direction of maximum backscattering declared by device manufacturers.

The measurement procedure includes the following steps:

- Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between test/CW antenna to DUT, insertion loss and cable loss, etc.

- Position Reader and device such that the direction of maximum backscattering faces the Reader according to the declaration from device manufacturers.

- Device is assumed charged during the measurement according to device declaration on the required energy conditions.

- Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set such that the received power at DUT’s antenna is larger than minimum reference sensitivity requirement of device. CW transmits in θ-polarization to ensure CW incident power at the device antenna is [x]dB higher than the receiver sensitivity requirement.

- Reader receives in  θ-polarization and ϕ-polarization either simultaneously or sequentially and record received power levels. Repeat step 2) and 3) with CW transmits in ϕ-polarization.

- Calculate backscattered power from the device: Pout = CW=θ,ϕ {(EIRPθ-Pathloss-Gainantenna)+(EIRPϕ-Pathloss- Gainantenna)}

Where

Pathloss : free space propagation loss between test antenna and device

Pout : backscattered power

Pin : CW incident power at device

PCW,θ and PCW,ϕ : CW transmit power in θ-polarization and the ϕ-polarization, respectively

EIRPθ and EIRPϕ ：Reader received power in θ-polarization and the ϕ-polarization, respectively

Gainantenna : test antenna gain

Sensitivity

Sensitivity is measured over a partial sphere of [y] degrees in elevation or θ-direction with [30] degree steps in both θ- and ϕ-directions.

The measurement procedure includes the following steps:

- Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Place DUT based on UE positioning guidelines.

- Device is assumed charged during the measurement according to device declaration on the required energy conditions.

- Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set such that the received power at DUT’s antenna is a few dB above minimum reference sensitivity requirement of device.

- Reader transmits in θ-polarization, but receives in θ-polarization and ϕ-polarization either simultaneously or sequentially.

- CW transmits in θ-polarization to ensure CW incident power at the device antenna is [x]dB higher than the receiver sensitivity requirement.

- Initial Reader transmit power shall be high enough for Reader to receive and decode responses from device successfully.

- Then Reader transmit power level is reduced to achieve 90% response decode success rate by determining whether DUT can send correct response in D2R channel within correct timing relationship.

- Repeat step 5) to 8) for all grid points over hemisphere and record the Reader received power levels.

- Change Reader configuration to: Reader transmits in ϕ-polarization, but receives in both θ-polarization and ϕ-polarization.

- Change CW configuration to: CW transmits in ϕ-polarization to ensure device reception

- Repeat step 5) and 8) for all grid points over hemisphere and record the Reader received power levels

- For every measured grid points, calculate total transmit power from Reader in both polarizations in dB:

  - Pθ (θi, ϕj) = PTx(θ-polarization) + Gainantenna – Pathloss

  - Pϕ (θi, ϕj) = PTx(ϕ-polarization) + Gainantenna – Pathloss

- Calculate sensitivity at grid point (i, j) using linear values:

Refsens(θi, ϕj) = [1Plinearθ(θi, ϕj)+1Plinearϕ(θi, ϕj)]-1

Proposal 5: consider to adopt test procedures for backscattered power and sensitivity in R4-2509036.

# Conclusion

This contribution makes the following proposals.

Proposal 1: measure backscattering power in a single direction declared by manufacturers

Proposal 2: measure device sensitivity over a solid angle, e.g. hemisphere or partial sphere.

Proposal 3: select a single bandwidth for OTA tests.

Proposal 4: The OTA tests for other requirements can be conducted based on the baseline methodology for sensitivity and backscattering. The detailed updating/simplification for specific requirements can be discussed in conformance requirements.

Proposal 5: consider to adopt test procedures for backscattered power and sensitivity in R4-2509036.

# References

- R4-2507907 WF on A-IOT device requirements (RAN4-115)

- R4-2505230 WF on A-IOT device requirements (RAN4-114bis)

- R1-2504894 Chair notes RAN1#121 (9.4 R19 Ambient IoT)

- R1-2503020 Summary #1 for coding aspects of physical channel design, RAN1#120bis