3GPP TSG-RAN WG4 Meeting #116												R4-2509037

Bengaluru, India, 25 – 29 August, 2025

Agenda item:			7.10.3.3

Source:	Huawei, HiSilicon

Title:	on MIMO OTA tests

Document for:	approval

# Introduction

One of the remaining issues in MIMO OTA test is how to define performance metric [1]. This contribution examines this issue and proposes options for consideration. In addition, RP-251566 [2],  a CR on channel model enhancement for 38.901 [3], was approved in in RAN#108 and may require updates on the channel models used by RAN4 as UMa and UMi models are impacted by the CR.

# Discussion

In the previous RAN4 meetings, various proposals on MIMO performance metrics are made. For example, average of throughput, weighted sum of throughput CDF at 10%-ile, 50%-ile and 90%-ile, and others. The main reason for the proposals is to balance the wide range of absolute values across the throughput CDF as the throughput values at higher percentiles would domain over those at lower percentiles. Artificial weights could be applied to the throughput values at various percentiles. However, it is difficult to remove potential bias due to the artificial nature of the weights.

One approach to come out this conundrum is to use relative throughput as performance metric. Namely the absolute throughput value at a given percentile is normalised to the average throughput at that percentile. Then the normalised values at various percentiles are summed up. The average throughput at any specific percentile can be obtained from throughput  measurement across a pool of devices.

The above proposal can be expressed in formula as follows.

PM=TP10%TPnominal10%+TP50%TPnominal50%+TP90%TPnominal90%

Where

TPx% denotes measured throughput at x%-ile

TPnominalx% represents averaged throughput at x%-ile from a pool of devices

TPnominal can be the average throughput at a given percentile from a pool of devices. In this case, the ratio TP/TPnominal could be larger than one. Alternatively, TPnominal can use other values such as the maximum. How to select TPnominal can be further discussed during the meeting.

Proposal 1: use the sum of normalised throughput at different percentiles on throughput CDF as performance metric.

As shown in RAN1 LS [4], RP-251566 [2] was approved in RAN#108 and impacts the following in 38.901 [3], viz.

- Table 7.5-2: cluster numbers 6 and 7 are added

- Table 7.5-4: cluster numbers 6, 7, 14, 16 are added

- Table 7.5-6: parameter values have been updated

- Clause 7.7.5 has been updated

- A.3, A.4 and A.5 have been added to Annex A.

The above together with other updates have been applied to TR 38.901 v19.0 [3]. Although RAN4 should not change the UMa and UMi channel models in Release 19 work items, RAN4 should take those updates into account in the next release if deemed necessary.

Observation 1: RAN4 should consider the updates in RP-251566 for TR 38.901 v19.0 in MIMO work item for release 20.

# Conclusion

This contribution provides the following proposal and observation.

Proposal 1: use the sum of normalised throughput at different percentiles on throughput CDF as performance metric.

Observation 1: RAN4 should consider the updates in RP-251566 for TR 38.901 v19.0 in MIMO work item for release 20.

# Reference

- R4-2508782 Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3

- RP-251566 Introduction of Rel-19 7-24 GHz channel model enhancements

- TS 38.901 v.19.0 Study on channel model for frequencies from 0.5 to 100 GHz

- R4-2509011 LS to CTIA MOSG on TR38.901 updates