3GPP TSG-RAN WG4 Meeting #116	R4-2509057

Bengaluru, India, August 25th – August 29th, 2025

Agenda item:			7.19.1

Source:	Moderator (Nokia)

Title:	Topic summary for [116][214] FS_NR_AIML_Mob_Part1

Document for:	Information

# Introduction

This summary thread addresses the issues related to AI/ML mobility SI in RAN4. The document is split into 3 high-level topics which are further split into issues.

Moderator’s recommendations are to discuss the following issues, listed in the order of priority.

- Issue 2-1: Impacts of measurement error on prediction accuracy

- Issue 2-2: Relative RSRP accuracy definition

- Issue 2-7: Ground truth definition for RSRP Accuracy for FR1

- Issue 1-1: Spatial domain prediction

- Issue 3-1: Performance metric for indirect measurement event prediction

- Issue 3-2: Performance metric for direct event prediction use case

- Issue 2-4: Impacts on beam/cell level measurements

- Issue 2-8: Impacts from observation and prediction windows

- Issue 2-9: Inter-frequency prediction scenario

- Issue 2-5: Performance metric for AI/ML based RRM measurement prediction

- Issue 2-6: Accuracy requirements for the predicted RSRPs

Furthermore, we will also discuss Issue 1-4 TR-Framework.

# Topic #1: General Aspects

This section addresses general aspects and priorities of different scenarios and sub use-cases identified in the draft TR 38.744.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509431 | Apple | General aspects on AI/ML for mobility in NR Observation 1: RRM measurement predictions provide several advantages, including reducing measurement gaps, increasing the periodicity of SMTC windows, minimizing scheduling and measurement restrictions, decreasing measurement delays, and shortening measurement gap lengths The two spatial prediction cases serve fundamentally different roles in the NR system. Case 1 (AI/ML BM spatial prediction) is designed for intra-cell beam management using immediate link feedback, while Case 6 focuses on neighbour prediction and proactive mobility support using cross cell measurements and AI/ML model at the network side. Spatial prediction for mobility (Case 6) is more challenging than beam management (BM Case 1) because it must estimate KPIs for multiple neighbor cells across varied bands, antenna configurations, and SINR conditions without access to real-time neighbor beam measurements. Instead, it relies on limited serving-cell measurements and potentially outdated neighbor reports, forcing the model to infer performance from incomplete information. This demands larger, more diverse training datasets and strong generalization across deployments, while any misprediction can directly harm handover quality. Further discussion is needed in RAN4 regarding future requirements for RLF design: - Should the indication be reported from the UE to the network, or only from the UE's lower layers to higher layers? - If a direct or indirect indication is sent from the UE to the network: - To be determined (FFS): How should such an RLF indication be triggered to the network? - For a direct indication, should the UE report a predicted RLF to the network when all measured and/or predicted downlink radio link quality values on all configured RLM-RS resources fall below a threshold (Qout)? - For an indirect indication, should the UE report all predicted SINR values with corresponding RS indices to the network when the measured and/or predicted downlink radio link quality on all configured RLM-RS resources is worse than Qout? Alternatively, should the UE indicate the predicted SINR with the relevant RS indices as long as at least one configured RLM-RS resource shows measured and/or predicted downlink radio link quality? Proposal 1: The conclusions, model designs, and deployment approaches validated for Case 1 (AI/ML for BM) cannot be assumed valid for Case 6 AI/ML for mobility. Case 6 demands a broader spatial awareness, cross-cell learning, and prediction mechanisms that are unique to mobility scenarios beyond the scope of beam management addressed in Case 1 Proposal 2: Spatial-domain prediction for mobility requires complex cross-cell modeling across diverse antenna configurations, frequency bands, and SINR regimes, and is highly sensitive to both measurement errors and generalization errors. Addressing these challenges demands large multi-cell training datasets and robust error-tolerant model designs. We propose postponing this study to the 6G timeframe to allow sufficient time for developing scalable solutions, with good generalization, and defining comprehensive evaluation methods. Proposal 3: RAN4 should consider a new use case within the AI/ML-based RRM prediction framework: “Temporal Prediction of Beam Pair Changes for RRM Optimization.” This use case focuses on predicting the timing of beam transitions, rather than the best beam index, differentiating it from existing beam management and spatial prediction cases. The goal is to reduce RRM measurement efforts in the temporal domain by enabling on-demand measurement activation. The study should define performance requirements, accuracy metrics, and error-handling procedures to ensure robust operation, while aligning with the first study goal of minimizing measurement overhead |
| R4-2509924 | Nokia | On General aspects in AIML Mobility Proposal 1: RAN4 to study the specification impact of intra-frequency spatial domain prediction in beam dimension for UE sided models. Proposal 2: RAN4 to capture the following error modelling principles in the TR: - RF and baseband measurement error modelling may be considered. - A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband. - Alternative modelling approaches (such as based on link-level simulations) are not precluded and can be explored in the Work Item phase. |
| R4-2510106 | MediaTek Inc. | Discussion on general aspects for AI mobility Proposal 1: If to derive the ground truth through transmitted power at TE, additional margin should be considered due to different L1 filtering and L3 filtering may be adopted by UE vendors. The value of the margin can be obtained through simulations. Proposal 2: If to use reported RSRP as ground truth, the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. The value of Y is relevant to UE implementation. Proposal 3: During the SI, RAN4 not to do down-selection on the two Alternatives of ground truth definition. Proposal 4: When do the simulations, the data containing cells with too low SNR (e.g., <-6dB) should be excluded from the dataset. Proposal 5: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), discuss whether co-located cells on different frequencies are with varied BS configurations (e.g., antenna pattern, downtilt angle) or not. Proposal 6: For scenario 3 (inter-frequency measurement prediction), discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network. Proposal 7: For AI mobility, in FR1, measurement error should be added as: - BB error: - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR. - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement. - RF error: - Modelled as Gaussian distribution under ±3 dB RF error (μ=0, σ2=3). - A random RF error is generated for all the beams of the same frequency layer per SMTC. Observation 1: The impact of measurement error is marginal. Observation 2: For generalization performance evaluation, RAN2 didn’t touch measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP. Proposal 8: RAN4 shall keep evaluating the generalization performance over measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP and etc. Observation 3: The performance with different L3 filtering configurations diverge. Proposal 9: Further discuss how to define RSRP accuracy requirements for different L3 filtering configurations. Observation 4: For legacy L3 measurement, RAN4 only defined test cases for event triggered L3 measurement report. Proposal 10: Prioritize the cases without legacy UE impact, e.g., Case A. Proposal 11: For temporal domain prediction, measurement event prediction should be the focus and RAN4 only need to define test cases for measurement event prediction. |
| R4-2510167 | CMCC | Discussion on general part for AI/ML for mobility Proposal 1: it is proposed to firstly focus on scenario 2, 3, 4. Proposal 2: it is proposed to consider all three sub-use cases. Proposal 3: it is proposed that RAN4 not to study RRM impact on RLF prediction in Rel-19, sine RAN2 agreed that RLF prediction will not be studied in Rel-19. Proposal 4: for Data Collection for NW sided model, it is proposed that legacy accuracy requirements are applied to the logging data, e.g. L3 cell level measurements, L3 beam level measurement, L1-filtered beam level measurement. |
| R4-2510486 | vivo | Discussion on general aspects for AI mobility Proposal 1: RAN4 only to consider collocated cases for frequency domain prediction. Accordingly, the above editor's note regarding non-collocated cases in the existing SID needs to be deleted. Proposal 2: RAN4 not to study RRM impact on RLF prediction in Rel-19 |
| R4-2510577 | Huawei, Hi Silicon | Discussion on genereal aspects in AIML mobility Proposal 1: Conclude that no issues are identified for interoperability in AI-mobility. Proposal 2: Generalization can be ensured by defining several test conditions. Test conditions will not change during one test. Test conditions will be discussed case by case. Proposal 3: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy. Pros and Cons need to be analyzed. - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4. - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4. Proposal 4: RAN4 will study the following two potential options for defining the testing and training dataset. - Option 1: Use the channel model and the sampling rules. - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner. |
| R4-2510445 | ZTE Corporation, Sanechips | Discussion on General Aspects of AIML Mobility Proposal 1: Align with RAN2 and RLF prediction will not be studied in Rel-19. Proposal 2: RAN4 to consider all three sub-use cases with no prioritization. Proposal 3: RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case in cell dimension and beam dimension. Proposal 4: RAN4 to study the RRM impact of intra-frequency spatial domain prediction (scenario 6) in beam dimension. |
| R4-2510762 | Ericsson | General discussion on AI/ML for mobility Proposal 1: Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements. - RF error in dB for FR1 is characterized by Gaussian distribution with μ=0, σ2= 2.5 truncated at ±2.5. - RF error in dB for FR2 is characterized by Gaussian distribution with μ=0, σ2= 4 truncated at ±4. Proposal 2: Provided no new measurements are identified by RAN2, RAN4 should further discuss whether the existing requirements are applicable for data collection for network sided model. Proposal 3: If new measurement is defined by RAN2 for data collection for network sided model, requirements for the new measurement is specified by RAN4. |

## Open issues summary

### Issue 1-1: Spatial domain prediction

- Background:

Following agreement was reached in RAN2#130:

Agreements RAN2#130: - The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts. Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s). - The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

- Background:

- Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 to study the specification impact of intra-frequency spatial domain prediction in beam dimension for UE sided models.

  - Proposal 2 (ZTE Corporation, Sanechips): RAN4 to study the RRM impact of intra-frequency spatial domain prediction (scenario 6) in beam dimension.

  - Proposal 3 (Apple): Spatial-domain prediction for mobility requires complex cross-cell modeling across diverse antenna configurations, frequency bands, and SINR regimes, and is highly sensitive to both measurement errors and generalization errors. Addressing these challenges demands large multi-cell training datasets and robust error-tolerant model designs. We propose postponing this study to the 6G timeframe to allow sufficient time for developing scalable solutions, with good generalization, and defining comprehensive evaluation methods.

  - Proposal 4 (ZTE Corporation, Sanechips): RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case in cell dimension and beam dimension.

  - Proposal 5 (CMCC): it is proposed to firstly focus on scenario 2, 3, 4.

- Candidate Options:

  - Option 1: RAN4 to study the specification impact of intra-frequency spatial domain prediction (Scenario 6) in beam dimension for UE sided models

  - Option 2: RAN4 to study the specification impact of intra-frequency spatial domain prediction (Scenario 6)  in beam dimension for UE sided models as second priority.

  - Option 2: RAN4 to postpone the study on the impact of intra-frequency spatial domain (Scenario 6) to the 6G timeframe to allow sufficient time for developing scalable solutions, with good generalization, and defining comprehensive evaluation methods

- Recommended WF:

RAN4 to discuss the above-listed candidate options.

### Issue 1-2: sub-use case priorities

- RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model). In RAN2, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Proposals and Observations:

  - Proposal 1 (CMCC, ZTE Corporation, Sanechips,): RAN4 to consider all three sub-use cases with no prioritization.

- Recommended WF:

  - RAN4 to consider all three sub-use cases with no prioritization.

### Issue 1-3: Radio Link Failure (RLF) prediction use case

- Proposals and Observations:

  - Proposal 1 (ZTE, vivo, CMCC): RAN4 not to study RRM impact on RLF prediction in Rel-19

- Recommended WF:

  - RAN4 doesn’t need to define requirements for RLF prediction use case.

### Issue 1-4: TR framework

- Proposals and Observations:

  - Proposal 1 (OPPO, Nokia): Adopt the text proposal for TR 38.744 in [R4-2509672] to capture the RAN4 study.

  - Proposal 2 (Ericsson): Adopt the text proposal for TR 38.744 in [R4-2510761] to capture the RAN4 study.

- Recommended WF:

  - Discuss the TR framework in R4-2509672 and R4-2510761.

### Issue 1-5: Requirements for Data Collection for NW sided model

- Proposals and Observations:

  - Proposal 1 (Ericsson): Provided no new measurements are identified by RAN2, RAN4 should further discuss whether the existing requirements are applicable for data collection for network sided model

  - Proposal 2 (Ericsson): If new measurement is defined by RAN2 for data collection for network sided model, requirements for the new measurement is specified by RAN4.

- Recommended WF:

  - RAN4 to discuss the above proposals.

the above-listed candidate options.

### Issue 1-6: Simulation settings

- Proposals and Observations:

  - Proposal 1 (MediaTek): When do the simulations, the data containing cells with too low SNR (e.g., <-6dB) should be excluded from the dataset.

- Recommended WF:

  - RAN4 to discuss the above proposal.

### Issue 1-7: Generalization

- Proposals and Observations:

  - Proposal 1 (MediaTek):

    - For generalization performance evaluation, RAN2 didn’t touch measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP.

    - Proposal 8: RAN4 shall keep evaluating the generalization performance over measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP and etc.

- Recommended WF:

  - RAN4 to discuss the above proposal.

### Issue 1-8: Use case priorities

- Proposals and Observations:

  - Proposal 1 (MediaTek): For temporal domain prediction

    - Prioritize the cases without legacy UE impact, e.g., Case A.

    - Measurement event prediction should be the focus and RAN4 only need to define test cases for measurement event prediction.

- Recommended WF:

  - RAN4 to discuss the above proposal.

# Topic #2: RAN4 requirements for RRM Measurement prediction

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2510578 | Huawei, Hi Silicon | On RAN4 Impacts for RRM measurement prediction in AIML Mobility Proposal 1: RAN4 to focus on the discussion on scenario 2, 3 and 4 with high priority in this release. Proposal 2: Not consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy. Proposal 3: Due to limited time in SI, Relative RSRP accuracy for Beam level measurement can be not be considered in R19. Proposal 4: For prediction accuracy, in R19 SI phase it is premature to draw conclusion on - define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases; - whether the prediction accuracy shall at least meet the accuracy requirements that are defined for the non-AI/ML RRM measurement. Observation 1: Fading channel is to be employed for AI mobility measurement prediction. Observation 2: For either intra-frequency temporal domain case B and case A, although the configuration of PW, OW and skipping pattern are known to TE, the specific position in time domain of UE obtaining a L3 predicted result is unknown to TE. Observation 3: The number of L1 measurement samples is unknown to TE. Proposal 5: If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles: - The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE. - The number of L1 measurement samples is unknown to TE. Proposal 6: The ground truth in FR1 can be defined based on the reported measurement value under certain conditions. Proposal 7: For intra-frequency temporal domain prediction case A, RSRP prediction accuracy is to be defined under the side condition of certain OW and PW length. Proposal 8: To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing. Proposal 9: For intra-frequency temporal domain prediction case B, RSRP prediction accuracy is to be defined under the side condition of MRRT with limited PW length. Proposal 10: For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy in collocated scenario can be specified under certain channel correlation coefficient, e.g., ~0.9. Proposal 11: For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss - Specify RSRP prediction accuracy for cluster approach and single cell approach separately, or - Specify minimum RSRP prediction accuracy to accommodate both cluster and single cell approach |
| R4-2511226 | Samsung | Discussion on AI mobility regarding RAN4 requirements impact Proposal 1: There is no need to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy in AI mobility Proposal 2: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction Proposal 3: For the RRM impact on prediction window, at least the following may need to be discussed - Scheduling restriction - Measurement latency for MO (s), MO (s) reduction Proposal 4: If to consider RSRP prediction accuracy, the applicability of accuracy definition should be further discussed by taking at least the following into account: - Cell-specific and/or cluster prediction approaches Proposal 5: Regarding measurement error model on prediction accuracy in AI mobility - For RF error, take the outcome of the error modelling approach in PHY AI/ML as a baseline - For BB error, LLS approach involving SNR/SINR is considered to determine the distribution - Different properties of channel propagation across neighbor cells would be captured as a factor |
| R4-2509302 | CATT | Discussion on impacts of AIML RRM measurement prediction on RRM requirements Proposal 1: For scenario 4 (2nd study goal), the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes. Proposal 2: For scenarios with 1st study goal, RAN4 discuss whether a relaxed prediction accuracy requirement is defined. Proposal 3: RAN4 to evaluate the impact of baseband error using link-level simulation with different SNR levels considered. For RF error, a uniform or Gaussian distribution can be used. Proposal 4: RAN4 to capture all options in the TR for further discussions. Proposal 5: RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed. |
| R4-2509432 | Apple | On RAN4 Impacts for RRM measurement prediction in AIML Mobility Observation 1: Relative RSRP accuracy evaluates how well the predicted difference between two cells matches the ground-truth difference, providing a measure of consistency between cell comparisons. While a small relative-accuracy error suggests that the prediction and measurement are mutually reliable, it does not reveal which cell is actually stronger. This creates a risk when the two values are taken at different times (t₁ for one cell and t₂ for the other): channel changes, mobility, or fading can cause the relative ranking of cells to change between these moments. As a result, even with perfect relative accuracy, the network could incorrectly conclude that one cell is stronger and make a sub-optimal handover decision. Alternative 1, where the FR1 ground truth is derived at the TE from transmitted or received power (legacy approach), is not well suited for AI/ML mobility scenarios. In fading CDL channels, rapid signal fluctuations make precise time alignment between measured and predicted samples difficult, leading to potential inaccuracies in ground truth. Furthermore, the TE lacks knowledge of the UE’s proprietary L1 filtering parameters, such as filtering coefficients and sample counts, which further limits its ability to replicate the UE’s measurement context. Alternative 2, where the FR1 ground truth is derived from UE-reported measurements under defined conditions, is better suited for AIML Mobility because channel variations and L1 filtering effects are inherently accounted for by the UE when producing the ground truth measurements The two spatial prediction cases serve fundamentally different roles in the NR system. Case 1 (AI/ML BM spatial prediction) is designed for intra-cell beam management using immediate link feedback, while Case 6 focuses on neighbour prediction and proactive mobility support using cross cell measurements and AI/ML model at the network side. Proposal 1: A similar definition of relative RSRP accuracy exists in legacy, but it does not account for any time alignment. As per the FFS item on whether and how to consider the time difference between the two reported RSRPs when applying this definition, RAN4 should clarify what differs in the AI/ML case compared to legacy. RAN4 is invited to consider that if the definition of intra-frequency relative RSRP accuracy allows for a non-zero time difference between the two RSRPs used in the calculation, either explicit timestamps should be required as part of the conformance testing framework to ensure traceable and bounded time alignment, or the reporting periodicity and measurement alignment rules should be clearly defined to implicitly control the maximum allowed time difference (Δt). This will ensure that any variation in the relative accuracy metric reflects the prediction or measurement performance, rather than uncontrolled environmental changes due to asynchronous reporting or prediction instances. RAN4 is invited to study mechanisms for handling time differences between the two RSRP values used in intra-frequency relative RSRP accuracy calculations. The following options are proposed: Option 1: Enforce strict time alignment, requiring both RSRPs to be measured or predicted at the same or near-identical time. Option 2: Define a fixed maximum Δt threshold, excluding RSRP pairs exceeding this limit from KPI evaluation. Option 3: Use scenario-dependent Δt thresholds based on UE mobility class or deployment scenario to better match realistic radio dynamics. Option 4: Permit post-processing compensation for larger Δt by applying a bounded correction factor that accounts for expected RSRP variation over time, allowing more relaxed alignment constraints while preserving KPI validity. RAN4 is invited to study the definition and testing of relative RSRP accuracy at the beam level for intra-frequency scenarios, focusing on four key areas: (1) determining whether existing cell-level thresholds, filtering, and reporting rules are suitable for the faster dynamics of beam-level RSRP or if tighter beam-specific definitions are needed; (2) clarifying reference beam selection, including whether it should always be the serving beam, any beam in the same cell, or a best-vs-secondary beam comparison; (3) assessing the need for stricter time alignment or Δt compensation methods given rapid variability of beam-level channels; and (4) defining how KPI evaluation should account for frequent intra-cell beam switching RAN4 is invited to address the impact of time misalignment (Δt) on inter-frequency relative RSRP accuracy. Two risks are noted: (1) Small error masking poor decisions, KPI can appear perfect even if Δt would lead the network to select a weaker cell; (2) Large error from timing artifacts , KPI may show large deviations caused by real channel changes over Δt, despite each value being accurate at its own time. To ensure the metric reflects true prediction/measurement quality rather than Δt effects, RAN4 should study whether to (i) enforce strict time alignment, (ii) allow a bounded Δt with exclusions, or (iii) apply compensation models for larger Δt. Clear Δt handling rules are needed for conformance testing RAN4 to adopt a layered anti-cheating framework for Alt-2 GT in FR1, with the primary method as a single-run twin-cell setup where the TE drives two synchronized cells , the UE predicts Cell-A and measures Cell-B simultaneously, and the setup includes documented channel-correlation methods plus a tolerance model for residual mismatch. At least one secondary check is required: (i) profile-comparison sweep with predefined design and pass/fail tolerance, or (ii) multi-UE collocated consistency with a specified δ per band/BW/test mode. RAN4 to adopt a measurement error with a compact four-term model. The first term is a slow UE-wide bias that is constant for one handset on a given carrier band. A second term adds a small residual unique to each transmit beam, whose correlation across beams is controlled by a single knob. The third term is fast thermal-and-estimator noise, redrawn for every measurement occasion with variance taken from the existing SINR-dependent link-level table or through LLS simulations. A fourth term introduces additional zero-mean variance proportional to a chosen neighbor-power metric, modelling interference. RAN4 to conduct simulation studies for calibration and to better align the frequency-domain channel correlation coefficients. This should be based on the modeling assumptions for large-scale and small-scale parameters across multiple frequencies, and antenna pattern differences. The goal is to create a consistent simulation baseline that can support more accurate evaluation of inter-frequency prediction models RAN4 is invited to study the impact of clustering on AI/ML-based inter-frequency measurement prediction performance. Multi-cell inputs provide richer spatial and frequency-domain information, improve robustness against anomalies in individual cell measurements, and enable better generalization across deployment scenarios. The study should assess clustering benefits under varying mobility conditions, network topologies, and correlation levels between cells, with the objective of determining whether clustering can reduce measurement overhead while maintaining or improving prediction accuracy RAN4 is invited to study the model robustness across scenarios of AI/ML models for AI/ML for mobility by evaluating three training approaches: (1) a universal model trained on a combined dataset of multiple deployment scenarios, (2) stand-alone models trained per scenario, and (3) a shared backbone model with lightweight scenario-specific experts. The study will assess performance on both training scenarios (Seen) and scenarios not used in training (Unseen) through systematic testing across diverse deployments. The outcome will clarify whether AI/ML for mobility requires many specialized models or if a shared architecture with minimal specialization can handle realistic deployment variations, guiding model deployment strategies and standardization in RAN4. RAN4 is invited to study the generalization capability of AI/ML-based inter-frequency measurement prediction models during the WI stage (Re-20). The study should evaluate how models trained on measurement data from specific inter-frequency deployment scenarios (e.g., macro–micro, FR1–FR1, FR1–FR2) perform when applied to unseen scenarios with different frequency layer combinations, cell layouts, and propagation environments. This includes assessing the impact of clustering multi-cell measurements as input features, the portability of models across varying network configurations, and the trade-off between prediction accuracy and measurement gap reduction. The conclusions, model designs, and deployment approaches validated for Case 1 (AI/ML for Beam Management) cannot be assumed valid for Case 6 (AI/ML for Mobility). Case 6 requires broader spatial awareness, cross-cell learning, and prediction mechanisms that address inter-frequency, inter-cell, and longer time-horizon mobility scenarios beyond the scope of beam management. It is therefore proposed to study these distinctions explicitly within the Re-20 6G WI, ensuring that AI/ML methods for mobility are evaluated under representative spatial, temporal, and multi-cell conditions to confirm their robustness and generalization capability across varied deployment scenarios. In case A temporal prediction, OW/PW length, UE speed, and MRRT affect accuracy, but the exact number of measurements in the OW is UE-proprietary. RAN4 should therefore base KPIs on standardized parameters (OW, PW, MRRT, UE speed) without considering internal L1/L3 counts. RAN4 is invited to agree on a set of pre-selected reference testing scenarios for temporal domain prediction case B, covering representative combinations of OW, PW, MRRT, and UE speed. This approach will limit the exponential growth of possible test configurations. RAN4 is invited to study whether MRRT configuration for temporal domain prediction should include visibility into the UE’s measurement skipping pattern. Two options are proposed: (1) Keep the pattern UE proprietary, with the TE applying only the MRRT value and verifying KPI compliance; or (2) Have the UE report supported patterns so the TE can explicitly configure one during testing. |
| R4-2509646 | Tejas Network Limited | Study of RAN4 impacts for RRM measurement prediction for AIML mobility in NR Observation 1: For FR2, the reported measurements will be used for ground truth extraction. Proposal 1: RAN4 to clarify the applicability of relative RSRP definition in the case FR2 testing. Proposal 2: RAN4 to clarify the applicability of relative RSRP definition in the case FR1 testing. Proposal 3: RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction. Proposal 4: RAN4 not study the impact of cluster-based and non-cluster-based inter-frequency prediction scenarios. Proposal 5: No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario. Observation 1: For FR2, the reported measurements will be used for ground truth extraction. |
| R4-2509750 | LG Electronics Inc. | Discussion of RAN4 impacts for RRM measurement prediction Proposal 1: RAN4 need to restrict the time difference between the two reported RSRPs and the additional accuracy margin caused by the measurement time difference also necessary. Proposal 2: Support proposal 1. Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes. Proposal 3: RAN4 to define one prediction accuracy requirements to cover 3 cell level sub-use cases. Proposal 4: Alternative 2 is more reasonable and feasible method for ground truth definition for RSRP accuracy for FR1. Proposal 5: RAN4 to consider only Gaussian error model for RF and BB error model. |
| R4-2509763 | Xiaomi | Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility Proposal 1: Don’t need to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy Observation 1: When timing mismatch increases to 200ms, RSRP difference increases to 5.5dB. Observation 2: Prediction interval for sliding and non-sliding widow is different, which will have impact on degerming the reported time instance. Proposal 2: Timing between predicated L3-RSRP and ground truth L3-RSRP needs to be aligned, how to align timing is highly dependent on reporting scheme designed by RAN2. If timing alignment is infeasible, incorporate additional margins into accuracy thresholds. Observation 3: L1 filtering sample number will have impact on deriving ideal L3-RSRP value from TE side. Observation 4: if 3 samples are used to derive L1 filtered RSRP, RSRP delta will be within 3dB for 90% cases when compared with that case that 5 samples are used. Proposal 3: L1 filtering number needs to be aligned, otherwise, extra margin needs to be considered. Observation 5: If the L3 filtering factor α is not zero, the L1/L3 filtering method will influence the derivation of the ideal L3-RSRP value from the TE side. Observation 6: At the same time T, ideal L3 filtered RSRP difference between non-sliding and sliding mode can be small than 1.8dB for k=4 in 90% cases. If K=0, no L3 filtering is applied and there is no RSRP difference. Proposal 4: L3 filtering impact can be removed by setting k=0, extra margin needs to be considered. Proposal 5: If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports. Test Equipment (TE) can employ multiple anti-cheating strategies such as: - Multi-instance channel scrambling: Randomizing the order of predefined channel sequences. - Insertion of redundant channels: Introducing randomly generated non-functional channels. - Randomized temporal offset: Reconfiguring channels at an unknown time offset within a specified Proposal 6: Observation window and prediction window defined in RAN2 will have RRM impact: - Observation window will have impact on measurement duration - Prediction window will have impact on scheduling restriction interval to disrupt UE's pattern recognition. Observation 7: Observation window and prediction length is dependent on different L1/L3 filtering method. Observation 8: Observation window and prediction length is dependent on different sub-cases. Observation 9: Observation window and prediction length is dependent on different patterns. Proposal 7: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window. Observation 10: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period. Proposal 8: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period. Observation 11: EPRE may not have impact on prediction accuracy if EPRE is relative static. Observation 12: The channel correlation coefficient is significantly influenced by factors such as antenna pattern differences, bandwidth variations, LOS probability variations, frequency scaling of large-scale parameters, cluster delays and angles, independent shadowing, and blockage. Observation 13: For cluster based approach, intra-frequency cluster Focuses on more about temporal relation while Inter-frequency cluster focus more about frequency domain relation. How to choose multiple cells in the same cluster will have impact on the final prediction accuracy. Observation 14: Extended measurement intervals degrade inter - frequency prediction performance by causing data obsolescence, which fails to capture rapid channel changes and results in inaccurate RSRP predictions. Proposal 9: RAN4 to study the factor and impact of measurement error on RRM prediction. |
| R4-2509922 | Nokia | On RAN4 Impacts for RRM measurement prediction in AIML Mobility Observation 1: It is expected Case-B temporal domain prediction with applied L3 filtering to include both combination of measured and predicted values and comply with accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement. Observation 2: Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE. Observation 3: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered by UE while performing measurements that serve as ground truth. Observation 4: The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT with multiple UE speed cases. Observation 5: For inter-frequency prediction scenario, it was agreed in RAN2 to use PCC to demonstrate the results. Also, it was agreed to restrict to co-located deployments only. Observation 6: RAN2 results demonstrate that the higher the CC is between two frequency layers, the higher the prediction accuracy. Besides, the prediction accuracy is comparable when predicting higher-to-lower and lower-to-higher frequency. Observation 7: R2407092 defines CC as measure of frequency distance. The results show that the higher the frequency distance the worse the prediction performance. Observation 8: Different aspects, e.g., beam pattern/configuration, channel model in each frequency, might have an impact on the currently used definition for CC. Hence, it might be necessary to redefine CC when such aspects are taken into consideration. Observation 9: In RAN4, there is no clear definition of CC. Unless RAN4 defines a common dataset, it is not feasible to study the impact of CC. Hence, it’s important to define CC for RAN4 requirements. Observation 10: For inter-frequency prediction scenario, it is possible that different beam patterns might show different performance. Therefore, RAN4 need to take this into account when defining the requirements. Proposal 1: For intra-frequency L3-RSRP relative accuracy definition, RAN4 to cover comparison between two RSRP levels from different beams in the same cell. Proposal 2: RAN4 to not consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy. It can be further investigated as a testability issue. Proposal 3: For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement. Proposal 4: Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions. Proposal 5: RAN4 to not consider number of L1/L3 measurements performed during OW and its impacts on the prediction accuracy. Proposal 6: For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed. Proposal 7: If skipping pattern information is not shared by the UE, RAN4 to only consider MRRT, OW and PW to UE as test configuration and set up a requirement on prediction accuracy. Proposal 8: If skipping pattern information is shared by the UE, TE should select the supported skipping pattern related to the MRRT to test. The test configuration should then include skipping pattern, OW and PW. Proposal 9: For inter-frequency prediction scenario, RAN4 should consider the RAN2 definition of correlation coefficient (CC) as a starting point. Proposal 10: In RAN2, inter-frequency prediction focuses on the co-located scenario. Hence, RAN4 can take the same assumption to define the requirements. Proposal 11: In inter-frequency prediction scenario, it is possible that different/same frequency pairs might have different CCs in co-located and/or non-co-located scenario. Hence, RAN4 requirements can be defined accordingly. Proposal 12: To define correlation coefficient (CC) for inter-frequency predictions, RAN4 to consider the channel models in different frequency layers. In this case, one way to define the CC is the correlation between PDPs of different frequency layers. |
| R4-2510168 | CMCC | Discussion on RAN4 impacts for RRM measurement prediction Proposal 1: it is proposed not to consider the time difference between the two reported RSRPs in the definition of relative RSRP accuracy. If needed, the time difference between the two reported RSRPs can be consdiered in the test design. Proposal 2: it is proposed that the accuracy requirements for predicted RSRP is no less than legacy accuracy requirements. Proposal 3: for impact on measurement error on prediction accuracy, the discussion on beam prediction can be used as baseline. In detail, the BB error can be derived based on link level simulation assumption, and RF error can be modelled as truncated Gaussian distribution. Proposal 4: for cluster approach, it is proposed to follow RAN2 agreements that model takes measurements from more than one cell as inputs. For cluster approach, not to consider the case that measurement from single cell in one carrier frequency is used as an model input. Proposal 5: it is proposed to discuss whether to introduce RAN4 requirements about MRRT and MRRS. Proposal 6: for RRM measurement prediction, the measurement delay requirements need to be defined/updated - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured, as a result, SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy - For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements |
| R4-2509673 | OPPO | Study of impacts on RAN4 requirements for RRM measurement prediction Proposal 1: Keep the existing relative RSRP accuracy definition in the TP and leave the two FFS parts to WI stage. Proposal 2: Use Alt2 (i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1. Proposal 3: Take the following option 1 as baseline to model the measurement error and the details can be left to WI when discussing the simulation: - Option 1: - BB error: - Use link level simulation assumptions to generate L3-RSRP difference as baseband error. - RF error: - Alt1: Modelled as uniform distribution - Alt2: Modelled as truncated Gaussian distribution Under ±[X] dB RF error (μ=0, σ^2=[X]). Proposal 4: Further discuss during WI stage: - The detailed assumptions for system level and link level simulation - Whether and how to simulate different SINR values for deriving baseband error - Whether and how to align the procedure for adding different errors on the ideal dataset Proposal 5: Discuss the measurement period requirements in WI stage considering: - the impact of OW/PW length and the skipped measurement instances (e.g., based on MRRT) for intra-frequency measurement. - the impact of skipped measurement frequencies or instances on CSSF calculation for inter-frequency measurement. Proposal 6: Leave the LCM related requirements discussion to WI stage and wait for more progress from RAN2 AI mobility. Proposal 7: Take the TP in [4 R4-2509672] as baseline and further refine based on the conclusions in this meeting. |
| R4-2510444 | ZTE Corporation, Sanechips | - Study of RAN4 impacts for RRM measurement prediction - Proposal 1: If down selection is needed, prefer to use Alt 2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1. Meanwhile, how to avoid UE cheating can wait the progress in R19 AI beam management test. - Proposal 2：RF and baseband measurement error modeling need to be considered. And RAN4 can use the outcome of the error modeling approach in AI/ML for NR Air Interface as baseline. - Proposal 3: RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed. - Proposal 4: Compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements. And RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility. |
| R4-2510763 | Ericsson | On requirements for RRM measurement prediction for AI/ML based mobility Observation 1: For a fixed MRRT, prediction accuracy for temporal prediction case-B may differ based on the measurement skipping pattern within OW. Proposal 1: RAN4 to consider the impact of measurement skipping pattern within the OW on prediction accuracy requirements for the temporal prediction case-B for a fixed MRRT. Proposal 2: Relative accuracy of predicted L3-RSRP measurements for intra-cell scenario (temporal prediction) is defined as: - Relative accuracy of predicted L3-RSRP measurement = (reported predicted L3-RSRP1 of a cell at time T1 – reported L3-RSRP2 of the same cell at time T2) – (ground truth of L3-RSRP1 of a cell at time T1 – ground truth of L3-RSRP2 of the same cell at time T2). - Reported L3-RSRP2 can be predicted or measured. - T1 refers to time instance for predicted L3-RSRP1. - T2 refers to time instance for measured L3-RSRP2 or predicted L3-RSRP2 Proposal 3: RAN4 to conclude on the time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy for intra and inter frequency L3-RSRP predictions in inter-cell scenario after the reporting framework for predicted L3-RSRP measurement is concluded in RAN2. |
| R4-2510487 | vivo | Discussion on RAN4 impacts for RRM measurement prediction Proposal 1: On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline: - For BB error modeling: - Link-level simulation approach is considered - For RF error, - For error modeling, the following two modelings can be the candidates. Further down-selection can be done in WI phase - Alt 1: Uniform distribution - Alt 2: Truncated Gaussian distribution(μ=0, σ^2=[X]), value [X] can be further determined respectively for FR1 and FR2 - RF error application, each of Tx beam is independent Proposal 2: The measurement error analysis can consider the impact on prediction performance - [Case 1]: when errors are only added to the model input during the prediction phase - [Case 2]: When errors are added to the model input both during both the training phase and prediction phase - Whether to consider adding errors to the ground truth depends on the discussion on the definition of ground truth |
| R4-2510706 | NTT DOCOMO, INC. | Discussions on requirements for RRM measurement prediction on AIML mobility Observation 1: Relative RSRP evaluation doesn’t depend on time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy. Proposal 1: Relative accuracy of predicted L3-RSRP for intra-frequency L3-RSRP can be specified without considering time difference between the two reported RSRPs. |
| R4-2511516 | Qualcomm Incorporated | Impact of AI based mobility on RRM measurement prediction Definition of ground truth for AI/ML-based RRM measurement prediction Proposal 1: For the definition of ground truth in L3-RSRP prediction, RAN4 to further clarify the two alternatives agreed upon at RAN4#114, as below: - For Alt 1 “Ground truth for FR1 will be based on the transmitted or reception power (agreed at RAN4#114)” - Rephrase Alt 1 as “Ground truth is defined as the expected measurement result” - Add the following clarifications: - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate. - The value of the margin to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc. needs a further study, and it can vary for different measurement configurations, etc. - For Alt 2 “Ground truth for FR1 will be based on the reported measurement value under certain conditions (agreed at RAN4#114)” - Rephrase Alt 2 as “Ground truth is defined as the measurement report from the UE” - Add the following clarifications: - The UE measurement reports used as ground truth are collected under the same channel fading conditions as those used for the L3-RSRP prediction reports. - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc. - The channel power of the target prediction reference signal remains stable within ±Z dB during the time window from X - Y ms to X + Y ms. - The S(I)NR of the target cell during the ground truth measurement may or may not be the same as that used during AI/ML based prediction result measurement. - The above ground truth definition of L3-RSRP is used to define the following prediction accuracy metrics, where applicable: - Absolute L3-RSRP prediction accuracy - Relative L3-RSRP prediction accuracy - Measurement even prediction accuracy Proposal 2: The test method for collecting UE measurement reports to serve as the Alt 2-based ground truth should meet the following criteria: - The approach is scalable across all use cases, including: - Intra-frequency temporal domain RRM measurement prediction (based on Case A and Case B), - Inter-frequency frequency domain RRM measurement prediction, - Measurement event prediction. - It is applicable to both FR1 and FR2. - It is applicable to both UE sided and NW sided models. - It is applicable to LCM performance test (AI/ML performance monitoring at both UE and NW sides) Proposal 3: For the Alt 2-based ground truth measurement during performance testing, the following method can be considered: - Step 1: Conduct two rounds of testing under a fading channel without AI/ML enabled to determine the testing SNR to be used in Step 2. - Both rounds should collect UE measurement reports with the same periodicity for the target cell and reporting configuration. - The two rounds differ by SNR: - The first round is conducted with a sufficiently high SNR. - The second round uses a lower SNR than the first. - Based on the L3-RSRP differences reported for the same or closely timed instances between the two rounds, RAN4 can determine an appropriate testing SNR for AI/ML-based RRM measurement/event prediction. - Step 2: The performance of AI/ML-based prediction is then evaluated under the testing SNR selected in Step 1. - The ground truth values for the prediction results are defined as the measurement reports from the second round in Step 1, matched to the same or closely timed instances. - Due to the dynamic nature of fading channel evolution, the above procedures begin at a specific time instance when the fading channel becomes active. The concept of 'same or closely timed instances' across different rounds applies starting from that time instance. Performance metrics for AI/ML-based RRM measurement prediction Proposal 4: RAN4 to consider the following as one of the performance metrics for AI/ML-based L3-RSRP prediction: - Top-1 (%): The percentage of ‘the Top-1 strongest cell is Top-1 predicted cell’ - Top-K/1 (%): The percentage of ‘the Top-1 strongest cell is one of the Top-K predicted cells’ - Top-1/K (%): The percentage of ‘the Top-1 predicted cell is one of the Top-K strongest cells’ - Top-K (%): The percentage of the maximum ground truth L3-RSRP among the Top-K predicted cells being larger than ‘the ground truth L3-RSRP of the strongest cell’ - X dB - A successful rate for the correct prediction which is considered as the ground-truth L3-RSRP of Top-1 predicted L3-RSRP >= the strongest ground-truth L3-RSRP of the strongest genie-aided L3-RSRP – X dB - Note: Top-1 strongest cell and Top-K strongest cells are determined by the metric of ground truth L3-RSRP. Potential impact on RRM requirements Proposal 5: The potential impacts on RRM core requirements due to AI/ML-based RRM measurement/event prediction and LCM/performance monitoring include, but are not necessarily limited to, the following: - Prediction report latency including model activation/switching, inference delay, etc. - Monitoring performance - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc. - Measurement and scheduling restrictions in the observation and prediction windows - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation. - Side conditions such as requirement applicability rules. - Overall budget in terms of mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements. - CSSF - MG sharing mechanism among different inter-frequency layers |

## Open issues summary

In this topic different companies’ proposals about the RRM requirements for RRM Measurements prediction use case are summarized.

### Issue 2-1: Impacts of measurement error on prediction accuracy

- Background:

The following agreement was reached in RAN4#114 meeting:

Agreement: For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

Furthermore, following WF was suggested in RAN4#114bis meeting:

WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

- Proposals and Observations:

  - Proposal 1 (vivo): On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline:

    - For BB error modeling:

      - Link-level simulation approach is considered

    - For RF error,

      - For error modeling, the following two modelings can be the candidates. Further down-selection can be done in WI phase

        - Alt 1: Uniform distribution

        - Alt 2: Truncated Gaussian distribution(μ=0, σ^2=[X]), value [X] can be further determined respectively for FR1 and FR2

    - RF error application, each of Tx beam is independent

  - Proposal 2 (vivo): The measurement error analysis can consider the impact on prediction performance

    - [Case 1]: when errors are only added to the model input during the prediction phase

    - [Case 2]: When errors are added to the model input both during both the training phase and prediction phase

    - Whether to consider adding errors to the ground truth depends on the discussion on the definition of ground truth

  - Proposal 3 (CMCC): for impact on measurement error on prediction accuracy, the discussion on beam prediction can be used as baseline. In detail, the BB error can be derived based on link level simulation assumption, and RF error can be modelled as truncated Gaussian distribution.

  - Proposal 4 (Xiaomi): RAN4 to study the factor and impact of measurement error on RRM prediction.

  - Proposal 5 (LG Electronics Inc.): RAN4 to consider only Gaussian error model for RF and BB error model.

  - Proposal 6 (Apple): RAN4 to adopt a measurement error with a compact four-term model. The first term is a slow UE-wide bias that is constant for one handset on a given carrier band. A second term adds a small residual unique to each transmit beam, whose correlation across beams is controlled by a single knob. The third term is fast thermal-and-estimator noise, redrawn for every measurement occasion with variance taken from the existing SINR-dependent link-level table or through LLS simulations. A fourth term introduces additional zero-mean variance proportional to a chosen neighbor-power metric, modelling interference.

  - Proposal 7 (CATT): RAN4 to evaluate the impact of baseband error using link-level simulation with different SNR levels considered. For RF error, a uniform or Gaussian distribution can be used.

  - Proposal 8 (Nokia): RAN4 to capture the following error modelling principles in the TR:

    - RF and baseband measurement error modelling may be considered.

    - A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband.

    - Alternative modelling approaches (such as based on link-level simulations) are not precluded and can be explored in the Work Item phase.

  - Proposal 9 (MediaTek): For AI mobility, in FR1, measurement error should be added as:

    - BB error:

      - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR.

      - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement.

    - RF error:

      - Modelled as Gaussian distribution under ±3 dB RF error (μ=0, σ2=3).

      - A random RF error is generated for all the beams of the same frequency layer per SMTC.

  - Proposal 10 (Ericsson): Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements.

    - RF error in dB for FR1 is characterized by Gaussian distribution with μ=0, σ2= 2.5 truncated at ±2.5.

    - RF error in dB for FR2 is characterized by Gaussian distribution with μ=0, σ2= 4 truncated at ±4.

  - Proposal 11 (OPPO): Take the following option 1 as baseline to model the measurement error and the details can be left to WI when discussing the simulation:

    - Option 1:

      - BB error:

        - Use link level simulation assumptions to generate L3-RSRP difference as baseband error.

      - RF error:

        - Alt1: Modelled as uniform distribution

        - Alt2: Modelled as truncated Gaussian distribution Under ±[X] dB RF error (μ=0, σ^2=[X]).

  - Proposal 12 (OPPO): Further discuss during WI stage:

    - The detailed assumptions for system level and link level simulation

    - Whether and how to simulate different SINR values for deriving baseband error

    - Whether and how to align the procedure for adding different errors on the ideal dataset

  - Proposal 13 (Samsung): Regarding measurement error model on prediction accuracy in AI mobility:

    - For RF error, take the outcome of the error modelling approach in PHY AI/ML as a baseline

    - For BB error, LLS approach involving SNR/SINR is considered to determine the distribution

    - Different properties of channel propagation across neighbor cells would be captured as a factor

  - Proposal 14 (Huawei): Use LLS to generate the measurement error model for each SINR, where the error is calculated by comparing the L1-filtered and L3-filtered of the measured L1-RSRP versus L3-RSRP ground truth.

  - Proposal 15 (Huawei): Use Gaussian distribution to model RF error, the mean value and variance of which will be decided during WI.

- Recommended WF:

RAN4 to capture the following factors to be considered for error models to consider the impact of measurement errors on prediction accuracy:

- RF and baseband measurement error modeling may be considered.

- For BB error, following AI/ML beam management approach, use link level simulation assumptions to generate L3-RSRP difference as baseband error.

- For RF error model, use a uniform or a Gaussian distribution.

- Alternative modeling approaches are not precluded and can be explored in the Work Item phase.

### Issue 2-2: Relative RSRP accuracy definition

- Background:

Following agreement was reached in RAN4#115 meeting:

Agreement: - For intra-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress. - For inter-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

- Proposals and Observations:

  - Proposal 1 (Ericsson): RAN4 to conclude on the time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy for intra and inter frequency L3-RSRP predictions in inter-cell scenario after the reporting framework for predicted L3-RSRP measurement is concluded in RAN2.

  - Proposal 2 (OPPO): Keep the existing relative RSRP accuracy definition in the TP and leave the two FFS parts to WI stage.

  - Proposal 3 (Samsung, Huawei, Xiaomi): There is no need to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy in AI mobility

  - Proposal 4 (NTT DOCOMO, INC.): Relative accuracy of predicted L3-RSRP for intra-frequency L3-RSRP can be specified without considering time difference between the two reported RSRPs.

  - Proposal 5 (Nokia): RAN4 to not consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy. It can be further investigated as a testability issue.

  - Proposal 6 (LG Electronics Inc.): RAN4 need to restrict the time difference between the two reported RSRPs and the additional accuracy margin caused by the time difference measurement, also necessary.

  - Proposal 7 (Tejas Network Limited): RAN4 to clarify the applicability of relative RSRP definition in the case FR2 testing.

  - Proposal 8 (Tejas Network Limited): RAN4 to clarify the applicability of relative RSRP definition in the case FR1 testing.

  - Proposal 9 (Apple): A similar definition of relative RSRP accuracy exists in legacy, but it does not account for any time alignment. As per the FFS item on whether and how to consider the time difference between the two reported RSRPs when applying this definition, RAN4 should clarify what differs in the AI/ML case compared to legacy.

    - RAN4 is invited to consider that if the definition of intra-frequency relative RSRP accuracy allows for a non-zero time difference between the two RSRPs used in the calculation, either explicit timestamps should be required as part of the conformance testing framework to ensure traceable and bounded time alignment, or the reporting periodicity and measurement alignment rules should be clearly defined to implicitly control the maximum allowed time difference (Δt). This will ensure that any variation in the relative accuracy metric reflects the prediction or measurement performance, rather than uncontrolled environmental changes due to asynchronous reporting or prediction instances.

    - RAN4 is invited to study mechanisms for handling time differences between the two RSRP values used in intra-frequency relative RSRP accuracy calculations. The following options are proposed:

      - Option 1: Enforce strict time alignment, requiring both RSRPs to be measured or predicted at the same or near-identical time.

      - Option 2: Define a fixed maximum Δt threshold, excluding RSRP pairs exceeding this limit from KPI evaluation.

      - Option 3: Use scenario-dependent Δt thresholds based on UE mobility class or deployment scenario to better match realistic radio dynamics.

      - Option 4: Permit post-processing compensation for larger Δt by applying a bounded correction factor that accounts for expected RSRP variation over time, allowing more relaxed alignment constraints while preserving KPI validity

- Candidate Options:

  - Option 1: No need to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy.

  - Option 2: Consider time difference between the reported RSRP for intra-frequency case when applying the definition of the relative RSRP accuracy.

  - Option 3: Consider time difference between the reported RSRP for intra-frequency and inter-frequency cases when applying the definition of the relative RSRP accuracy.

  - Option 4: Keep the existing relative RSRP accuracy definition in the TP and leave the two FFS parts to WI stage

  - Option 5: Use following options for handling time differences between the two RSRP values used in intra-frequency relative RSRP accuracy calculations:

    - Option 5a: Enforce strict time alignment, requiring both RSRPs to be measured or predicted at the same or near-identical time.

    - Option 5b: Define a fixed maximum Δt threshold, excluding RSRP pairs exceeding this limit from KPI evaluation.

    - Option 5c: Use scenario-dependent Δt thresholds based on UE mobility class or deployment scenario to better match realistic radio dynamics.

    - Option 5d: Permit post-processing compensation for larger Δt by applying a bounded correction factor that accounts for expected RSRP variation over time, allowing more relaxed alignment constraints while preserving KPI validity

- Recommended WF:

RAN4 to discuss the above-listed candidate options.

### Issue 2-3: Relative RSRP accuracy definition (temporal prediction)

- Proposal 1 (Ericsson): Relative accuracy of predicted L3-RSRP measurements for intra-cell scenario (temporal prediction) is defined as:

- Relative accuracy of predicted L3-RSRP measurement = (reported predicted L3-RSRP1 of a cell at time T1 – reported L3-RSRP2 of the same cell at time T2) – (ground truth of L3-RSRP1 of a cell at time T1 – ground truth of L3-RSRP2 of the same cell at time T2).

  - Reported L3-RSRP2 can be predicted or measured.

  - T1 refers to time instance for predicted L3-RSRP1.

  - T2 refers to time instance for measured L3-RSRP2 or predicted L3-RSRP2.

- Recommended WF:

  - Discuss proposal 1.

### Issue 2-4: Impacts on beam/cell level measurements

- Background:

The following agreement was reached in RAN4#115 meeting:

Agreement: Consider L3 beam-level (Point E) measurement prediction depending upon the progress in RAN2. Note: RAN4 didn’t study impact of L3 beam level measurements. However, it doesn’t preclude the discussion on L3 beam level measurements in the work item phase.

Furthermore, following agreement was reached in RAN4#114bis meeting:

Agreement: - For Rel. 19 study: - RAN4 to not consider L1 beam-level (Point A1) measurement prediction.

Furthermore, the following agreement was achieved in RAN4#112bis meeting:

Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

The figure below from TS 38.300 (Figure 9.2.4-1) should be considered as a reference for this discussion.

- Proposals and Observations:

  - Proposal 1 (Apple): RAN4 is invited to study the definition and testing of relative RSRP accuracy at the beam level for intra-frequency scenarios, focusing on four key areas: (1) determining whether existing cell-level thresholds, filtering, and reporting rules are suitable for the faster dynamics of beam-level RSRP or if tighter beam-specific definitions are needed; (2) clarifying reference beam selection, including whether it should always be the serving beam, any beam in the same cell, or a best-vs-secondary beam comparison; (3) assessing the need for stricter time alignment or Δt compensation methods given rapid variability of beam-level channels; and (4) defining how KPI evaluation should account for frequent intra-cell beam switching.

  - Proposal 2 (LG Electronics Inc.): RAN4 to define one prediction accuracy requirements to cover 3 cell level sub-use cases.

  - Proposal 3 (Nokia): For intra-frequency L3-RSRP relative accuracy definition, RAN4 to cover comparison between two RSRP levels from different beams in the same cell.

  - Proposal 4 (Huawei, HiSilicon): Due to limited time in SI, Relative RSRP accuracy for Beam level measurement can be not be considered in R19.

- Candidate Options:

  - Option 1: RAN4 to not consider beam level impacts in the SI.

  - Option 2: RAN4 to consider beam level impacts only in the WI phase.

  - Option 3: RAN4 to agree on the definition of intra-frequency L3-RSRP relative accuracy definition for two different beams.

    - Option 3a: Use the agreed definition for cell level L3-RSRP relative accuracy definition as the baseline.

- Recommended WF:

RAN4 to discuss the above-listed non mutually exclusive options.

### Issue 2-5: Performance metric for AI/ML based RRM measurement prediction

- Proposals and Observations:

  - Proposal 1 (Qualcomm): RAN4 to consider the following as one of the performance metrics for AI/ML-based L3-RSRP prediction:

    - Top-1 (%): The percentage of ‘the Top-1 strongest cell is Top-1 predicted cell’

    - Top-K/1 (%): The percentage of ‘the Top-1 strongest cell is one of the Top-K predicted cells’

    - Top-1/K (%): The percentage of ‘the Top-1 predicted cell is one of the Top-K strongest cells’

    - Top-K (%): The percentage of the maximum ground truth L3-RSRP among the Top-K predicted cells being larger than ‘the ground truth L3-RSRP of the strongest cell’ - X dB

    - A successful rate for the correct prediction which is considered as the ground-truth L3-RSRP of Top-1 predicted L3-RSRP >= the strongest ground-truth L3-RSRP of the strongest genie-aided L3-RSRP – X dB

    - Note: Top-1 strongest cell and Top-K strongest cells are determined by the metric of ground truth L3-RSRP.

- Candidate Options:

  - Option 1: RAN4 to not consider any other performance metric than Absolute and Relative accuracies of predicted RSRPs.

  - Option 2: RAN4 to consider above mentioned metrics in Proposal 1 in addition to absolute and relative accuracies of predicted RSRPs.

- Recommended WF:

Option 1?

### Issue 2-6: Accuracy requirements for the predicted RSRPs

- Proposals and Observations:

  - Proposal 1 (Huawei, HiSilicon): For prediction accuracy, in R19 SI phase it is premature to draw conclusion on define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases; whether the prediction accuracy shall at least meet the accuracy requirements that are defined for the non-AI/ML RRM measurement.

  - Proposal 2 (CMCC): it is proposed that the accuracy requirements for predicted RSRP is no less than legacy accuracy requirements.

  - Proposal 3 (Nokia): For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement.

  - Proposal 4 (LG Electronics Inc.): Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes.

  - Proposal 5 (CATT): For scenario 4 (2nd study goal), the predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurements for mobility purposes.

  - Proposal 6 (CATT): For scenarios with 1st study goal, RAN4 discuss whether a relaxed prediction accuracy requirement is defined.

  - Proposal 7 (MediaTek):

    - The performance with different L3 filtering configurations diverge.

    - Further discuss how to define RSRP accuracy requirements for different L3 filtering configurations.

- Candidate Options:

  - Option 1: For temporal domain Case-B, consider same requirement targets as in legacy

  - Option 2: For 2nd study goal (improved handover performance), consider same requirement targets as in legacy

  - Option 3: For 1st study goal (measurement reduction), consider relaxing accuracy requirements.

  - Option 4: Accuracy requirements for predicted RSRPs shouldn’t be less than legacy accuracy targets.

  - Option 5: Not enough information available to conclude on this issue. Postpone this discussion to WI phase.

- Recommended WF:

RAN4 to discuss the above-listed candidate options.

### Issue 2-7: Ground truth definition for RSRP Accuracy for FR1

- Background:

Following agreement was reached in RAN4#114bis meeting:

Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Furthermore, the following agreement was reached in RAN4#114 meeting:

- Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

- Proposals and Observations:

  - Proposal 1 (ZTE Corporation, Sanechips): If down selection is needed, prefer to use Alt 2(i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1. Meanwhile, how to avoid UE cheating can wait the progress in R19 AI beam management test

  - Proposal 2 (OPPO): Use Alt2 (i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1.

  - Proposal 3 (Qualcomm): For the definition of ground truth in L3-RSRP prediction, RAN4 to further clarify the two alternatives agreed upon at RAN4#114, as below:

    - For Alt 1 “Ground truth for FR1 will be based on the transmitted or reception power (agreed at RAN4#114)”

      - Rephrase Alt 1 as “Ground truth is defined as the expected measurement result”

      - Add the following clarifications:

        - The ground truth of L3-RSRP predicted for the time instance X is derived based on the expected reception power of the reference signal at UE assuming the measurement rate defined in RRM spec, plus a margin to accommodate uncertainties in the actual UE L3 measurement rate.

        - The value of the margin to accommodate the measurement error due to noisy channel, RF calibration error, UE Rx beamforming gain, etc. needs a further study, and it can vary for different measurement configurations, etc.

    - For Alt 2 “Ground truth for FR1 will be based on the reported measurement value under certain conditions (agreed at RAN4#114)”

      - Rephrase Alt 2 as “Ground truth is defined as the measurement report from the UE”

      - Add the following clarifications:

        - The UE measurement reports used as ground truth are collected under the same channel fading conditions as those used for the L3-RSRP prediction reports.

        - The ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X.

        - Y can be differently defined depending on factors such as UE mobility speed, channel conditions, the distance between the UE and the target cell, etc.

        - The channel power of the target prediction reference signal remains stable within ±Z dB during the time window from X - Y ms to X + Y ms.

        - The S(I)NR of the target cell during the ground truth measurement may or may not be the same as that used during AI/ML based prediction result measurement.

    - The above ground truth definition of L3-RSRP is used to define the following prediction accuracy metrics, where applicable:

      - Absolute L3-RSRP prediction accuracy

      - Relative L3-RSRP prediction accuracy

      - Measurement even prediction accuracy

  - Proposal 4 (Qualcomm): The test method for collecting UE measurement reports to serve as the Alt 2-based ground truth should meet the following criteria:

    - The approach is scalable across all use cases, including:

      - Intra-frequency temporal domain RRM measurement prediction (based on Case A and Case B),

      - Inter-frequency frequency domain RRM measurement prediction,

      - Measurement event prediction.

    - It is applicable to both FR1 and FR2.

    - It is applicable to both UE sided and NW sided models.

    - It is applicable to LCM performance test (AI/ML performance monitoring at both UE and NW sides)

  - Proposal 5 (Qualcomm): For the Alt 2-based ground truth measurement during performance testing, the following method can be considered:

    - Step 1: Conduct two rounds of testing under a fading channel without AI/ML enabled to determine the testing SNR to be used in Step 2.

      - Both rounds should collect UE measurement reports with the same periodicity for the target cell and reporting configuration.

      - The two rounds differ by SNR:

        - The first round is conducted with a sufficiently high SNR.

        - The second round uses a lower SNR than the first.

      - Based on the L3-RSRP differences reported for the same or closely timed instances between the two rounds, RAN4 can determine an appropriate testing SNR for AI/ML-based RRM measurement/event prediction.

    - Step 2: The performance of AI/ML-based prediction is then evaluated under the testing SNR selected in Step 1.

      - The ground truth values for the prediction results are defined as the measurement reports from the second round in Step 1, matched to the same or closely timed instances.

    - Due to the dynamic nature of fading channel evolution, the above procedures begin at a specific time instance when the fading channel becomes active. The concept of 'same or closely timed instances' across different rounds applies starting from that time instance.

  - Proposal 6 (Huawei, HiSilicon): If the ground truth in FR1 is defined based on the TE transmitted or reception power (Alter 1), the following aspects are obstacles:

    - The specific position in time domain of UE obtaining a L3 predicted result is unknown to TE.

    - The number of L1 measurement samples is unknown to TE.

  - Proposal 7 (Huawei, HiSilicon): The ground truth in FR1 can be defined based on the reported measurement value under certain conditions.

  - Proposal 8 (Nokia): Alternative 2, where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

  - Proposal 9 (Xiaomi): If the ground truth of L3-RSRP is reported by the UE, RAN4 should discuss whether and how to prevent the UE from providing cheating reports. Test Equipment (TE) can employ multiple anti-cheating strategies such as:

    - Multi-instance channel scrambling: Randomizing the order of predefined channel sequences.

    - Insertion of redundant channels: Introducing randomly generated non-functional channels.

    - Randomized temporal offset: Reconfiguring channels at an unknown time offset within a specified

  - Proposal 10 (LG Electronics Inc.): Alternative 2 is more reasonable and feasible method for ground truth definition for RSRP accuracy for FR1.

  - Proposal 11 (Apple): RAN4 to adopt a layered anti-cheating framework for Alt-2 GT in FR1, with the primary method as a single-run twin-cell setup where the TE drives two synchronized cells , the UE predicts Cell-A and measures Cell-B simultaneously, and the setup includes documented channel-correlation methods plus a tolerance model for residual mismatch. At least one secondary check is required: (i) profile-comparison sweep with predefined design and pass/fail tolerance, or (ii) multi-UE collocated consistency with a specified δ per band/BW/test mode.

  - Proposal 12 (MediaTek):

    - If to derive the ground truth through transmitted power at TE, additional margin should be considered due to different L1 filtering and L3 filtering may be adopted by UE vendors. The value of the margin can be obtained through simulations.

    - If to use reported RSRP as ground truth, the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. The value of Y is relevant to UE implementation.

    - During the SI, RAN4 not to do down-selection on the two Alternatives of ground truth definition.

- Recommended WF:

RAN4 to use Alternative 2 (reported measurement value under certain conditions) as baseline to define the ground truth for absolute and relative accuracies for predicted RSRPs in FR1.

### Issue 2-8: Impacts from observation and prediction windows

- Background:

The following agreement was reached in RAN4#114 meeting:

Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

- Proposals and Observations:

  - Proposal 1 (Ericsson): RAN4 to consider the impact of measurement skipping pattern within the OW on prediction accuracy requirements for the temporal prediction case-B for a fixed MRRT

  - Proposal 2 (ZTE Corporation, Sanechips): RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed

  - Proposal 3 (OPPO): Discuss the measurement period requirements in WI stage considering:

    - the impact of OW/PW length and the skipped measurement instances (e.g., based on MRRT) for intra-frequency measurement.

    - the impact of skipped measurement frequencies or instances on CSSF calculation for inter-frequency measurement

  - Proposal 4 (Samsung): For the RRM impact on prediction window, at least the following may need to be discussed

    - Scheduling restriction

    - Measurement latency for MO (s), MO (s) reduction

  - Proposal 5 (Huawei, HiSilicon): For intra-frequency temporal domain prediction case A, RSRP prediction accuracy is to be defined under the side condition of certain OW and PW length.

  - Proposal 6 (Huawei, HiSilicon): To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing.

  - Proposal 7 (Huawei, Hi Silicon): For intra-frequency temporal domain prediction case B, RSRP prediction accuracy is to be defined under the side condition of MRRT with limited PW length.

  - Proposal 8 (CMCC): it is proposed to discuss whether to introduce RAN4 requirements about MRRT and MRRS

  - Proposal 9 (Nokia): RAN4 to not consider number of L1/L3 measurements performed during OW and its impacts on the prediction accuracy.

  - Proposal 10 (Nokia): For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as UE speed.

  - Proposal 11 (Nokia): If skipping pattern information is not shared by the UE, RAN4 to only consider MRRT, OW and PW to UE as test configuration and set up a requirement on prediction accuracy.

  - Proposal 12 (Nokia): If skipping pattern information is shared by the UE, TE should select the supported skipping pattern related to the MRRT to test. The test configuration should then include skipping pattern, OW and PW.

  - Proposal 13 (Xiaomi): Observation window and prediction window defined in RAN2 will have RRM impact:

    - Observation window will have impact on measurement duration

    - Prediction window will have impact on scheduling restriction interval to disrupt UE's pattern recognition.

  - Proposal 13 (Xiaomi): For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

  - Proposal 13 (CATT): RAN4 to discuss the impact of UE speed, MRRT and OW and PW lengths in the work item phase unless any issues are identified to be discussed.

  - Proposal 14 (Apple): In case A temporal prediction, OW/PW length, UE speed, and MRRT affect accuracy, but the exact number of measurements in the OW is UE-proprietary. RAN4 should therefore base KPIs on standardized parameters (OW, PW, MRRT, UE speed) without considering internal L1/L3 counts

  - Proposal 15 (Apple): RAN4 is invited to agree on a set of pre-selected reference testing scenarios for temporal domain prediction case B, covering representative combinations of OW, PW, MRRT, and UE speed. This approach will limit the exponential growth of possible test configurations.

  - Proposal 16 (Apple): RAN4 is invited to study whether MRRT configuration for temporal domain prediction should include visibility into the UE’s measurement skipping pattern. Two options are proposed: (1) Keep the pattern UE proprietary, with the TE applying only the MRRT value and verifying KPI compliance; or (2) Have the UE report supported patterns so the TE can explicitly configure one during testing.

- Candidate Options:

  - For the impact of number of measurements in the OW

    - Option 1: RAN4 to not consider number of L1/L3 measurements performed during OW and its impacts on the prediction accuracy.

    - Option 2: keep the earlier agreement unchanged.

  - Impact of MRRT, OW, PW and the skipping pattern

    - Option 1: Further discuss the impact of MRRT, OW and PW during the WI phase.

    - Option 2: RAN4 to identify reference testing configuration for temporal domain Case B relating to OW, PW and MRRT as well as UE speed

      - If the skipping pattern is shared by the UE, RAN4 to also consider selecting the supported skipping pattern related to the MRRT to test

    - Option 3:

      - For intra-frequency temporal domain prediction case A, RSRP prediction accuracy is to be defined under the side condition of certain OW and PW length.

        - To guarantee prediction accuracy for FR2 intra-frequency temporal domain case A, limited doppler shift, limited PW length and proper OW length shall be set in testing

      - For intra-frequency temporal domain prediction case B, RSRP prediction accuracy is to be defined under the side condition of MRRT with limited PW length.

- Recommended WF:

RAN4 to discuss the above-listed candidate options.

### Issue 2-9: Inter-frequency prediction scenario

- Background:

The following agreement was reached in RAN4#115 meeting:

Agreement: RAN4 to study whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

Furthermore, the following agreement was reached in RAN4#114bis meeting:

Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

- Proposals and Observations:

  - Proposal 1 (Qualcomm): RAN4 to consider the impact on MG sharing mechanism among different inter-frequency layers

  - Proposal 2 (Qualcomm): RAN4 to consider the impact on Carrier-Specific Scaling Factor (CSSF) in case of inter-frequency prediction

  - Proposal 3 (Samsung): RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction and CSSF enhancement for inter-frequency prediction

  - Proposal 4 (Huawei, HiSilicon): For inter-frequency inter-cell frequency domain prediction, RSRP prediction accuracy in collocated scenario can be specified under certain channel correlation coefficient, e.g., ~0.9.

  - Proposal 5 (Huawei, HiSilicon): For inter-frequency inter-cell frequency domain prediction, RAN4 to discuss:

    - Specify RSRP prediction accuracy for cluster approach and single cell approach separately, or

    - Specify minimum RSRP prediction accuracy to accommodate both cluster and single cell approach

  - Proposal 6 (CMCC): for cluster approach, it is proposed to follow RAN2 agreements that model takes measurements from more than one cell as inputs.  For cluster approach, not to consider the case that measurement from single cell in one carrier frequency is used as an model input.

  - Proposal 7 (Nokia): For inter-frequency prediction scenario, RAN4 should consider the RAN2 definition of correlation coefficient (CC) as a starting point.

  - Proposal 8 (Nokia): In RAN2, inter-frequency prediction focuses on the co-located scenario. Hence, RAN4 should take the same assumption to define the requirements.

  - Proposal 9 (Nokia): To define correlation coefficient (CC) for inter-frequency predictions, RAN4 to consider the channel models in different frequency layers. In this case, one way to define the CC is the correlation between PDPs of different frequency layers.

  - Proposal 10 (Xiaomi): For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

  - Proposal 11 (Tejas Network Limited): No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario.

  - Proposal 12 (Apple): RAN4 is invited to study the generalization capability of AI/ML-based inter-frequency measurement prediction models during the WI stage (Re-20). The study should evaluate how models trained on measurement data from specific inter-frequency deployment scenarios (e.g., macro–micro, FR1–FR1, FR1–FR2) perform when applied to unseen scenarios with different frequency layer combinations, cell layouts, and propagation environments. This includes assessing the impact of clustering multi-cell measurements as input features, the portability of models across varying network configurations, and the trade-off between prediction accuracy and measurement gap reduction.

  - Proposal 13 (Apple):RAN4 to conduct simulation studies for calibration and to better align the frequency-domain channel correlation coefficients. This should be based on the modeling assumptions for large-scale and small-scale parameters across multiple frequencies, and antenna pattern differences. The goal is to create a consistent simulation baseline that can support more accurate evaluation of inter-frequency prediction model

  - Proposal 14 (Apple): RAN4 is invited to study the impact of clustering on AI/ML-based inter-frequency measurement prediction performance. Multi-cell inputs provide richer spatial and frequency-domain information, improve robustness against anomalies in individual cell measurements, and enable better generalization across deployment scenarios. The study should assess clustering benefits under varying mobility conditions, network topologies, and correlation levels between cells, with the objective of determining whether clustering can reduce measurement overhead while maintaining or improving prediction accuracy

  - Proposal 15 (MediaTek):

    - For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), discuss whether co-located cells on different frequencies are with varied BS configurations (e.g., antenna pattern, downtilt angle) or not.

    - For scenario 3 (inter-frequency measurement prediction), discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network.

- Candidate Options:

  - Correlation Coefficient:

    - Option 1: RSRP prediction accuracy in collocated scenario can be specified under certain channel correlation coefficient, e.g., ~0.9.

    - Option 2: RAN4 should consider the RAN2 definition of correlation coefficient (CC) as a starting point.

    - Option 3: RAN4 to define the CC as the correlation between PDPs of different frequency layers by considering channel models in different frequency layers

    - Option 4: Further details on the impact of CC should be considered during the WI phase

  - CSSF:

    - Option 1: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period

    - Option 2: Not needed

  - Impact of cluster approach:

    - Option 1: Specify RSRP prediction accuracy for cluster approach and single cell approach separately

    - Option 2: Specify minimum RSRP prediction accuracy to accommodate both cluster and single cell approach

  - Impact on Measurement Gaps:

    - Option 1: RAN4 to consider the impact on MG sharing mechanism among different inter-frequency layers

    - Option 2: Not needed

- Recommended WF:

RAN4 to discuss the above-listed candidate options for Correlation Coefficient, CSSF, Cluster approach and measurement gaps one by one.

### Issue 2-10: LCM related requirements

- Proposals and Observations:

  - Proposal 1 (OPPO): Leave the LCM related requirements discussion to WI stage and wait for more progress from RAN2 AI mobility.

- Recommended WF:

RAN4 to discuss if Proposal 1 is agreeable.

# Topic #3: RAN4 requirements for measurement event prediction

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509433 | Apple | - Observation 1: Measuring only precision is less costly because the tester’s work is limited to checking short time windows around the UE’s own predictions, classifying each as correct or false, with effort and data storage scaling only to the number of predictions made. In contrast, computing recall requires a full sweep of the entire measurement log to identify every ground-truth A3 event, even during long stretches where the UE made no predictions, and then cross-matching each event against the prediction list to find missed cases - Observation 2: Case A (FR2, mobility optimisation): The benefit comes from providing the network with early notice of an upcoming A3 trigger. Therefore, the metric pair should be precision + minimum lead-time (L-min). A prediction is counted as “true” only if it occurs at least L-min milliseconds before the ground-truth trigger, giving the gNB enough time to prepare the handover. - Observation 3: Case B (FR1, measurement reduction): The benefit comes from accurately reconstructing skipped measurements, not from early prediction. The metric pair should be precision + maximum Event-Time-Deviation (E-max), where predictions are valid if they fall within a tight symmetric window (±E-max) around the ground-truth event. Lead-time bias is unnecessary here because the objective is maintaining measurement accuracy, not triggering early actions. - RAN4 should set similar performance requirements for indirect measurement-event prediction in Temporal Case A and Case B, but with different emphasis. Case A should focus more on prediction lead time and reliability to enable earlier, safe handover preparation. Case B should focus more on RSRP reconstruction accuracy and correctness of the derived event decision to ensure measurement-overhead reduction without mobility degradation - For measurement event prediction Case A replace the F1 score with two limits: precision ≥ P-min and lead-time ≥ L-min. Precision curbs false A3 alarms, lead-time guarantees each accepted hit arrives early enough for handover preparation. This simple pair avoids periodic/event-based recall ambiguities and roughly halves test workload by eliminating the need to log every missed event. - For the measurement-reduction use case B, replace F1 with two limits, precision ≥ P-min and absolute Event-Time-Deviation ≤ E-max for every accepted hit. Precision prevents superfluous A3 reports that would negate the overhead savings, while the tight, symmetric E-max window guarantees each prediction accurately fills the skipped-measurement gap without harming mobility. |
| R4-2509303 | CATT | Direct measurement event prediction Observation 1: Based on RAN2 agreements, RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Proposal 1: RAN4 to reuse the accuracy requirements defined for RRM measurement prediction for the inputs of measurement event prediction. Proposal 2: For the outputs of measurement event prediction, RAN4 consider to use a pre-defined successful prediction rate, e.g., 90%, as the performance metric. Indirect measurement event prediction Proposal 3: RAN4 to define delay requirement for indirect case and discuss the details in WI phase. Proposal 4: RAN4 do not define accuracy requirements when the final result report contains the expected occurrence time of a measurement event. Impact on core requirements Proposal 5: With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details in WI phase. |
| R4-2509674 | OPPO | Proposal 1: For indirect prediction, if the report includes the predicted RSRPs, the requirements for measurement prediction can be reused. Proposal 2: For indirect prediction, discuss whether the predicted RSRP reporting delay and the event triggered reporting delay is needed in work item based on RAN2 design. Proposal 3: For indirect prediction, the following approaches can be considered as possible KPI to evaluate the performance: - Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.) - Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges) - Alt 3: The handover failure rate per UE under certain time durations - Other alternatives are not precluded. Proposal 4: For direct and indirect prediction, RAN4 to discuss how to define the performance metrics and requirements in work item based on further RAN2 progress. |
| R4-2509764 | Xiaomi | Observation 1: In legacy, RAN4 defined event triggered L3 reporting delay in core part. Proposal 1: Measurement event prediction may have impact on predicated event triggered L3 reporting. Observation 2: The time interval from measurement to prediction increase, the prediction accuracy decreases. The prediction accuracy of individual measurement results in a time window could be quite different. Proposal 2: For event-triggered reporting, we propose waiting for RAN2 progress to define requirements based on finalized formats, clarifying rules for multi-value scenarios and inter-report relative accuracy. Proposal 3: For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances. Observation 3: In RAN2, performance metric F1 score is defined for event prediction accuracy, which considered both precision and recall. Observation 4: In RAN2, a timing difference between predicated event and ground truth event is defined. Observation 5: In legacy RAN4, in test for verify event prediction accuracy, only precision = 90% is used and no timing difference between predicated event and ground truth event is considered. Proposal 4: RAN4 to only consider precision and not consider recall in event prediction accuracy KPI. |
| R4-2509923 | Nokia | Observation 1: The following scenarios have been considered in RAN2 for indirect measurement event prediction using temporal domain predictions: 1) Temporal domain Case A: The UE predicts a probable future measurement event, which may be used in mobility optimization decisions, but there is no measurement reduction. 2) Temporal domain Case B: The UE is predicting measurements for measurement reduction, which should not have significant impact on the mobility performance. Observation 2: The choice between periodic and event-based prediction affects the timing and interpretation of performance counters for F1 score metric. Observation 3: From a network perspective, false event detections are generally more detrimental than missed event detections. However, this asymmetry in impact is not reflected in the F1 score calculation, as the metric assigns equal weight to precision and recall. Observation 4: Testing the UE with F1 score as the metric for A3 measurement event prediction is challenging due to significant testing overhead. Observation 5: For a measurement event prediction with time domain case A which is used in mobility optimization predicting A3 events ahead of the actual ground-truth trigger is often desirable. Observation 6: For the time domain case B measurement event prediction, a similar performance metric definition as that of temporal domain case A. Proposal 1: RAN4 to consider following performance metrics for indirect measurement event prediction. - Event Prediction Accuracy: Captures both the correctness of the predicted outcome and the alignment in time between the predicted and actual event occurrence. It consists of two components: - Outcome Accuracy: whether the prediction correctly identifies event occurrence or non-occurrence. - Temporal Accuracy: Measures how closely the predicted event occurrence time matches the actual event time, within a defined tolerance. - Event Prediction Reporting RSRP Accuracy: Refers to the accuracy of the reported relative /absolute predicted RSRP values associated with the event, compared to ground truth value. Proposal 2: RAN4 to consider following configurations for indirect measurement event prediction requirements - Event Prediction Window (Prediction Horizon): Refers to the future time interval within which the UE predicts the entry and/or exit conditions of a configured measurement event (e.g., A3). - Event Prediction Reporting/Inference Delay: Defined as the time interval between the point at which the UE initiate the event prediction and the point at which the UE transmits the corresponding event prediction report over the air interface. - Number of Cells/SSBs for which Prediction is Supported: Indicates the capability of the UE to simultaneously predict measurement events for multiple cells or SSBs. Observation 7: Similar accuracy definition and requirements have been considered for Even-based measurement reporting and periodic measurement in TS 38.133. Proposal 3: For indirect measurement event prediction use case, for potential report that includes the predicted RSRPs, RAN4 to assume a similar RSRP accuracy definition considered for predicted measurements. The applicable tolerance or accuracy levels may be influenced by several key factors, including length of the observation window, the duration of the prediction window, etc. |
| MediaTek | R4-2510107 | Proposal 1: For measurement event prediction, it is up to UE implementation when to activate the AI model for prediction. Observation 1: The longer PW is, the worse the absolute RSRP accuracy is. Proposal 2: Prefer not to define a set of requirements for different TTT values. Further discuss which TTT value to define requirements for. Propose 3: For measurement event prediction, the assumed UE behaviour is that UE starts to predict when enter condition of A3 event is met. |
| CMCC | R4-2510169 | Proposal 1: for impact on core requirements for indirect measurement event prediction, it is proposed to reuse the conclusion on the absolute and/or relative accuracy of predicted RSRP for RRM measurement prediction. Proposal 2: for impact on core requirements for the direct measurement event prediction, it is proposed to discuss whether and how to define RRM requirements for the final output (e.g. the probability of event occurrence for direct prediction or expected occurrence time of a event for indirect prediction). Proposal 3: Whether measurement error is considered for event prediction use case can be considered case by case. - For indirect measurement event prediction, the conclusion on how to handle the measurement error for RRM prediction mode can be reused. - For direct measurement event prediction, whether to consider the impact due to measurement error depends on the model input. - If the model input does not include measured RSRP, no need to consider measurement error. Otherwise, measurement error may have impact on prediction perfromance. |
| R4-2510446 | ZTE Corporation, Sanechips | Proposal 1: Measurement error should be considered into indirect measurement event prediction use case during WI phase, and Error model can be similar to that of RRM measurement prediction use case. Proposal 2: For measurement event prediction, RAN4 should at least consider scenarios 2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A). Proposal 3: For event-based measurement reporting based on indirect event prediction, RAN4 to consider a similar accuracy definition as measurement prediction. Proposal 4: RAN4 should consider whether and how to define accuracy for direct event prediction and indirect event prediction based on RAN2’s KPIs, such as precision, recall, F1 score and ETD. |
| R4-2510488 | vivo | Observation 1: In legacy, the time when the event condition is met must always precede the UE’s transmission of the measurement report, ensuring that the UE can confirm the event condition is satisfied during measurement. While for the measurement event prediction, the expected occurrence time of the measurement event corresponds to a future moment. It will be later than the UE’s transmission of the prediction report Proposal 1: RAN4 not to study the delay requirement for the predicted event triggered reporting Proposal 2: F1 score under certain fixed ETD conditions can be served as the candidate performance metric for evaluating the accuracy of measurement event prediction. Proposal 3: F1 score under certain fixed occurrence window length with fixed probability conditions can be served as a candidate performance metric for evaluating the accuracy of measurement event prediction. |
| R4-2510579 | Huawei, HiSilicon | Proposal 1: If the starting point of legacy handover delay is reused (i.e, when UE receives HO CMD), the AI event prediction has no impact on the requirements of handover delay. Proposal 2: RAN4 to discuss the following options for indirect event prediction - Option 1: Define accuracy for indirect event prediction - Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD compared with ground truth event, under the condition that precision and recall or F1 score consistently is above a value. - Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score under the condition of certain fixed ETD. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently above a value, under the condition that measurement event is predicted within certain ETD compared with ground truth event. - Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied. Proposal 3: RAN4 to discuss the following options for defining accuracy for direct event prediction: - Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that precision and recall or F1 score is consistently above a value. - Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1]. Proposal 4: RAN4 to study the requirement for event prediction delay, where event prediction delay is the time between the current time (at t0) where event is predicted, and the future time (at t1) of the occurrence of predicted event. |
| NTT DOCOMO | R4-2510707 | Observation 1: The report doesn’t include the predicted RSRPs in case of indirect measurement event prediction use case. Proposal 1: No need to specify additional requirements on relative accuracy of predicted RSRP for measurement event prediction. |
| R4-2510764 | Ericsson | Observation 1: Reporting delay requirement defined for the legacy event triggered reporting may not hold true for the predicted event triggered reporting. Proposal 1: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting for indirect event prediction use case. Observation 2: Temporal domain prediction of RRM measurements is used for indirect event prediction. Observation 3: Accuracy of temporal domain prediction has an impact on the accuracy indirect event prediction. Proposal 2: RAN4 to reuse accuracy requirement for temporal prediction of RRM measurement for indirect event prediction. Proposal 3: RAN4 to consider defining requirement for direct event prediction provided that the reporting framework for direct event prediction triggered reporting is agreed to be supported by RAN2. |
| R4-2511517 | Qualcomm Incorporated | Performance metrics for AI/ML-based RRM event prediction Proposal 1: The performance metrics for AI/ML-based measurement event prediction include, but are not necessarily limited to, the following: - L3-RSRP accuracy: - RAN4 to further clarify “In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP” agreed upon at RAN4#114 as follows: - For the absolute and/or relative accuracy of the predicted L3-RSRP, if the predicted event report includes predicted L3-RSRPs, the corresponding ground truth should be taken from the predicted event occurrence time, not the actual ground truth event occurrence time. - Event prediction latency (i.e., how far ahead of the actual ground truth event occurrence time the event is predicted): - It is defined as the time gap between the report time of the predicted event and the actual occurrence time of the ground truth event. - Note: The report time of the predicted event should precede the actual occurrence time of the ground truth event. - Positive event prediction accuracy: - The below two metrics can be considered as event prediction requirements either separately or jointly (e.g., via F1 score). - Successful event detection rate: The proportion of actual positive ground truth events that were correctly predicted by the UE within a tolerable prediction window of a certain length. - False alarm rate: The proportion of predicted positive ground truth events that did not actually occur within a tolerable prediction window of a certain length. - The tolerable prediction window length is defined as the time gap between the predicted event occurrence time and the actual positive ground truth event occurrence time. The specific value of this window length is subject to further discussion in Rel-20. - The actual ground truth event occurrence time is defined according to the ground truth definition used in L3-RSRP prediction. - Note: All of the above performance metrics (L3-RSRP accuracy, Event prediction latency, Positive event prediction accuracy, Tolerable prediction window) may be interdependent. |

## Open issues summary

In this section, different companies’ proposals about the RRM requirements for Measurement event prediction use case are summarized.

Issue 3-1: Performance metric for indirect measurement event prediction

- Background:

The following agreement was reached in RAN4#115 meeting:

Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

Furthermore, following agreement was  reached inRAN4#114bis meeting:

Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

- Proposals and Observations:

  - Proposal 1 (NTT Docomo):

    - The report doesn’t include the predicted RSRPs in case of indirect measurement event prediction use case.

    - No need to specify additional requirements on relative accuracy of predicted RSRP for measurement event prediction.

  - Proposal 2 (Nokia):

    - RAN4 to consider following performance metrics for indirect measurement event prediction.

      - Event Prediction Accuracy: Captures both the correctness of the predicted outcome and the alignment in time between the predicted and actual event occurrence. It consists of two components:

        - Outcome Accuracy: whether the prediction correctly identifies event occurrence or non-occurrence.

        - Temporal Accuracy: Measures how closely the predicted event occurrence time matches the actual event time, within a defined tolerance.

      - Event Prediction Reporting RSRP Accuracy: Refers to the accuracy of the reported relative /absolute predicted RSRP values associated with the event, compared to ground truth value.

    - For indirect measurement event prediction use case, for potential report that includes the predicted RSRPs, RAN4 to assume a similar RSRP accuracy definition considered for predicted measurements. The applicable tolerance or accuracy levels may be influenced by several key factors, including length of the observation window, the duration of the prediction window, etc

  - Proposal 3 (Ericsson):

    - RAN4 to reuse accuracy requirement for temporal prediction of RRM measurement for indirect event prediction.

  - Proposal 4 (OPPO):

    - For indirect prediction, if the report includes the predicted RSRPs, the requirements for measurement prediction can be reused.

    - For indirect prediction, the following approaches can be considered as possible KPI to evaluate the performance:

      - Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.)

      - Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges)

      - Alt 3: The handover failure rate per UE under certain time durations.

    - Other alternatives are not precluded.

    - For indirect prediction, RAN4 to discuss how to define the performance metrics and requirements in work item based on further RAN2 progress.

  - Proposal 5 (Huawei):

    - RAN4 to discuss the following options for indirect event prediction

      - Option 1: Define accuracy for indirect event prediction

      - Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD compared with ground truth event, under the condition that precision and recall or F1 score consistently is above a value.

      - Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score under the condition of certain fixed ETD. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently above a value, under the condition that measurement event is predicted within certain ETD compared with ground truth event.

      - Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

  - Proposal 6 (CATT): Indirect measurement event prediction:

    - RAN4 do not define accuracy requirements when the final result report contains the expected occurrence time of a measurement event.

  - Proposal 7 (Qualcomm):

    - The performance metrics for AI/ML-based measurement event prediction include:

      - L3-RSRP accuracy:

        - RAN4 to further clarify “In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP” agreed upon at RAN4#114 as follows:

        - For the absolute and/or relative accuracy of the predicted L3-RSRP, if the predicted event report includes predicted L3-RSRPs, the corresponding ground truth should be taken from the predicted event occurrence time, not the actual ground truth event occurrence time.

      - Event prediction latency (i.e., how far ahead of the actual ground truth event occurrence time the event is predicted):

        - It is defined as the time gap between the report time of the predicted event and the actual occurrence time of the ground truth event.

        - Note: The report time of the predicted event should precede the actual occurrence time of the ground truth event.

  - Proposal 8 (CMCC):

    - For impact on core requirements for indirect measurement event prediction, it is proposed to reuse the conclusion on the absolute and/or relative accuracy of predicted RSRP for RRM measurement prediction.

    - For impact on core requirements for the direct measurement event prediction, it is proposed to discuss whether and how to define RRM requirements for the final output (e.g. the probability of event occurrence for direct prediction or expected occurrence time of a event for indirect prediction).

  - Proposal 9 (ZTE):

    - For event-based measurement reporting based on indirect event prediction, RAN4 to consider a similar accuracy definition as measurement prediction.

    - RAN4 should consider whether and how to define accuracy for direct event prediction and indirect event prediction based on RAN2’s KPIs, such as precision, recall, F1 score and ETD.

  - Proposal 10 (Apple):

    - RAN4 should set similar performance requirements for indirect measurement-event prediction in Temporal Case A and Case B, but with different emphasis. Case A should focus more on prediction lead time and reliability to enable earlier, safe handover preparation. Case B should focus more on RSRP reconstruction accuracy and correctness of the derived event decision to ensure measurement-overhead reduction without mobility degradation

    - Case A and Case B shall use windowed precision over a lead-time window [L_min, L_max]. A prediction is counted true only if it arrives at least L_min and at most L_max before the ground-truth A3 trigger T0, ensuring the gNB has enough,but not excessive,time to prepare the handover. Replace the F1 score with two limits: (i)windowed precision ≥ P_min and (ii) every counted hit satisfies the lead-time constraint (L_min, L_max). Precision curbs false A3 alarms; the lead-time bound guarantees accepted hits are early enough to be actionable. This pair avoids periodic/event-based recall ambiguities and roughly halves test workload by eliminating the need to log every missed event.

    - For Case B, apply the same windowed-precision rule over [L_min, L_max] and the same lead-time constraint, but with a relaxed performance target to reflect the reduced (skip-aware) prediction interval pattern

- Candidate options:

- For indirect case:

  - Option 1: RAN4 to define new performance metrics for indirect measurement event prediction

  - Option 2: RAN4 to reuse accuracy requirement for temporal prediction of RRM measurement for indirect event prediction.

  - Option 3: No need to specify additional requirements on relative accuracy of predicted RSRP for measurement event prediction

- If RAN4 defines a new performance metric (Option 1 above), the potential options include, but not limited to:

  - Option 1: Define Event Prediction Accuracy to capture both the correctness of the predicted outcome and the alignment in time between the predicted and actual event occurrence

    - Option 1-1: Consider Outcome Accuracy (whether the prediction correctly identifies event occurrence or non-occurrence) and Temporal Accuracy (Measures how closely the predicted event occurrence time matches the actual event time, within a defined tolerance).

    - Option 1-2: Consider ETD under the conditions of certain fixed precision and recall or F1 score, where the measurement event is predicted within certain ETD compared with ground truth event, under the condition that precision and recall or F1 score consistently is above a value.

    - Option 1-3: Consider L3-RSRP accuracy and Event prediction latency (i.e., how far ahead of the actual ground truth event occurrence time the event is predicted).

    - Option 1-4: In Case A, replace the F1 score with two limits: precision ≥ P-min and lead-time ≥ L-min. Precision curbs false A3 alarms, lead-time guarantees each accepted hit arrives early enough for handover preparation. This simple pair avoids periodic/event-based recall ambiguities and roughly halves test workload by eliminating the need to log every missed event.

    - Option 1-5: In Case B, replace F1 with two limits, precision ≥ P-min and absolute Event-Time-Deviation ≤ E-max for every accepted hit. Precision prevents superfluous A3 reports that would negate the overhead savings, while the tight, symmetric E-max window guarantees each prediction accurately fills the skipped-measurement gap without harming mobility.

  - Option 2: Define Event Prediction Reporting RSRP Accuracy as an accuracy of the reported relative /absolute predicted RSRP values associated with the event, compared to ground truth value.

  - Option 3: Define handover failure rate per UE under certain time durations.

  - Option 4: Further discuss new performance metric for indirect case during the WI phase.

- Recommended WF

  - Discuss above listed Candidate Options.

Issue 3-2: Performance metric for direct event prediction use case

- Background:

- Proposals and Observations:

  - Proposal 1 (Mediatek):

    - For measurement event prediction, it is up to UE implementation when to activate the AI model for prediction.

    - Prefer not to define a set of requirements for different TTT values. Further discuss which TTT value to define requirements for.

    - For measurement event prediction, the assumed UE behaviour is that UE starts to predict when enter condition of A3 event is met.

  - Proposal 2 (Ericsson):

    - RAN4 to consider defining requirement for direct event prediction provided that the reporting framework for direct event prediction triggered reporting is agreed to be supported by RAN2.

  - Proposal 3 (Xiaomi):

    - For event-triggered reporting, we propose waiting for RAN2 progress to define requirements based on finalized formats, clarifying rules for multi-value scenarios and inter-report relative accuracy.

    - For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances.

    - RAN4 to only consider precision and not consider recall in event prediction accuracy KPI since no timing difference between predicated event and ground truth event is considered in RAN4

  - Proposal 4 (OPPO):

    - For direct prediction, RAN4 to discuss how to define the performance metrics and requirements in work item based on further RAN2 progress.

  - Proposal 5 (Huawei):

    - RAN4 to discuss the following options for defining accuracy for direct event prediction:

      - Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that precision and recall or F1 score is consistently above a value.

      - Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1].

  - Proposal 6 (CATT): Direct measurement event prediction:

    - RAN4 to reuse the accuracy requirements defined for RRM measurement prediction for the inputs of measurement event prediction.

    - For the outputs of measurement event prediction, RAN4 consider to use a pre-defined successful prediction rate, e.g., 90%, as the performance metric.

  - Proposal 7 (Qualcomm):

    - The performance metrics for AI/ML-based measurement event prediction include:

      - Positive event prediction accuracy:

        - The below two metrics can be considered as event prediction requirements either separately or jointly (e.g., via F1 score).

          - Successful event detection rate: The proportion of actual positive ground truth events that were correctly predicted by the UE within a tolerable prediction window of a certain length.

          - False alarm rate: The proportion of predicted positive ground truth events that did not actually occur within a tolerable prediction window of a certain length.

        - The tolerable prediction window length is defined as the time gap between the predicted event occurrence time and the actual positive ground truth event occurrence time. The specific value of this window length is subject to further discussion in Rel-20.

      - The actual ground truth event occurrence time is defined according to the ground truth definition used in L3-RSRP prediction.

  - Proposal 8 (ZTE):

    - RAN4 should consider whether and how to define accuracy for direct event prediction and indirect event prediction based on RAN2’s KPIs, such as precision, recall, F1 score and ETD.

  - Proposal 9 (vivo):

    - F1 score under certain fixed ETD conditions can be served as the candidate performance metric for evaluating the accuracy of measurement event prediction.

    - Proposal 3: F1 score under certain fixed occurrence window length with fixed probability conditions can be served as a candidate performance metric for evaluating the accuracy of measurement event prediction.

- Candidate options:

- For direct case:

  - Option 1: RAN4 to discuss in the WI phase how to define the performance metrics and requirements based on RAN2 progress

  - Option 2: RAN4 to reuse the accuracy requirements defined for RRM measurement prediction for the inputs of measurement event prediction.

  - Option 3: RAN4 to discuss the following options for defining accuracy for direct event prediction:

    - Option 3-1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that precision and recall or F1 score is consistently above a value.

    - Option 3-2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1].

  - Option 4: RAN4 to define Positive event prediction accuracy as a performance metric for AI/ML-based measurement event prediction, including successful event detection rate and false alarm rate.

- Recommended WF

  - Discuss above listed Candidate Options.

Issue 3-3: Impact of measurement error for event prediction use case

- Proposals and Observations:

  - Proposal 1 (ZTE):

    - Measurement error should be considered into indirect measurement event prediction use case during WI phase, and Error model can be similar to that of RRM measurement prediction use case.

  - Proposal 2 (CMCC):

    - Whether measurement error is considered for event prediction use case can be considered case by case.

      - For indirect measurement event prediction, the conclusion on how to handle the measurement error for RRM prediction mode can be reused.

      - For direct measurement event prediction, whether to consider the impact due to measurement error depends on the model input.

      - If the model input does not include measured RSRP, no need to consider measurement error. Otherwise, measurement error may have impact on prediction perfromance.

Candidate options:

  - Option 1: Measurement error should be considered into indirect measurement event prediction use case during WI phase.

  - Option 2: Measurement error is considered for event prediction use case can be considered case by case.

  - Option 2-1: For indirect measurement event prediction, the conclusion on how to handle the measurement error for RRM prediction mode can be reused.

  - Option 2-2: For direct measurement event prediction, if the model input does not include measured RSRP, no need to consider measurement error.

- Recommended WF

  - Discuss above listed Candidate Options.

Issue 3-4: Impact on core requirements for event prediction use case

- Proposals and Observations:

  - Proposal 1 (CATT):

    - RAN4 to define delay requirement for indirect case and discuss the details in WI phase.

    - With AI/ML based measurement event prediction, some components in conditional handover delay requirements can be updated or removed, e.g., Tprocessing and Tmeasure. FFS details in WI phase.

  - Proposal 2 (Huawei):

    - If the starting point of legacy handover delay is reused (i.e, when UE receives HO CMD), the AI event prediction has no impact on the requirements of handover delay.

  - Proposal 3 (vivo):

    - RAN4 not to study the delay requirement for the predicted event triggered reporting

  - Proposal 4 (Nokia):

    - RAN4 to consider following configurations for indirect measurement event prediction requirements

      - Event Prediction Window (Prediction Horizon): Refers to the future time interval within which the UE predicts the entry and/or exit conditions of a configured measurement event (e.g., A3).

      - Event Prediction Reporting/Inference Delay: Defined as the time interval between the point at which the UE initiate the event prediction and the point at which the UE transmits the corresponding event prediction report over the air interface.

      - Number of Cells/SSBs for which Prediction is Supported: Indicates the capability of the UE to simultaneously predict measurement events for multiple cells or SSBs.

  - Proposal 5 (Ericsson):

    - RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting for indirect event prediction use case.

  - Proposal 6 (Oppo):

    - For indirect prediction, discuss whether the predicted RSRP reporting delay and the event triggered reporting delay is needed in work item based on RAN2 design.

  - Proposal 7 (Huawei):

    - RAN4 to study the requirement for event prediction delay, where event prediction delay is the time between the current time (at t0) where event is predicted, and the future time (at t1) of the occurrence of predicted event.

- Candidate options:

  - Option 1: RAN4 not to define delay requirements for predicted event triggered reporting

  - Option 2: RAN4 to study the delay requirement for event prediction, where event prediction delay is the time between the current time (at t0) where event is predicted, and the future time (at t1) of the occurrence of predicted event.

  - Option 3: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting in indirect event prediction use case

  - Option 4: RAN4 to defer the discussion on delay requirement for indirect measurement event prediction until WI phase

- Recommended WF

  - Discuss above listed Candidate Options.

Issue 3-5: Scenario priorities for event prediction

- Proposals and Observations:

  - Proposal 1 (ZTE): For measurement event prediction, RAN4 should at least consider scenarios 2(FR1 to FR1 intra-frequency temporal domain caseB), scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), and scenario 4 (FR2 to FR2 intra-frequency temporal domain case A).

  - Proposal 2 (Nokia): RAN4 to study the requirements and performance metrics for indirect measurement event prediction temporal domain Case A and Case B

  - Proposal 3 (Apple): RAN4 should set similar performance requirements for indirect measurement-event prediction in Temporal Case A and Case B, but with different emphasis.

- Candidate options:

  - Option 1: RAN4 to study the requirements for indirect measurement-event prediction in Temporal Case A and Case B

- Recommended WF

  - RAN4 to discuss if Option 1 is agreeable.