3GPP TSG-RAN WG4 Meeting # 116					       			       			R4-2509064

Bengaluru, India, August 25-29, 2025

Agenda item:			7.23.1

Source:	Moderator (Huawei)

Title:	Topic summary for [116][221] Netw_Energy_NR_enh_Part2

Document for:	Information

Introduction

This topic summary includes RRM core requirements for Rel-19 network energy saving (7.23.3.2 and 7.27.3.3).

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-1-1: Applicable scenario for SSB adaption.

Issue 1-3-1: Colliding with MG

Issue 1-2-1: L1 requirements at transition - general

Issue 2-1-1: RRM requirements impacts for OD-SIB1

Topic #1: Adaptation of common signal/channel transmission

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509035 | Qualcomm Technologies Ireland | Observation 1: Legacy behavior in RAN2 is that as long as the SCell is deactivated, the UE does not monitor PDCCH on the SCell nor for the SCell. Therefore, DCI indication for SSB adaptation can only occur on activated SCell for the same cell and/or on other activated SCells. Hence, SCell activation based on SSB adaptation is not supported in Rel-19. Proposal 1: RAN4 should stop discussing SCell activation based on SSB adaptation as long as RAN1 or RAN2 has not agreed to support this scenario. Observation 2: Based on the legacy behavior in RAN2 it also means that these agreements and proposals for SCell activation based on SSB adaptation do not apply until RAN1 or RAN2 has agreed to change legacy behavior. |
| R4-2509283 | CATT | Proposal 1: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can also apply. Proposal 2: After SSB adaptation, if the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, UE performs the measurement within gap, otherwise the UE performs the measurement outside gap. |
| R4-2509452 | Apple | Observation 1: gap cancellation takes up to 5ms after DCI, which is longer than the agreed additional delay requirement in TfirstSSB. Besides, gap cancellation is an optional UE feature. Proposal 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell. |
| R4-2509453 | Apple | DraftCR of NR intra-frequency measurement due to SSB adaptation |
| R4-2509627 | LG Electronics Inc. | Proposal 1: UE is allowed to skip the SSB adaptation measurement if SMTC is parialliy overlapped with MG occasion Proposal 2: RAN4 to define intra-frequency with gap requirement for SSB adpataion |
| R4-2509695 | OPPO | - Observation 1: The adapted SSB on neighbor cell is measured based on legacy SMTC, while for serving cell it is based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity. - Proposal 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell. Proposal 2: RAN4 not to include the case of R18 Scell activation with L3 reporting in this WI.. |
| R4-2509796 | Xiaomi | Proposal 1: R18 Scell activation with L3 reporting can be defined if WI competition timeline allowed. Proposal 2: Deprioritize the multiple SCells activation scenario with SSB adaptation Proposal 3: If the SMTC occasion with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell. |
| R4-2509951 | Nokia, Nokia Shanghai Bell | Scenarios for SSB adaptation Proposal 1: RAN4 to add the scenario of Rel-18 SCell activation with L3 reporting to the applicable SCell activation scenarios with SSB adaptation. Proposal 2: RAN4 to exclude multiple SCells activation scenario with SSB adaptation from Rel-19 NES enhancements. Proposal 3: RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered group common DCI and RRC signaling, the latter in combination with normal SCell activation or direct SCell activation or PUCCH SCell activation or any further agreed SCell activation scenarios. Layer 1 measurement transition requirements Proposal 4: RAN4 to send liaison to RAN1, asking them to include the additional SSB adaptation delay in the calculation of the earliest possible occurrence of the first SSB occasion belonging to the new SSB periodicity in clause 11.6 of TS 38.213. Proposal 5: For measurement restrictions, when adapted SSB collides with CSI-RS, existing L1 measurement restriction in clauses 9.5.5.1 and 9.5.5.2 apply. Layer 3 measurement impact Proposal 6: RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG based on measCycleSCell. Proposal 7: RAN4 to agree, if the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell. SCell activation requirements with concurrent SSB adaptation Proposal 8: RAN4 to define SCell activation requirements in case of concurrent SCell activation and SSB adaptation command in same RRC message, e.g. for known SCell in FR1, based on - Tactivation_time = TFirstSSB + X, with X=6ms for measurement period ≤ 2400ms, - Tactivation_time = TFirstSSB_MAX + Trs + X, with X=6ms for measurement period > 2400ms. |
| R4-2510028 | CMCC | Proposal 1: When the selected additional SMTC is overlapping with MG, the legacy measurement principle and scaling factor Kp shall be reused. Proposal 2: The SSB adaptation based SCell activation requirements can not be applied for ‘Direct SCell activation’. |
| R4-2510091 | China Telecom | Proposal 1: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply. Proposal 2: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell. |
| R4-2510191 | vivo | Proposal 1: For SSB adaptation, measurement restriction for SSB based L1-RSRP can be the same as that of legacy when SSB collides with CSI-RS. Proposal 2: For the SMTC configuration colliding with MG after adaptation, option 1 is ok. |
| R4-2510598 | Huawei, HiSilicon | Observation 1: The first SSB with new periodicity can be located based on the configured SFN offset and half-frame index. Observation 2: Based on RAN2 existing MAC spec, whether SSB adaptation triggered by DCI 2-9 can happen in deactivated SCell and SCell activation procedure is not clear. Proposal 1: RAN4 to postpone the discussion on SSB adaption for deactivated SCell measurement and SCell activation. If RAN1/2 achieve further progress, RAN4 could further discuss the requirements accordingly which does not impact the WI completion. Observation 3: The occasionally triggered SSB adaption (maybe also for OD-SSB) may collide pre-defined Cell specific CSI-RS, and gNB may not be able to guarantee that both SSB and CSI-RS are available to UE. Per existing requirements, if UE chooses to measurement CSI-RS, it may cause wrong UE behaviour. Proposal 2: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. Proposal 3: SMTC occasion of the selected additional SMTC configuration collides with MG occasion, legacy principle applies that UE is allowed to skip the SMTC when it collides with MG occasion. |
| R4-2510681 | ZTE Corporation, Sanechips | Proposal 1: After SSB adaptation, for the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, follow legacy collision handling, i.e. - For partial overlapping between SMTC and MG, the SMTC occasion is deprioritized by MG occasion, UE only perform measurement outside MG; - For fully overlapping between SMTC and MG, UE has to perform measurement within MG. Proposal 2: Besides the basic SCell activation, the applicable SCell activation requirements also include R18 SCell activation with L3 reporting. |
| R4-2510914 | Ericsson | Proposal 1: After UE receiving SSB adaptation indication, UE still follows legacy deactivated SCell measurement framework based on measCycleScell. Proposal 2: After SSB adaption, if the updated SMTC is collided with MG, the legacy MG collision rule is reused. Proposal 3: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can also apply. |
| R4-2510918 | Ericsson | draftCR on SSB adapation for SCell actiation |
| R4-2511082 | Samsung | Observation 1: If no any restrictions on applicable scenarios for DCI 2_9 based SSB burst periodicity adaptation for SCell, same framework as OD-SSB case#2 (same frequency but with different periodicities) can be reused by using adapted SSB with smaller periodicity indicated via DCI 2_9 Observation 2: Based on RAN1 agreement, the SFN offset and HF index are configured per additional SSB periodicity value, UE can apply the associated SFN offset and HF index to determine the time location of the first possible SSB burst after adaptation after receiving the DCI based SSB adaptation indication Proposal 1: - For the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the legacy principle to solve the collision can be reused: - If SMTC are fully overlapped with MG, gap sharing measurement, clarification on the existing definition of CSSFwithin_gap,i applies to SSB adaptation L3 measurement can be needed - If SMTC are partially overlapped with MG, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell, the measurements are only performed on SMTC occasions outside MG Proposal 2: Considering the limited TU and from requirement importance/ urgency perspective, for SSB adaptation based deactivated SCell measurement, - UE follows legacy framework based on measCycleSCell. Proposal 3: According to the DCI 2_9 based mechanism for adaptation of SSB in time domain for SCell, the SCell timing information is known to the UE, blind detection is not needed. Proposal 4: If to define SCell activation requirement for SSB adaptation when SSB adaptation command comes at the same time as SCell activation command, d and additional processing time need to be contained in TfirstSSB Proposal 5: When to define SCell activation requirement for SSB adaptation when SSB adaptation command comes earlier than SCell activation command, it would be better to ensure the first possible SSB burst after SSB adaptation was acquired before SCell activation command reception - FFS: Condition related to slot m+d+∆ and slot n Proposal 6: The existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1 SSB adaptation – General

Issue 1-1-1: Applicable scenario for SSB adaption.

Background:

TS 38.321 1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell.

- Proposals

  - Option 1: RAN4 to postpone the discussion on SSB adaption for deactivated SCell measurement and SCell activation. If RAN1/2 achieve further progress, RAN4 could further discuss the requirements accordingly which does not impact the WI completion (Huawei)

    - Option 1a: RAN4 should stop discussing SCell activation based on SSB adaptation as long as RAN1 or RAN2 has not agreed to support this scenario. (QC)

- Recommended WF:

  - Agree on option 1.

Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Background

RAN4 R4-2508441 Issue 1-1-1: Processing time for SSB adaption Agreement RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation. - FR1: 2ms FR2: 3ms - FFS L3 transition period measurement

Issue 1-2-1: L1 requirements at transition - general

Proposals

  - Option 1(Nokia):RAN4 to send liaison to RAN1, asking them to include the additional SSB adaptation delay in the calculation of the earliest possible occurrence of the first SSB occasion belonging to the new SSB periodicity in clause 11.6 of TS 38.213.

- Recommended WF:

  - Discuss whether there is need to send LS to RAN1 in additional to RAN4 L1 transition period requirements.

Issue 1-2-2: L1 requirements – Measurement restriction impact

- Proposals

  - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei)

  - Option 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply. (CATT, Nokia, Ericsson, Samsung, CTC, vivo)

- Recommended WF:

  - Agree on option2.

Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: Colliding with MG

- Proposals

  - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell. (Apple, LGE, OPPO, Xiaomi, Nokia, Ericsson, Samsung, CTC, ZTE, vivo, CATT)

    - Option 1a: RAN4 to define intra-frequency with gap requirement for SSB adaptation (LGE).

    - Option 1b: After SSB adaptation, if the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, UE performs the measurement within gap, otherwise the UE performs the measurement outside gap. (CATT)

- Recommended WF:

  - Agree on option 1.

Issue 1-3-2: L3 requirements impacts

- Proposals

  - Option 1: After UE receiving SSB adaptation indication, UE still follows legacy deactivated SCell measurement framework based on measCycleScell (Ericsson, Samsung—from requirement definition perspective)

- Recommended WF:

  - Depends on conclusion of issue 1-1-1.

Sub-topic 1-4 SSB adaptation – Other requirements impacts

Issue 1-4-1: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

- Proposals

  - Option 1: R18 Scell activation with L3 reporting

    - No requirements: Oppo

    - Define requirements: Nokia, Huawei, Xiaomi, ZTE

  - Option 2: multiple SCells activation scenario with SSB adaptation

    - No requirements: Nokia, Xiaomi

  - Option 3: Direct SCell activation

    - No requirements: CMCC

- Recommended WF:

  - Depends on conclusion of issue 1-1-1.

Issue 1-4-3: SCell activation requirement for SSB adaptation – Others

- Proposals

  - Option 1: RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered group common DCI and RRC signaling, the latter in combination with normal SCell activation or direct SCell activation or PUCCH SCell activation or any further agreed SCell activation scenarios. (Nokia)

  - Option 2: 	RAN4 to define SCell activation requirements in case of concurrent SCell activation and SSB adaptation command in same RRC message, e.g. for known SCell in FR1, based on

    - -	Tactivation_time = TFirstSSB + X, with X=6ms for measurement period ≤ 2400ms,

    - -	Tactivation_time = TFirstSSB_MAX + Trs + X, with X=6ms for measurement period > 2400ms.

  - Option 3: According to the DCI 2_9 based mechanism for adaptation of SSB in time domain for SCell, the SCell timing information is known to the UE, blind detection is not needed. (Samsung)

  - Option 4: If to define SCell activation requirement for SSB adaptation when SSB adaptation command comes at the same time as SCell activation command, d and additional processing time need to be contained in TfirstSSB (Samsung)

  - Option 5: When to define SCell activation requirement for SSB adaptation when SSB adaptation command comes earlier than SCell activation command, it would be better to ensure the first possible SSB burst after SSB adaptation was acquired before SCell activation command reception (Samsung)

    - FFS: Condition related to slot m+d+∆ and slot n

- Recommended WF:

  - Depends on conclusion of issue 1-1-1.

Topic #2: OD-SIB1

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509696 | OPPO | DraftCR on requirements for interruption in paging reception |
| R4-2509953 | Nokia, Nokia Shanghai Bell | Paging interruption Proposal 1: RAN4 to consider following change to paging interruption during cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency and inter-frequency cell re-selection, the interruption time may be longer if the target cell is a NES cell supporting on-demand SIB1 transmission.” in clause 4.2.2.6 of TS 38.133. Cell reselection delay Proposal 2: RAN4 to consider following change to cell reselection delay in idle mode, when UE performs cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency cell re-selection, Tdetect,NR_Intra , Tmeasure,NR_Intra and Tevaluate,NR_Intra may be longer if the target Cell is a NES Cell supporting on-demand SIB1 transmission.” for intra-frequency NR cells in clause 4.2.2.3 of TS 38.133 and the same phrase in clause 5.1.2.3 for inactive mode. Add similar statements in clauses 4.2.2.4 and 5.1.2.4 for inter-frequency NR cells and in clauses 4.2.2.5 and 5.1.2.5 for inter-RAT E-UTRAN cells. UL-WUS Msg1 retransmission attempts Proposal 3: RAN4 to consider the impact on RRM core requirements for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured. |
| R4-2509954 | Nokia, Nokia Shanghai Bell | Draft CR38.133 Introduction of RRM core requirements for OD-SIB1 |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 RRM requirements impacts for OD-SIB1

Issue 2-1-1: RRM requirements impacts for OD-SIB1

RAN4 agreements in R4-2508441 Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

- Proposals

  - Proposal 1: 	RAN4 to consider following change to paging interruption during cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency and inter-frequency cell re-selection, the interruption time may be longer if the target cell is a NES cell supporting on-demand SIB1 transmission.” in clause 4.2.2.6 of TS 38.133. (Nokia)

  - Proposal 2: 	RAN4 to consider following change to cell reselection delay in idle mode, when UE performs cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency cell re-selection, Tdetect,NR_Intra , Tmeasure,NR_Intra and Tevaluate,NR_Intra may be longer if the target Cell is a NES Cell supporting on-demand SIB1 transmission.” for intra-frequency NR cells in clause 4.2.2.3 of TS 38.133 and the same phrase in clause 5.1.2.3 for inactive mode. Add similar statements in clauses 4.2.2.4 and 5.1.2.4 for inter-frequency NR cells and in clauses 4.2.2.5 and 5.1.2.5 for inter-RAT E-UTRAN cells. (Nokia)

  - Proposal 3: 	RAN4 to consider the impact on RRM core requirements for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured. (Nokia)

- Recommended WF:

  - For paging interruption requirements, directly discuss the CR based on previous agreements.

  - For P2 and P3, discuss whether there is RRM impact needs to be captured in RAN4 spec.

CR handling

CR for SSB adaptation.

| Tdoc | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2509453 | DraftCR of NR intra-frequency measurement due to SSB adaptation | Apple | Discuss the CR directly. |
| R4-2510918 | draftCR on SSB adapation for SCell actiation | Ericsson | Depends on the conclusion of issue 1-1-1. |

CR for OD-SIB1

| Tdoc | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2509696 | DraftCR on requirements for interruption in paging reception | OPPO | Discuss the CR directly and cooperate R4-2509954 |
| R4-2509954 | Draft CR38.133 Introduction of RRM core requirements for OD-SIB1 | Nokia, Nokia Shanghai Bell | Discuss the CR directly and cooperate R4-2509696 |