3GPP TSG-RAN4 Meeting #116	R4-2509106

Bengaluru, India, 25 – 29, August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.191 | CR | 00xx | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for split TRP/TRS measurement grids method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | First introduction of OTA test methodology for ambient IoT devices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define test procedure and performance metrics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambient IoT devices cannot be tested for their performances. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | none |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

### 8.4	Anechoic Chamber method

8.4.1 General

Test frequency band in clause 8.1.1 is used for tests described in this clause. A device shall be positioned according to the positioning guideline in clause 8.3. Device manufacturers shall declare direction of maximum backscattering to enable efficient measurement as this eliminates the need for spherical scan to find the direction of maximum backscattering.

During tests, device is placed on a platform with either combined axis or distributed axis at the origin of a Cartesian coordinate. Test antenna with two linear orthogonal polarizations support both CW and Reader, namely CW and Reader share the same antenna with CW occupying one polarization and Reader using both polarizations.

Polarization orientation of CW may need to be chosen in such a way to avoid complete polarization mismatch between CW and device as both use linear polarization.

8.4.2 Backscattering measurement procedure

Backscattered power is only measured at the direction of maximum backscattering declared by device manufacturers.

The measurement procedure includes the following steps:

- Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between test/CW antenna to DUT, insertion loss and cable loss, etc.

- Position Reader and device such that the direction of maximum backscattering faces the Reader according to the declaration from device manufacturers.

- Device is assumed charged during the measurement according to device declaration on the required energy conditions.

- Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set such that the received power at DUT’s antenna is larger than minimum reference sensitivity requirement of device. CW transmits in θ-polarization to ensure CW incident power at the device antenna is [x]dB higher than the receiver sensitivity requirement.

- Reader receives in  θ-polarization and ϕ-polarization either simultaneously or sequentially and record received power levels. Repeat step 2) and 3) with CW transmits in ϕ-polarization.

- Calculate backscattered power from the device: Pout = CW=θ,ϕ {(EIRPθ-Pathloss-Gainantenna)+(EIRPϕ-Pathloss- Gainantenna)}

Where

Pathloss : free space propagation loss between test antenna and device

Pout : backscattered power

Pin : CW incident power at device

PCW,θ and PCW,ϕ : CW transmit power in θ-polarization and the ϕ-polarization, respectively

EIRPθ and EIRPϕ ：Reader received power in θ-polarization and the ϕ-polarization, respectively

Gainantenna : test antenna gain

8.4.3 Sensitivity

Sensitivity is measured over a partial sphere of [y] degrees in elevation or θ-direction with [30] degree steps in both θ- and ϕ-directions.

The measurement procedure includes the following steps:

- Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Place DUT based on UE positioning guidelines.

- Device is assumed charged during the measurement according to device declaration on the required energy conditions.

- Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set such that the received power at DUT’s antenna is a few dB above minimum reference sensitivity requirement of device.

- Reader transmits in θ-polarization, but receives in θ-polarization and ϕ-polarization either simultaneously or sequentially.

- CW transmits in θ-polarization to ensure CW incident power at the device antenna is [x]dB higher than the receiver sensitivity requirement.

- Initial Reader transmit power shall be high enough for Reader to receive and decode responses from device successfully.

- Then Reader transmit power level is reduced to achieve 90% response decode success rate by determining whether DUT can send correct response in D2R channel within correct timing relationship.

- Repeat step 5) to 8) for all grid points over hemisphere and record the Reader received power levels.

- Change Reader configuration to: Reader transmits in ϕ-polarization, but receives in both θ-polarization and ϕ-polarization.

- Change CW configuration to: CW transmits in ϕ-polarization to ensure device reception

- Repeat step 5) and 8) for all grid points over hemisphere and record the Reader received power levels

- For every measured grid points, calculate total transmit power from Reader in both polarizations in dB:

  - Pθ (θi, ϕj) = PTx(θ-polarization) + Gainantenna – Pathloss

  - Pϕ (θi, ϕj) = PTx(ϕ-polarization) + Gainantenna – Pathloss

- Calculate sensitivity at grid point (i, j) using linear values:

Refsens(θi, ϕj) = [1Plinearθ(θi, ϕj)+1Plinearϕ(θi, ϕj)]-1

<<< END OF CHANGES >>>