3GPP TSG-RAN WG4 Meeting #116	R4-2509110

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on Rel-19 NTN Demod performance requirements

Agenda item:	7.27.7

Document for:	Discussion

Introduction

In this document Nokia presents some perspectives on the ongoing Rel-19 work item on NR NTN in Rel-19.

Some views are presented based on the Way Forward from RAN4#115 [2].

Discussion

OCC Parameters

During RAN4#115 it was agreed the inter-slot OCC length of 2 would be defined, along with PUSCH type A repetitions.

The Way Forward indicates that it is FFS on whether inter-slot length for OCC of 4 should be defined.

As the work item of NTN in Rel-19 specifies that that OCC should be defined for at least 2 or 4 UEs in accordance with the WID Nokia believes that to be in line with the WID and direction from RAN Plenary that RAN4 should define PUSCH requirements with inter-slot OCC length 4.

The WID indicates at least 2 and 4 should be defined for OCC inter-slot length

RAN4 shall define PUSCH requirements with inter-slot OCC length 4

PUSCH parameters

Regarding the parameters for PUSCH Nokia concurs that for a 15 kHz SCS, only 5MHz requirements should be defined, similarly for 30 kHz SCS, only 10 MHz

RAN4 shall use SCS=15KHz with 5MHz CBW and SCS=30KHz with 10MHz CBW

For channel model, a single channel model can be used, and it would make sense for this to be the most stringent channel model out of those used in previous cases, NTN-TDLA100-200 Low

RAN4 shall use NTN-TLDA100-200 Low for all cases

Regarding frequency offset, it may be appropriate to define 200 Hz, however Nokia also notes that pre-compensation is assumed and other test cases will be able to assess the SANs capability to do so, therefore equally a no frequency offset could be defined.

It could be appropriate to use either 200Hz Frequency offset or none

RAN4 may use 200 Hz Frequency offset

Regarding MCS choice, a sensible choice would be MCS 4, given previous requirements, equally the UEs will have similar link budgets if in the same cell, so the MCS should be consistent for all.

RAN4 shall use MCS4 for all cases

For number of Rx, as the SAN may be configured in either 1Rx or 2Rx is makes sense to define requirements for both.

RAN4 shall use both 1Rx and 2Rx for PUSCH requirements

RedCap/eRedCap

Regarding PDSCH requirements for eRedCap, Nokia believes that it would be appropriate to reuse appropriate Rel-17 NTN PDSCH requirements without BW reduction, as there will be little to no performance difference.

RAN4 shall Reuse Rel-17 NTN PDSCH demodulation requirements without BW reduction for eRedCap

Conclusion

The paper discusses the topic of NR NTN in RAN4 for performance requirements derivation, and presents some of the Nokia views on this topic for discussion during RAN4#116.

The following Observations and Proposals were made:

Observation 1: The WID indicates at least 2 and 4 should be defined for OCC inter-slot length

Proposal 1: RAN4 shall define PUSCH requirements with inter-slot OCC length 4

Proposal 2: RAN4 shall use SCS=15KHz with 5MHz CBW and SCS=30KHz with 10MHz CBW

Proposal 3: RAN4 shall use NTN-TLDA100-200 Low for all cases

Observation 2: It could be appropriate to use either 200Hz Frequency offset or none

Proposal 4: RAN4 may use 200 Hz Frequency offset

Proposal 5: RAN4 shall use MCS4 for all cases

Proposal 6: RAN4 shall use both 1Rx and 2Rx for PUSCH requirements

Proposal 7: RAN4 shall Reuse Rel-17 NTN PDSCH demodulation requirements without BW reduction for eRedCap

- References

- RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Thales, CATT, RAN#106, Madrid, December 2024

- R4-2508455 “Way Forward for [115][333] NR_NTN_Ph3_demod”, Ericsson, RAN4#115, Malta, May 2025