| 3GPP TR 38.753 V1.1.0 (2025-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on spatial channel model for demodulation performance Requirements for NR; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Background	10

4.1	General	10

5	Spatial Channel Modelling Approaches	11

5.1	CDL Approach	11

5.1.1	Antenna Panel Configuration	11

5.1.2	Antenna Array Virtualization	11

5.1.3	Channel Models	12

5.1.4	Channel Power Normalization	15

5.2	TDL Approaches	15

5.2.1	MIMO correlation and angular spread	15

5.2.2	Multi-cluster TDL channel via TX-RX beam steering	16

5.2.3	Example cluster model	18

6	Comparison of spatial channel Models	20

6.1	PDSCH performance under SU-MIMO scenario	20

6.2	PMI performance under SU-MIMO scenario	23

6.3	Channel Properties	31

6.3.1	CDL	31

6.3.2	TDL	33

6.4	Channel Model Statistics Comparison	34

6.4.1	Metric definitions and direct measurement from channel realization	35

6.4.2	Theoretical SDPD curves from CDL model parameters	35

6.4.3	Theoretical SDPD curves from multi-cluster TDL model parameters	36

6.4.4	Numerical results	36

6.4.5	Single-cluster time correlation properties	40

7	Alignment of Spatial Channel Models	42

7.1	 rCDL-C1 model results alignment	42

7.2	 xTDL-C1 model results alignment	45

8	Summary	48

8.1	General	48

8.2	Summary of SU-MIMO Results	48

8.2.1	PDSCH	48

8.2.2	PMI	48

8.3	Comparison of SCM Candidates	48

A.1	Measurement Configurations	50

A.1.1	Measurement campaign A: a 4-port transmitter and a 4-port receiver system	50

A.1.2	Measurement campaign B: a 4-port transmitter and a 32-port receiver	50

A.2	Measurement Results	51

A.2.1	Channel Eigenmodes	52

A.2.2	Spatial Properties	53

B.1	Introduction	58

B.2	Common Execution steps to all CDL models	58

B.3	Derivation of CDL Models for RAN4 Demod	60

B.3.1 CDL Cluster Reduction	60

B.4	Information on RAN4 CDL model derivation	62

B.4.1 CDL Cluster Reduction results for validation	62

C.1	TDL Approaches	64

C.1.1	Multi-cluster TDL channel for 2-CW fixed-MCS testing	64

C.1.1.1	Example 1	64

C.1.1.2	Example 2	66

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document contains the findings of the Study on spatial channel model for demodulation performance requirements for NR, with the detailed objectives as follows:

-	Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements:

-	Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance

-	Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches

-	For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes.

-	Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased.

-	The methodology shall be for FR1 (conducted).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-241610, Study on spatial channel model for demodulation performance requirements for NR, RAN#104

[3]	RP-250577, Revised SID: Study on spatial channel model for demodulation performance requirements, RAN#107 meeting, Nokia, BT Plc

[4]	RP-160210, SID: Study on channel model for frequency spectrum above 6 GHz, RAN#71, Samsung, Nokia, March 2016

[5]	3GPP TR 38.900, Study on channel model for frequency spectrum above 6 GHz

[6]	RP-170379, SID: Study on New Radio Access Technology, NTT DOCOMO INC., RAN#75, March 2017

[7]	3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

[8]	3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

[9]	R4-1808400, NR Test Methods SI ad-hoc meeting notes, RAN4#87, Intel Corporation

[10]	R4-1812001, RAN4#88 Meeting report, RAN4#88

[11]	RP-181325, New SID on Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs, RAN#80, CATR, OPPO, Samsung

[12]	3GPP TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

[13]	3GPP TS 38.104, Base Station (BS) radio transmission and reception

[14]	R4-2017678, “Way Forward on PMI Reporting Requirement for NR eMIMO”, RAN4#97-e

[15]	R4-2214397, “WF on UE demodulation and CSI requirements for feMIMO”, RAN4#104b-e

[16]	R4-2210669, “WF on CSI requirement for Rel-17 feMIMO”, RAN4#103-e

[17]	R4-2402277, Discussion on 8Rx general demodulation aspects and spatial channel models, RAN4#110, Nokia, Nokia Shanghai Bell, BT

[18]	R4-2509413, Results collection table for NR SCM, RAN4#116, BT

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

bearing angle

downtilt angle

slant angle

azimuth angle

zenith angle

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AAV	Antenna Array Virtualization

AOA	Azimuth angle Of Arrival

AOD	Azimuth angle Of Departure

AS	Angular Spread

ASA	Azimuth angle Spread of Arrival

ASD	Azimuth angle Spread of Departure

BS	Base Station

CDL	Clustered Delay Line

DS	Delay Spread

FR1	Frequency Range 1

FR2	Frequency Range 2

GCS	Global Coordinate System

LOS 	Line Of Sight

LCS	Local Coordinate System

MIMO	Multiple Input Multiple Output

NLOS	Non-LOS

PAD	Power Angular Distribution

PDP	Power Delay Profile

TDL	Tapped Delay Line

UE	User Equipment

XPR	Cross-Polarization Ratio

ZOA	Zenith angle Of Arrival

ZOD	Zenith angle Of Departure

ZSA	Zenith angle Spread of Arrival

ZSD	Zenith angle Spread of Departure

# 4	Background

## 4.1	General

In the RAN#104 meeting, the SID [2] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#109 in Sep 2025 [3]. The present document is a technical report based upon the study for a practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements for both FR1 (conducted) and FR2 (wireless cable) for RAN4 in 3GPP Rel-19, which provides evaluations on both CDL-based and TDL-based channel modelling approaches.

Generally, the channel model with spatial property has been discussed at the beginning of NR. In Rel-14, two study items were progressed carried out: the first one analyzing how to define a channel model representing the channel for frequencies up to 100 GHz [4], whose outcome was documented in 3GPP TR 38.900 [5], and the second one aiming to develop an NR access technology to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC [6], including the channel model definition that was finally documented in TR 38.901 [7]. RAN1 are using geometric based SCM(E) defined in 38.901 for system level simulation since this channel model is more realistic. 38.901 CDL channel model are derived from SCM(E) with randomness reduction on some parameters, and 38.901 TDL are further derived from 38.901 CDL without spatial property [6]. In Rel-15, RAN4 decided to use simplified TDL model [8] for NR FR1 demodulation test in May 2018 [9] and simplified TDL model for NR FR2 demodulation test in Aug 2018 [10] since majority companies acknowledge TDL is enough for demodulation performance requirements and it is hard to align simulation results using 38.901 CDL at that stage. A New SID [11] on the radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs was approved in June 2018, which targeting for TR 38.827 [12] approval in Dec 2019 and actually approved in June 2020.

Until now, the demodulation performance requirements in RAN4 [8][13] are all based on TDL channel modelling. There are two types of correlation matrices defined for TDL channel modelling, i.e., Uniform Linear Array (ULA), Cross Polarized Antennas (X-pol). Different correlation factors and beam steering approach are also defined to control the correlation levels and steering directions during the tests. Even with applying correlation matrices on these TDL models, all taps would use the same correlation matric which also could not reflect the spatial fading on different clusters. Therefore, it is hard for RAN4 to show the performance gains verified by RAN1 for features need separate directions, such as Rel-16 NR_eMIMO [14], Rel-17 NR_feMIMO [15][16], and even unexpected performance for 8Rx [17]. In order to achieve perfect matching with realistic propagation conditions, separate spatial components for different clusters/taps should be introduced for RAN4 SCM.

# 5	Spatial Channel Modelling Approaches

## 5.1	CDL Approach

The Clustered Delay Line (CDL) channel model is a widely used framework for simulating realistic wireless propagation environments. CDL models capture the spatial, temporal, and frequency-dependent characteristics of radio channels by representing multipath propagation through discrete clusters of scatterers, each contributing multiple delay paths. The details of the CDL based approach employed in the study are provided in this section with the relevant parameter configurations. The model employed in the study is applicable for 3.5 GHz frequency in urban macro scenario.

Figure 5.1.1-1: CDL Modeling Execution

### 5.1.1	Antenna Panel Configuration

The antenna panel is one of the primary components for CDL modeling methodology. The antenna panel configuration employed in this study are captured in simulation assumptions Table 6.1-2.

### 5.1.2	Antenna Array Virtualization

The antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays. One subarray is a unique set of co-polarized antenna elements mapped exclusively to one antenna port through a virtualizer (set of complex values).

The antenna panel (M,N,P,Ms,Ns) and AAV (Ms,Ns) configurations used in the study item are listed below, according to the terminology defined in Annex B.2 (Step 2).

Two different AAV configurations for base station have been considered in the study.

BS AAV Configurations:

- BS AAV(8,1) configuration:

    - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports

    - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

- BS AAV(1,1) configuration (single element subarray):

    - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

    - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

Only one UE AAV(1,1) configuration (single element subarray) has been considered in the study:

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4 RX

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8 RX

Figure 5.1.2-1: Example AAV(1,1) and AAV(8,1) configurations

### 5.1.3	Channel Models

The CDL-C model from 38.901 in Table 7.7.1-3 is taken as a starting point. The parameters for desired angle spread, RMS delay spread are derived based on urban macro (UMa) scenario for 3.5 GHz carrier. The randomness introduced by the channel generation based on 38.901 is alleviated by randomness reduction framework adopted from 38.827 [12].

#### 5.1.3.1	Parameters for Angle Scaling

Note: The angle scaling procedure included in this section was used during the work in Rel-19 study item, and it is based on the companies’ agreed interpretation of the procedure, not captured in the version of TR 38.901 (v18) available during the derivation of the rCDL model. The angular scaling procedure for CDL models was since revised in TR 38.901 (v19), and the procedure in 38.901 shall be applicable to rCDL in future releases. The procedure here is captured as a reference for the procedure and rCDL table reproducibility only.

The angles need to be scaled for the scenario to achieve the desired angular spread and direction. The parameters for scaling the angles of arrival and departure are provided here. The CDL tables provided in 5.1.3.3 include the delay scaling for RMS delay spread of 365ns and desired angle scaling. The scaled cluster (φn,scaled) and scaled ray (φn,m,scaled) angles and cluster specific angular spread are obtained as described below for AOD, AOD, ZOA, and ZOD.

φn,scaled=wrap(ASdesiredASmodel.wrap(φn,model-μφ,model)+μφ,model)                         (5.1.3.1-1)

c{ASA, ASD,ZSA,ZSD}_scaled= ASdesiredASmodel. c{ASA, ASD,ZSA,ZSD}_model                                  (5.1.3.1-2)

φn,m,scaled= φn,scaled+c{ASA, ASD,ZSA,ZSD}_scaledαm                               (5.1.3.1-3)

φn,model	is the tabulated CDL cluster angle from 38.901

ASmodel	is the rms angular spread of the tabulated CDL from 38.901 including the offset ray angles provided in Table 5.1.3.1-1

μφ,model	is the power weighted mean angle of the tabulated CDL from 38.901, calculated using the definition in Annex A.2 of 38.901[7]

cASA, ASD,ZSA,ZSD_model is the tabulated cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD from Table 7.7.1-3 of 38.901 [7].

ASdesired	is the desired rms angular spread provided in Table 5.1.3.1-1

φn,scaled	is the resulting scaled cluster angle.

φn,m,scaled	is the resulting scaled ray angle.

cASA, ASD,ZSA,ZSD_scaled is the scaled cluster-wise RMS angular spread ASA, ASD, ZSA, or ZSD in Table 5.1.3.1-1.

αm			is the offset angle per ray from Table 7.5-3 of 38.901[7]

y = wrap(x) makes y to fall between -180 and +180 degrees so that y = (x+180) mod 360 - 180

μφ=arg{n=1Nm=1Mexp(jφn,m)Pn,m}

Pn,mφn,m

Table 5.1.3.1-1: Angle scaling parameters

| AS |  | ASmodel / ASdesired [deg] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ASD |  | ASA | ZSD | ZSA |
| Model |  | 37.4036 |  | 71.4535 | 4.0665 | 10.4182 |
| Desired |  | 25.7620 |  | 74.1138 | 4.8978 | 18.2050 |

#### 5.1.3.2	Parameters for Randomness Reduction

The randomness reduction framework employs fixed coupling pattern for ray angles and fixed initial phases for polarization matrices in the channel coefficient generation from 38.827 [12].

Within a cluster the AOD angles are coupled to AOD angles using the fixed coupling pattern as specified in Table 7.2-6 in 38.827 [12] and captured in Table 5.1.3.2-1. The same fixed coupling pattern is applied for all clusters n.

Table 5.1.3.2-1: Fixed coupling pattern of ray angles to be applied for each cluster

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

The initial phase {Φn,mθθ,Φn,mθφ,Φn,mφθ,Φn,mφφ} for each ray m of each cluster n and for four different polarization combinations (θθ, θϕ, ϕθ, ϕϕ) are fixed as specified in Table 7.2-8 of 38.827 [12] and captured in Table 5.1.3.2-2.

Table 5.1.3.2-2: Fixed initial phases for 2x2 polarization matrices

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

#### 5.1.3.2	CDL Cluster Reduction

The CDL model is reduced to 12 clusters based on the procedure provided in Annex B.3.1. The channel model parameters in 5.1.3.3-1 is derived based on the angle scaling procedure and parameters in 5.1.3.1.1

#### 5.1.3.3	CDL Channel Models

5.1.3.3.1 rCDL-C1

The rCDL-C1 channel model parameters are derived based on 38.901 CDL-C, Uma scenario parameters in 38.827, 3.5 GHz frequency with the desired delay and angle spread applied and truncated to 12 significant clusters.

Table 5.1.3.3.1-1: Channel model parameters for rCDL-C1

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

### 5.1.4	Channel Power Normalization

To maintain the SNR definition used in RAN4 performance requirements, the channel power must be normalized. However, when directional antennas or antenna array virtualization are used, the channel could introduce a power gain. To account for this gain, a single normalization factor must be applied across all Rx chains. This normalization factor is calculated based on the long-term average PDP for the CDL channel, as well as the spatial response of the antenna and the gain achieved with antenna virtualization, i.e., the AAV gain including AE pattern is taken away as part of the channel normalization.

## 5.2	TDL Approaches

The novel multi-cluster TDL approach builds on top of the widely used and well tested legacy RAN4 TDL channel models. In the legacy models, spatial selectivity is introduced by applying MIMO correlation matrices on top of uncorrelated fading MIMO channel. However, the legacy correlation models (TS 38.101-4: Annex B.2.3) amplify the signal only in the broadside direction (AOD/AOA) of the TX and RX antenna arrays. As a result, the spatial degrees of freedom remain limited, and only a relatively low number of spatial layers can be supported. In order to support a higher number of layers, multiple separate spatial signal directions should exist. This can be achieved by introducing spatial clusters, where each cluster has its own separable mean AOD/AOA.

The multi-cluster TDL models aim to be an easier-to-use and easier-to-configure alternative to CDL models. There is just a handful of high-level parameters that can be chosen to achieve desired channel properties, and to match the scope of different performance tests. To further improve the test coverage, the spatial properties of the clusters (in terms of AOA/AOD) could be defined to be time-varying.

### 5.2.1	MIMO correlation and angular spread

For both LTE and NR, RAN4 demodulation and CSI reporting tests until 3GPP Release 18 employ TDL fading channel models. In many of those tests, spatial selectivity is introduced by applying MIMO correlation matrices on top of an uncorrelated fading MIMO channel. Correlation models are defined for two different antenna array assumptions: Uniform Linear Array (ULA) and Cross-polarized Antennas (X-pol).

MIMO correlation (TDL) and angular spread (CDL) approaches are alternative ways to generate spatial selectivity. High angular spread corresponds to low MIMO correlation and vice versa. As can be seen in Figure 5.2.1-1, a typical 3GPP correlation model generates spatial selectivity that resembles that of a single-cluster CDL channel model. As a result of the real-valued spatial correlation matrix, the strongest beam direction is always at 0⁰ AOD/AOA angle (broadside) of each linear array dimension.

The spatial selectivity of a MIMO channel can be depicted as power angular distribution (PAD) that represents the average channel power response as function of TX or RX direction. Figure 5.2.1-1(a) illustrates the (PAD) measured from an 8TX-8RX X-pol medium correlated TDL channel model (α1=0.3, β=0.6). As can be seen, this channel favors the AODs and AOAs close to zero degrees. The PAD at the TX side is slightly wider than at the RX side because the TX-correlation is lower than the RX-correlation (α < β). Similarly, Figure 5.2.1-1(b) shows the PAD measured from a single-cluster 8TX-8RX CDL channel model with mean AOA = 0 degrees. Here, the angular spread is at the TX side CASD = 1.3710 and at the RX side CASA = 15.632 degrees, and the assumed antenna element radiation patterns is flat.

(a)                                                                                          (b)

Figure 5.2.1-1: Spatial selectivity of X-pol correlated TDL channel (a) and single-cluster CDL channel (b).

### 5.2.2	Multi-cluster TDL channel via TX-RX beam steering

The multi-cluster TDL channel model re-uses and extends the concept of beam steering that has earlier been specified for RAN4 PMI reporting tests ([X] TS 38.101-4: B.2.3.2.3 and B.2.3.2.3A). In TX-RX beam steering, the average AOA and AOD present in a correlated TDL MIMO channel are shifted to desired directions by post-processing. Note that beam steering is a part of the channel model, it is not an action taken by the transmitter or the receiver.

An example of multi-cluster channel is illustrated in Figure 5-2.

Figure 5.2.2-1: TX-RX beam steering channel with two spatial clusters.

The general channel model employs one or more statistically independent time-varying legacy TDL channel instances using some existing spatial correlation model. For the purpose of TX and RX beam steering, the delay taps of each TDL channel instance are split into spatial clusters. The delay taps in a spatial cluster are steered corresponding to common cluster-specific AOD/AOA angles. For mathematical notation, let n(k,m) denote the cluster index corresponding to delay tap m of TDL channel instance k.

The MIMO channel impulse response matrix with dimensions Nrx× Ntx at some time instant is

Hτ=k=1Km=1MPk∙pm∙Dn(k,m)(rx)Hk,mDn(k,m)(tx)∙δτ-τm                                (5.2.2-1)

where

k is the TDL channel instance index, m is the delay tap index, and τ is the delay variable

K is the number of legacy TDL channel instances, and M is the number of delay taps in the TDL model

pm is the power of delay tap m from the TDL power-delay profile, and Pk the power weight of TDL model instance k so that kmPk∙pm=1

τm is the propagation delay of tap m

Hk,m is the spatially correlated fading MIMO channel matrix of delay tap m of TDL channel instance k with unit average power response and dimensions Nrx× Ntx

Dn(k,m)(rx) is the diagonal RX steering matrix of cluster n(k,m) with dimensions Nrx× Nrx

Dn(k,m)(tx) is the diagonal TX steering matrix of cluster n(k,m) with dimensions Ntx× Ntx

δτ is the Dirac delta function.

The implementation complexity of the channel model grows with the number of underlying TDL channel instances K. It is anticipated that one or two TDL channels is adequate for most testing purposes. Due to the diagonal structure of the steering matrices, the matrix multiplications in equation 5.2.2-1 can be implemented with low complexity as element-wise complex phase rotations of matrix Hk,m.

#### 5.2.2.1	Steering matrices for cross-polarized antenna arrays

Steering parameters with cross-polarized (X-pol) antenna array assumption:

- gNB TX-array dimensions: Ntx1 and Ntx2 are the number of antenna elements with the same polarization in the first and the second array dimension. The total number of TX antennas is Ntx=2∙Ntx1∙Ntx2.

- UE RX-array dimensions: Nrx1 and Nrx2 are the number of antenna elements with the same polarization in the first and the second array dimension. The total number of TX antennas is Nrx=2∙Nrx1∙Nrx2. Note that 3GPP X-pol model (TS 38.101-4 Annex B.2.3.2) supports only 1D RX-arrays so that Nrx2=1.

- TX phases: θtx1(n) and θtx2(n) per cluster n

- RX phases: θrx1(n) and θrx2(n) per cluster n

The steering matrices are defined as:

Dn(tx)=1001⊗Dθtx1(n),Ntx1⊗Dθtx2(n),Ntx2

Dn(rx)=1001⊗Dθrx1(n),Nrx1⊗Dθrx2(n),Nrx2

where the steering matrix of size N× N as function of phase θ is defined as

Dθ,N=diag1ejθ⋯ejθ(N-1)

Assuming the antenna arrays employ uniform distance between elements in each linear dimension, the phase shift θ corresponds to AOA or AOD angle φ so that θφ,dλ=2π∙dλ∙sinφ, where d is the distance between adjacent antenna elements and λ is the carrier wavelength.

The steering phases θtx1(n), θtx2(n), θrx1(n), θrx2(n) may be specified either directly or by specifying the corresponding directional angles φtx1(n), φtx2(n), φrx1(n), φrx2(n).

The beam steering phases can be either constant or (slowly) time-varying. In general, the model parameters can be chosen to match the scope of each test case. For example, the number of clusters can be selected so that the desired number of layers (rank) is supported. Optionally, cluster-specific Doppler shifts can also be specified.

### 5.2.3	Example cluster model

In this section, parameterization of a four-cluster channel employing two TDLC channel instances is given. In model xTDL-C1 in Table 5.2.3-1, the two TDLC instances are each further split into two clusters so that delay taps 1-4 are grouped into one and taps 5-12 into another cluster.

Assuming 8TX-8RX array setup with (Ntx1, Ntx2, Nrx1, Nrx2)= (4, 1, 4, 1), the steering phases both at TX and RX side are chosen so that the TX and RX beams become mutually orthogonal between clusters. Orthogonality between the beams maximizes the spatial multi-layer capacity of the channel for a given set of power weights.

The cluster power distribution also affects the spatial properties of the channel. For a set of orthogonal beams, assigning equal power for all clusters results in the highest channel capacity. The total power of a cluster depends on the power weight of the corresponding TDL channel instance, and on the sum power of the delay taps included in the cluster. Thus, the cluster power distribution can be adjusted by two different methods: by setting the power weights and by splitting the delay taps into clusters so that a desired distribution is achieved.

Table 5.2.3-1: Cluster model xTDL-C1: 2 TDLC channel instances, 4 clusters

| TDLC-300 / XP-high |  |  | TX-RX beam steering |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | TDL channel 1 Power: 0 dB |  |  | TDL channel 2 Power: -9 dB |  |  |
|  |  |  | Cluster index | θtx1 | θrx1 | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 | 3 | π | π |
| 2 | 65 | 0 |  |  |  |  |  |  |
| 3 | 70 | -7.7 |  |  |  |  |  |  |
| 4 | 190 | -2.5 |  |  |  |  |  |  |
| 5 | 195 | -2.4 | 2 | -π/2 | -π/2 | 4 | π/2 | π/2 |
| 6 | 200 | -9.9 |  |  |  |  |  |  |
| 7 | 240 | -8.0 |  |  |  |  |  |  |
| 8 | 325 | -6.6 |  |  |  |  |  |  |
| 9 | 520 | -7.1 |  |  |  |  |  |  |
| 10 | 1045 | -13.0 |  |  |  |  |  |  |
| 11 | 1510 | -14.2 |  |  |  |  |  |  |
| 12 | 2595 | -16.0 |  |  |  |  |  |  |

φn,scaled=wrap(ASdesiredASmodel.wrap(φn,model-μφ,model)+μφ,model)

φn,model

ASmodel

μφ,model

ASdesired

φn,scaled

μφ=arg{n=1Nm=1Mexp(jφn,m)Pn,m}

Pn,mφn,m

{Φn,mθθ,Φn,mθφ,Φn,mφθ,Φn,mφφ}

|  | Φmθθ | Φmϕθ | Φmθϕ | Φmϕϕ |
| --- | --- | --- | --- | --- |

-

-

Nrx×Ntx

Hτ=k=1Km=1MPk∙pm∙Dn(k,m)(rx)Hk,mDn(k,m)(tx)∙δτ-τm

τ

M

pmPkkmPk∙pm=1

τm

Hk,mNrx×Ntx

Dn(k,m)(rx)Nrx×Nrx

Dn(k,m)(tx)Ntx×Ntx

δτ

Ntx1Ntx2

Ntx=2∙Ntx1∙Ntx2

Nrx1Nrx2Nrx2=1

Nrx=2∙Nrx1∙Nrx2

θtx1(n)θtx2(n)

θrx1(n)θrx2(n)

Dn(tx)=1001⊗Dθtx1(n),Ntx1⊗Dθtx2(n),Ntx2

Dn(rx)=1001⊗Dθrx1(n),Nrx1⊗Dθrx2(n),Nrx2

N×Nθ

Dθ,N=diag1ejθ⋯ejθ(N-1)

θφθφ,dλ=2π∙dλ∙sinφ

θtx1(n)θtx2(n)θrx1(n)θrx2(n)φtx1(n)φtx2(n)φrx1(n)φrx2(n)

(Ntx1Ntx2Nrx1Nrx2)=

|  |  |  |  | θtx1 | θrx1 |  | θtx1 | θrx1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | π | π |
|  |  |  |  | -π/2 | -π/2 |  | π/2 | π/2 |

# 6	Comparison of spatial channel Models

## 6.1	PDSCH performance under SU-MIMO scenario

This section provides comparison for different spatial channel models with common simulation assumptions captured in Table 6.1-1.

Table 6.1-1: Common Simulation assumptions

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Rank |  |  | 4, 8 |
| Antenna configuration |  |  | Rank4: 4T4R Rank8: 8T8R |
| MCS |  |  | 13 (64 QAM table) |
| Channel model |  |  | rCDL-C1 xTDL-C1 TDLC300-Low TDLC300-ULA Med TDLC300-ULA High TDLC300-XP Med TDLC300-XP High |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | Single Panel Type I; Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook. |
|  |  | Codebook configuration | (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | Rank 4: SNR@70% of maximum throughput Rank8: SNR(dB) @ 30% and 70% of max throughput for each codeword |

Simulation assumptions for rCDL-C1 channel specifically are captured in Table 6.1-2:

Table 6.1-2: Simulation assumptions for rCDL-C1 channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank8 |  |
| FR / Carrier frequency |  | FR1, 3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

The following comparison test cases are included

- FR1 SU-MIMO PDSCH 4Tx 4Rx 4 layers

- FR1 SU-MIMO PDSCH 8Tx 8Rx 8 layers

Table 6.1-3: Simulation result summary of SNR@70% of maximum throughput for FR1 SU-MIMO PDSCH 4Tx 4Rx with 4 layers

| Channel model | UE speed/Doppler | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rCDL-C1 | 3 kph | 15.5 | 14.3 | 16.48 | 15.6 | 15.9 | 16.6 | 14.7 | 15.6 |  |
| xTDL-C1 | 10 Hz |  |  |  | 13.5 | 15.9 |  | 14 | 15.4 | 15.8 |
| TDLC300-Low | 10 Hz | 14 |  | 13.21 |  | 14.7 | 14.3 | 13 | 13.6 |  |
| TDLC300-ULA Med | 10 Hz | N/A |  |  |  | 49.2 | N/A |  | N/A |  |
| TDLC300-ULA High | 10 Hz |  |  |  |  |  |  |  | N/A |  |
| TDLC300-XP Med | 10 Hz | 15.3 |  | 14.95 |  | 16.3 |  |  | 15.5 |  |
| TDLC300-XP High | 10 Hz |  |  |  |  |  |  |  | 27.6 |  |
| rCDL-C1 | 30 kph | 16.3 | 15.34 | 17.17 |  |  | 17.9 | 15.2 | 17.5 |  |
| xTDL-C1 | 100 Hz |  |  |  |  |  |  | 14.5 |  |  |
| TDLC300-Low | 100 Hz | 14.5 |  | 14.05 |  | 15.6 |  | 13.3 |  |  |
| TDLC300-ULA Med | 100 Hz | N/A |  |  |  | N/A |  |  |  |  |
| TDLC300-ULA High | 100 Hz |  |  |  |  |  |  |  |  |  |
| TDLC300-XP Med | 100 Hz | 16.1 |  | 14.95 |  | 17.4 |  |  |  |  |
| TDLC300-XP High | 100 Hz |  |  |  |  | 15.9 | 16.6 | 14.7 | 15.6 |  |

Table 6.1-4: Simulation result summary of SNR@70% of maximum throughput for FR1 SU-MIMO PDSCH 8Tx 8Rx with 8 layers (sources #1-#5)

| Channel model | UE speed/Doppler | Source #1 |  | Source #2 |  | Source #3 |  | Source #4 |  | Source #5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CW1 | CW2 | CW1 | CW2 | CW1 | CW2 | CW1 | CW2 | CW1 | CW2 |
| rCDL -C 1 | 3 kph |  |  | 16.9 | 23.7 | 14.2 | 26 | 16.5 | 24.4 | 16.7 | 24 |
| xTDL-C1 | 10 Hz |  |  |  |  |  |  | 14.5 | 21.2 | 11.1 | 19.7 |
| TDLC300-Low | 10 Hz |  |  |  |  |  |  |  |  | 15.7 | 15.8 |
| TDLC300-ULA Med | 10 Hz |  |  |  |  |  |  |  |  | N/A | N/A |
| TDLC300-ULA High | 10 Hz |  |  |  |  |  |  |  |  |  |  |
| TDLC300-XP Med | 10 Hz |  |  |  |  | N/A | N/A |  |  | N/A | 35.1 |
| TDLC300-XP High | 10 Hz |  |  |  |  |  |  |  |  |  |  |
| rCDL -C 1 | 30 kph |  |  | 18.13 | 23.91 | 15.32 | 28.54 |  |  |  |  |
| xTDL-C1 | 100 Hz |  |  |  |  |  |  |  |  |  |  |
| TDLC300-Low | 100 Hz |  |  |  |  |  |  |  |  | 16.4 | 16.4 |
| TDLC300-ULA Med | 100 Hz |  |  |  |  |  |  |  |  | N/A | N/A |
| TDLC300-ULA High | 100 Hz |  |  |  |  |  |  |  |  |  |  |
| TDLC300-XP Med | 100 Hz |  |  |  |  | N/A | N/A |  |  | N/A | 39.1 |
| TDLC300-XP High | 100 Hz |  |  |  |  |  |  |  |  |  |  |

Table 6.1-5: Simulation result summary of SNR@70% of maximum throughput for FR1 SU-MIMO PDSCH 8Tx 8Rx with 8 layers (sources #6-#9)

| Channel model | UE speed/Doppler | Source #6 |  | Source #7 |  | Source #8 |  | Source #9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CW1 | CW2 | CW1 | CW2 | CW1 | CW2 | CW1 | CW2 |
| rCDL -C 1 | 3 kph | 17.5 | 24.7 | 17.2 | 23.4 | 18.5 | 24.4 |  |  |
| xTDL-C1 | 10 Hz |  |  | 15.4 | 16.6 | 14.8 | 20.4 |  |  |
| TDLC300-Low | 10 Hz | 15.9 | 15.9 | 15.5 | 15.7 | 14.4 | 14.3 |  |  |
| TDLC300-ULA Med | 10 Hz | N/A | N/A |  |  | N/A | N/A |  |  |
| TDLC300-ULA High | 10 Hz |  |  |  |  | N/A | N/A |  |  |
| TDLC300-XP Med | 10 Hz |  |  |  |  | 27.4 | N/A |  |  |
| TDLC300-XP High | 10 Hz |  |  |  |  | N/A | N/A |  |  |
| rCDL -C 1 | 30 kph | 18.6 | 25.5 | 17.4 | 23.8 | 21.1 | 26.4 |  |  |
| xTDL-C1 | 100 Hz |  |  | 16 | 17.4 |  |  |  |  |
| TDLC300-Low | 100 Hz |  |  | 16.4 | 16.6 |  |  |  |  |
| TDLC300-ULA Med | 100 Hz |  |  |  |  |  |  |  |  |
| TDLC300-ULA High | 100 Hz |  |  |  |  |  |  |  |  |
| TDLC300-XP Med | 100 Hz |  |  |  |  |  |  |  |  |
| TDLC300-XP High | 100 Hz |  |  |  |  |  |  |  |  |

For more results with full curves refer to [18].

Regarding legacy TDL the following observations can be drawn:

- For TDL Low there is an overly optimistic channel matrix condition number

- For TDL nonLow a heuristic correlation tuning is required to show expected performance gain

Regarding rCDL-C1 the following observations can be drawn:

- For rCDL-C1 a spatial channel effect is observed

Regarding xTDL-C1 the following observations can be drawn:

- For xTDL-C1 a spatial channel effect is observed

|  |  | 65°,90° |  |
| --- | --- | --- | --- |

## 6.2	PMI performance under SU-MIMO scenario

This section provides comparison results for different spatial channel modes with simulation assumptions captured in Table 6.2-1.

Table 6.2-1: Common simulation assumptions for PMI

| Parameter |  |  | Unit |  | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz |  | 40 |  |
| Subcarrier spacing |  |  | kHz |  | 30 |  |
| Duplex Mode |  |  |  |  | TDD |  |
| TDD DL-UL configurations |  |  |  |  | 7D1S2U S=6D+4G+4U |  |
| Antenna configuration |  |  |  |  | 8 x 4 (M, N, P, Ms, Ns) = (1, 4, 2, 1, 1) |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  |  | 8 |  |
|  | CDM Type |  |  |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  |  | Row 8, (4,6) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | Row 8, (5) |  |
|  | CSI-RS interval and offset |  | slot |  | 10, 1 |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  |  | Not configured for eType II Wideband for Type I |  |
| Sub-band Size |  |  | RB |  | 8 |  |
| csi-ReportingBand |  |  |  |  | 11111111111111 |  |
| Codebook configuration | Codebook Type |  |  |  | typeII-r16 typeI-SP |  |
|  | eType II CB config | paramCombination-r16 |  |  | 6 (L =4, pν =1/2, β=1/2 ) |  |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |
|  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  |  | Rank 2: 0010 Rank 4: 1000 |  |
| Physical channel for CSI report |  |  |  |  | PUSCH |  |
| CQI/RI/PMI delay |  |  | ms |  | 7 |  |
| Maximum number of HARQ transmission |  |  |  |  | 4 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  | Type I: Random and Follow PMI. eType II: Folow PMI |  |
| Note: Use DM-RS based FOE and compensation. |  |  |  |  |  |  |

Table 6.2-2: Simulation assumptions for CDL channel

| Parameter |  |  | Value |  |
| --- | --- | --- | --- | --- |
| FR / Carrier frequency |  |  | FR1,3.5GHz |  |
| UE speed and movement direction |  |  | 3km/h, (65°,90°) |  |
| Channel Geometry | LCS UE |  | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB |  | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE |  | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB |  | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation |  | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern |  | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation |  | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern |  | Omnidirectional |  |
|  | Antenna Panel Placement |  | YZ Plane |  |

The BS antenna configuration for CDL comparison is one antenna element per subarray.

- 8Tx : (M, N, P, Ms, Ns) = (1, 4, 2, 1, 1).

The following comparison test cases are included this chapter:

- FR1 SU-MIMO PMI 8Tx 4Rx 4 layers with Type I.

- FR1 SU-MIMO PMI 8Tx 4Rx 4 layers with eType II.

- FR1 SU-MIMO PMI 8Tx 4Rx 2 layers with Type I.

- FR1 SU-MIMO PMI 8Tx 4Rx 2 layers with eType II.

Table 6.2-3: Simulation result summary for FR1 SU-MIMO Follow PMI 8Tx 4Rx with 4 layers

| Channel Model | Doppler/Speed | PMI | Gamma at 90% Norm. Throughput |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| rCDL-C1 | 3km/h | Type I | 1.8 |  | 1.8 |  | 1.8 | 1.8 |  | 1.7 |  |
|  |  | eType II | 1.8 |  | 1.8 |  | 1.8 | 1.7 |  | 1.4 |  |
| xTDL-C1 | 10Hz | Type I |  |  |  |  | 1.6 |  |  | 1.4 |  |
|  |  | eType II |  |  |  |  | 1.3 |  |  | 1.3 |  |
| TDLC-300 low | 10Hz | Type I | 1.3 |  |  |  | 1.1 | 1.1 |  |  |  |
|  |  | eType II | 1.3 |  |  |  | 1.0 | 0.9 |  |  |  |
| TDLC-300 Med | 10Hz | Type I |  |  |  |  | N/A | N/A |  |  |  |
|  |  | eType II |  |  |  |  | N/A | N/A |  |  |  |
| TDLC-300 XP Med | 10Hz | Type I | 1.9 |  | 1.92 |  | 1.8 | 1.8 |  |  |  |
|  |  | eType II | 1.8 |  | 1.92 |  | 1.7 | 1.2 |  |  |  |
| TDLC-300 XP High | 10Hz | Type I | 2.0 |  |  |  |  |  |  | 2.1 |  |
|  |  | eType II | 2.0 |  |  |  |  |  |  | 2.0 |  |

Table 6.2-4: Simulation result summary for FR1 SU-MIMO Follow PMI 8Tx 4Rx with 4 layers

| Channel Model | Doppler/Speed | PMI | SNR at 70% Norm. Throughput |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| rCDL-C1 | 3km/h | Type I | 12.4 | 11.0 | 9.7 | 10.3 | 10.2 | 9.7 | 9.5 | 10.2 | 9.8 |
|  |  | eType II | 14.0 | 14.3 | 8.4 | 11.6 | 10.1 | 10.8 | 8.2 | 13.8 | 10.8 |
| xTDL-C1 | 10Hz | Type I |  |  |  | 12.3 | 13.0 |  | 11.3 | 10.8 |  |
|  |  | eType II |  |  |  | 13.5 | 14.7 |  | 10.9 | 11.6 |  |
| TDLC-300 low | 10Hz | Type I | 12.6 |  |  |  | 14.4 | 13.2 |  |  |  |
|  |  | eType II | 12.8 |  |  |  | 15.0 | 15.7 |  |  |  |
| TDLC-300 Med | 10Hz | Type I |  |  |  |  | N/A | N/A | N/A |  |  |
|  |  | eType II |  |  |  |  | N/A | N/A | N/A |  |  |
| TDLC-300 XP Med | 10Hz | Type I | 12.7 |  | 13 |  | 15.2 | 18.9 |  |  |  |
|  |  | eType II | 12.8 |  | 12.66 |  | 15.3 | 24.5 |  |  |  |
| TDLC-300 XP High | 10Hz | Type I | 18.6 |  |  |  |  |  |  | 23.4 |  |
|  |  | eType II | 17.4 |  |  |  |  |  |  | 24.6 |  |

Table 6.2-5: Simulation result summary for FR1 SU-MIMO Follow PMI 8Tx 4Rx with 2 layers

| Channel Model | Doppler/Speed | PMI | Gamma at 90% Norm. Throughput |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| rCDL-C1 | 3km/h | Type I | 2.0 |  | 2.4 |  | 2.3 | 2.3 |  | 1.7 |  |
|  |  | eType II | 1.9 |  | 2.9 |  | 2.4 | 2.4 |  | 1.6 |  |
| xTDL-C1 | 10Hz | Type I |  |  |  |  | 1.7 |  |  | 1.5 |  |
|  |  | eType II |  |  |  |  | 1.9 |  |  | 1.6 |  |
| TDLC-300 low | 10Hz | Type I | 1.5 |  |  |  | 1.3 |  |  |  |  |
|  |  | eType II | 1.8 |  |  |  | 1.8 |  |  |  |  |
| TDLC-300 Med | 10Hz | Type I |  |  | 2.21 |  | 2.1 | 1.05 |  |  |  |
|  |  | eType II |  |  | 2.3 |  | 2.1 | 1.4 |  |  |  |
| TDLC-300 XP Med | 10Hz | Type I | 2.8 |  | 3.1 |  | 2.5 | 2.1 |  |  |  |
|  |  | eType II | 2.6 |  | 3.16 |  | 2.5 | 2 |  |  |  |
| TDLC-300 XP High | 10Hz | Type I | 6.1 |  |  |  |  |  |  | 3.7 |  |
|  |  | eType II | 3.3 |  |  |  |  |  |  | 2.3 |  |

Table 6.2-6: Simulation result summary for FR1 SU-MIMO Follow PMI 8Tx 4Rx with 2 layers

| Channel Model | Doppler/Speed | PMI | SNR at 70% Norm. Throughput |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Source #9 |
| rCDL-C1 | 3km/h | Type I | 4.7 |  | 0.2 |  | 1.7 | 0.9 |  | 2.2 |  |
|  |  | eType II | 5.3 |  | -0.8 |  | 1.5 | 1.5 |  | 3.3 |  |
| xTDL-C1 | 10Hz | Type I |  |  |  |  | 4.1 |  |  | 4.3 |  |
|  |  | eType II |  |  |  |  | 2.8 |  |  | 3.5 |  |
| TDLC-300 low | 10Hz | Type I | 5.1 |  |  |  | 5.6 |  |  |  |  |
|  |  | eType II | 4.4 |  |  |  | 4.1 |  |  |  |  |
| TDLC-300 Med | 10Hz | Type I |  |  | 11.37 |  | 14.1 | 1.5 |  |  |  |
|  |  | eType II |  |  | 10.79 |  | 14.0 | 1.3 |  |  |  |
| TDLC-300 XP Med | 10Hz | Type I | 2.1 |  | 0.76 |  | 2.7 | 3.8 |  |  |  |
|  |  | eType II | 2.7 |  | 0.49 |  | 2.8 | 5.4 |  |  |  |
| TDLC-300 XP High | 10Hz | Type I | 2.1 |  |  |  |  |  |  | 1.1 |  |
|  |  | eType II | 2.9 |  |  |  |  |  |  | 4.5 |  |

For more results with full curves refer to [18].

Regarding legacy TDL the following observations can be drawn:

- For TDL Low there is an overly optimistic channel matrix condition number

- For TDL Low no performance impact from the PMI choice

- For TDL nonLow a heuristic correlation tuning is required to show expected performance gain

Regarding rCDL-C1 the following observations can be drawn:

- For rCDL-C1 a spatial channel effect is observed

Regarding xTDL-C1 the following observations can be drawn:

- For xTDL-C1 a spatial channel effect is observed

|  |  | 65°,90° |  |
| --- | --- | --- | --- |

## 6.3	Channel Properties

Spatial channel properties were analysed qualitatively across TDL and CDL models in reference to provided field measurements, including angular distribution in Tx and Rx directions (stability and diversity) and spatial layer properties. Measurement results provided to this study are included in Annex A.

### 6.3.1	CDL

#### 6.3.1.1	Spatial properties

Estimated direction of arrivals (DoAs) from field measurements demonstrate a limited number of stable directions (environment properties).

Figure 6.3.1.1-1: Three primary AoAs for SRS from UE in Locations A and C using MUSIC algorithm.

DoAs of the TR 38.753 based CDLC channel provide a limited number of mid-term stable directions (clusters), that slowly fade in and out over time (or rather with RE distance).

| Slot 20 | Slot 1002 |
| --- | --- |

Figure 6.3.1.1-2: rCDL-C1 Bartlett DoA analysis vs. relative “RE distance” (x-axis is DoA):
4x1 Xpol ULA assumption.

DoAs of the TR 38.753 based CDLC channel, we observe a limited number of mid-term stable directions (clusters), that slowly fade in and out over time (or rather with RE distance).

| Slot 1 | Slot 100 |
| --- | --- |

Figure 6.3.1.1-3: TR 38.827 based CDLC Uma Bartlett DoA analysis vs. relative “RE distance” (x-axis is DoA):
4x1 Xpol ULA assumption.

#### 6.3.1.2	SINR distributions

Per layer post-EQ SINR of each MIMO layer measured after the application of a baseband receiver/equalizer on the channel facing receive ports has been evaluated for CDL and TDL models in reference to measurements from field deployments. Post-EQ processed SINR distributions from field measurements, demonstrate that each spatial layer exhibits individual loss in a realistic deployment [R4-2402277, R4-2411557].

Figure 6.3.1.2-1: Histograms of the per layer SINR assuming SVD precoding and combining for measurement locations with a strong line of sight (B), and for non-line of sight position (D).

The post-EQ SINR distributions are derived with both random and fixed TypeI precoding and assuming MMSE-IRC receivers [R4-2509395]. The PDSCH post-EQ SINR profiles, when using TDL channel models do not match measurements. CDL both shows typical post-EQ SINR profiles and typical deployment spatial components, as shown by measured spatial properties presented in this sub clause, this is observed by 2 companies. 1 company observes that if TDL is configured with suitable MIMO correlation and antenna array configuration, spatial properties of typical deployments can be observed, no results are presented for this in the TR for this observation.

Medium Correlation and ULA configuration is not practical for 8T8R configurations with 8 layers, as observed by 4 companies.

| TDLC low | TDLC Med | 38.753 CDLC |
| --- | --- | --- |

Figure 6.3.1.2-2: Post-EQ SINR distributions for channel candidates under random (top row) and fixed (bottom row) precoding.

#### 6.3.1.3	Observations

Following observations can be drawn:

- The spatial properties of rCDL-C1 matches presented measurements of a MIMO deployment captured in this sub clause and Annex A, as observed by 3 companies.

- CDL (link level) models are based on the same paradigm that is extensively used for system-level simulations by RAN1 and regularly used for link-level simulations by RAN1 to develop MIMO related features.

- rCDL-C1 corresponds to a single possible physical environment example with static long-term spatial properties in line with the UMa CDL-C realisation parameters as included in TR 38.901 v18.0.0.

- In this study item, RAN4 contributors spent considerable effort to clarify and align the understanding and implementation of CDL models.

- If RAN4 considers the application of rCDL to settings that involved time-domain prediction, further consideration on the deterministic behaviour of rCDL, and if necessary, countermeasures are to be considered as observed by 3 companies.

### 6.3.2	TDL

#### 6.3.2.1	Spatial properties

Looking at the DoAs of a 3GPP (low correlation) TDL channel, the large-scale spatial preference of the per RE channels is seen to fully decorrelate after about 2.5ms (5 slots in our simulation); it could be argued that this already occurs after only 1ms

| Slot 1 | Slot 5 |
| --- | --- |

Figure 6.3.2.1-1: TDLC300-100 (low) Bartlett DoA analysis vs. relative “RE distance”:

DoAs of a 3GPP TDLC300-100 MedA channel, we can directly see the limitation to broadside spatial preference, which remains unchanged indefinitely.

| Slot 1 | Slot 5 |
| --- | --- |

Figure 6.3.2.1-2: TDLC300-100 (MedA) Bartlett DoA analysis vs. relative “RE distance”:

#### 6.3.2.2	Observations

Following observations can be drawn:

- Spatial properties of legacy channel models do not match the measured typical deployment MIMO characteristics

- The PDSCH post-EQ SINR profiles, when using TDL channel models do not match measurements. SDM processing does not impact performance, when using TDL channel models.

- TDL channel models are very simple and extensively used in RAN4 demodulation and CSI testing.

- Multi-cluster TDL models builds on top of the well-known and well-aligned legacy TDL models.

- Legacy TDL correlation models and related correlation derivation models introduce strong spatial selectivity so that higher transmission ranks are either infeasible or require unreasonably high SNR or low MCS.

- The multi-cluster TDL model reduces the spatial limitations of the underlying spatially correlated legacy TDL model so that higher ranks can be supported.

- The multi-cluster TDL model does not alter the Doppler spread or the frequency selectivity of the underlying legacy TDL model.

- The multi-cluster TDL model can be configured using a limited number of beam-steering parameters to match desired test behaviour. The steered beam directions and the relative beam power offsets are artificially configured.

## 6.4	Channel Model Statistics Comparison

In this chapter, channel statistics for evaluation, comparison, alignment, and verification of channel models are defined and illustrated. Channel statistics can be gathered without the tput simulation burden and uncertainty resulting from different receiver algorithms implementations.

To specify the channel metrics, the following notations are defined. Arrange MIMO channel as a complex-valued 7-dimensional array:

hn,k,m1,m2,l,f, t , where

  - n=0,1,…, Nrx1-1 is RX-antenna index within one polarization group,

  - k=0,1 is the RX-antenna polarization group index,

  - m1=0,1,…, Ntx1-1 is 1st TX-antenna index within one polarization group,

  - m2=0,1,…, Ntx2-1 is 2nd TX-antenna index within one polarization group,

  - l=0,1 is the TX-antenna polarization group index,

  - f is frequency (subcarrier) index for channel frequency response (CFR), and

  - t is time (symbol) index.

Here it is assumed that Ntx1 is the TX-array size in horizontal direction and Ntx2 in vertical direction. For 4TX and 8TX alignment cases, Ntx2=1. It is also worth noting that channel impulse response (CIR) representation can alternatively be used (except for frequency coherence) by replacing the frequency dimension with delay tap index dimension.

### 6.4.1	Metric definitions and direct measurement from channel realization

Spatial domain power density (SDPD) is defined at RX side as

SDPDrx1θ=Ek,m1,m2,l, f,t1Nrx1n=0Nrx1-1hn,k,m1,m2,l, f, t∙e-j∙θ∙n2

and at TX side as

SDPDtx1θ=En,k,m2,l, f,t1Ntx1m1=0Ntx1-1hn,k,m1,m2,l, f, t∙e-j∙θ∙m12

Here, the range for phase shift θ is (-π,π) or equivalently (0,2π). FFT can be utilized for fast calculation of SDPD. Notation E(index list) denotes statistical mean over the listed indexes/dimensions.

The complex-valued time coherence (TC) is defined as

TC∆t=En,k,m1,m2,l, f,th*n,k,m1,m2,l, f, t∙hn,k,m1,m2,l, f, t+∆t

where ∆t=0, 1, 2,…Nt symbols (or multiples of symbols).

The complex-valued frequency coherence (FC) is defined as

FC∆f=En,k,m1,m2,l, f,th*n,k,m1,m2,l, f, t∙hn,k,m1,m2,l, f+∆f, t

where ∆f=0, 1, 2,…Nf subcarriers (or multiples of subcarriers).

The average channel power is assumed to be normalized to unity so that FC0=TC0=1. As a result, the mean value of SDPD is also one.

### 6.4.2	Theoretical SDPD curves from CDL model parameters

For CDL models, the theoretical expected SDPD can directly be calculated from model parameters such as AOD, ZOD, AOA ray angles φn,mtx1 , φn,mtx2 , φn,mrx1 and ray powers pn,m, where n is the cluster index and m is the ray index. The ray indexing here is assumed to include the effect of applying the fixed coupling pattern of ray angles given in Table 5.1.4.2-1.

The per-ray channel processes are assumed uncorrelated so that their power responses can be summed. At TX side:

SDPDtx1θ=1Ntx1n,mpn,mk=0Ntx1-1expj∙k∙θn,mtx1-θ2

where θn,mtx1=π∙sinφn,mtx1-αtx and αtx=0 is the horizontal TX antenna array rotation angle. At RX side:

SDPDrx1θ=1Nrx1n,mpn,mk=0Nrx1-1expj∙k∙θn,mrx1-θ2

where θn,mrx1=π∙sinφn,mrx1-αrx and αrx=180 is the horizontal RX antenna array rotation angle. The ray powers include the tabulated cluster powers as well as the attenuation effect of the TX-side antenna radiation pattern Aφn,mtx1-αtx,φn,mtx2-βtx, where βtx=10 is the TX-array downtilt angle and φn,mtx2 is the ZOD angle of the ray. The powers are assumed normalized so that n,mpn,m=1.

### 6.4.3	Theoretical SDPD curves from multi-cluster TDL model parameters

For multi-cluster TDL models, the expected SDPD can directly be calculated from the cluster-specific TX-RX steering parameters θtx1(n), θrx1(n) and from the spatial correlation matrices of the underlying TDL model. For notational convenience, define column vector d of length N as function of phase shift x: dx,N=1ejx⋯ejx(N-1)T. At TX side:

SDPDtx1θ=1Ntx1npn∙dHθtx1(n)-θ,Ntx1∙Ctx1∙dθtx1(n)-θ,Ntx1

where Ctx1 is the Ntx1×Ntx1 spatial correlation matrix in the first TX-array dimension with correlation parameter α1=0.9 for XP-High. Here, power pn of cluster n is the aggregate power of all the channel taps of the cluster. The powers are assumed normalized so that npn=1.

Similarly, at RX side:

SDPDrx1θ=1Nrx1npn∙dHθrx1(n)-θ,Nrx1∙Crx1∙dθrx1(n)-θ,Nrx1

where Crx1 is the Nrx1×Nrx1 spatial correlation matrix in the first RX-array dimension with correlation parameter β=0.9 for XP-High.

### 6.4.4	Numerical results

In this section, metrics for CDL and TDL models are plotted.

Here, the final truncated 12-cluster model rCDL-C1 is referred to as A4, and its untruncated 24-cluster version is referred to as A2. Figure 6.4.4-1 depicts the SDPD of CDL-C A2 and A4. As can be seen, the TX side is much more spatially selective than the RX side. The difference between models A2 and A4 is very small. The theoretical and measured curves match very well at TX side, while at RX side some difference remains.

Figure 6.4.4-2 shows the absolute value of FC for CDL-C A2 and A4. As can be seen, the frequency coherence properties of the 24-cluster A2 and the 12-cluster A4 models are somewhat different. This is mainly due to the post-truncation delay scaling.

Figure 6.4.4-3 shows the TC of CDL-C A2 and A4. As can be seen, TC is complex-valued. This implies that the Doppler spectrum is not symmetric around zero frequency. The shapes of TC for A2 and A4 are similar.

Figure 6.4.4-1: Measured and theoretical SDPD at TX (left) and at RX (right) for CDL-C.

Figure 6.4.4-2: Measured frequency coherence (FC) for CDL-C.

Figure 6.4.4-3: Measured time coherence (TC) for CDL-C with 3 km/h UE speed.

The left side of Figure 6.4.4-4 depicts the SDPD of two multi-cluster TDL model configurations, as detailed in Table 5.2.3-1 for xTDL-C1, and in Table C.1.1.2-1 for the alternative xTDL-C model. Note that the models are defined to be symmetrical so that the phase shifts and correlation coefficients at the TX and RX side are the same. Therefore, also the SDPD at the TX and the RX sides are identical with 8TX-8RX antenna arrays. In general, multi-cluster TDL models need not be symmetrical. The channels are spatially selective, and the theoretical curves match well with the measured curves. The spatial selectivity pattern can be tailored to suit any test purpose by properly selecting the cluster-specific phase shifts and powers.

For comparison, the right side of Figure 6.4.4-4 depicts the SDPD of legacy TDL channels with different correlation levels. As can be seen, spatial correlation results in spatial selectivity so that the signal is amplified at beam phase zero. The theoretical curves match with the measured curves very closely.

Figure 6.4.4-5 shows the absolute value of FC for the two multi-cluster TDL models. As can be seen, their frequency coherence properties are identical. This is because the channel tap delays and powers are directly inherited from the underlying TDL-C model.

Figure 6.4.4-6 shows the TC for the two multi-cluster TDL models. Time coherence is in practice real-valued (imaginary part is very close to zero) and it follows the shape of Bessel function. This is due to the symmetric Jakes Doppler spectrum inherited from the underlying TDL-C channel.

Figure 6.4.4-4: SDPD of multi-cluster TDL-C channels (left) and legacy TDL-C channels (right).

Figure 6.4.4-5: Measured FC (absolute value) of multi-cluster TDL-C channels.

Figure 6.4.4-6: Measured TC (real part) of multi-cluster TDL-C channels with 10Hz Doppler.

### 6.4.5	Single-cluster time correlation properties

One company studied the temporal correlation properties of a single cluster in the CDL profile. For this analysis the dominant cluster (cluster with the strongest power) was selected, and its temporal autocorrelation (according to the textbook definition) is represented in Figure 6.4.5.-1. The UE speed is 3km/h and the equivalent Doppler spread is 10Hz

In general, the time evolution of the CDL coefficients results to be deterministic, given the complex gain per each ray at time t=0. That is because, the radiation pattern gains, analog beamforming gains, polarization leakage, initial phases, etc, can all be combined into ray gains that depend on the realization (initial seed) but are static for every run. Each cluster is then composed by a set of 20 rays, which sum up to the cluster average power according to the CDL profile table.

The following observations can be drawn based on the results:

- The autocorrelation shows a periodic nature, which can be expected due to the inherent periodicity of the component rays.

- The autocorrelation does not show a decay with time and strong peaks persist for an unrealistic and unwanted duration (beyond the figure 4s window).

- There is a pronounced correlation between the in-phase and quadrature coefficient components.

- The autocorrelation does not depend on the initial seed, as the time evolution of the cluster rays is deterministic.

Based on the results and the observation above, it is necessary to further study the deterministic behaviour of rCDL and, if necessary, identify countermeasures before considering the applications of rCDL to settings that involve time-domain prediction.

Figure 6.4.5-1: Time correlation of single-cluster CDL channel with Doppler spread equivalent to 10 Hz for 
[-2,2] seconds and [10, 15] seconds

# 7	Alignment of Spatial Channel Models

Simulation results and analysis of different test cases with different channel models are captured in chapter 6 for model comparison. Chapter 7 for alignment is to capture companies’ simulation results on agreed performance metric per channel model and also the span and average value of companies results.

The following alignment test cases are included in this chapter:

FR1 SU-MIMO PDSCH 4Tx4Rx with 4 layers. The SNR of 30% and 70% maximum throughput are captured. The detailed parameter assumptions can be found in Table 6.1-1 and 6.1-2.

FR1 SU-MIMO PDSCH 8Tx8Rx with 8 layers. The SNR of 30% and 70% maximum throughput per codeword are captured. The detailed parameter assumptions can be found in Table 6.1-1 and 6.1-2.

FR1 SU-MIMO PMI 8Tx4Rx with 4 layers and Type-I codebook. The SNR of 70% and 90% maximum throughput are captured. The detailed parameter assumptions can be found in Table 6.2-1 and 6.2-2.

FR1 SU-MIMO PMI 8Tx4Rx with 4 layers and eType-II codebook. The SNR of 70% and 90% maximum throughput are captured. The detailed parameter assumptions can be found in Table 6.2-1 and 6.2-2.

There are two Spatial Channel Models, one CDL based model, known as rCDL-C1 (reduced CDL-C) and one TDL based model known as xTDL-C1 (extended TDL-C) which are captured for alignment simulation. Details of these models are in chapters 5.1 and 5.2 respectively.

The BS antenna configuration for CDL alignment is one antenna element per subarray.

4Tx case: (M, N, P, Ms, Ns) = (1, 2, 2, 1, 1).

8Tx case: (M, N, P, Ms, Ns) = (1, 4, 2, 1, 1).

The Doppler shift configurations are 3km/h for CDL based model and 10Hz for TDL based model.

## 7.1		rCDL-C1 model results alignment

Table 7.1-1 Simulation result summary for FR1 SU-MIMO PDSCH 4Tx4Rx with 4 layers

| SNR at Norm. Throughput [dB] | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30% | 8.5 |  | 6 |  | 6.4 |  | 6.6 |  | 6.4 |  | 7 |  | 5.9 |  | 6.7 |  | 6.7 |  | 2.6 |  |
| 70% | 15.5 |  | 14.3 |  | 16.5 |  | 15.6 |  | 15.9 |  | 16.6 |  | 14.7 |  | 15.6 |  | 15.6 |  | 2.3 |  |

Observations for rCDL-C1

- 7 out of 8 sources could achieve SNR span < 2.5dB at both 30% and 70% normalized throughput.

Table 7.1-2 Simulation result summary for FR1 SU-MIMO PDSCH 8Tx8Rx with 8 layers

| SNR at Norm. Throughput [dB] |  |  | Source #1 |  | Source #2 |  | Source #3 |  | Source #4 |  | Source #5 |  | Source #6 |  | Source #7 |  | Average |  |  | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CW1 | 30% | 6.4 |  | 4.9 |  | 4.8 |  | 6.1 |  | 6.8 |  | 6.9 |  | 6.3 |  | 6.0 |  | 2.1 |
|  | 70% | 16.9 |  | 14.2 |  | 16.5 |  | 16.7 |  | 17.5 |  | 17.2 |  | 18.5 |  | 16.8 |  | 4.3 |
| CW2 | 30% | 10.1 |  | 15.7 |  | 10 |  | 9.9 |  | 11.6 |  | 12.3 |  | 12.4 |  | 11.7 |  | 5.8 |
|  | 70% | 23.7 |  | 26 |  | 24.4 |  | 24 |  | 24.7 |  | 23.4 |  | 24.4 |  | 24.4 |  | 2.6 |

Observation for rCDL-C1:

- 6 out of 7 sources could achieve SNR span <2.5dB for both CW1 and CW2 at both 30% and 70% normalized throughput.

Table 7.1-3 Simulation result summary for FR1 SU-MIMO PMI 8Tx4Rx with 4 layers, follow PMI

| SNR at Norm. Throughput [dB] |  |  | Source #1 |  |  | Source #2 |  |  | Source #3 |  |  | Source #4 |  |  | Source #5 |  |  | Source #6 |  |  | Source #7 |  |  | Source #8 |  |  | Source #9 |  |  | Average |  |  | Span |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type-I | 70% |  | 12.4 |  |  | 11.0 |  |  | 9.7 |  |  | 10.3 |  |  | 10.2 |  |  | 9.7 |  |  | 9.5 |  |  | 10.2 |  |  | 9.8 |  |  | 10.4 |  |  | 2.9 |  |  |
|  | 90% |  | 14.4 |  |  | 13.1 |  |  | 12.5 |  |  | 12.8 |  |  | 12.6 |  |  | 12.6 |  |  | 12.0 |  |  | 12.5 |  |  | 12.3 |  |  | 12.7 |  |  | 2.4 |  |  |
| eType-II |  | 70% |  | 14.0 |  |  | 14.3 |  |  | 8.4 |  |  | 11.6 |  |  | 10.1 |  |  | 10.8 |  |  | 8.2 |  |  | 13.8 |  |  | 10.8 |  |  | 11.5 |  |  | 6.1 |  |
|  |  | 90% |  | 15.9 |  |  | 17.1 |  |  | 12.3 |  |  | 14.0 |  |  | 12.6 |  |  | 13.9 |  |  | 10.6 |  |  | 16.8 |  |  | 12.9 |  |  | 14.0 |  |  | 6.5 |  |

Observation for rCDL-C1:

- In type-I codebook case, 8 out of 9 sources could achieve SNR span < 2.5dB at 70% and 90% normalized throughput.

Observation for rCDL-C1:

- In eType-II codebook case, three clusters of results can be observed:

- Cluster 1: include source #3, #7 are in 70% normalized throughput and source #7 in 90% normalized throughput.

- Cluster 2: include source #4, #5, #6, #9 in 70% normalized throughput and source #3, #4, #5, #6, #9 in 90% normalized throughput. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput.

- Cluster 3: include source #1, #2, #8. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput.

## 7.2		xTDL-C1 model results alignment

Table 7.3-1 Simulation result summary for FR1 SU-MIMO PDSCH 4Tx4Rx with 4 layers

| SNR at Norm. Throughput [dB] | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30% | 5.3 | 5.8 | 6 | 6 | 5.7 | 5.8 | 0.7 |
| 70% | 13.5 | 15.9 | 15.4 | 15.8 | 14 | 14.9 | 2.4 |

Observation for xTDL-C1:

- 5 out of 5 sources could achieve SNR span < 2.5dB for both 30% and 70% normalized throughput.

Table 7.3-2 Simulation result summary for FR1 SU-MIMO PDSCH 8Tx8Rx with 8 layers

| SNR at Norm. Throughput [dB] |  | Source #1 | Source #2 | Source #3 | Source #4 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CW1 | 30% | 4 | 2.3 | 5.7 | 7.1 | 4.8 | 4.8 |
|  | 70% | 14.5 | 11.1 | 14.8 | 15.4 | 14 | 4.3 |
| CW2 | 30% | 10.8 | 11 | 11 | 8.4 | 10.3 | 2.6 |
|  | 70% | 21.2 | 19.7 | 20.4 | 16.6 | 19.5 | 4.6 |

Observation for xTDL-C1:

- For CW1 at 30% normalized throughput, 4 sources results are almost evenly distributed with ~1.5dB offset.

- 3 out of 4 sources could achieve SNR span < 2.5dB for CW1 at 70% normalized throughput.

- 3 out of 4 sources could achieve SNR span < 2.5dB for CW2 at both 30% and 70% normalized throughput.

Table 7.3-3 Simulation result summary for FR1 SU-MIMO PMI 8Tx4Rx with 4 layers, follow PMI

| SNR at Norm. Throughput [dB] |  | Source #1 | Source #2 | Source #3 | Source #4 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type-I | 70% | 12.3 | 13.0 | 10.8 | 11.3 | 11.9 | 2.2 |
|  | 90% | 14.0 | 15.0 | 13.6 | 12.8 | 13.9 | 2.2 |
| eType-II | 70% | 13.5 | 14.7 | 11.6 | 10.9 | 12.7 | 3.8 |
|  | 90% | 17.0 | 16.8 | 17.3 | 12.4 | 15.9 | 4.9 |

Observation for xTDL-C1:

- In type-I codebook case, 4 out of 4 sources could achieve SNR span< 2.5dB at both 70% and 90% normalized throughput.

Observation for xTDL-C1:

- In eType-II codebook case, there are two clusters of result that can be observed at 70% normalized throughput. Sources #1 and #2 are in one cluster and sources #3 and #4 are in another cluster. The span of each cluster could achieve SNR span< 2.5dB. 3 out of 4 sources could achieve SNR span< 2.5dB at 90% normalized throughput.

# 8	Summary

## 8.1	General

This Technical Report has studied candidate spatial channel models for NR demodulation performance requirements in FR1, considering SU-MIMO scenarios, and evaluating CDL-based and Multi cluster TDL-based modelling approaches. The study has:

- Investigated methodology to generate repeatable spatial channel effects with manageable test complexity

- Compared performance outcomes across candidate models against agreed test cases

- Collected alignment results from multiple contributors to determine the span and average for key performance metrics.

The following subsections capture the preliminary consensus points and highlight areas requiring further discussion.

## 8.2	Summary of SU-MIMO Results

### 8.2.1	PDSCH

#### 8.2.1.1	Rank 8

For rank-8 SU-MIMO PDSCH scenarios, for rCDL-C1 6 out of 7 sources could achieve SNR span <2.5dB for both CW1 and CW2 at both 30% and 70% normalized throughput, for xTDL-C1 3 out of 4 sources could achieve SNR span < 2.5dB for CW1 at both 30% and 70% normalized throughput.

#### 8.2.1.1	Rank 4

For rank-4 SU-MIMO PDSCH scenarios, for rCDL-C1 7 out of 8 sources could achieve SNR span <2.5dB for both CW1 and CW2 at both 30% and 70% normalized throughput, for xTDL-C1 5 out of 5 sources could achieve SNR span < 2.5dB for CW1 at both 30% and 70% normalized throughput.

### 8.2.2	PMI

#### 8.2.1.1	4 layer, type-I

For PMI testing with 4 layer transmission, with rCDL-C1, In type-I codebook case, 8 out of 9 sources could achieve SNR span < 2.5dB at 70% and 90% normalized throughput, for xTDL-C1 4 out of 4 sources could achieve SNR span < 2.5dB for both 30% and 70% normalized throughput.

#### 8.2.1.1	4 layer, eType-II

For PMI testing with 4 layer transmission, with rCDL-C1, in eType-II codebook case, three clusters of results can be observed; Cluster 1: include source #3, #7, Cluster 2: include #4, #5, #6, #9. The span of this cluster is < 2.5dB for both 70% and 90% normalized throughput percentiles, and Cluster 3 include source #1, #2, #8. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput percentiles.

For PMI testing with 4 layer transmission, with xTDL-C1, in eType-II codebook case, 2 out of 3 sources could achieve SNR span < 2.5dB at 70% normalized throughput and 3 out of 4 sources could achieve SNR span < 2.5dB at 90% normalized throughput.

## 8.3	Comparison of SCM Candidates

Regarding rCDL-C1 the following observations can be drawn:

- The study has worked on alignment of rCDL-C1 implementations and has verified that there is a reasonable level of alignment.

- This level of alignment is expected to allow RAN4 to work on normative requirements.

- If the group considers the application of rCDL to settings that involve time-domain prediction it is necessary to further review the deterministic behaviour of rCDL and, if found necessary, identify countermeasures.

Regarding xTDL-C1 the following observations can be drawn:

- The study has worked on alignment of xTDL-C1 implementations and has verified that there is a reasonable level of alignment, among a lower number of contributors than other SCM candidates.

- This level of alignment likely allows RAN4 to work on normative requirements.

Regarding all channel models:

- Applicability of all channel models for band agnostic requirements to be evaluated for specific use case.

- All spatial channel models in this study have fixed angular properties and have non-uniform PMI statistics, hence a fixed precoder may perform better than a channel adaptive precoder found by PMI selection algorithm. Countermeasures can be identified if necessary, including but not limited to, e.g. the legacy beam steering approach defined in LTE and NR specifications [TS 38.101-4 section B.2.3.2.3].

Annex A (informative):
Channel Measurements

During SCM channel measurements have been provided to demonstrate properties of sample MIMO channels from operational networks. This section summarises contributed material, observations and conclusions.

For ease of reference, Table A.1 below lists and summarizes some contributions to the study, and relevant channel properties reported.

Table A-1: Summary of provided measurement results

| Tdoc number | Channel properties reported | Observations |
| --- | --- | --- |
| R4-2402277, Sec. 2.5 | Singular value distributions of measured 4x4 MIMO channels | Variability in the quality of spatial layers observed. |
| R4-2411557 | Long-term stability of singular values, cross-polarisation discrimination | Variations in channel eigenmodes characterised, long-term stability consistent with expected behaviour in a measurement scenario is observed. |
| R4-2415673 | Power delay profile | The power delay profiles from reflections of the transmitted reference signals exhibit stability across time (~250ms in this specific measurement sample) for static scenarios |
| R4-2419338 | Angle of Arrival | MIMO radio channels for stationary UE in typical deployments exhibit multiple long-term stable spatial directions of arrivals |
| R4-2501745 | Eigenmodes for 32x4 channels from commercial deployments; Angular spread | In typical MIMO deployments, dominant eigenmodes remain within a narrow high-gain range, while weaker eigenmodes are more spread out, leading to performance disparities among codewords |
| R4-2504431 | PMI statistics for sample measured 32x4 channels | The UE's preference for a specific PMI subset is observed in measured MIMO channels from typical deployments |

# A.1	Measurement Configurations

Results were provided based on two sets of measurement campaigns for:

- a 4-port transmitter and a 4-port receiver system, and

- a 4-port transmitter and a 32-port receiver.

## A.1.1	Measurement campaign A: a 4-port transmitter and a 4-port receiver system

The MIMO channel measurements were conducted using a typical commercial MIMO deployment using a base station with 4 transceiver ports and user equipment with 4 ports. Measurements were conducted in a 15MHz channel at 2162.2MHz centre frequency, over several locations in a campus-type environment shown in Figure A.1.1-1. Reported samples were collected in static positions during office hours, with some movement of people and vehicles in the coverage sector, yet not in the immediate vicinity of the measurement setup.

The output of the measurements were complex channel gains per transmitter and receiver port pair (the H matrices), including all system gains and losses. Noise power was estimated at the receiving equipment, while the transmitter power setting was 15 dBm per resource element (15 kHz subcarrier spacing).

## A.1.2	Measurement campaign B: a 4-port transmitter and a 32-port receiver

Uplink sounding reference signals (SRS) were used for evaluation of respective MIMO channel between a typical commercial MIMO base station and a UE.  Measurements were conducted using a 40MHz channel in n78 band (3680 – 3720 MHz) in four locations A to D shown in Figure A.1.2-1. The output of the measurements were complex channel gains per transmitter and receiver port pair (the H matrices), including all system gains and losses. Noise power was estimated at the receiving equipment, while the transmitter power setting was 15 dBm per resource element (15 kHz subcarrier spacing).

Figure A.1.1-1: Measurement point locations for the sample MIMO channels from a typical 4T4R base station for measurement campaign A.

Figure A.1.2-1: Measurement point locations for the sample MIMO channels measurement campaign B.

# A.2	Measurement Results

Analysis of sample MIMO channel measurements was provided covering following channel properties:

- Channel Eigenmodes

- Power Angular Distribution

- Time Coherence

- Frequency Coherence

- PMI Preference

## A.2.1	Channel Eigenmodes

Histograms in Figure 6.3.1.2-1illustrate the distribution of singular values of measured MIMO channels in locations B and D, in conditions outlined in Section A.1.1.

Table A.2.1-1 Average eigenmode gains relative to the strongest spatial preference per measurement location

| Location | Average gain relative strongest eigenmode, dB |  |  |
| --- | --- | --- | --- |
|  | Eigenmode 2 | Eigenmode 3 | Eigenmode 4 |
| A | -4.85 | -20.70 | -33.18 |
| B | -5.84 | -12.94 | -24.38 |
| C | -7.48 | -15.71 | -27.86 |
| D | -12.74 | -18.65 | -30.67 |
| E | -7.92 | -14.89 | -24.89 |
| F | -7.84 | -15.17 | -27.00 |
| G | -7.91 | -18.58 | -31.92 |
| Average | -7.80 | -16.66 | -28.56 |

Empirical CDFs of eigenmodes for Measurement campaign B are provided in Figures A.2.1-1 and A.2.1-2.

| Figure A.2.1-1: Empirical CDF of Eigen Modes for Locations A and C. |  |
| --- | --- |
| Figure A.2.1-2 Empirical CDF of Eigen Modes for Locations B and D. |  |

## A.2.2	Spatial Properties

Spatial properties were analysed based on measurement campaign B.

Angle of Arrival stability

Figure 6.3.1.1-1 and Figure A.2.2-1 illustrate the estimated AoAs over time. In this analysis, the focus was on exploring whether MIMO channels in typical field deployments have long-term stable spatial preferences. Presence of three dominant arrivals was assumed and a simple implementation of MUSIC algorithm was applied to measured MIMO channel matrices.

| Figure A.2.2-1: Three primary AoAs for SRS from UE in Location B and D. |  |
| --- | --- |

Angular Spread in MIMO channels

| Figure A.2.2-2: Empirical CDF of Angular Spread for Locations A and C |  |
| --- | --- |

| Figure A.2.2-3. Empirical CDF of Angular Spread for Locations B and D. |  |
| --- | --- |

The empirical CDF of angular spread has been computed for 4 different locations, as shown in Figure A.2.2-2 and Figure A.2.2-3, to analyze spatial clustering characteristics. The Angles of Arrival (AoAs) were estimated using the MUSIC algorithm, identifying three dominant AoAs per location. The empirical CDF of angular spread is then computed separately around each of these AoAs. The analysis is conducted in the azimuth domain, where correlations are derived from the horizontal spatial relationships between antenna elements. To enhance statistical robustness, each estimate combines 25 consecutive SRS receptions, representing an observation period of 500 ms. This approach ensures a reliable representation of angular spread variations across different locations.

The analysis of angular spread CDFs around primary, secondary and tertiary AoAs suggests the presence of different types of spatial clusters in the MIMO channel environement. Primary clusters, characterized by a narrow angular spread, are crucial for high-SNR transmissions, while diffuse clusters with a broader angular spread enable higher-rank MIMO transmission. This presence of distinct spatial clusters indicates a spatial preference in MIMO channels, which is more acurately captured by CDL models.

Time Correlation Analysis

The time correlation across SRS transmissions is computed over the full duration of the dataset by evaluating the similarity between channel snapshots separated by forward time lags of up to 2 seconds. The general time correlation function R(Δt)  is given by the normalized inner product between channel vectors separated by a time lag Δt,  averaged over all valid time indices:

RΔt =E{h(t)h*(t + Δt)}E{h(t)h*(t )}E{h(t + Δt)h*(t + Δt)},

where h(t) is the vectorized channel at time t, and Δt=Δ TSRS with TSRS=20 ms and Δ∈{0,1,2,…,100}. The expectation E[⋅] is approximated by a sample average across all antenna pairs and frequency realizations.

At each time lag Δt, the correlation is calculated between the channel at time t and t+Δt. The results from all valid time indices t for which both channel snapshots (t and Δt) are available are then averaged to obtain the average time correlation function RΔt . The coherence time Tc is defined as the minimum time lag (in seconds) at which the average magnitude of the time correlation function drops below a specified threshold γ:

Tc=minΔt  RΔt|≤γ}.

Figure A.2.2-4:  Average Time Correlation Across Four Locations Highlighting Time Lag (s).

Table A.2.2-1: Temporal Channel Metrics: Coherence Time and Doppler Spread (γ=0.5).

|  | Coherence Time, Tc [s] | Doppler Spread, fD≈12Tc , [Hz] |
| --- | --- | --- |
| Location A | 1.44 | 0.3472 |
| Location B | 0.1 | 5 |
| Location C | 0.12 | 4.166 |
| Location D | 0.1 | 5 |

Figure A.2.2-4 shows the average time correlation as a function of time lag for four different measurement locations (A–D). Location A displays the most temporally stable channel, with correlation remaining above 0.5 for lags approaching 2 seconds, indicating low Doppler spread and minimal channel variation over time. In contrast, Locations B and D exhibit steep correlation decay within the first 0.1 seconds, consistent with higher Doppler spread and more dynamic propagation environments, potentially due to complex scattering. Location C shows slightly slower decay than B and D but still indicates a fast-varying channel.

Frequency Correlation Analysis

To evaluate the frequency selectivity of the wireless channel, frequency correlation function (FCF) was computed across subcarriers and extract the corresponding coherence bandwidth. The normalized frequency correlation function is calculated as:

Rf(Δf)=E[hf⋅hf+Δf*]Ehf2⋅Ehf+Δf2

where h(f) is the channel frequency response at subband f. The expectation E[⋅] is approximated by a sample average across all antenna pairs and time realizations, and Δf is the frequency lag in subbands. The coherence bandwidth fc is then defined as the frequency lag corresponding to Δf  at which the average magnitude of the FCF falls below a certain threshold γ:

fc=minΔf  |RfΔf|≤γ}.

Figure A.2.2-5: Average Frequency Correlation Across Four Locations Highlighting Frequency Lag.

Table A.2.2-2: Channel Metrics: Coherence Bandwidth and Delay Spread (γ=0.5, subcarrier spacing= 30 KHz, 1 Subband = 2 PRBs).

|  | Coherence bandwidth, fc [Hz] | Delay Spread, TD≈12 πfc , [ns] |
| --- | --- | --- |
| Location A | 7.92 MHz | 20.1 |
| Location B | 720 KHz | 221 |
| Location C | 2.16 MHz | 73.7 |
| Location D | 4.32 MHz | 36.8 |

Figure A.2.2-5 represents the frequency correlation functions computed across four measurement locations (A–D), plotted as a function of frequency lag in subbands. The results highlight distinct frequency selectivity characteristics, with location A exhibiting the slowest decorrelation across frequency, indicative of a flat (frequency-nonselective) channel. In contrast, locations B and C display a faster decorrelation, likely due to multipath delay spread or scattering richness. Location D exhibits intermediate behaviour with moderate correlation decay.

Table A.2.2-2 summarizes the estimated coherence bandwidths and corresponding delay spreads at a threshold of 0.5. The coherence bandwidth is determined as the minimum positive frequency lag at which the frequency correlation magnitude falls below the threshold. The delay spread is then estimated using the relation T_D≈1/(2πf_c ).  These results underscore the spatial variability in frequency-domain characteristics.

PMI statistics

Figure A.2.2-6 presents the PMI statistics for four locations. This analysis aims to determine if there is a spatial preference experienced by a UE, as indicated by the PMI values in a channel state information (CSI) report. We input the time samples of the channel matrix from the first subband (captured by the SRS) into an in-house simulator. The simulator processes each H matrix to generate an NR 3GPP Release-15 Type-1 report. From this report, we extract the port-to-beam mapping (represented by PMI1) and the beam-to-layer mapping (represented by PMI2). The actual precoder for the PDSCH is the product of PMI1 and PMI2.

| a) | b) |
| --- | --- |
| c) | d) |
| Figure A.2.2-6: PMI Statistics Across Four Locations Highlighting Spatial Preferences in UE |  |

The results indicate that the UE prefers the highest rank (rank 4) for transmission among the PMI hypotheses {1,2,3,4}. For a rank of 4, there are 512 possible PMI1 hypotheses and two corresponding PMI2 hypotheses for each PMI1. The histograms reveal that the UE reports a subset of PMI1s, with one particular PMI being reported significantly more frequently than others. This suggests a spatial preference consistent with the PMI statistics shown in Figure 17 of R4-2418043. Furthermore, this observation is consistent with the Angle of Arrival (AoA) analysis detailed in R4-2419338, where the primary AoA remained constant over time.

Annex B:
Modelling execution for CDL Channel Models

## B.1	Introduction

Unless otherwise specified, the procedure below is applied to all CDL models studied in this TR.

Parameters including for example as:

-	CDL Tables

-	Antenna Panel Configuration

-	Antenna Array Virtualizer

are left up to the CDL model configuration.

## B.2	Common Execution steps to all CDL models

All the definition, indices, coordinate systems used in this section, unless otherwise specified, follow the relative definition as included in 3GPP TR 38.901 v19.0.0.

Step 0: Set environment, network layout, antenna array parameters

a)	Give number of BS and UT. 3D location in the GCS of UT and BS are not necessary as LOS information are already included in the CDL table (see step d).

b)	Give speed and direction of motion of UT in the global coordinate system

c)	Specify system centre frequency  and bandwidth

d) Specify CDL Table (includes already Angular spread and scaling)

Step 1: Scaling and Translation of Angles

The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle μϕ,model can be changed to μϕ,desired  and angular spread ASmodel can be changed to match ASdesired by scaling. The translated and scaled ray angles can be obtained according to the equations 7.7-5 and related (a, b, c, d) in TR 38.901 v19.0.0.

Step 2: Spatial Filtering: use Antenna Array Virtualization procedure to obtain the BS and UT antenna field patterns Frx and Ftx in the global coordinate system and array geometries

The antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays. One subarray is a unique set of co-polarized antenna elements mapped exclusively to one antenna port through a virtualizer (set of complex values), and can also be identified with the name of AAV.

The antenna panel configuration is defined as (M, N, P, Ms, Ns), according to the description below:

- M is the number of antenna elements per panel with the same polarization in each column (vertical size)

- N is the number of antenna elements per panel with the same polarization in each row (horizontal size)

- P is the number of polarizations

- Ms is the number of antenna elements per subarray with the same polarization in each column (vertical size)

- Ns is the number of antenna elements per subarray with the same polarization in each row (horizontal size)

Subarray/AAV, according to the convention above, can be uniquely described in a short form defined as AAV(Ms,Ns).

E.g. when the NR BS Antenna implements an antenna panel of size (8,1,2,8,1), the subarray can be identified as AAV(8,1). In this example, the Antenna Panel is composed of 8 cross-polarized antenna elements arranged in a single column, and each column of 8 elements with the same polarization belongs to one subarray/AAV.

Elements of each AAV are combined to form a beam by setting certain weights per element. Weight vector for the first polarization and for the second polarization is

α1,1…αMs,Ns=1MsNsexp-j2πrtx,max T ∙ dtx,ms,nsλ0∈C1×MsNs

Where

- dtx,ms,ns is the location vector of each ms, ns transmit antenna element in the subarray

- rtx,max is a spherical unit vector denoting the virtualizer direction

If no virtualizer direction is specified, it is customary to consider broadside-emission at the AAV, thus the weight vector is real valued and all the elements are equal, e.g. [1,1 … 1]. Scaling factors can be applied only if they are consistent and equal for all AAVs in the panel.

The procedure to apply spatial filtering to CDL channels is also illustrated in Section 7.7.4.2 in TR 38.901 v 19.0.0, where it is used to generate TDL channel models. The same computation can be used to spatially filter the CDL profile using the AAV spatial response as specified in this section, retaining the cluster angles information (unlike when generating TDL profiles) for the next steps.

Step 3: Couple rays within the cluster for both azimuth and elevation according to fixed coupling

The AOD angles are coupled to AOA angles within a cluster n. The coupling is performed using the fixed coupling pattern specified in Table 5.1.3.2-1.  The same fixed coupling pattern is applied for all clusters n.

Step 4: Compute cross polarization power ratio

The linear cross polarization power ratios (XPR)  are calculated for each ray m of each cluster n as

,

where X is the per-cluster XPR in dB from Tables 7.7.1-1 – 7.7.1-5 of TR38.901.

Step 5: Set the fixed initial phases for each ray in the cluster

The set of fixed initial phases is same for all clusters, i.e. Φn,mθθ=Φmθθ ∀ n=1,…,N etc. for all four polarization combinations. These 20×4 initial phase values are specified in Table 5.1.3.2-2.

Step 6: Generate channel coefficients for each cluster n and each receiver and transmitter element pair u, s.

Hu,s,nNLOSt=PnMm=1Mns=1Nsms=1MsFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,ms,ns,θθn,m,ZOD,φn,m,AODFtx,s,ms,ns,φθn,m,ZOD,φn,m,AOD

αs,ms,nsexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,s,ms,nsλ0expj2πrrx,n,mT.vλ0t

Note:	that the patterns are given in the GCS and therefore include transformations with respect to antenna orientation.  is the spherical unit vector with azimuth arrival angle ϕn,m,AOA and elevation arrival angle θn,m,ZOA, given by

,

where n denotes a cluster and m denotes a ray within cluster n.  is the spherical unit vector with azimuth departure angle ϕn,m,AOD and elevation departure angle θn,m,ZOD, given by

,

where n denotes a cluster and m denotes a ray within cluster n. Also, is the location vector of receive antenna element u and is the location vector of transmit antenna element s, κn,m is the cross polarisation power ratio in linear scale, and λ0 is the wavelength of the carrier frequency. If polarisation is not considered, the 2x2 polarisation matrix can be replaced by the scalar  and only vertically polarised field patterns are applied.

The Doppler frequency component depends on the arrival angles (AOA, ZOA), and the UT velocity vector with speed v, travel azimuth angle ϕv, elevation angle θv and is given by

## B.3	Derivation of CDL Models for RAN4 Demod

### B.3.1 CDL Cluster Reduction

The Clustered Delay Line (CDL) profiles used in this study are simplified from the TR 38.901 CDL models. The simplification steps are shown below for information. These steps are only used when new delay profiles are created.

Step 0: The CDL profile from TR 38.901 is the starting point. If the profile includes set of sub-clusters (defined in TR 38.901 as clusters generated from the two strongest clusters) with the same angle and fixed delay offsets, each sub-clusters set is combined(Note 1) in a single cluster with the minimum delay among the sub-clusters and power equal to the sum of the sub-clusters.

Step 1: Angle spread scaling (to match the ASdesired ) and angle translation (to match the μϕ,desired) is applied to the CDL profile after sub-cluster combining, in order to extract the new cluster angles (ϕn,scaled) according to the procedure specified in TR 38.901 v19.1.0 Section 7.7.5.1.

-	The values for s can be extracted from the table 7.7.5.1-1 according to the input desired (AoD, AoA, ZoA, ZoD) spreads or computed using the scaling factor calculation method described in 38.901 v19.1.0 Annex A.5

-	Angle translation is effectively a rotation of the cluster from the point of view of the UE.

Step 2: Apply spatial filtering to the CDL profile generated in Step 1. This step ensures that the power-based reduction of the clusters considers the combined effect of the spatial channel and the AAV spatial response, and it is achieved by scaling each cluster average power with the AAV gain computed towards the respective mean cluster angle (see Annex B.3 for a detailed breakdown of the spatial filtering computation)

- 	The AAV gain assumes single AAV (subarray or single element) is used at the transmitter and/or at the receiver (where applicable).

- 	In the computation of the AAV gain the AAS orientation (downtilt, bearing, slant if applicable) needs also to be considered to obtain the correct AAV Gain

Step 3: Re-normalize cluster powers in the spatially filtered profile generated in Step 2 such that the strongest cluster is at 0dB.

Step 4: Find the clusters in the spatially filtered profile generated in Step 3 with power below [-30] dB, and remove the corresponding cluster index from the CDL profile generated in Step 1.

Step 5: If the number of clusters in the spatially filtered profile after Step 4 is larger than 12, find the cluster with the lowest power, and remove the corresponding cluster index from the CDL profile generated in Step 1. Repeat this step until the number of clusters equals 12.

Do not consider the spatially filtered profile generated in Steps 2-3 further after this step. The following modifications are to be applied on the surviving clusters (Note 2) in the CDL profile after Step 5.

Step 6: Calculate the effective delay spread of the CDL profile generated in Step 1 based only on the surviving clusters after Step 5, DStruncated (ns), to be used in the next step by computing the root mean squared (rms) delay spread of the remaining clusters, using the following expressions:

τ=iτipiipi

DStruncated=iτi2piipi-τ2

Where:

- τi is the normalized delay value of the i-th cluster in the CDL profile

- pi is the power value of the i-th cluster in the CDL profile

Step 7: Perform delay scaling according to the procedure described below, based on the scaling procedure from clause 7.7.3 in TR 38.901 v19.1.0 but considering the impact of truncation using τrmstruncated.

τn, scaled= τn, model ∙DSdesiredDStruncated

Where:

- τn, scaled is the delay value of the i-th cluster in the truncated CDL profile

- DSdesired is the wanted delay spread in ns

Step 8: Apply the quantization to the delay resolution of 5 ns (for FR1) and [TBD] ns for FR2. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution. -	To ensure that there is reduced impact from the use of a regularly spaced grid in the delay position, further improvements to delay quantization (e.g. dithering) can be considered

Step 9: If the delay spread has slightly changed due to the delay quantization, adjust the final delay spread by increasing or decreasing the power of the last cluster so that the delay spread matches the pre-quantization value.

Step 10: Round the amplitude of the last cluster to one decimal (e.g. -8.78 dB  -8.8 dB).

Note 1: Combining the clusters with the same AoD, AoA, ZoD, ZoA in CDL may impact the wide sense stationary (WSS) assumption of very large system bandwidth.

Note 2: The angular spread of the CDL Model after cluster reduction may not correspond to the ASdesired value used to scale in step 1. That is because the removal of lower energy clusters might lead to minor differences in the final angular spread.

## B.4	Information on RAN4 CDL model derivation

### B.4.1 CDL Cluster Reduction results for validation

This section contains an overview of the intermediate result of the step by step cluster reduction procedure included in section B.3.1, with the purpose to allow cross-validation of the CDL cluster reduction algorithm implementation.

The example provided here is computed assuming the Receiver NR UE is using a single omnidirectional antenna with polarization slant of 0 deg, while the Transmitting NR BS is using an antennal panel of size = (8,1,1,8,1) with polarization slant of -45 deg.

The virtualizer used for AAV(8,1) is described as follows and correspods to the default broadside emission (no subarray level electrical steering). A normalization or scaling factor may be applied.

α1,1…α8,1=[1, 1, … 1]

| Cluster # | Input cluster power | Element Gain | AAV Gain (Array Factor) | Output Cluster Power | Normalized Output Cluster Power | Cluster retention flag |
| --- | --- | --- | --- | --- | --- | --- |
|  | After combining, downtilting, polarization slant | Computed based on scaled and translated cluster mean angle (ϕn,scaled) in TR 38.901 7.7-5d |  |  |  |  |
| 1 | -7.4792 | 3.88 | 8.9504 | 5.35 | -11.50 | Yes |
| 2 | -1.2668 | 6.64 | 8.9874 | 14.36 | -2.49 | Yes |
| 3 | -5.5704 | 4.75 | 8.5662 | 7.75 | -9.11 | Yes |
| 4 | 0 | 7.86 | 8.9925 | 16.85 | 0.00 | Yes |
| 5 | -10.495 | 2.58 | 2.3295 | -5.58 | -22.44 | Yes |
| 6 | -10.208 | 0.79 | 8.7867 | -0.63 | -17.48 | Yes |
| 7 | -13.807 | 1.32 | -1.3293 | -13.81 | -30.67 | No |
| 8 | -14.25 | -7.05 | 9.0026 | -12.29 | -29.15 | Yes |
| 9 | -8.1939 | 2.60 | 4.9713 | -0.63 | -17.48 | Yes |
| 10 | -9.9091 | 0.99 | 3.0401 | -5.87 | -22.73 | Yes |
| 11 | -11.829 | -2.35 | 8.2854 | -5.89 | -22.75 | Yes |
| 12 | -16.341 | -3.44 | -1.1187 | -20.90 | -37.75 | No |
| 13 | -17.033 | -2.93 | 8.998 | -10.97 | -27.82 | Yes |
| 14 | -17.018 | -0.72 | 7.9434 | -9.79 | -26.65 | Yes |
| 15 | -18.954 | -8.53 | 8.8396 | -18.65 | -35.50 | No |
| 16 | -20.228 | -1.08 | -7.9129 | -29.22 | -46.08 | No |
| 17 | -19.125 | -1.71 | 8.1326 | -12.70 | -29.55 | No |
| 18 | -18.842 | -4.93 | 8.677 | -15.10 | -31.95 | No |
| 19 | -24.762 | -7.92 | -16.988 | -49.67 | -66.52 | No |
| 20 | -25.979 | -14.98 | -4.2836 | -45.25 | -62.10 | No |

Hu,s,nNLOSt=PnMm=1Mne=1Neme=1MeFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,me,ne,θθn,m,ZOD,φn,m,AODFtx,s,me,ne,φθn,m,ZOD,φn,m,AOD

αs,me,neexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,s,me,neλ0expj2πrrx,n,mT.vλ0t

Annex C:
Modelling execution for TDL Channel Models

# C.1	TDL Approaches

## C.1.1	Multi-cluster TDL channel for 2-CW fixed-MCS testing

As one example, testing of two-codeword (up to 8 layers) MIMO demodulation – so that consistently different channel quality per CW is maintained – has turned out to be challenging with current RAN4 correlation models. This section presents examples how to construct PDSCH demodulation tests with fixed rank, fixed Type I PMI, and different fixed MCS per CW.

### C.1.1.1	Example 1

For fixed PMI testing, the per-cluster phase shifts: θtx1(n) and θtx2(n) can be chosen to be fixed and to match the beams of the transmit PMI. In Type I codebook, each CW often maps to a subset of the orthogonal PMI-beams. This is illustrated below for the 5-layer precoder (TS 38.214: 5.2.2.2.1). Consequently, per-CW channel quality levels can be adjusted by adjusting the cluster-specific channel powers.

To minimize cross-layer/CW interference at the UE, the RX-phases θrx1(n) can be chosen so that the corresponding steering vectors are orthogonal. Alternatively, by choosing non-orthogonal RX steering vectors, cross-layer/CW interference can be increased if so desired. Finally, the selection of the X-pol spatial correlation model parameters (α1, α2, β) determines how strictly the mean spatial directions are maintained. Higher correlation is analogous to narrower angular spread per cluster.

Figure C.1.1.1-1: Illustration of a codebook for 5-layer CSI reporting

To demonstrate the concept, a rank 5 PDSCH setup as detailed in Table C.1.1.1-1 was simulated in the channel model defined in Table C.1.1.1-2. The results are depicted in Figure C.1.1.1-1. Here, CW0 employs 16QAM while CW1 has QPSK, but as clusters corresponding to CW0 are 6dB stronger, the BLER performances of the two CWs are almost on par. Thus, it can be concluded that CW-specific TX-RX beam steering clusters can effectively be used to scale CW-specific demodulation and decoding performance.

Table C.1.1.1-1: Simulation setup

| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | 2, 2, 4, 1 |
| --- | --- |
| Rank 5 Type1 TX-PMI: i1,1, i1,2, i2 1st dim PMI beam indexes: l, l’, l” 2nd dim PMI beam indexes: m, m’, m” | 0, 0, 0 0, O1, O1 => phase 2πl/O1N1 = 0, π, π 0, 0, O2 => phase 2πm/O2N2 = 0, 0, π |
| MCS / CW0 MCS / CW1 | MCS13, 16-QAM, code rate 0.48 MCS7, QPSK, code rate 0.51 |

Table C.1.1.1-2: Cluster model with 3 TDL-A channel instances and 3 clusters

| TDLA-30 delay profile 10Hz Doppler XP-high correlation | TX-RX beam steering |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TDL channel 1 / cluster 1 Power: 0 dB |  |  | TDL channel 2 / cluster 2 Power: -6 dB |  |  | TDL channel 3 / cluster 3 Power: -6 dB |  |  |
|  | θtx1 | θtx2 | θrx1 | θtx1 | θtx2 | θrx1 | θtx1 | θtx2 | θrx1 |
|  | 0 | 0 | 0 | π | 0 | π/2 | π | π | π |

Figure C.1.1.1-2: 2: CW fixed MCS case in three-cluster TDL-A channel.

### C.1.1.2	Example 2

A model with just one TDL channel instance can also satisfy the 2-CW fixed MCS testing requirements by setting mapping relation between clusters and taps to control the 2CWs power and delay difference in the reasonable level. Take channel model in Table C.1.1.2-1 as an example, where cluster 3 and cluster 4 correspond to last four taps, leading to large power and delay difference with cluster 1 and 2. Specific precoder mapping the 1st CW to first two clusters and 2nd CW to second two clusters leads to large SINR difference between two CWs. In the case, different MCSs could be configured to match performance difference.

Table C.1.1.2-1: Cluster model with one TDL-C channel instance and 4 clusters

| TDLC-300 / XP-high / 100Hz Doppler |  |  | TX-RX beam steering |  |  |
| --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |
| 3 | 70 | -7.7 | 2 | -π/2 | -π/2 |
| 4 | 190 | -2.5 |  |  |  |
| 5 | 195 | -2.4 |  |  |  |
| 6 | 200 | -9.9 |  |  |  |
| 7 | 240 | -8.0 |  |  |  |
| 8 | 325 | -6.6 |  |  |  |
| 9 | 520 | -7.1 | 3 | -π | -π |
| 10 | 1045 | -13.0 |  |  |  |
| 11 | 1510 | -14.2 | 4 | π/2 | π/2 |
| 12 | 2595 | -16.0 |  |  |  |

A simulation is performed to verify the characteristic with simulation assumptions captured in Table C.1.1.2-2. For rank 8 with fixed Type I Single panel precoder i1,1=i1,2=i2=0, the 2nd and the 3rd cluster are mapped to the last four channel taps with low power and large delay. This leads to a big performance difference between two codewords as shown in Figure C.1.1.2-1.

Table C1.1.2-2: Simulation setup

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,4,1) for 8TX-8RX (1-dimensional antenna arrays) |
| Precoder | Type I Single panel, Fixed precoder i1,1=i1,2=i2=0 |
| Rank | 8 |
| MCS | 13 |

Figure C.1.1.2-1: 2-CW fixed MCS case in four-cluster TDL-C channel

θtx1(n)θtx2(n)

θrx1(n)

|  | θtx1 | θtx2 | θrx1 | θtx1 | θtx2 | θrx1 | θtx1 | θtx2 | θrx1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -π | 0 | π/2 | -π | -π | π |

-

|  |  |  |  | θtx1 | θrx1 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | -π/2 | -π/2 |
|  |  |  |  | -π | -π |
|  |  |  |  | π/2 | π/2 |

-

Annex D (informative):
MU-MIMO

The study of SCM for MU MIMO scenarios was part of the initial WID and interest in this aspect was voiced by a number of contributors, including operators. MU MIMO has not been part of the alignment or comparison discussions, however comments and proposals have been made, and discussions have been observed throughout the study.

The discussions highlighted two types of MU scenarios. Firstly, the legacy version with a single transmitter and a single receiver, hence necessitating creation of a single SU CDL channel and addition of MU interference via simultaneous transmission of two transport blocks using two precoders. Secondly, a version with a single transmitter and two receivers, hence necessitating creation of two single SU CDL channels (potentially with spatial consistency constraints on the both).

It was not concluded whether the "two receiver" version is useful and/or necessary for MU performance requirement derivation.

No agreement on SCM, implementation, alignment, usage in MU MIMO has been reached in this phase of SCM study.

Spatial channel models for MU-MIMO cases were evaluated based on the table Section 3 in [1].

Table D.1: Spatial channel models for MU-MIMO cases

| Parameter |  | MU-MIMO PDSCH (FR1) |
| --- | --- | --- |
| Duplex mode |  | TDD |
| TDD Slot Configuration Pattern |  | 7DS2U |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |
| UE receiver type |  | IRC, E-IRC |
| Number of Tx Ports |  | 4 |
| Number of Rx Ports/Antennas |  | 4 |
| Number of layers |  | 2+2 |
| PMI |  | UE1: Random UE2: Random and not equal to UE1, Orthogonal |
| Waveform |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |
| MCS |  | 13 (64 QAM table) |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
|  | Resource allocation type | Type 0 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |
| Codebook configuration | CodebookType | type-I-SP |
|  | Codebook configuration | (N1,N2,O1,O2) = (2,1,4,1) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |
|  | Number of CSI-RS ports (X) | TBD |
|  | Density (ρ) | 1 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |
| CQI/RI/PMI delay |  | N/A |
| Number of HARQ Processes |  | 8 |
| Maximum HARQ transmissions |  | 4 |
| Channel Models |  | The purpose of the study item. |
| Testing Metric |  | Full Throughput Curves |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |
|  | BS Antenna Polarisation | (+45, -45) |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |
|  | UE Antenna Polarisation | (0, 90) |
|  | UE Antenna Radiation Pattern | Omnidirectional |
|  | Antenna Panel Placement | YZ Plane |

Annex E (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-10 | RAN4#112-bis | R4-2415297 |  |  |  | Initial TR skeleton | 0.0.1 |
| 2025-06 | RAN4#115 | R4-2506178 |  |  |  | Update | 1.0.0 |
| 2025-09 | RAN4#116 | R4-2509114 |  |  |  | In preparation for RAN Plenary | 1.1.0 |