3GPP TSG-RAN4 Meeting #116	R4-2509115

# Bengaluru, India, 25th - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR | TBD | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.901 on the extension of the Scaling of Angles procedure to Cluster angles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_demod_SCM |  |  |  |  |  | Date: |  |  | 2025-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### CHANGE #1

### 7.7.5	Extension for MIMO simulations

Extended MIMO link-level channel models can be constructed according to two alternative methods described in the following.

#### 7.7.5.1	CDL extension: Scaling of angles

The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle μϕ,model can be changed to μϕ,desired  and angular spread ASmodel can be changed to match ASdesired by scaling.

The translated and scaled cluster (ϕn,scaled) and ray (ϕn,m,scaled) angles can be obtained according to the following equation:

ϕn,m,scaled=ϕn,scaledϕn,intermediate-μϕ,intermediate+μϕ,desired+c{ASA, ASD,ZSA,ZSD}αm	(7.7-5)

ϕLOS,scaled=ϕLOS,intermediate-μϕ,intermediate+μϕ,desired	(7.7-5a)

ϕn,intermediate=s∙WrapTo180ϕn,model-μϕ,model	(7.7-5b)

ϕLOS,intermediate=s∙WrapTo180ϕLOS,model-μϕ,model	(7.7-5c)

ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired	(7.7-5)

in which:

-	μϕ,model is the cluster mean angle calculated using Annex A.4 with ϕn,model  and ϕLOS,model as the input cluster angles,

-	μϕ,intermediate is the intermediate cluster mean angle calculated using Annex A.4 with ϕn,intermediate and ϕLOS,intermediate as input cluster angles,

-	s is a scale factor chosen to change the distribution of the angles based on calculation in Annex A.5. Table 7.7.5.1-1 shows required scale factor for typical desired angular spread values for AOD, AOD, ZOA, and ZOD,

-	ϕn,model	is the tabulated CDL cluster angle,

-	ASmodel	is the rms cluster angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A.3,

-	μϕ,desired	is the desired cluster mean angle,

-	ASdesired	is the desired rms cluster angular spread,

-	ϕn,scaled	is the scaled CDL cluster angle of the nth cluster,

-	ϕn,m,scaled is the resulting scaled ray angle that corresponds to mth subpath angle of nth cluster,

-	cASA, ASD,ZSA,ZSD is the cluster-wise RMS angular spread of either custer ASA, ASD, ZSA, or ZSD from Table 7.7.1-1, 7.7.1-2, 7.7.1-3, 7.7.1-4, and 7.7.1-5,

-	αm is the offset angles from Table 7.5-3,

-	WrapTo180⋅ is a function which wraps an azimuth angle to the half-open interval (-180, 180].

Table 7.7.5.1-1: Scale factor values for each CDL model

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-A | 5 | 0.0680 | 30 | 0.3531 | 5 | 0.0352 | 1 | 0.0352 |
|  | 10 | 0.1360 | 45 | 0.5268 | 10 | 0.1056 | 3 | 0.1056 |
|  | 15 | 0.2041 | 60 | 0.6981 | 15 | 0.1761 | 5 | 0.1761 |
|  | 25 | 0.3405 |  |  |  |  |  |  |
| CDL-B | 5 | 0.1238 | 30 | 0.5417 | 5 | 0.1940 | 1 | 0.1940 |
|  | 10 | 0.2475 | 45 | 0.8081 | 10 | 0.5822 | 3 | 0.5822 |
|  | 15 | 0.3710 | 60 | 1.0709 | 15 | 0.9705 | 5 | 0.9705 |
|  | 25 | 0.6168 |  |  |  |  |  |  |
| CDL-C | 5 | 0.1281 | 30 | 0.4307 | 5 | 0.3643 | 1 | 0.3643 |
|  | 10 | 0.2568 | 45 | 0.6447 | 10 | 1.0929 | 3 | 1.0929 |
|  | 15 | 0.3864 | 60 | 0.8585 | 15 | 1.8219 | 5 | 1.8219 |
|  | 25 | 0.6513 |  |  |  |  |  |  |
| CDL-D | 5 | 0.3231 | 30 | 9.8888 | 5 | 0.4477 | 1 | 0.4477 |
|  | 10 | 0.6652 | 45 | N/A | 10 | 1.3469 | 3 | 1.3469 |
|  | 15 | 1.0594 | 60 | N/A | 15 | 2.2579 | 5 | 2.2579 |
|  | 25 | 5.8637 |  |  |  |  |  |  |
| CDL-E | 5 | 0.3950 | 30 | 2.9733 | 5 | 0.9714 | 1 | 0.9714 |
|  | 10 | 0.8009 | 45 | N/A | 10 | 2.9180 | 3 | 2.9180 |
|  | 15 | 1.2330 | 60 | N/A | 15 | 4.8774 | 5 | 4.8774 |
|  | 25 | 2.3627 |  |  |  |  |  |  |
| NOTE: Values of Table 7.7.5.1-1 were computed based on scaling factor calculation method described in Annex A.3. |  |  |  |  |  |  |  |  |

Alternatively, for CDL-D and CDL-E, if maintaining the LOS angle is desired, the translated and scaled ray angles can be obtained according to the following equation instead of using equation (7.7-5) and (7.7-5a):

ϕn,m,scaled=ϕn,intermediate-ϕLOS,intermediate+ϕLOS,model ϕn,scaled+c{ASA, ASD,ZSA,ZSD}αm	(7.7-6)

ϕLOS,scaled=ϕLOS,model	(7.7-6a)

ϕn,scaled= ϕn,intermediate-ϕLOS,intermediate+ϕLOS,model	(7.7-6b)

in which:

-	ϕLOS,model is the LOS path angle of the model.

-	ϕn,scaled	is the scaled CDL cluster angle of the nth cluster,

The angular scaling is applied on the cluster angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model.

Example scaling values are:

-	AOD spread (ASD) for each CDL model: {5, 10, 15, 25} degrees.

-	AOA spread (ASA) for each CDL model: {30, 45, 60} degrees.

-	ZOA spread (ZSA) for each CDL model: {5, 10, 15} degrees.

-	ZOD spread (ZSD) for each CDL model: {1, 3, 5} degrees.

The angular scaling and translation can be applied to some or all of the azimuth and zenith angles of departure and arrival.

Note: The azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees.