3GPP TSG-RAN WG4 Meeting #116										                    R4-2509116

Bangalore, India, August 25th – 29th, 2025

Source:	Murata

Title:	ΔR1R for PC3 RedCap

Agenda item:	4.2.2

Document for:	Approval

# 1.	Introduction

In this contribution, we will discuss the ΔR1R for PC3 Redcap.

# 2.	Discussion

Section 2.1 shows the proposed change of ΔR1R for PC3 RedCap. Section 2.2 shows the justification for any possible change.

## Proposed Change

Current Specification for PC3 Redcap 1RX from 2RX is shown below in Table 2.1-1:

Table2.1-1: RedCap PC3 ΔR1R in 38.101-1

The proposed change [1] is quite significant and is shown in Table 2.1-2.

Table2.1-2: Proposed PC3 ΔR1R from [1].

The presideing claim is that self-interfering TX is much worse than expected for low bands, especially for low bands supporting larger BWs in NR release 17 compared to LTE and earlier releases where lower channel BWs were used. While increasing MSD due to TX self-interference is certainly justified, the question remains whether the amount of change is justified to warrant a signifcant change in ΔR1R.

## 2.2  De-sense, ΔR1R formulation, and Justification

In FDD bands, larger bandwidths are subjected to self-interfering TX noise. Excessive TX OOB emissions reach the lower duplex offsets relative to the TX centre frequency more so in low bands, especially bands that have close TX-RX spacings. The RX SINR degrades due to the interference. The de-sense or MSD is related to the level of interference above the noise floor. In 3GPP, a 2-port receiver is used to compute the de-sense, but a closer examination is necessary to determine whether ΔR1R should change appreciably when subject to the excessive interference.

There are 3 main factors that determine the MSD or amount of de-sense using the MRC combiner. They are the actual amount of interference above the thermal noise floor, the level of imbalance of the interference arriving at the 2 RX ports, and the strength of correlation of the interfering data streams at each RX port. Since there is only one self-interfering source of 1TX, especially in RedCap, then there could be a strong justification for non-zero covariance and positive correlation for the TX interference. This is an important consideration when combining the weighted amplitudes at the MRC combiner, where the weights of the combiner are inversely proportional to the noise-plus-interference.

So, figure 2.2-1 shows the MSD as a function of the interference to thermal noise ratio, I/Nth using 6dB interference imbalance. The figure clearly shows when there is high correlation between the interference at the 2 RX ports, less interference (low I/Nth) is required to obtain the same MSD level (blue curve) than for the case of assuming no correlation.

Figure 2.2-1: 2RX MSD Vs Interference/Thermal noise ratio Vs degree of correlation given 6dB interference imbalance

Figure 2.2-2 shows another plot can be created to determine the ΔR1R requirement Vs degree of correlation Vs the I/Nth ratio. The graph shows that larger BWs with large self-interference requires much lower ΔR1R when there is a high degree of correlation of the interference arriving at the 2 RX ports. For example, only 4.0dB delta R in the correlated case Vs 6.0dB of ΔR1R in the uncorrelated case for the same amount of I/Nth degradation.

Figure 2.2-2: ΔR1R curves for correlated case Vs uncorrelated case given 6dB interference imbalance

In fact, now we can accurately predict that the ΔR1R in [1] using the same assumptions, which are zero correlation and 6dB interference imbalance and compare this result to the result using MRC combining assuming strong correlation. The comparison is shown in Table 2.2-1 and Table 2.2-2. The worst-case scenarios (Band n12 with 15MHz UL BW) give at the most 3.8dB increase in ΔR1R Vs a 6.3 dB increase in ΔR1R for no correlation, which is a large difference. Band n5 20MHz example is shown in Table 2.2-1, which shows 3.3dB increase Vs a 5.6dB increase.

| NR Band n5 | High correlation | No correlation |
| --- | --- | --- |
| Channel BW | 20 MHz | 20 MHz |
| PC3 2Rx REFSENS (dBm) | -86.8 | -86.8 |
| PC3 MRC noise (dBm) | -85.8 | -85.8 |
| PC3 primary Rx noise (dBm) | -87.8 | -87.8 |
| PC3 diversity Rx noise (dBm) | -87.8 | -87.8 |
| PC3 Primary Tx noise (dBm) | -84.1 | -81.1 |
| PC3 Diversity Tx noise (dBm) | -90.1 | -87.1 |
| PC3 Primary total noise (dBm) | -82.5 | -80.2 |
| PC3 Diversity total noise (dBm) | -85.8 | -84.4 |
| PC3 MRC noise (dBm) | -85.8 | -85.8 |
| PC3 ΔR1R | 3.3 | 5.6 |

Table 2.2-1: n5 20MHz ΔR1R analysis as a function of interference correlation at RX ports 1 and 2

|  | ΔR1R (High correlation) |  |  | ΔR1R (No correlation) |  |
| --- | --- | --- | --- | --- | --- |
| Band | 15 MHz | 20 MHz |  | 15 MHz | 20 MHz |
| n5 |  | 3.3 |  |  | 5.6 |
| n8 |  | 3.3 |  | 3.4 | 5.6 |
| n12 | 3.8 | N/A |  | 6.3 | N/A |
| n20 |  |  |  | 3.6 | 3.6 |
| n26 |  | 3.1 |  |  | 5.1 |
| n28 |  |  |  |  | 4.0 |
| n71 |  | 3.3 |  | 3.4 | 5.6 |
| n74 |  | 3.1 |  |  | 5.3 |
| n85 | 3.8 | N/A |  | 6.3 | N/A |
| n105 |  | 3.1 |  | 3.4 | 5.3 |

Table 2.2-2: Actual ΔR1R as a function of the interference correlation at RX ports 1 and 2.

Apart from the proposed change, there are various arguments to leave the current specification intact or modify the specification in according to [1].

Proponents to the change argue that actual deployed UEs may use more sophisticated RX combining such as MMSE (minimum mean squared error) which tends to decorrelate the interference arriving at the 2 RX ports, so that the actual derived REFSENS has the effect of using uncorrelated interference into an MRC combiner thus giving the larger ΔR1R. However, the quality of MMSE combining and hence the quality of decorrelating the interference using an MRC combiner could be a function of the UE tier level chosen or UE implementation.

Opponents to the change argue that B71, B20, B28, and B74 in LTE already specifies REFSENS for 15MHz and 20MHz BWs since release 15 [2] using a ΔR1R of 3dB before NR RedCap was introduced in release 17. Also, from an operator point of view, R17 devices have already been deployed in the field, and the concern is that deploying devices from a later release with less conforming standards can impact the coverage and network performance. Additionally, REFSENS in 38.101-1 for all bands has been derived from various companies using varying degrees of correlation. Using the increased ΔR1R in [1] then becomes very questionable. We illustrate this in Figure 2.2-3. The actual REFSENS, given the same interference level and interference imbalance and different correlation, will yield a different absolute REFSENS, but when averaged gives the 2RX REFSENS level as indicated in TS38.101-1. When the different ΔR1R values are calculated based on the degree of correlation, the final 1RX REFSENS will obviously be the same. But, adding the ΔR1R in [1] to the existing 38.101-1 REFSENS is considered as an over-relaxation for both correlated and uncorrelated cases as shown in the figure. The delta value must be lower, but the question remains on how much lower needs to rest on the final decision in RAN4 from both UE vendors and network operators.

Figure 2.2-3: ΔR1R and REFSENS behaviour in the presence of the same interference level and 6dB interference imbalance.

In summary, measurements at both the UE and network level would be advantageous to sorting out a possible compromise.

Observation 1: The expected change in ΔR1R is less than 0.8dB in the worst-case scenario when the interfering data streams arriving at the 2RX ports from a single TX source are correlated.

Observation 2: Since R17 devices have been deployed, there is a concern that the introduction to devices with less conforming standards can degrade the network. LTE bands B71, B20, B28, B74 have used larger channel BWs since release 15 with existing ΔR1R.

Observation 3: REFSENS has been derived using MRC combining with varying degrees of interference correlation from various companies. The benefit of the increased ΔR1R depends on how the absolute 2RX REFSENS was derived.

Proposal 1: Because of observation 1, 2, and 3, further measurement investigation at both the UE level and network level is required to justify any ΔR1R increase for PC3 RedCap.

# Conclusion

Observation 1: The expected change in ΔR1R is less than 0.8dB in the worst-case scenario when the interfering data streams arriving at the 2RX ports from a single TX source are correlated.

Observation 2: Since R17 devices have been deployed, there is a concern that the introduction to devices with less conforming standards can degrade the network. LTE bands B71, B20, B28, B74 have used larger channel BWs since release 15 with existing ΔR1R.

Observation 3: REFSENS has been derived using MRC combining with varying degrees of interference correlation from various companies. The benefit of the increased ΔR1R depends on how the absolute 2RX REFSENS was derived.

Proposal 1: Because of observation 1, 2, and 3, further measurement investigation at both the UE level and network level is required to justify any ΔR1R increase for PC3 RedCap.

# 4.  References:

- R4-2505443, “Corrections on RedCap UE PC3 FDD band 1Rx REFSENS”, Apple, Skyworks, RAN4#115, Malta, May 19th – 23rd, 2025.

- 3GPP TS 36.101 V15.26.0 (2025-06)

# Appendix