3GPP TSG-RAN WG4 Meeting #116										                    R4-2509117

Bangalore, India, August 25th – 29th, 2025

Source:	Murata

Title:	Power Domain Enhancement Single Carrier

Agenda item:	7.1.2.2.1

Document for:	Approval

# 1.	Introduction

In this contribution, we will present views on the agreed WF [1] for Power Domain Enhancement.

# 2.	Discussion

The following topics are addressed in this discussion: Extension values and ratios (sec. 2.1), guard-band for extended CBW (sec 2.2), derivation of new inner RB region in extended CBW (sec 2.3), derivation and relaxation of the extended IBE (sec 2.4). The perceived OOBE requirements are presented in the appendix in section 5.

## Extension Values and Ratios

The excerpt from the WF from the last meeting reads as follows:

- Agreement in main session:

- At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases

- It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides.

- It is FFS on details of the single side case.

Table 2.1-1 shows our understanding of the cases. The table shows how the extended BW is related to the UE BW for the given extension ratios. Highlighted are the symmetrical cases that seem plausible to be completed at the close of the work item this meeting, and there are 4 asymmetrical cases for the approved fixed ½ and ¼ fixed extensions with the assumption of zero extension on either the lower side or upper side.  Also shown is the ΔFOOBoffset which shows how the general emission requirements in section 5 will be affected for each use case. The key takeaway is that the general requirements are simply positionally impacted by the parameter ΔFOOBoffset. All these requirements are referenced to the edge of the UE channel bandwidth [5].

Table 2.1-1: List of the symmetrical and asymmetrical cases to be considered

Observation 1: SEM, ACLR, and Spurious Emission requirements for a given UE BW  are only positionally impacted by ΔFOOBoffset which is a function of the single or dual sided extension ratio listed in Table 2.1-1.

Figure 2.1-1 compares a 50% dual-sided symmetric case with a non-standard 33% single sided asymmetric case for the same transmission BW and RB allocation for a fixed available extended BW.  As expected, the asymmetric case requires more available spectrum to give inner type MPR. The asymmetric case requires the UE BW to be larger which in turn forces the ΔFOOBext region is larger which pushes out the start of the FOOB frequency for spurious emissions.

Figure 2.1-1: OOBE Requirement definition example for 50% dual-sided symmetric extension (top) and 33% single sided asymmetric extension (bottom) for the same RB allocation to give full power boost.

Observation 2: Boosting the power for a given transmission BW requires more available spectrum in the asymmetric case compared to the symmetric case.

Generalizing, we can look at the percentage of inner RB allocations or the percentage of RB allocations available for power boosting Vs the configured UE channel BW without fixing the extended BW. The configured UE channel BW can be expressed as a percentage of the extended BW use. The simulation and case studies using the new inner RB derivation (section 2.3) are shown in Figure 2.1-2. Figure 2.1-3 shows the performance plot indicating that symmetric cases have a higher percentage of available RB allocations that can be used for power boosting.

Figure 2.1-2: Symmetric Cases (left) and Asymmetric Cases (Right)

Figure 2.1-3: Percentage of RB allocations available for power boosting

Symmetry takes full advantage of the adjusted general requirements. After all, the definition of MPR is maximum power reduction only to meet the general requirements. Based on the plot in figure 2.1-3, the MPR reduction feature should only concentrate on the symmetric cases because those cases make good use of the extended BW based on the percentage of allocations available for boosting due to the general requirements being symmetric. The only advantage of using the asymmetric case is to meet the UE-UE coexistence requirement or to provide guard-band and prevent the need for AMPR. But this is already indirectly done in the 38.101-1 specification. A frequency offset or guard-band involving RB restriction is used to prevent AMPR for those scenarios. On top of this RB restriction, further asymmetric extension would be required for a boosted waveform.

Observation 3: Symmetric cases give a higher percentage of RB allocations available for power boosting than the asymmetric cases for a given extended BW.

Observation 4: Asymmetric cases are indirectly used to meet existing UE-UE coexistence spurious requirements, which is the same as providing one sided free RBs or guard-band to prevent AMPR. Further asymmetric extension would be required for a boosted waveform.

Proposal 1: To ease UE implementation, dual sided symmetrical cases are the preferred choice for MPR reduction for this release.

## Guard-band for Extended CBW

The following is the excerpt from the WF regarding guard-band for the extended channel BW.

- WF

- GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification, and it can be calculated by extended CBW, UE CBW, NRB and extended NRB

- Extended CBW is not limited to the existing UE CBW

- RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests

The first sub-bullet is basically the OOBE requirements in section 5. The frequency separation between the edge of the UE channel bandwidth and the extended channel bandwidth is defined as the ΔFOOBoffset parameter that determines how much to shift the OOBE region from the edge of the UE bandwidth to meet the general requirements for power boosting in the extended case. It is unclear what is meant by the 2nd sub-bullet, but the assumption is that RAN4 can study the need to extend the existing guard-band of the UE bandwidth in the asymmetric case to avoid edge-case MPR as indicated in Table 6.2.1-1 of 38.101-1 for PC2 UEs.

Only a single-sided extension of the UE channel bandwidth will see the edge RB impact. The edge RB impact is seen at low LCRB (<=2), where the PSD emission is high. The edge RB MPR is due to the lack of spectral containment when shaping the time domain symbol. Although the WOLA (Weighted overlap and add) effect is implementation dependent, measurements can show that for a 2% WOLA, the edge MPR is not necessary if the effective guard band were increased. Figure 2.2-1 shows the guard band required to avoid edge MPR in 1RB edge cases for the single sided asymmetric case for varying power class and MPR.

Figure 2.2-1: Guard-band required to meet WOLA emission for single sided asymmetric cases.

Observation 5: Figure 2.2-1 shows PC3 MPR 0 and PC3 MPR 1 do not need any additional guard band for asymmetric cases. Asymmetric PC2 MPR 0 requires an extra guard-band.

Proposal 2: Extend guard band for only PC2 single sided asymmetric cases.

## Derivation of new inner RB region in extended CBW

Ultimately, it is up to RAN4 to decide the best approach for inner RB derivation from an implementation and specification point of view. Although very reasonable, it is unclear whether a new co-ordinate system approach is necessary. Several approaches have already been proposed that also seem very feasible such as the one described in [2] [5].

- WF

- The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well

  - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

- Other approach is not precluded

- The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios

Figure 2.3-1: RB mapping for defining the new inner region shown by orange border in the triangle

Referring to figure 2.3-1, where p and q are the free RBs in the extended region surrounding the UE bandwidth.The UE BW containing allocation, RBstart,LCRB, is considered a new inner allocation if the allocation is within the following region indicated in proposal 3. Generalizations can be made as shown in the proposal. Note, for symmetric extensions, p = q, and for single sided asymmetric extensions, either p or q = 0. In the generalization where p≠q, p=ΔRBshift,low and q=ΔRBshift,high.

Proposal 3: For a given UE BW containing allocation, RBstart,LCRB, use the following generalized inner RB definition:

RBstart_ext,low≤RBstart_ext≤RBstart_ext,high

LCRB≤ceilNRBext2

Where

RBstart_ext=p+RBstart

RBstart_ext,low=p+max⁡(1+floor(LCRB2))

RBstart_ext,high=NRBext-RBstart_ext,low-LCRB

NRBext=p+NRB+q

p=ΔRBshift,low

q=ΔRBshift,high

## Derivation and Relaxation of the extended IBE

The following was the WF agreement.

- Agreement in main session:

- Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed)

  - The details and the related test cases can be discussed and decided later.

  - If significant issue is identified, other options can be further considered.

- Options

  - Option 1: using BPSK EVM in the IBE formula for all extended RBs

  - Option 2: no relaxation is needed.

- WF

  - If there is no obvious system performance impact observed, option 1 is adopted

In this section, we discuss the system performance impact as mentioned in the last bullet of the WF. It was mentioned in previous meetings by a few companies to use a relaxed fixed version of the IBE as in the WF or even propose no requirement [3] [4] in the extended region. We provide justification for not arbitrarily setting an emission mask in the extended region as follows:

Figure 2.4-1 shows a MPR 0 fully allocated waveform with the proposed IBE masks in a 50% dual-sided symmetric extended region along with the normalized SEM mask referred to a common or minimum measurement BW. The flat extended BPSK IBE mask in the WF was arbitrarily chosen so that the emission limit in the extended region did not exceed the SEM mask in the OOBE region which seems quite reasonable at first glance. However, after closer inspection, arbitrarily setting this emission limit to this level can be misleading as well as redundant from a compliance perspective. Hypothetically, if the UE were allowed to generate emissions to this limit, it would not comply with the regulatory occupied bandwidth (OBW) specification from FCC and ITU. Excerpt from rule 1.153 from ITU-R SM.328:

Using the WF proposed emission mask in the extended region implies that the UE BW cannot contain more than 99% of the transmitted power for the dual sided extension case. The table in figure 2.4-1 shows the emission power in the regions outside of the PRB transmission relative to total transmitted power for 0.5 dual-sided symmetric cases. The SEM mask is fine, the QPSK MPR 0 waveform is fine, but the proposed IBE emission limits in the extended region show OBW non-compliance, even though the outside emission will never be generated that high. The IBE mask is only intended for unallocated PRB locations where there is potential PRB transmission within the UE channel BW, so to avoid the redundancy and hypothetical non-compliance, having no requirement is the most logical option since OBW already takes care of any missing requirement in the extended gap.

Figure 2.4-1: Emissions in the extended region for a dual sided symmetric case with 50% extension.

In addition to regulatory reasoning, one could argue having no requirement by simple extension. Since there are no emission requirements in the guard-band of the regular UE channel BW, then by simple extension, there should be no requirements in the extended gap since the gap could be viewed as a much larger extended guard band.

Observation 6: Arbitrarily setting the emission limit to the IBE mask is misleading, and emissions generated in the extended region are of no consequence if the measured OBW follows regulations.

Proposal 4: No emission requirement is necessary for the extended region.

# Conclusion

Observation 1: SEM, ACLR, and Spurious Emission requirements for a given UE BW  are only positionally impacted by ΔFOOBoffset which is a function of the single or dual sided extension ratio listed in Table 2.1-1.

Observation 2: Boosting the power for a given transmission BW requires more available spectrum in the asymmetric case compared to the symmetric case.

Observation 3: Symmetric cases give a higher percentage of RB allocations available for power boosting than the asymmetric cases for a given extended BW.

Observation 4: Asymmetric cases are indirectly used to meet existing UE-UE coexistence spurious requirements, which is the same as providing one sided free RBs or guard-band to prevent AMPR. Further asymmetric extension would be required for a boosted waveform.

Observation 5: Figure 2.2-1 shows PC3 MPR 0 and PC3 MPR 1 do not need any additional guard band for asymmetric cases. Asymmetric PC2 MPR 0 requires an extra guard-band.

Observation 6: Arbitrarily setting the emission limit to the IBE mask is misleading, and emissions generated in the extended region are of no consequence if the measured OBW follows regulations.

Proposal 1: To ease UE implementation, dual sided symmetrical cases are the preferred choice for MPR reduction for this release.

Proposal 2: Extend guard band for only PC2 single sided asymmetric cases.

Proposal 3: For a given UE BW containing allocation, RBstart,LCRB, use the following generalized inner RB definition:

RBstart_ext,low≤RBstart_ext≤RBstart_ext,high

LCRB≤ceilNRBext2

Where

RBstart_ext=p+RBstart

RBstart_ext,low=p+max⁡(1+floor(LCRB2))

RBstart_ext,high=NRBext-RBstart_ext,low-LCRB

NRBext=p+NRB+q

p=ΔRBshift,low

q=ΔRBshift,high

Proposal 4: No emission requirement is necessary for the extended region.

# 4.  References:

- R4-2507930, “WF on NR_ENDC_RF_Ph4_part1”, Huawei, RAN4#115, Malta, May 19th – 23rd, 2025.

- R4-2505962, “Power Boosting and MPR Reduction”, Qualcomm, RAN4#115, Malta, May 19th – 23rd, 2025.

- R4-2506194, “Power domain enhancement for a UE-specific CHBW within a single carrier”, Ericsson, RAN4#115, Malta, May 19th – 23rd, 2025.

- R4-2507127, “Further Views on MPR Reduction”, Sony, RAN4#115, Malta, May 19th – 23rd, 2025.

- R4-2505585, “Draft CR for TS 38.101-1 to introduce MPR reduction for single carrier”, Huawei, RAN4#115, Malta, May 19th – 23rd, 2025.

# Appendix (OOBE Requirements Summary)

## SEM

For SEM, when UE indicates MPRreduction-R19, ΔFOOB is modified by an offset, ΔFOOBoffset = ± x*BWchannel, where the indicated extension ratios are listed in Table 2.1-1.

ΔFOOBext = ΔFOOBoffset + ΔFOOB.

Or from [5] with ΔFOOBoffset = extendedCBW-Low or extendedCBW-High

For a UE indicates support for UE capability [MPRreduction-singleCC] and if IE [extendedCBW-Low] and [extendedCBW-High] are respectively set to a UE channel bandwidth specified in Table 5.3.2-1, the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the frequency span comprises of the bandwidth configured by [extendedCBW-Low], the assigned NR channel bandwidth and the bandwidth configured by [extendedCBW-High] instead.

Table 2.1.1-2: General NR spectrum emission mask (Table 6.5.2.2-1 in 38.101-1)

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |

## 5.2 Spurious

For Spurious domain, the OOB boundary FOOB is adjusted by the same offset for UE indicating MPR reduction-R19.

FOOBext = FOOB + ΔFOOBoffset

Or from [5] with ΔFOOBoffset = extendedCBW-Low or extendedCBW-High

For a UE indicates support for UE capability [MPRreduction-singleCC] and if IE [extendedCBW-Low] and [extendedCBW-High] are respectively set to a UE channel bandwidth specified in Table 5.3.2-1, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the frequency span comprises of the bandwidth configured by [extendedCBW-Low], the channel bandwidth and the bandwidth configured by [extendedCBW-High] instead.

Table 2.1.1.2: Boundary between NR out of band and general spurious emission domain (Table 6.5.3.1-1 in 38.101-1)

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWChannel + 5 |

## 5.3 ACLR

For ACLR, the wording in sub-clause 6.5.2.4.1 needs an extra paragraph as indicated in [5]:

For a UE indicates support for UE capability [MPRreduction-singleCC] and if IE [extendedCBW-Low] and [extendedCBW-High] are respectively set to a UE channel bandwidth specified in Table 5.3.2-1, the adjacent channel starts from the  edge of the frequency span comprises of the bandwidth configured by [extendedCBW-Low], the assigned NR channel bandwidth and the bandwidth configured by [extendedCBW-High] instead.