3GPP TSG-RAN WG4 Meeting #116										                    R4-2509118

Bangalore, India, August 25th – 29th, 2025

Source:	Murata

Title:	LP WUR UE RX REFSENS

Agenda item:		7.24.3.2

Document for:	Approval

# 1.	Introduction

In this contribution, we will discuss the LP-WUR REFSENS. The focus of the discussion will be on how to formulate REFENS for LP-WUR to account for NR band dependency.

# 2.	Discussion

## REFSENS

One reason to calculate the LP-WUR REFSENS in terms of the main radio REFSENS is to not complicate the specification. The main radio assumes a baseline NF of 9dB but depending on the duplex type (FDD or TDD), TX-RX spacing, and frequency, the REFSENS per band can vary. The LP-WUR has agreed that the baseline NF+IM for OOK and OFDM based RX is 18dB and 13.5 dB respectively. The agreed SNR is -4.5dB. Also, the following was agreed as the equation for REFSENS [1] [2]:

REFSENSLP=PrefLP=-174+10logBWLP+NFLP+SNRLP+IM+[ASCSdesense]

The agreed REFSENS equation for LP-WUR does not show any band dependency. Considering that the LP-WUR does not apply the self-interfering TX until the main radio wakes up, the question remains, whether to derive the LP-WUR REFSENS from the main radio REFSENS using a ∆RLP offset parameter. After all, the LP-WUR hardware is most likely going to be a subset of the main radio hardware sharing the same RX filters. Since filters are designed for each band uniquely to overcome a closely spaced TX band or some adjacent coexistence requirement, the approach taken in this contribution is not to consider any self-interfering TX but consider any potential roll-off of the RX filter regardless of the radio duplexing. To accomplish this, the REFSENS must consider an adjustment relative to the minimum supported UEBW reported REFSENS in Table 7.3.2-1 of 38.101-1, since it has been well established that the minimum supported UEBW REFSENS in a FDD band is not dependent on the TX self-interference.

In summary, the following pre-conditions are applied in determining the REFSENS using the main radio REFSENS as a baseline.

- Any UE BW larger than the minimum supported UE channel BW can be configured in any TDD or FDD band except SDL and unlicenced bands containing a fixed LP-WUS bandwidth.

- No self-interfering TX for FDD bands.

- The requirement must consider the existing practical filter implementation to overcome adjacent coexistence issues and in-band roll-off due to small TX-RX spacing.

Observation 1: It is expected that the main radio and LP-WUR will share common RX filters. LP-WUR thermal noise will be band dependent due to varying filter requirements across bands.

The REFESENS difference between LP-WUR and main radio at the minimum supported UE channel BW can be determined as follows:

PrefM,minBW=-174+10logBWM,minBW+NFM+SNRM+IM-DG

∆RLP=PrefLP-PrefM,minBW

∆RLP=10log10BWLPBWM,minBW+NFLP-NFM+SNRLP-SNRM+DG

∆RLP=∆Roffset+ 10log10BWLPBWM,minBW

∆Roffset= NFLP-NFM+SNRLP-SNRM+DG

PrefLP=PrefM,minBW+∆Roffset+10log10BWLPBWM,minBW+[ASCSdesense]

In terms of NRB and ignoring the [ASCS de-sense] by assuming the requirement is tested without adjacent subcarriers:

PrefLP=PrefM,minBW+∆Roffset+10log10NRBLP*SCSLPNRBM,minBW*SCSM,minBW

It is preferable to specify the REFSENS in generalized form to avoid table complexity since the LP-WUS could be a different SCS than the main radio provided the alignment with the PRB grid is maintained. Even if the SCS is the same for both radios, the BW ratio in the equation simplifies to the NRB ratio. The tabulated LP-WUR REFSENS values per band and RX type for 15KHz SCS of the main radio and low power radio are shown in the appendix.

Proposal 1: Specify LP-WUR radio reference in terms of the Main Radio REFSENS at the minimum supported UE channel BW to reduce adding extra tables and added complexity.

PrefLP=PrefM,minBW+∆Roffset+10log10NRBLP*SCSLPNRBM,minBW*SCSM,minBW

∆Roffset= 8.5 for OOK based RX

∆Roffset= 4.0 for OFDM based RX

- PrefM,minBW  is the main radio REFSENS at the minimum supported UE channel BW in table 7.3.2-1a and 7.3.2-1b.

- NRBM,minBW at SCSM,minBW is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 at the minimum supported UE channel BW in table 7.3.2-3.

- NRBLP at SCSLP is the maximum transmission bandwidth configuration as defined in Table 5.3M.2-1 at the minimum supported UE channel BW in table 7.3.2-3.

# Conclusion

Observation 1: It is expected that the main radio and LP-WUR will share common RX filters. LP-WUR thermal noise will be band dependent due to varying filter requirements across bands.

Proposal 1: Specify LP-WUR radio reference in terms of the Main Radio REFSENS at the minimum supported UE channel BW to reduce adding extra tables and added complexity.

PrefLP=PrefM,minBW+∆Roffset+10log10NRBLP*SCSLPNRBM,minBW*SCSM,minBW

∆Roffset= 8.5 for OOK based RX

∆Roffset= 4.0 for OFDM based RX

- PrefM,minBW  is the main radio REFSENS at the minimum supported UE channel BW in table 7.3.2-1a and 7.3.2-1b.

- NRBM,minBW at SCSM,minBW is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 at the minimum supported UE channel BW in table 7.3.2-3.

- NRBLP at SCSLP is the maximum transmission bandwidth configuration as defined in Table 5.3M.2-1 at the minimum supported UE channel BW in table 7.3.2-3.

# 4.  References:

- R4-2507936, “WF on NR_LPWUS_UERF”, Vivo, RAN4#115, Malta, May 19th  – 23rd, 2025.

- TR38.774 V0.4.0, 2025-05, Sub-clause 7.1.4.

# Appendix

Table 5.1: LP-WUR sensitivity for FDD bands

|  |  | Main Radio |  | LP-WUR OOK | LP-WUR OFDM |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | (dBm) | (dBm) |
| n1 | 15 |  | -100.0 | -95.1 | -99.6 |
| n2 | 15 |  | -98 | -93.1 | -97.6 |
| n3 | 15 |  | -97.0 | -92.1 | -96.6 |
| n5 | 15 | -100.2 | -98.0 | -93.1 | -97.6 |
| n7 | 15 |  | -98.0 | -93.1 | -97.6 |
| n8 | 15 |  | -97.0 | -92.1 | -96.6 |
| n12 | 15 | -99.2 | -97.0 | -92.1 | -96.6 |
| n13 | 15 |  | -97.0 | -92.1 | -96.6 |
| n14 | 15 |  | -97.0 | -92.1 | -96.6 |
| n18 | 15 |  | -100.0 | -95.1 | -99.6 |
| n20 | 15 |  | -97.0 | -92.1 | --96.1 |
|  | 30 |  |  |  |  |
| n24 | 15 |  | -100.0 | -95 | -99.5 |
|  | 30 |  |  |  |  |
| n25 | 15 |  | -96.5 | -91.6 | -96.1 |
|  | 30 |  |  |  |  |
| n26 | 15 | -99.7 | -97.5 | -92.6 | -97.1 |
|  | 30 |  |  |  |  |
| n28 | 15 | -100.2 | -98.5 | -93.6 | -98.1 |
|  | 30 |  |  |  |  |
| n30 | 15 |  | -99.0 | -94.1 | -98.6 |
|  | 30 |  |  |  |  |
| n31 | 15 | -95.7 | -93.5 | -88.5 | -93 |
| n65 | 15 |  | -99.5 | -94.6 | -99.1 |
|  | 30 |  |  |  |  |
| n66 | 15 |  | -99.5 | -94.6 | -99.1 |
|  | 30 |  |  |  |  |
| n68 | 15 |  | -98.5 | -93.5 | -98 |
|  | 30 |  |  |  |  |
|  | 15 |  | -100.0 | -95.1 | -99.6 |
| n70 | 30 |  |  |  |  |
| n71 | 15 |  | -97.2 | -92.3 | -96.8 |
|  | 30 |  |  |  |  |
| n72 | 15 | -95.7 | -93.5 | -88.5 | -93 |
| n74 | 15 |  | -99.53 | -94.6 | -99.1 |
|  | 30 |  |  |  |  |
| n85 | 15 | -99.2 | -97.0 | -92.1 | -96.6 |
| n87 | 15 | -95.7 | -93.5 | -88.5 | -93 |
| n88 | 15 | -95.7 | -93.5 | -88.5 | -93 |
| n100 | 15 | -102.2 | -100 | -95 | -99.5 |
| n105 | 15 |  | -97.2 | -92.3 | -96.8 |
|  | 30 |  |  |  |  |
| n106 | 15 | -99.2 |  | -92 | -96.5 |
| n110 | 15 | -102.2 |  | -95 | -99.5 |

Table 5.2: LP-WUR sensitivity for TDD bands

| Operating band | SCS kHz | Channel bandwidth (MHz) | Main Radio REFSENS (dBm) | LP-WUR OOK (dBm) | LP-WUR OFDM (dBm) |
| --- | --- | --- | --- | --- | --- |
| n34 | 15 | 5 | -100 |  | -99.6 |
|  | 30 | 10 | -97.1 |  |  |
| n38 | 15 | 5 | -100 |  | -99.6 |
|  | 30 | 10 | -97.1 |  |  |
| n39 | 15 | 5 | -100 |  | -99.6 |
|  | 30 | 10 | -97.1 |  |  |
| n40 | 15 | 5 | -100 |  | -99.6 |
|  | 30 | 10 | -97.1 |  |  |
| n41 | 15 | 10 | -94.8 |  | -97.5 |
|  | 30 | 10 | -95.1 |  |  |
| n48 | 15 | 5 | -99 |  | -98.6 |
|  | 30 | 10 | -96.1 |  |  |
| n50 | 15 | 5 | -100 |  |  |
|  | 30 | 10 | -97.1 |  |  |
| n51 | 15 | 5 | -100 |  |  |
| n53 | 15 | 5 | -100 |  |  |
|  | 30 | 10 | -97.1 |  |  |
| n54 | 15 | 5 | -100 |  |  |
| n77 | 15 | 10 | -95.3 |  |  |
|  | 30 | 10 | -95.6 |  |  |
| n78 | 15 | 10 | -95.8 |  |  |
|  | 30 | 10 | -96.1 |  |  |
| n79 | 15 | 10 | -95.8 |  |  |
|  | 30 | 10 | -96.1 |  |  |
| n91 | 15 | 5 | -100 |  |  |
| n92 | 15 | 5 | -100 |  |  |
|  | 30 | 10 | -97.1 |  |  |
| n93 | 15 | 5 | -100 |  |  |
| n94 | 15 | 5 | -100 |  |  |
|  | 30 | 10 | -97.1 |  |  |
| n101 | 15 | 5 | -100 |  |  |
|  | 30 | 10 | -97.1 |  |  |
| n104 | 15 | 20 | -90.7 |  |  |
|  | 30 | 20 | -90.8 |  |  |
| n109 | 15 | 5 | -100 |  |  |
|  | 30 | 10 | -97.1 |  |  |