3GPP TSG-RAN4 Meeting #116	R4-2509132

# Bengaluru, India, 25th Aug.2025 - 29th Aug.2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0077 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to introduce a new annex to 36.102 for time varying doppler shift and propagation delay modelling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is necessary to capture the agreements on the time varying doppler shift and propagation delay modelling for NGSO as normative conditions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Created a new Annex E and captured following agreements: - Calculation procedures and formulae based on Newton method - Calculation procedures and formulae based on 4th order Runge-Kutta method - Examples of initial conditions of NTN Ephemeris information - Descriptions on the influence of the Earth’s rotation on propagation delay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normative conditions remain undefined regarding the methods to calculate time varying doppler shift and propagation delay for NGSO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New) Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

Annex D (normative): 
Propagation conditions

# D.1	Multi-path fading propagation conditions

The multipath propagation conditions consist of several parts:

-	A delay profile in the form of a "tapped delay-line", characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency.

## D.1.1	Delay profiles

The delay profiles are derived from the TR 38.811 [13] NTN-TDL models for the desired delay spread and tap resolution. After scaling the normalized delay spread values for each tap by the desired RMS delay spread, the tap delays are quantized to a delay resolution of 5ns by rounding to the nearest multiple of the delay resolution.

Table D.1.1-1: Delay profiles for IoT NTN channel models

| Type | Model | Delay spread (r.m.s.) | Delay resolution |
| --- | --- | --- | --- |
| NLOS | NTN-TDLA100 | 100 ns | 5 ns |
| LOS | NTN-TDLC5 | 5 ns | 5 ns |

Table D.1.1-2: NTN-TDLA100 (DS = 100 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.7 | Rayleigh |
| 3 | 285 | -6.5 | Rayleigh |

Table D.1.1-3 NTN-TDLC5 (DS = 5 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |
| Note 1: Tap #1 follows a Rician distribution. |  |  |  |

## D.1.2	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., NTN-TDLA<DS>-<Doppler>, or NTN-TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table D.1.2-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment for NLOS and LOS propagation conditions.

Table D.1.2-1: Channel model parameters for NTN

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLA100-10 | NTN-TDLA100 | 10 Hz |
| NTN-TDLA100-200 | NTN-TDLA100 | 200 Hz |
| NTN-TDLC5-30 | NTN-TDLC5 | 30 Hz |
| NTN-TDLC5-200 | NTN-TDLC5 | 200 Hz |

Annex E (Normative):  
Modelling of time varying doppler shift and propagation delay for NGSO

E.1	General

This annex specifies the methodologies for time varying doppler shift and propagation delay modelling for NGSO.

E.2	Modelling of time varying doppler shift and propagation delay

Starting from the following equation of motion in ECI (Earth Centred Inertial) frame, two kinds of methodologies are applicable to model the time varying doppler shift and propagation delay by a test equipment, i.e. Newton-Raphson method and 4th order Runge-Kutta method.

A difference between these two methods is there are cases that calculations are carried out in ECI (Earth-Centred Inertial) frame or ECEF (Earth-Centred Earth-Fixed) frame with the Newton-Raphson method while only ECEF frame is applied with the Runge-Kutta method during its calculation flow. Detailed calculation flows with each method are described in the following sub-clauses.

Equation of motion (ECI Frame)

ma=F

a=d2rdt2

Solve Kepler Eq. (ECI Frame)

M=E-e sin⁡(E)

Transform position, velocity and acceleration from (ECI Frame) to (ECEF Frame)

r→r

v→v+ω×r

a→a+2 ω×v+ω×(ω×r)

Determine SV orbit (ECI Frame)

rt, vt@(ECI Frame)

Transform

(ECI Frame) to (ECEF Frame)

{ rt, vt} @(ECEF Frame)

Eq. of motion in ECEF Frame

d2rdt2 =-μ r2 r r +2 v×ω +ω×(r×ω )

Solve Differential equation by Runge-Kutta method.  (ECEF Frame)

{ rt, vt} @(ECEF Frame)

Calculate Delay and Doppler Frequency (ECEF Frame)

Newton-Raphson method

4th order Runge-Kutta method

Figure E.2-1: Two methodologies for modelling the time varying doppler shift and propagation delay

E.2.1	Newton-Raphson method

E.2.1.1 Keplerian model based estimation

Step 1	Derive the orbital parameters (a, e, i, Ω, ω, M0)

Step 1-0	Convert the initial position/velocity state vectors from ECEF format to ECI format

Note we assume the x-axis of ECI and ECEF the same for simplicity.

r0=r0ECEF=r0,xECEFr0,yECEFr0,zECEF

v0=v0,xv0,yv0,z=v0ECEF+00ωE×r0ECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

Step 1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0=r0,x2+r0,y2+r0,z2

v=v0=v0,x2+v0,y2+v0,z2

vr=v0⋅r0r=r0,xv0,x+r0,yv0,y+r0,zv0,zr

h=hxhyhz=r×v=r0,yv0,z-r0,zv0,yr0,zv0,x-r0,xv0,zr0,xv0,y-r0,yv0,x

h=h=hx2+hy2+hz2

Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω is called as ‘Longitude of ascending node’ in TS 36.331 [6] ephemerisInfo-r17.

Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0-r0⋅v0v0=1μv2-μrr0,xr0,yr0,zT-r⋅vrv0,xv0,yv0,zT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0e⋅r=cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr≥02π-cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

Step 2	Determine the satellite position and velocity at time t (sec)

Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton’s method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-2-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-2-5: If Et(n+1)-Et(n)<ϵ, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n := n+1.

Note ϵ=10-6, for example.

Note n=0, 1, 2, …

Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

Step 2-4	Convert the orbital elements to the state vector in ECI frame

Convert to the state vector in perifocal frame,

rt,xpqwrt,ypqwrt,zpqw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0=a1-e21+ecosνtcosνtsinνt0

vt,xpqwvt,ypqwvt,zpqw=μh-sinνt e+cosνt0=μa1-e2-sinνt(e+cosνt)0

Convert the state vector from perifocal frame to ECI.

rt,xECIrt,yECIrt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001rt,xpqwrt,ypqwrt,zpqw

vt,xECIvt,yECIvt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001vt,xpqwvt,ypqwvt,zpqw

Convert the state vector from ECI to ECEF.

rt,xECEFrt,yECEFrt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos(-ωEt)0001rt,xECIrt,yECIrt,zECI

vt,xECEFvt,yECEFvt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos-ωEt0001vt,xECIvt,yECIvt,zECI-0-ωE0ωE00000rt,xECEFrt,yECEFrt,zECEF

E.2.2	4th order Runge-Kutta method

E.2.2.1 Equations of motion followed by satellites in ECEF frame

A motion followed by satellites in the ECEF frame, in other words CTRS (Conventional Terrestrial Reference System), can be expressed by the following vector equation.

d2rdt2 =-μ  r2 rr +2  v×ω +ω×r×ω#E.2.2.1-1

After this, to express the angular velocity factor with the general expression, ω is replaced by Ω (dot omega).

d2rdt2 =-μ  r2 rr +2  v×Ω +ω×r×Ω#E.2.2.1-1'

r=x,y,z#E.2.2.1-2

r=x2+y2+z2 #E.2.2.1-3

Here Ω is the WGS-84 Earth rotation rate, Ω=0, 0, 7.2921151467×10-5 radsec., μ is the Earth gravitational parameter (μ= GM ) μ=3.986005×1014[m3/sec.2], and x, y, z are a coordinate of satellite position state vector in ECEF.  And r and v are position and velocity with vector representation.

Transforming the expression of the second-order ordinary differential equation (E.2.2.1-1’) into the form of a simultaneous equation yields the following:

dvdt=-μ  r2 rr +2 v×Ω+Ω×r×Ω#E.2.2.1-4

drdt=v#E.2.2.1-5

E.2.2.2 Numerical solution of differential equation

There are several methods to numerically solve the differential equation, e.g. Euler method, Predictor-Corrector method, Runge Kutta method, etc. Here, we solve the simultaneous equations (E.2.2.1-4) and (E.2.2.1-5) utilizing the 4th order Runge-Kutta method. Calculation flow is described as follows.

Refer initial satellite position and velocity based on SIB-31 ephemeris information.

1st step: Calculation of k(1,v) and k(1,r)  (See following explanation in this sub-clause.)

2nd step: Calculation of k(2,v) and k(2,r) using the outcome of the 1st step.

3rd step: Calculation of k(3,v) and k(3,r) using the outcome of the 2nd step.

4th step: Calculation of k(4,v) and k(4,r) using the outcome of the 3rd step.

5th step: Calculation of rn+1 and vn+1 at time tn+1. Go to 1st step and use rn+1 and vn+1 in the calculation of k(1,v) and k(1,r). Repeat till the end of measurement time.

Figure E.2.2.2-1: Calculation flow by 4-th order Runge-Kutta method

First, the equation (E.2.2.1-4) in sub-clause E.2.2.1 can be expressed as the acceleration vector since (E.2.2.1-4) is the differential of the velocity.

dvdt=a rn ,vn  =-μ rn2  rnrn +2 vn×Ω+Ω×rn×Ω#E.2.2.2-1

Here

rn= xn ,yn ,zn #E.2.2.2-2

vn=vx, n ,vy,n ,vz,n#E.2.2.2-3

rn=xn2+yn2+zn2#E.2.2.2-4

The vector function a( rn ,vn  ) with a condition Ω=0,0,Ω can be expressed in concrete form by the following equations:

ax xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  xnrn +2 vy,n Ω+xnΩ2 #E.2.2.2-5

ay xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  ynrn -2 vx,n Ω+ynΩ2 #E.2.2.2-6

az xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  znrn #E.2.2.2-7

Note that the condition Ω=0,0,Ω is set when using the ECEF frame such that the centre of earth's gravity is the origin, and the rotation axis is the z-axis.

Next based on the 4th order Runge-Kutta method, we can calculate from k(1,v) to k(4,v), and from k(1,r) to k(4,r) with step-by-step manner. In the next step 1 to 5, Δt is a step size of time. From the position and velocity vector (rn,vn) at time tn(=n Δt), equations from (E.2.2.2-8) to (E.2.2.2-23) are successively calculated to determine (rn+1,vn+1) at time tn+1. Once the (rn+1,vn+1) is obtained at step 5, then we go back to step 1 and use these values in the equations (E.2.2.2-8) and (E.2.2.2-9) to obtain the position and velocity vector at the next step time. Continuing these calculations until the end of measurement time will provide position and velocity information throughout the measurement period with the step size granularity.

Step 1: Calculate following k(1,v) and k(1,r).

k1,  v=a rn,vn  Δt#E.2.2.2-8

k1, r=vn Δt#E.2.2.2-9

If we transform equations (E.2.2.2-8) and (E.2.2.2-9) to the concrete form like (E.2.2.2-5), (E.2.2.2-6) and (E.2.2.2-7), vector functions above can be transformed to the following equations.

kvx=ax xn,yn,zn,vx,n , vy,n ,vz,n ×Δt=-μ rn2  xnrn +2 vy,n Ω+xnΩ2×Δt#E.2.2.2-10

kvy=ay xn,yn,zn,vx,n , vy,n ,vz,n ×Δt=-μ rn2  ynrn -2 vx,n Ω+ynΩ2×Δt #E.2.2.2-11

kvz=az xn,yn,zn,vx,n , vy,n ,vz,n ×Δt=-μ rn2  znrn ×Δt#E.2.2.2-12

krx=vx,n×Δt#E.2.2.2-13

kry=vy,n×Δt#E.2.2.2-14

krz=vz,n×Δt#E.2.2.2-15

Calculations can be carried out also from step 2 to step 5 with the same manner. For the simplicity of the expression, equations are described by the form of vector function after this.

Step 2: Calculate following k(2,v) and k(2,r) using the outcome of Step 1.

k2, v=a rn+12k1, r ,vn+12k1, v Δt#E.2.2.2-16

k2,  r=vn+12k1, vΔt#E.2.2.2-17

Step 3: Calculate following k(3,v) and k(3,r) using the outcome of Step 2.

k3, v=a rn+12k2, r ,vn+12k2, v Δt#E.2.2.2-18

k3,  r=vn+12k2, vΔt#E.2.2.2-19

Step 4: Calculate following k(4,v) and k(4,r) using the outcome of Step 3.

k4, v=a xn+k3, r ,vn+k3, v  Δt#E.2.2.2-20

k4,  r=vn+k3, v Δt#E.2.2.2-21

Step 5: Calculate following vn+1 and rn+1 using the outcome from Step 1 to Step 4.

vn+1=vn+16k1,  v+2k2,  v+2k3,  v+k4,  v#E.2.2.2-22

rn+1=rn+16k1,  r+2k2,  r+2k3, r+k4,  r#E.2.2.2-23

Go back to step 1 and continue calculation to derive position and velocity till the end of measurement period.

E.2.3 Initial conditions for numerical solution

Position and velocity of a LEO satellite and position of a UE can be referred in TS 36.508 [14] as initial conditions. Note these values are described in ECEF frame in Table 5.6.2.1-2 of [14] and described in the ellipsoidal coordinate system in sub-clause 5.6.1 of [14]. And following conditions are extracted as examples.

Table E.2.3-1: Example of SIB31- – eMTC NTN Ephemeris Information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm. |  |  |  |

UE location (referred from TS 36.508 [14])

5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

The position and velocity of the satellite described in ECEF Frame need to be converted based on the definitions in TS 36.331 [6] as EphemerisInfo field descriptions.

EphemerisInfo field descriptions

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

The converted satellite coordinate in ECEF Frame (X_SV, Y_SV, Z_SV):

(X_SV, Y_SV, Z_SV) = ( -3315105.300000 [m], 5396426.100000 [m], 2941944.200000 [m])

The converted satellite velocity in ECEF Frame (VX, VY, VZ):

(VX, VY, VZ) = (1318.200000 [m/s], -2889.840000 [m/s], 6846.540000 [m/s])

Also, the position of the UE needs to be converted to ECEF Frame by the following equations since the definitions of the UE location in TS 36.508 above are described based on the ellipsoidal coordinate system.

xyz=N+hcosϕcosλN+hcosϕsinλN1-e2+hsinϕ#

Here f is geodetic latitude, l is east longitude, h is altitude from geoid, a is average earth equatorial radius, e=f(2-f)  where f is the earth’s declination, and

ϕ=π180UElatitude

λ=π180UElongitude

N=a1-e2sin2ϕ1 2

The converted UE coordinate in ECEF Frame:

(X_UE, Y_UE, Z_UE) = (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

E.2.4	Influence of the Earth’s rotation on propagation delay

As shown in Figure E.2.4-1 and Figure E.2.4-2, an influence of the receiver movement needs to be considered when calculating the propagation delay. For a downlink case, rotation of the Earth affects the delay during the transmission of signals from a satellite to a UE.

Figure E.2.4-1: Propagation delay in downlink case (Signal transmission timing based)

Figure E.2.4-2: Propagation delay in downlink case (Signal receive timing based)

For the downlink case, propagation delay can be expressed as follows.

ΔtDL=RDLtRc1-1cVSVtR⋅RDLtRRDLtR#

where

SV (Satellite)		: Transmitter

UE 				: Receiver

∆tDL= tR- tT  : Downlink total propagation delay

tT					: Time when signals are transmitted from satellite

tR					: Time when signals are received by UE

rR 					: UE position vector in ECI Frame at either tT or tR depending on the reference point.

rT 					: SV position vector in ECI Frame at either tT or tR depending on the reference point.

vUE 				: UE velocity vector at transmitted time tT in ECI Frame

VSV				: Satellite velocity vector at received time tR in ECI Frame

RDL=rR-rT	: Distance between satellite and UE at either tT or tR depending on the reference point.

c = 2.99792458 x 108 [m/s]

Annex F (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4#104-e | R4-2215118 |  |  |  | Initial Skeleton | 0.1.0 |
| 2022-10 | RAN4#105 | R4-2218376 |  |  |  | Added approved TPs in RAN4#104-bis-e including: R4-2217750, R4-2217752, R4-2217753, R4-2217755, R4-2217807, R4-2217810 | 0.2.0 |
| 2022-11 | RAN4#105 | R4-2218377 |  |  |  | Added approved TPs in RAN4#105 including: R4-2218767, R4-2220803, R4-2220804, R4-2220805, R4-2220806, R4-2220812, R4-2220828, R4-2220835, R4-2220836 | 0.3.0 |
| 2022-12 | RAN#98-e | RP-223233 |  |  |  | 1-step Approval of version 1.0.0 | 1.0.0 |

<<End of change >>