3GPP TSG-RAN4 #116	R4-2509134

Bengaluru, India, 25th Aug. – 29th Aug. 2025

Title:	FR2 test setup for AI/ML beam management evaluation

Source:			Anritsu Corporation

Agenda Item:			7.17.6

Document for:			Approval

# 1.	Introduction

During the release 19 activities, we continued a discussion on the AI/ML beam management test setup. However, due to the time constraint we could not have an official discussion for some meetings in this year. A part of the associated previous way forward [1] is extracted for reference.

Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

In this contribution, we share our views on test setup for AI/ML beam management evaluation and its associated channel model.

# 2.	Discussion

# 2.1 General view on AI/ML beam management

RAN4 continued a discussion on AI/ML beam management for several meetings. When we consider test cases for the beam management evaluation, we need to note that TR 38.843 [2] and the current WID [8] are assuming two kinds of sub-use cases, i.e. BM-Case 1 and BM-Case 2. Although we can see in the feature lead summary in RAN1 [7] that RAN1 discussed both BM-case 1 and case 2, it appears that RAN4 was discussing the associated test setup only based on BM-Case 1, described as follows.

BM-Case 1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams

BM-Case 2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams

To fulfill its test coverage of beam management, we think the associated test setup needs to consider both BM-Case 1 and Case 2 along with their channel models to be applied. Thus, we need to clarify all the necessary conditions for both BM-Case 1 and Case 2. Also in these days, some companies are mentioning that tests need to ensure a prediction performance of an AI model in the field. From that perspective, considering the UE behavior in the actual field, there might be a need to conduct tests with mixed conditions of BM-Case 1 and Case 2 and run the tests in series. For example, we conduct a test with BM-Case 1 condition as a first step and then another test with BM-Case 2 condition follows later based on the predicted results from the first BM-Case 1 test. Additional tests with BM-Case 1 condition might also be run at some points during the BM-Case 2 test. Companies are encouraged to bring views on the necessity of such mixed conditional tests.

Observation 1: TR 38.843 and the WID for AI/ML air interface is assuming both BM-Case 1 and BM-Case 2.

Proposal 1: Clarify whether the test setup for the beam management feature shall support both BM-Case 1 and Case 2 tests.

Observation 2: Considering the UE behavior in the actual field, there might be a need to conduct tests with mixed conditions of BM-Case 1 and Case 2 and run the tests in series. For example, we conduct a test with BM-Case 1 condition as a first step and then another test with BM-Case 2 condition follows later based on the predicted results from the first BM-Case 1 test. Additional tests with BM-Case 1 condition might also be run at some points during the BM-Case 2 test.

Proposal 2: Companies are encouraged to bring views on the necessity of such mixed conditional tests. (i.e. BM-Case 1 and/or BM-Case 2)

Also, from the perspective of ensuring the field performance by the AI model, we’d like to make the following two comments. The first point is that we have not concluded regarding channel models to apply to the BM tests in RAN4. Since the group has not completed analyzing which type of channel model can ensure the field performance of the AI model, it is not clear at this moment whether the legacy CDL channel model is necessary or the simplified CDL model can suffice. We think RAN4 must derive the conclusion before deciding a test setup since an outcome of the analysis will have a huge impact on the test setup and test procedures. The second point is that we think we need to conduct tests with multiple channel models. In a real field propagation path, there can be variations in the direction of arrival of each path (direction of clusters), etc. So, testing under a single condition may not be sufficient. Therefore, it can be said that a test result does not fully reflect its performance in the real field when an AI/ML BM-enabled UE is tested using only one channel model. From this viewpoint, we propose defining multiple channel models like the existing CDL models.

Observation 3: Since the group has not completed analyzing which type of channel model can ensure the field performance of the AI model, it is not clear at this moment whether the legacy CDL channel model is necessary or the simplified CDL model can suffice.

Observation 4: Depending on the results of the required channel model analysis, the test system configuration and test procedures may be significantly affected.

Observation 5: In a real field propagation path, there can be variations in the direction of arrival of each path (direction of clusters), etc. Therefore, a test result may not fully reflect its performance in the real field when an AI/ML BM-enabled UE is tested using only one channel model.

Proposal 3: Study the necessary channel models to ensure the prediction performance of the AI model in the field. (Legacy CDL or Simplified CDL, if we need to assume a situation that UE is moving, etc.)

Proposal 4: Define multiple channel models like the existing CDL models (e.g. CDL-A/B/C) to ensure the field performance of the AI model for beam management. FFS for the necessary number of channel models.

# 2.2 Agreements in RAN1 on AI model inputs

Next, we would like to clarify the RAN1 agreement on AI model inputs as of after the meeting in May. As can be seen from the agreement in the previous feature lead summary [6] below, RAN1 concluded that there are 3 options.

Agreement extracted from [6]

Agreement At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Option 2: Measurements of specific Rx beam(s) - Option 2a: Measurements of specific Rx beam(s) per model input sample - Option 2b: Measurements of specific Rx beam(s) for all model input sample - FFS how to select the specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample Other options are not precluded and can be reported by companies.

From these options, our understanding is that it is still up to UE implementation if the UE inputs its Rx beam information into its AI model for inference of the Tx beam ID. However, since this agreement was made in 2023, we think it is better that we confirm the current idea among companies. For example, as for Option 1 above, it is mentioning that the best Rx beam is selected from the measurement result of multiple Rx beams for each model input sample. To align the assumption among companies again, it is preferrable that we confirm whether there is really not a case that multiple Rx beam IDs are input to the AI model per Tx beam ID. Or if there is any other option now.

Observation 6: According to the previous RAN1 agreement, it is still up to UE implementation if the UE inputs its Rx beam information into its AI model for inference of the Tx beam ID.

Proposal 5: Confirm the latest assumptions among companies in RAN4 regarding valuations of Rx beam information to be input to the AI model per measurement of one Tx beam ID.

# 2.3 Further consideration on applicable test setup and test procedure

In this sub-clause, we’d like to consider the applicable test setup and test procedures a little deeper. As mentioned at Observation 1 to 4 above, depending on the results of the required BM-Cases and channel model analysis, the test system configuration and test procedures may be significantly affected.

For example, as for feasibility of the BM-Case 1 tests by multi-AoA test setup, it depends on the channel model to be applied to conduct the tests. If multiple channel models are necessary to evaluate by multi-AoA test setup, or in a case the group identifies the performance issue with the simplified CDL model, there will be a testability issue since the measurement probes in the multi-AoA test setup are fixed in an OTA chamber, and there is a limit on a number of measurement probes to reduce the system complexity.

On the other hand, although there is a benefit that 1AoA test setup has almost no restrictions on the channel model (or number of clusters) to simulate by fading simulator, it may not be able to handle a case where UE sweeps its Rx beam during the tests.

Observation 7: If multiple channel models are necessary to evaluate by multi-AoA test setup, or in a case the group identifies the performance issue with the simplified CDL model, there will be a testability issue.

Observation 8: Although there is a benefit that 1AoA test setup has almost no restrictions on the channel model (or number of clusters) to simulate by fading simulator, it may not be able to handle a case where UE sweeps its Rx beam during the tests.

Another example is related to BM-Case 2. According to the report attached in TR 38.843 [2], the link level simulation in RAN1 appears to apply an assumption of the channel model described in TR 38.901 clause 7.6.3.2 [4], i.e. Spatially-consistent UT/BS mobility modelling.

Observation 9: Link level simulation in RAN1 appears to apply the assumption of the channel model described in TR 38.901 clause 7.6.3.2 [4], i.e. Spatially-consistent UT/BS mobility modelling.

When considering the test method of this mobility modelling, we need to study how we can create the channel environment by the candidate test setups in a case the UE moves from one point to another.

Observation 10: We need to study how we can create the channel environment by the candidate test setups in a case that the UE moves from one point to another.

UE moves from one point to another during the BM-Case 2 test.

Figure 2.3-1: Example of channel model in TR 38.901 clause 7.6.3 – Spatial consistency

If we consider 1AoA test setup, it is possible to reproduce the situation that UE moves from one point to another. However, same as BM-Case 1, since the 1AoA test setup may not be able to treat a situation where UE sweeps its Rx beam, there is a testability issue.

Next, as for the multi-AoA setup, we anticipate that it also has some restrictions while testing BM-Case 2. For example, it would be possible to temporally change transmitted powers from measurement probes. However, the following limitations are expected.

- Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce.

- Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber.

Considering the restrictions above, both test setups may also face the testability issue in BM-Case 2.

Observation 11: For BM-Case 2, it is expected that there are testability issues with both the 1AoA setup and multi-AoA setup.

1AoA setup

- The 1AoA test setup may not be able to treat a situation where UE sweeps its Rx beam.

Multi-AoA setup

- Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce.

- Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber.

As a summary, depending on the outcome of the proposals in this paper, we think there are still cases where there is no comprehensive test solution to evaluate BM-Case 1 and/or BM-Case 2 at this moment. Therefore, we should leave an option as an alternative solution that we conduct the evaluation of these beam management AI performances by two steps, first we evaluate only the prediction performance of AI model without the Rx beam sweeping, and second, we evaluate the UE behavior of the Rx beam sweeping.

Observation 12: Depending on the outcome of the study, there may still be cases where there is no comprehensive test solution to evaluate BM-Case 1 and/or BM-Case 2 at this moment.

Proposal 6: Leave an option that we conduct the evaluation of these beam management AI performances by two steps, first we evaluate only the prediction performance of AI model without the Rx beam sweeping, and second, we evaluate the UE behavior of the Rx beam sweeping.

- Conclusion

In this contribution, we shared our views on test setup for AI/ML beam management evaluation and its associated channel model.

Observation 1: TR 38.843 and the WID for AI/ML air interface is assuming both BM-Case 1 and BM-Case 2.

Proposal 1: Clarify whether the test setup for the beam management feature shall support both BM-Case 1 and Case 2 tests.

Observation 2: Considering the UE behavior in the actual field, there might be a need to conduct tests with mixed conditions of BM-Case 1 and Case 2 and run the tests in series. For example, we conduct a test with BM-Case 1 condition as a first step and then another test with BM-Case 2 condition follows later based on the predicted results from the first BM-Case 1 test. Additional tests with BM-Case 1 condition might also be run at some points during the BM-Case 2 test.

Proposal 2: Companies are encouraged to bring views on the necessity of such mixed conditional tests. (i.e. BM-Case 1 and/or BM-Case 2)

Observation 3: Since the group has not completed analyzing which type of channel model can ensure the field performance of the AI model, it is not clear at this moment whether the legacy CDL channel model is necessary or the simplified CDL model can suffice.

Observation 4: Depending on the results of the required channel model analysis, the test system configuration and test procedures may be significantly affected.

Observation 5: In a real field propagation path, there can be variations in the direction of arrival of each path (direction of clusters), etc. Therefore, a test result may not fully reflect its performance in the real field when an AI/ML BM-enabled UE is tested using only one channel model.

Proposal 3: Study the necessary channel models to ensure the prediction performance of the AI model in the field. (Legacy CDL or Simplified CDL, if we need to assume a situation that UE is moving, etc.)

Proposal 4: Define multiple channel models like the existing CDL models (e.g. CDL-A/B/C) to ensure the field performance of the AI model for beam management. FFS for the necessary number of channel models.

Observation 6: According to the previous RAN1 agreement, it is still up to UE implementation if the UE inputs its Rx beam information into its AI model for inference of the Tx beam ID.

Proposal 5: Confirm the latest assumptions among companies in RAN4 regarding valuations of Rx beam information to be input to the AI model per measurement of one Tx beam ID.

Observation 7: If multiple channel models are necessary to evaluate by multi-AoA test setup, or in a case the group identifies the performance issue with the simplified CDL model, there will be a testability issue.

Observation 8: Although there is a benefit that 1AoA test setup has almost no restrictions on the channel model (or number of clusters) to simulate by fading simulator, it may not be able to handle a case where UE sweeps its Rx beam during the tests.

Observation 9: Link level simulation in RAN1 appears to apply the assumption of the channel model described in TR 38.901 clause 7.6.3.2 [4], i.e. Spatially-consistent UT/BS mobility modelling.

Observation 10: We need to study how we can create the channel environment by the candidate test setups in a case that the UE moves from one point to another.

Observation 11: For BM-Case 2, it is expected that there are testability issues with both the 1AoA setup and multi-AoA setup.

1AoA setup

- The 1AoA test setup may not be able to treat a situation where UE sweeps its Rx beam.

Multi-AoA setup

- Channel models described in TR 38.901 clause 7.6.3.2 (i.e. Spatially-consistent UT/BS mobility modelling.) might be difficult to reproduce.

- Positions and directions of clusters cannot be changed since measurement probes are fixed in the OTA chamber.

Observation 12: Depending on the outcome of the study, there may still be cases where there is no comprehensive test solution to evaluate BM-Case 1 and/or BM-Case 2 at this moment.

Proposal 6: Leave an option that we conduct the evaluation of these beam management AI performances by two steps, first we evaluate only the prediction performance of AI model without the Rx beam sweeping, and second, we evaluate the UE behavior of the Rx beam sweeping.

# 4.	References

[1] R4-2505105, “WF on requirements for AI/ML air interface normative work”, Moderator (Qualcomm), RAN4 #114-bis, Wuhan, China

[2] TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, V18.0.0

[3] R4-2503711, “FR2 test setup for AI/ML beam management evaluation”, Anritsu, RAN4 #114-bis, Wuhan, China

[4] TR 38.901, "Study on channel model for frequencies from 0.5 to 100 GHz", v18.0.0, 2024-03

[5] R4-2506281, “FR2 test setup for AI/ML beam management evaluation”, Anritsu, RAN4 #115, St. Julian’s, Malta

[6] R1-2303997, “FL summary #3 for AI/ML in beam management”, Samsung (Moderator), RAN1 #112bis-e

[7] R1-2504773, “FL summary #4 for AI/ML in beam management”, Samsung (Moderator), RAN1 #121, St. Julian’s, Malta

[8] RP-243244, “WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, Qualcomm, RAN #106, Madrid, Spain