3GPP TSG RAN WG4 Meeting #116                  	R4-2509150

Bengaluru, IN, 25th - 29th Aug, 2025

Title:	Discussion on UE MMSE-IRC receiver feature update

Source:	China Telecom

Agenda item:			4.5

Document for:	Discussion

- Introduction

This contribution provides views and possible solutions on UE MMSE-IRC receiver feature definition defined in Rel-17 WI NR_demod_enh2-Perf.

- Discussion

In the Rel-17 WI NR_demod_enh2-Perf, the UE demodulation and CSI reporting requirements for interference scenarios are defined in TS38.101-4.

RAN4 has also agreed the release independent and test applicability rules for the above requirements, i.e., optional only for Rel-15 and Rel-16 UEs based on declaration and mandatory for all Rel-17 and forward NR UEs (as specified in TS38.307 below).

Table 5.4-1: Additional requirements of other release independent features Feature; Release independent from; Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced); Further information. UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference; Rel-15; Table B.3.3-1; Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration..

Observation 1: In the Rel-17 WI NR_demod_enh2-Perf, RAN4 has defined PDSCH requirements with ICI/IUI interference with additional rule that ‘optional only for Rel-15 and Rel-16 UEs based on declaration and mandatory for all Rel-17 and forward NR UEs’.

According to the above RAN4 input, RAN2 has defined the following UE MMSE-IRC receiver feature in the NR feature list TR38.822. In addition, RAN2 has also introduced new Rel-15 and Rel-16 UE MMSE-IRC receiver capability as an optional without capability signalling feature in the TS38.306 (v15.x and v16.x).

Note that from v17.x TS38.306, there is no MMSE-IRC receiver capability definition because it becomes a mandatory without signalling feature.

5.2 UE receiver features Definitions for feature. SU-MIMO Interference Mitigation advanced receiver - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 [18].. MMSE-IRC (Minimum Mean Square Error - Interference Rejection Combining) receiver It is optional for UE to support MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference..

Observation 2: Based on the RAN4 input, RAN2 has also introduced Rel-15 and Rel-16 optional w/o signalling UE feature MMSE-IRC receiver as specified in TS38.306 and TR38.822.

According to the above information, the requirements under interference scenarios are completely bundled with the UE MMSE-IRC processing capability. In other words, from the specification point of view, there is no need for a R15/R16 UE to pass such tests if it supports other more advanced receivers, e.g., SU-MIMO R-ML receiver which is introduced in Rel-15.

One more information is that in the current RAN5 specification TS38.521-4, tests with interference are only applicable for R15/16 UEs supporting MMSE-IRC:

###### 5.2.2.1.16_1 2Rx FDD FR1 for PDSCH with intra cell inter user interference performance for baseline MMSE-IRC receiver – 2x2 MIMO for both NSA and SA 5.2.2.1.16_1.2 Test applicability Test 1-1 applies to all types of NR UEs and E-UTRAN UEs supporting EN-DC for release 15 and release 16 supporting MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference. Test 1-1 applies to all types of release 17 and forward NR UEs and E-UTRAN UEs supporting EN-DC.

Observation 3: The requirements under interference scenarios are completely bundled with the UE MMSE-IRC processing capability. From the specification point of view, a R15/R16 UE can skip tests with ICI/IUI interference, if it supports more advanced receiver, e.g., SU-MIMO R-ML receiver introduced in Rel-15.

From our understanding, there should be no doubt that a R15/R16 UE supporting SU-MIMO R-ML or other unspecified advanced receivers, can still pass the tests with interference, because such requirements have been mandatory for all NR UEs since R17. Therefore, we think there is necessity to update the existing NR specification to avoid the possible situation in Observation 3. 2 options are proposed shown below:

Proposal 1 (Preferred): Decouple the R17 requirements with interference with the UE MMSE-IRC receiver capability in the NR spec. The R17 requirements with interference can be optional for R15/16 UEs and can be executed simply based on UE declaration.

Therefore, RAN4 needs to send LS to RAN2 to:

- Update the existing UE feature list TR38.822 as below:

| Features | Index | Feature group | Components | Xxx |
| --- | --- | --- | --- | --- |
| 24. NR_demod_enh2-Perf | 24-6 | MMSE-IRC receiver for scenarios with inter-cell and intra-cell inter-user interference | Support of MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference | xxx |

- Remove the MMSE-IRC receiver capability definition in the RAN2 TS38.306 v15.x and v16.x.

If the Proposal 1 cannot be accepted, there is at least necessity to add additional test applicability that the test requirements with interference are also applicable for R15/R16 UEs supporting SU-MIMO R-ML receiver.

Proposal 2: If the proposal 1 cannot be accepted, add additional applicability note in TS38.101-4 that the test requirements with interference are also applicable for R15/R16 UEs supporting SU-MIMO R-ML receiver.

- Summary

Observation 1: In the Rel-17 WI NR_demod_enh2-Perf, RAN4 has defined PDSCH requirements with ICI/IUI interference with additional rule that ‘optional only for Rel-15 and Rel-16 UEs based on declaration and mandatory for all Rel-17 and forward NR UEs’.

Observation 2: Based on the RAN4 input, RAN2 has also introduced Rel-15 and Rel-16 optional w/o signalling UE feature MMSE-IRC receiver as specified in TS38.306 and TR38.822.

Observation 3: The requirements under interference scenarios are completely bundled with the UE MMSE-IRC processing capability. From the specification point of view, a R15/R16 UE can skip tests with ICI/IUI interference, if it supports more advanced receiver, e.g., SU-MIMO R-ML receiver introduced in Rel-15.

Proposal 1 (Preferred): Decouple the R17 requirements with interference with the UE MMSE-IRC receiver capability in the NR spec. The R17 requirements with interference can be optional for R15/16 UEs and can be executed simply based on UE declaration.

Therefore, RAN4 needs to send LS to RAN2 to:

- Update the existing UE feature list TR38.822 as below:

| Features | Index | Feature group | Components | Xxx |
| --- | --- | --- | --- | --- |
| 24. NR_demod_enh2-Perf | 24-6 | MMSE-IRC receiver for scenarios with inter-cell and intra-cell inter-user interference | Support of MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference | xxx |

- Remove the MMSE-IRC receiver capability definition in the RAN2 TS38.306 v15.x and v16.x.

Proposal 2: If the proposal 1 cannot be accepted, add additional note in TS38.307 that the test requirements with interference are also applicable for R15/R16 UEs supporting SU-MIMO R-ML receiver.

- References