3GPP TSG-RAN WG4 Meeting #116						R4-2509162

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	On remaining UEIBM requirements

Agenda item:	7.20.4.1

Document for:	Discussion

Introduction

In this document, we present our views regarding the remaining core requirements that RAN4 needs to discuss in relation to the UE-initiated beam management feature.

Discussion on reporting delay requirements with timer

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

- Option 1: (Qualcomm, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

In the last RAN4 meeting, there was a general agreement on how to define the measurement period with some open points.

RAN1 has defined a semi sliding window based approach where the UE starts the timer for the window when the first event instance occurs. The length of the window is network configured. The UE shall determine if the reporting condition is met when M event instances have occurred within this window. The UE shall have only a single timer running per beam at a given instance of time.

In the case of window-based approach, the UE can have only one timer running per beam at a time.

For UE to determine that a report is to be triggered in the window-based approach, M events for the same beam should occur before the expiry of the timer.

We see no issue in defining requirements taking the above RAN1 defined conditions into account. Hence for the sake of completeness of the work item, we propose to specify requirements for the window based approach where the requirements shall consider the window/timer and the M events that need to occur within the window for the reporting condition to be met.

Define reporting delay requirement also for the window/timer based approach, where the delay considers that the UE shall send the report after M event instances have occurred within the window/timer.

In the basic scenario for the window-based approach, the report is triggered within a timer that starts from reporting condition becoming met for the first time, similar as in the no window option. For this case, the reporting delay should be simply extended so that it covers the M event occasions that trigger the report. The delay may be up to the timer duration, but UE shall trigger the report at the first PUCCH occasion after M event occasions have occurred, within or after the timer.

For the window/timer based approach, when the timer is started for the first time and if the reporting condition for a beam is met before the expiry of the first timer, reporting delay is the time from when the condition is met over the air until the first PUCCH transmission.

However, if the report is not triggered by the first timer, a second timer for this beam would only be started after the first timer expires. It is not clearly defined how the UE shall decide about restarting the timer, so the starting point of this second timer is not obvious.

The M event instances may not be consecutive, and if the timer expires without a report being triggered, the UE can start another timer for the same beam when the first timer expires.

Therefore, the starting point of the reporting delay may not be the same as the basic scenario, where the delay should start from the moment the reporting condition becomes met over the air. We think it would be clearest if the reporting delay is defined to start from the beginning of the timer during which the report is actually triggered. Hence, RAN4 may need to define two alternative starting points for the window-based reporting delay depending on whether a timer is already running.

If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from the start of a subsequent timer during which reporting condition is met until the first PUCCH transmission.

The reporting delay shall take into account the evaluation delay for realizing the condition is met (if met for the first time), the time it takes for the M event occasions to occur and the time until the first PUCCH after that.

The reporting delay for the window-based approach shall not be larger than the evaluation delay + the time until the first PUCCH transmission after M event occasions have occurred within one timer.

Discussion on TCI switching requirements

Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

<Way forward >

- Option 1: (Nokia, Ericsson, MTK)

  - No need to capture this scenario in RAN4 requirements

- Option 2: (Xiaomi, Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

In the last RAN4 meeting, the following was left as open issue for discussion [1]:

On the topic of resetting the counting of beams for event trigger determination, RAN1 has defined that the UE needs to reset and restart the counting of all the new beams when the indicated TCI state is changed. In view of this, it is natural that the evaluation of the determination of the event triggering condition being fulfilled starts once the UE has switched to the target TCI state.

However, this is something that is to be defined by RAN1/2, not RAN4. From RAN4 point of view this would in our view be considered as a corner case, and hence we do not see a need for RAN4 to specifically define event-triggered reporting requirements or a corresponding test case for this scenario.

Issue 1-5: Whether to specify new unified TCI switching delay requirements?

Agreement:

- The existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management

  - Further discuss additional enhancements in the next meeting, and the discussion does not impact the WI core part completion.

If the indicated TCI state changes during the L1 measurement time, the evaluation of the event trigger determination shall start after the TCI state switch is completed. However, there is no need to capture this scenario in RAN4 requirements.

In RAN1#120 meeting various alternatives about beam application latency reduction after a UEI beam report is sent were discussed and agreed to be studied. No conclusion was reached about whether or which alternative to use. In the last RAN4 meeting, the topic was discussed, but companies had different views about whether to capture this aspect or not. In our view, this aspect is very useful, and it makes a lot of sense since the purpose of event-based L1 reporting is to lead to TCI state switch, and hence in most cases, the network will indeed send a following TCI state activation command after the UE has sent the report.

Support DL synchronization based on UEIBM report.

Based on the earlier proposals in RAN1, the intention is that the UE would be DL synchronized with the RS reported in the event-triggered report due to which the TCI states will be activated without the need for DL sync, i.e., within Tharq+3ms after the reception of the MAC-CE command for TCI state activation. From RAN4 requirements point of view, the reduction in the TCI state activation delay will depend on whether the UE has performed a DL synchronization with the RS reported in the UEI report before the reception of the TCI state switch command. There can be the following two scenarios where the UE can do the DL time/frequency synchronization before it receives the TCI state activation command:

- Prior to sending the UEI beam report while doing the measurement of the RS that triggered the UEI event.

- After sending the UEI beam report and before receiving the TCI state activation command.

Depending on which option applies and depending on the time period between sending the report and receiving the TCI state activation command, the UE may or may not have DL synchronization at the time of receiving the TCI state activation command. We think the latter assumption is more reasonable as the UE would only need to do the DL sync after report has been triggered. This means that the UE would require one SSB after triggering the report for DL sync. If such SSB has occurred before reception of the TCI state switching command, the UE is expected to have the DL sync and TCI state activation delay does not include the first SSB for DL sync (TOk = 0).

If UE initiated DL synchronization based on UEIBM report is supported, TOk = 0 (DL sync is skipped) in the TCI state switching delay, if the TCI state switching command is received after reception of the first SSB after sending the report.

We have captured a text proposal in our CR to cover the above.

UE capabilities

Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

<Way forward >

- Option 1: (Samsung, Apple, Huawei, Ericsson)

- For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Option 2: (Nokia, MTK)

- Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

Issue 1-9: Compatibility of capabilities with LTM

<Way forward >

- If such capabilities are defined in Issue 1-8, then discuss on:

- Option 1:

- The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO.

- Option 2:

- The capability for supported number of events triggered L1 measurement reporting is defined as common for Rel-19 mobility and Rel-19 MIMO.

For L3 event-triggered reporting, RAN4 has defined capabilities for the support of event-triggered reporting, where the UE shall be able to support in parallel per category up to Ecat reporting criteria according to table 9.1.4.2-1.

There have been discussions in the past few RAN4 meetings whether there is a need to update the existing table or add new table in section 9.1.4 for the number of events supported per category (Ecat) for L1 events.

Firstly, the L3 events per category obviously do not cover the L1 events. The L1 events the UE shall be able to support should be added on top of the existing L3 events, i.e., introduction of L1 events shall not impact the existing UE capabilities to support L3 events. Hence, to ensure that the network is aware of the number of events that can be configured for a UE, we think RAN4 needs to define either a new table for L1 events to be supported or add new rows to the existing Ecat table to cover intra- and inter-frequency UEIBM events to be supported.

Introduce the number of supported UEIBM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.

Within the NR_Mob_Ph4 work item, it was already agreed not to discuss common capabilities between mobility and MIMO work items, at least before the feature list discussion. We propose to make the same agreement for MIMO as was captured in NR_Mob_Ph4 way forward in RAN4#114bis (R4-2504965):

Issue 2-5: whether and how to coordinate with R19 MIMO Agreement: - RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

Reuse the agreement under NR_Mob_Ph4 regarding coordination between mobility and MIMO events: RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

Conclusion

In this paper, the following observations and proposals were made:

Observation 1: In the case of window-based approach, the UE can have only one timer running per beam at a time.

Observation 2: For UE to determine that a report is to be triggered in the window-based approach, M events for the same beam should occur before the expiry of the timer.

Proposal 1: Define reporting delay requirement also for the window/timer based approach, where the delay considers that the UE shall send the report after M event instances have occurred within the window/timer.

Proposal 2: For the window/timer based approach, when the timer is started for the first time and if the reporting condition for a beam is met before the expiry of the first timer, reporting delay is the time from when the condition is met over the air until the first PUCCH transmission.

Observation 3: The M event instances may not be consecutive, and if the timer expires without a report being triggered, the UE can start another timer for the same beam when the first timer expires.

Proposal 3: If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from the start of a subsequent timer during which reporting condition is met until the first PUCCH transmission.

Proposal 4: The reporting delay for the window-based approach shall not be larger than the evaluation delay + the time until the first PUCCH transmission after M event occasions have occurred within one timer.

Proposal 5: If the indicated TCI state changes during the L1 measurement time, the evaluation of the event trigger determination shall start after the TCI state switch is completed. However, there is no need to capture this scenario in RAN4 requirements.

Proposal 6: Support DL synchronization based on UEIBM report.

Proposal 7: If UE initiated DL synchronization based on UEIBM report is supported, TOk = 0 (DL sync is skipped) in the TCI state switching delay, if the TCI state switching command is received after reception of the first SSB after sending the report.

Proposal 8: Introduce the number of supported UEIBM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.

Proposal 9: Reuse the agreement under NR_Mob_Ph4 regarding coordination between mobility and MIMO events: RAN4 shall develop the two features independently at current stage. Further discussion on common capability is allowed when discussing UE feature list of R19.

- References

- R4-2508271     WF on RRM requirements for NR_MIMO_Ph5_Part1