3GPP TSG-RAN WG4 Meeting #116	R4-2509168

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.6.5.3

Source:	MediaTek Inc.

Title:	Discussion on demodulation performance requirements for NGSO channel model

Document for:	Discussion

# Introduction

In this contribution, we provide our views on remaining issues for NTN demodulation requirements considering time varying doppler shift and propagation delay models.

# Discussion

Issue 2-1-2 Test cases for NR NTN with applying time varying Doppler Shift and propagation delay model - agreement - Only apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2. Issue 2-1-3 Test case for NB-IoT NTN with applying the time-varying Doppler shift and propagation delay model - agreement - Only apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1. Issue 2-1-4 Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model - agreement - Only consider test 1 and test 2 for Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model.

Prior to Release-19, the demodulation requirements for NTN were established under the assumptions of zero Doppler shift and constant propagation delay. However, RAN4 recognizes the need to re-evaluate UE demodulation performance by incorporating a time-varying Doppler shift and a dynamic propagation delay model. To address this, RAN4 has agreed to reuse existing test parameters and select a subset of previous test cases for NR-NTN, NB-IoT NTN, and eMTC NTN. These selected cases will serve as the basis for defining new demodulation requirements, replacing the earlier assumptions of zero Doppler shift and constant delay with more realistic, time-varying conditions.

Issue 1-2-3 Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models - agreement - Launch simulation campaigns to assess the impact on demodulation requirements from time-varying propagation delay and Doppler shift. Considering the residual frequency error and timing error to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. - Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - residual frequency offset and time delay - option 1: Fix value - option 1a: frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s - option 1b: frequency error and timing error as 2Hz/s and 0.05us/s - option 2: time-varying propagation delay and Doppler shift. - FFS - Companies are encouraged to provide the simulation results during RAN4#116 meeting

RAN4 has nearly finalized the model for time varying Doppler shift and propagation delay for NSGO. Based on the agreements reached at RAN4#115, RAN4 will start launching simulation campaigns to assess the impact on demodulation requirements from time-varying propagation delay and Doppler shift. We referred to the Annex of WF [1] to generate the Doppler shift and propagation delay shown in Figure 1 for simulations.

and provide the corresponding simulation results summarized in Table 1. Since the simulation cannot run for a very long duration, we selected specific evaluation angles to conduct the simulation.

Figure 1. Doppler shift and delay considering satellite movement for NGSO-600 from evaluation angle -30 to 30 degrees

In our simulations, we assume UE receives the satellite ephemeris information every 10.24 seconds and leverage some satellite propagator model. Since the simulation cannot run for a very long duration, we selected some specific evaluation angles to conduct the simulation. We also choose cos(Ө) = 0.7 for the case of LOS channel.

From the simulation results in Table 1, it can be observed that the SNR required to achieve 70% of the maximum throughput is nearly identical for both the “zero Doppler and constant delay” scenario and the “time-varying Doppler shift and propagation delay” scenario. Additionally, the results remain consistent across different evaluation angles, showing no significant variation.

Table 1. Simulation results for Test 1-1 and Test 1-2 in TS 38.101-5 Clause 8.2.1.2.2.1when considering time varying Doppler shift and propagation delay (SNR achieving 70% maximum throughput)

|  | zero Doppler and constat delay | time varying Doppler shift and propagation delay |  |  | Current requirements |
| --- | --- | --- | --- | --- | --- |
|  |  | evaluation angle around -30 degrees | evaluation angle around 90 degrees | evaluation angle around 30 degrees |  |
| Test 1-1 | -1.23 dB | -1.23 dB | -1.23 dB | -1.23 dB | 0.3 dB |
| Test 1-2 | 5.19 dB | 5.18 dB | 5.16 dB | 5.19 dB | 7.6 dB |

Observation 1: Based on simulations for NR-NTN, the SNR achieving 70% of the maximum throughput is nearly identical for both the scenario of “zero Doppler and constant delay” scenario and the “time-varying Doppler shift and propagation delay”.

Compared to the current requirements with zero Doppler, there are at least 1.5dB margin when considering time varying Doppler shift and propagation delay model. However, as this meeting is the first time for simulation alignment, we think it is too early to determine whether to keep the same PDSCH requirements or to relax requirements (add some margin on top the current requirements) when we apply the new channel model. Therefore, we propose to check the results at this meeting first and then determine whether to keep or relax the current requirements.

Observation 2: From our simulation results, there are at least 1.5dB margin when considering time varying Doppler shift and propagation delay model

Proposal 1:  Check simulation result first and then determine whether to keep or relax the current requirements.

It is noted that Test 1-2 in TS 38.101-5 Clause 8.2.1.2.2.1 pertains to the LOS channel (NTN-TDLC). Figure 2 illustrates the Doppler power spectrum for NTN under LOS conditions. In addition to the time-varying Doppler shift fd,shift caused by the movement of satellite, a specific AoA (Ө) value must also be defined when running the simulation. For this analysis, we consider a time-varying channel with an evaluation angle of approximately 30 degrees and compare the performance for different fixed values of cos(Ө), as shown in Table 2. The results indicate that performance varies with the value of cos(Ө).

Table 2. Simulation results for Test 1-2 in TS 38.101-5 Clause 8.2.1.2.2.1 when considering time varying Doppler shift and propagation delay and different cos(Ө) (SNR achieving 70% maximum throughput)

|  | cos(Ө) = 1.0 | cos(Ө) = 0.7 | cos(Ө) = 0.2 |
| --- | --- | --- | --- |
| Test 1-2 | 5.28 dB | 5.18 dB | 4.88 dB |

Figure 2. Illustration of Doppler power spectrum in NTN in LOS conditions

Observation 3: For an NTN LOS channel, the demodulation performance varies with AoA (Ө) value.

Proposal 2: Use cos(Ө) = 0.7 to run simulations when using time varying Doppler shift and propagation delay model.

Issue 2-1-3 Test case for NB-IoT NTN with applying the time-varying Doppler shift and propagation delay model - agreement - Only apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1. Issue 2-1-4 Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model - agreement - Only consider test 1 and test 2 for Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model. Issue 2-1-5 Testing time of demodulation test with NGSO LEO-600 - agreement - RAN4 introduce the channel model for both NGSO LEO-600 and LEO-1200. If the LEO-600 model is not enough to cover the test time of NR and IoT NTN test cases defined in RAN4, it is up to RAN5 to decide whether LEO-1200 will be used for testing

At the RAN4#115 meeting, RAN4 agreed to leverage certain existing test setups and incorporate the new channel model to define updated test cases. Based on our understanding, these new test cases introduced in Rel-19 are intended to replace the existing Rel-17/Rel-18 test cases, which rely on an unrealistic channel model, such as a snapshot of the satellite link channel. In light of this, we propose the following:

- For Rel-19 UEs, if a UE successfully meets the new NGSO requirements using the updated channel model, it should be exempt from all NGSO requirements defined in Rel-17/Rel-18.

- Additionally, we believe that some Rel-17/Rel-18 UEs may also be capable of meeting the new NGSO requirements, which incorporate a time-varying Doppler shift and delay model. Therefore, we propose that if a Rel-17/Rel-18 UE successfully meets the new NGSO requirements using the updated channel model, it should likewise be exempt from all NGSO requirements defined in Rel-17/Rel-18.

Observation 4: New test cases defined in Rel-19 are used to replace the existing Rel-17/Rel-18 test case considering unrealistic channel model, i.e., snapshot of the satellite link channel.

Proposal 4: For Rel-19 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18.

Proposal 5: For Rel-17/Rel-18 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18.

# Conclusion

In this contribution, we provide our views on demodulation performance requirements for NGSO channel model. Observations and proposals are summarized as below.

Observation 1: Based on simulations for NR-NTN, the SNR achieving 70% of the maximum throughput is nearly identical for both the scenario of “zero Doppler and constant delay” scenario and the “time-varying Doppler shift and propagation delay”.

Observation 2: From our simulation results, there are at least 1.5dB margin when considering time varying Doppler shift and propagation delay model

Proposal 1:  Check simulation result first and then determine whether to keep or relax the current requirements.

Observation 3: For an NTN LOS channel, the demodulation performance varies with AoA (Ө) value.

Proposal 2: Use cos(Ө) = 0.7 to run simulations when using time varying Doppler shift and propagation delay model.

Observation 4: New test cases defined in Rel-19 are used to replace the existing Rel-17/Rel-18 test case considering unrealistic channel model, i.e., snapshot of the satellite link channel.

Proposal 4: For Rel-19 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18.

Proposal 5: For Rel-17/Rel-18 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18.

# Reference

- R4-2508625, Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod, Samsung