3GPP TSG-RAN WG4 Meeting #116	R4-2509169

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.27.7

Source:	MediaTek Inc.

Title:	Discussion on UE demodulation requirements for Rel-19 NR-NTN

Document for:	Discussion

# Introduction

There are lots of new features in the WI of Rel-19 NR-NTN phase 3. At RAN4#115, RAN4 agreed to only introduce UE demodulation requirements for RedCap/eRedCap with NR NTN operating FR1-NTN bands. We provide our view on remining issues in the WF [1].

- Discussion

Issue 3-2-1: PDSCH demodulation requirements for RedCap Agreement: For 2Rx UEs, - Reuse Rel-17 NTN PDSCH demodulation requirements. - Companies can check whether all the Rel-17 NTN features can be applicable for RedCap. Way forward: For 1Rx UE, - FFS whether to add new PDSCH requirements with NTN channel or not. Issue 3-2-4: Test case of PDSCH for RedCap 1Rx (if defined) Way forward: - Option 1: - MCS4, NTN-TDLA100-200, 1x1, Disabled HARQ - MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes - MCS4, NTN-TDLA100-200, 1x1, 32 HARQ processes - Option 2: - MCS4, NTN-TDLA100-200, 1x1, 52RBs - MCS13, NTN-TDLC5-200, 1x1, 52RBs - Option 3: Similar test configurations as the existing PDSCH cases

RAN4 has agreed to reuse the Rel-17 NTN PDSCH requirements to define the specifications for RedCap UEs operating over NR-NTN with 2Rx. For RedCap UEs operating over NR-NTN with 1Rx, we support the introduction of new PDSCH requirements tailored to the NTN channel by changing the number of Rx in current NR-NTN requirements from 2 to 1.

According to the applicability rules outlined in Table 8.2.1.1.2-1 of TS 38.101-5, NTN RedCap UEs are also required to pass TN RedCap test cases. To alleviate the testing burden, we propose minimizing the number of additional PDSCH requirements specified in TS 38.101-5 for RedCap UEs. Specifically, we recommend excluding the optional capability for 32 HARQ processes and disabling HARQ feedback.

As for the reference channel, “R.PDSCH.1-1.2  HD-FDD” in TS38.101-4 can be reused to the case of Modulation format and code rate “16QAM, 0.48”. For the case of Modulation format and code rate “QPSK, 0.30”, RAN4 needs to define a new reference channel.

Based on the above, we propose the following additional requirements for RedCap UEs with NR NTN operating FR1-NTN bands.

Proposal 1: Define PDSCH demodulation requirements for RedCap UEs with NR NTN operating FR1-NTN bands:

| Rx number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | HARQ Config | Propagation condition | Correlation matrix and antenna configuration | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | R.PDSCH.1-1.1 FDD in TS38.101-5 | 10 / 15 | QPSK, 0.30 | 16 HARQ Proc | NTN-TDLA100-200 | 1x2, ULA Low | Also applicable to eRedCap without reduced BW (Reuse Test 1-1 in TS38.105) |
|  | New for HD-FDD |  |  |  |  |  |  |
| 2Rx | R.PDSCH.1-2.1 FDD in TS38.101-5 | 10 / 15 | 16QAM, 0.48 | 16 HARQ Proc | NTN-TDLC5-200 | 1x2, ULA Low | (Reuse Test 1-2 in TS38.105) |
|  | R.PDSCH.1-1.2 HD-FDD in TS38.101-4 |  |  |  |  |  |  |
| 1Rx | R.PDSCH.1-1.1 FDD in TS38.101-5 | 10 / 15 | QPSK, 0.30 | 16 HARQ Proc | NTN-TDLA100-200 | 1x1, ULA Low | Also applicable to eRedCap without reduced BW (New) |
|  | New for HD-FDD |  |  |  |  |  |  |
| 1Rx | R.PDSCH.1-2.1 FDD in TS38.101-5 | 10 / 15 | 16QAM, 0.48 | 16 HARQ Proc | NTN-TDLC5-200 | 1x1, ULA Low | (New) |
|  | R.PDSCH.1-1.2 HD-FDD in TS38.101-4 |  |  |  |  |  |  |

Proposal 2: Define a new HD-FDD reference channel for Modulation format and code rate “QPSK, 0.30” and 52PRBs

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.PDSCH.x-y.z HD-FDD |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing | kHz | 15 |
| Number of allocated resource blocks | PRBs | 52 |
| Number of consecutive PDSCH symbols |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 |
| Allocated slots per 2 frames | Slots | 15 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 2664 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 4096 |
| Transport block CRC per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slot i = 0 | CBs | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 1 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 1 |
| Binary Channel Bits Per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slots i = 10, 11 | Bits | 13104 |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 8736 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 13728 |
| Max. Throughput averaged over 2 frames | Mbps | 2.785 |

Issue 3-2-2: PDSCH demodulation requirements for eRedCap Way forward: For 2Rx UEs, - Option 1: - Add new PDSCH requirements with BW reduction (25RB) with NTN channel. - Option 2: - Reuse Rel-17 NTN PDSCH demodulation requirements without BW reduction. - Applicable test cases (TBS ≤ 10,000 bits): TS 38.101-5 Table 8.2.1.2.2.1.1-3 Tests 1-1, 1-3, and 1-4. For 1Rx UE, - FFS whether to add new PDSCH requirements with NTN channel or not. Issue 3-2-6: Test case of PDSCH for eRedCap 1Rx (if defined) Way forward: - Option 1: - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes - MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes - MCS13, NTN-TDLA100-200, 1x1, without reduced BB BW (40RBs), 16 HARQ processes - Option 2: - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RBs) - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RBs) - Option 3: Similar test configurations as the existing PDSCH cases Issue 3-2-7: Test case of PDSCH for eRedCap 2Rx (if defined) Way forward: - Option 1: - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RB), Disabled HARQ - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 16 HARQ processes - MCS4, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 32 HARQ processes - MCS13, NTN-TDLA100-200, 1x2, without reduced BB BW (40RBs), 16 HARQ processes - Option 2: - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RBs) - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RBs) - Option 3: Check if the existing PDSCH requirements for 2Rx can be reused for eRedCap without reduced baseband

There are still lots of options eRedCap requirements. We propose the following additional requirements for eRedCap UEs with NR NTN operating FR1-NTN bands.

Proposal 3: Define PDSCH demodulation requirements for eRedCap UEs with NR NTN operating FR1-NTN bands:

| Rx number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | HARQ Config | Propagation condition | Correlation matrix and antenna configuration | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | R.PDSCH.1-1.1 FDD in TS38.101-5 | 10 / 15 | QPSK, 0.30 | 16 HARQ Proc | NTN-TDLA100-200 | 1x2, ULA Low | without reduced BW (same requirements as RedCap) |
|  | New for HD-FDD |  |  |  |  |  |  |
| 2Rx | R.PDSCH.1-25.1 FDD in TS38.101-4 | 10 / 15 | 16QAM, 0.48 | 16 HARQ Proc | NTN-TDLC5-200 | 1x2, ULA Low | Reduced BW (New) |
|  | R.PDSCH.1-2.3 HD-FDD in TS38.101-4 |  |  |  |  |  |  |
| 1Rx | R.PDSCH.1-1.1 FDD in TS38.101-5 | 10 / 15 | QPSK, 0.30 | 16 HARQ Proc | NTN-TDLA100-200 | 1x1, ULA Low | without reduced BW (same requirements as RedCap) |
|  | New for HD-FDD |  |  |  |  |  |  |
| 1Rx | R.PDSCH.1-25.1 FDD in TS38.101-4 | 10 / 15 | 16QAM, 0.48 | 16 HARQ Proc | NTN-TDLC5-200 | 1x1, ULA Low | Reduced BW (New) |
|  | R.PDSCH.1-2.3 HD-FDD in TS38.101-4 |  |  |  |  |  |  |

Proposal 4: For reference channel, reuse R.PDSCH.1-25.1 FDD and R.PDSCH.1-2.3 HD-FDD in TS38.101-4 for NR NTN eRedCap UEs with reduced bandwidth.

Issue 3-2-8: Applicability of time-varying Doppler shift and propagation delay model for NGSO Way forward: - Option 1: Apply the time-varying Doppler shift and propagation delay model for NGSO to (e)RedCap demodulation requirements. - Option 2: Not to apply the time-varying Doppler shift and propagation delay model for NGSO to (e)RedCap demodulation requirements in Rel-19.

At RAN4#116, companies will present simulation results based on the Release 19 time-varying Doppler shift and propagation delay model. Discussions will focus on whether an additional margin should be applied when evaluating results under a snapshot-based satellite channel assumption. We believe that RAN4 should adopt the time-varying channel model, as it provides a more accurate representation of real-world conditions. However, we are open to further discussions on whether the time-varying channel model should be applied specifically to the NR NTN RedCap/eRedCap requirements. We propose first reaching consensus on the detailed NR NTN RedCap/eRedCap configurations and awaiting the outcomes of the WI NR_IoT_NTN_req_test_enh performance evaluation.

Proposal 5:  We propose first reaching consensus on the detailed NR NTN RedCap/eRedCap configurations and awaiting the outcomes of the WI NR_IoT_NTN_req_test_enh performance evaluation to determine whether to apply the time-varying Doppler shift and propagation delay model for NGSO.

# Conclusion

In this contribution, we provide our views about performance requirements for Rel-19 NTN phase3 and proposals are summarized as follows.

Proposal 1: Define PDSCH demodulation requirements for RedCap UEs with NR NTN operating FR1-NTN bands:

| Rx number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | HARQ Config | Propagation condition | Correlation matrix and antenna configuration | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | R.PDSCH.1-1.1 FDD in TS38.101-5 | 10 / 15 | QPSK, 0.30 | 16 HARQ Proc | NTN-TDLA100-200 | 1x2, ULA Low | Also applicable to eRedCap without reduced BW (Reuse Test 1-1 in TS38.105) |
|  | New for HD-FDD |  |  |  |  |  |  |
| 2Rx | R.PDSCH.1-2.1 FDD in TS38.101-5 | 10 / 15 | 16QAM, 0.48 | 16 HARQ Proc | NTN-TDLC5-200 | 1x2, ULA Low | (Reuse Test 1-2 in TS38.105) |
|  | R.PDSCH.1-1.2 HD-FDD in TS38.101-4 |  |  |  |  |  |  |
| 1Rx | R.PDSCH.1-1.1 FDD in TS38.101-5 | 10 / 15 | QPSK, 0.30 | 16 HARQ Proc | NTN-TDLA100-200 | 1x1, ULA Low | Also applicable to eRedCap without reduced BW (New) |
|  | New for HD-FDD |  |  |  |  |  |  |
| 1Rx | R.PDSCH.1-2.1 FDD in TS38.101-5 | 10 / 15 | 16QAM, 0.48 | 16 HARQ Proc | NTN-TDLC5-200 | 1x1, ULA Low | (New) |
|  | R.PDSCH.1-1.2 HD-FDD in TS38.101-4 |  |  |  |  |  |  |

Proposal 2: Define a new HD-FDD reference channel for Modulation format and code rate “QPSK, 0.30” and 52PRBs

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.PDSCH.x-y.z HD-FDD |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing | kHz | 15 |
| Number of allocated resource blocks | PRBs | 52 |
| Number of consecutive PDSCH symbols |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 |
| Allocated slots per 2 frames | Slots | 15 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 2664 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 4096 |
| Transport block CRC per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 |
| Number of Code Blocks per Slot |  |  |
| For Slot i = 0 | CBs | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 1 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 1 |
| Binary Channel Bits Per Slot |  |  |
| For Slot i = 0 | Bits | N/A |
| For Slots i = 10, 11 | Bits | 13104 |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 8736 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 13728 |
| Max. Throughput averaged over 2 frames | Mbps | 2.785 |

Proposal 3: Define PDSCH demodulation requirements for eRedCap UEs with NR NTN operating FR1-NTN bands:

| Rx number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | HARQ Config | Propagation condition | Correlation matrix and antenna configuration | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | R.PDSCH.1-1.1 FDD in TS38.101-5 | 10 / 15 | QPSK, 0.30 | 16 HARQ Proc | NTN-TDLA100-200 | 1x2, ULA Low | without reduced BW (same requirements as RedCap) |
|  | New for HD-FDD |  |  |  |  |  |  |
| 2Rx | R.PDSCH.1-25.1 FDD in TS38.101-4 | 10 / 15 | 16QAM, 0.48 | 16 HARQ Proc | NTN-TDLC5-200 | 1x2, ULA Low | Reduced BW (New) |
|  | R.PDSCH.1-2.3 HD-FDD in TS38.101-4 |  |  |  |  |  |  |
| 1Rx | R.PDSCH.1-1.1 FDD in TS38.101-5 | 10 / 15 | QPSK, 0.30 | 16 HARQ Proc | NTN-TDLA100-200 | 1x1, ULA Low | without reduced BW (same requirements as RedCap) |
|  | New for HD-FDD |  |  |  |  |  |  |
| 1Rx | R.PDSCH.1-25.1 FDD in TS38.101-4 | 10 / 15 | 16QAM, 0.48 | 16 HARQ Proc | NTN-TDLC5-200 | 1x1, ULA Low | Reduced BW (New) |
|  | R.PDSCH.1-2.3 HD-FDD in TS38.101-4 |  |  |  |  |  |  |

Proposal 4: For reference channel, reuse R.PDSCH.1-25.1 FDD and R.PDSCH.1-2.3 HD-FDD in TS38.101-4 for NR NTN eRedCap UEs with reduced bandwidth.

Proposal 5:  We propose first reaching consensus on the detailed NR NTN RedCap/eRedCap configurations and awaiting the outcomes of the WI NR_IoT_NTN_req_test_enh performance evaluation to determine whether to apply the time-varying Doppler shift and propagation delay model for NGSO.

# Reference

- R4-2508755, Way Forward for [115][333] NR_NTN_Ph3_demod, Ericsson