3GPP TSG-RAN WG4 Meeting # 116	R4-2509192

Bengaluru (Bangalore), India, 25th – 29th August 2025

Agenda Item:	7.24.4

Source: 	Nokia

Title: 	BS RF requirements for low-power wake-up signal for NR

Document for:	Approval

# 1.	Introduction

The revised WI for low-power wake-up signal and receiver for NR was approved at TSG RAN#108 [1]. One of the objectives of this WI is to:

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline

This topic was discussed at TSG RAN4#115 [2] and the WF was agreed [3].

This contribution provides proposals to progress the open issues according to the agreed WF.

# 2.	Discussion

2.1	Whether to define EVM requirements for LP-WUS

It is stated in the agreed WF as follows:

- Options - Option 1: - No BS LP-WUS EVM requirement is specified. - The background and consideration of LP-WUS transmitted signal quality are captured in TR 38.774. - Option 2: - BS LP-WUS EVM requirement as follows, no conformance test for this requirement. - For BS type 1-C and 1-H, the EVM levels of each NR carrier for OOK-4 modulation scheme on LP-WUS is specified in table 6.5.2.2-2. Table 6.5.2.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier Modulation scheme for LP-WUS; Required EVM. OOK-4; [FFS] %. - WF - Down select the options in next meeting

On option 2, the simulation results on the LP-WUS BLER Vs SNR with OOK-based (ED) and OFDM-based (IQ) receivers and different EVM levels (EVM%) using the latest RAN1 agreements [7] are provided in figure 1. In particular, the following simulation assumptions are used to obtain these results:

-	Guard tones used is 6 on either side (30KHz SCS).

-	EVM is added as AWGN that corrupts the transmit constellation.

-	The overlay sequence is based on ZC sequence as agreed in RAN1. The number of overlay sequences are based on RAN1 assumption, i.e., 16 for M=1, 8 for M=2 and 4 for M=4 using 5 information bits for evaluation.

-	Power boosting is not considered.

-	Power pooling (pooling power across MC encoded symbols) is not considered for M=1. However, for M=2 and 4, as the ON/OFF occurs within an OFDM symbol, power pooling is used within OFDM symbol.

(a) M=2										(b) M=4

Figure 1: LP-WUS BLER Vs SNR with different EVM levels

It can be observed that the impact on missed detection error (MDR) is minor for EVM up to 30%, as such the EVM requirement do not seem relevant for LP-WUS based on RAN1 assumptions for LP-WUS operation (in low SNR) and design. Therefore, there is no need to specify the EVM requirement for LP-WUS/LP-SS. Even if the EVM requirement is specified for LP-WUS/LP-SS, it should be sufficiently relaxed such that there is no need to test this requirement, as BS that meets the current EVM requirements for PDSCH specified in TS 38.104 [5] should be able to pass the test without difficulty.

2.2	Restriction on the applicability of LP-WUS operation

It is stated in the agreed WF as follows:

- WF - Option 1: - It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance. - Option 2 - No restriction of LP-WUS operation, this is BS implementation issue. - WF - Down select the options in next meeting

On option 2, the simulation results on the PAPR (at different percentiles) with different transmission schemes (different combinations of LP-WUS and/or NR QAM) using the latest RAN1 agreements [7] are provided in figure 2. In particular, the following simulation assumptions are used to obtain these results:

-	Guard tones used is 0 or 6 on either side (30KHz SCS).

-	The overlay sequence assumption is based on RAN1 agreement, where the 5 information bits are mapped directly to ZC sequence (with 2 roots).

-	Power boosting is not considered.

-	Power pooling (pooling power across MC encoded symbols) is not considered for M=1. However, for M=2 and 4, as the ON/OFF occurs within an OFDM symbol, power pooling is used within OFDM symbol.

(a) 99%ile PAPR

(b) 99.9%ile PAPR

(c) 99.99%ile PAPR

(d) 99.999%ile PAPR

(e) 99.9999%ile PAPR

(f) 99.99999%ile or 100%ile PAPR

Figure 2: PAPR with different transmission schemes

It can be observed that the impact on overall PAPR (at different percentiles) is less than 0.5 dB when LP-WUS is transmitted together with the QAM signals for NR, and this would have some impact on NR EVM depending on the clipping algorithm applied to limit the PAPR of the overall transmit signal. However, clipping algorithm is an implementation issue and LP-WUS operation with higher order modulation is a scheduling decision of the BS. Therefore, there is no restriction of LP-WUS operation with higher order modulation, this is BS implementation issue.

# 3.	Conclusion

This contribution has provided proposals to progress the issues according to the agreed WF in last RAN4 meeting.

Proposal 1: There is no need to specify the EVM requirement for LP-WUS/LP-SS.

Proposal 2: There is no need to test the EVM requirement for LP-WUS/LP-SS even if it is specified.

Proposal 3: There is no restriction of LP-WUS operation with higher order modulation.

# References

[1]	RP-251200, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO.

[2]	R4-2507840, “Topic summary for [115][308] NR_LPWUS”, Huawei.

[3]	R4-2508668, “Way Forward for [115][308] NR_LPWUS”, Huawei.

[4]	RP-251199, “Status report for WI: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO.

[5]	3GPP TS 38.104 v19.0.0: “NR; Base Station (BS) radio transmission and reception”.