3GPP TSG RAN WG4 Meeting #116		R4-2509204

Bengaluru, India, August 25th-29th, 2025

Agenda item:	7.10.4.1

Source:	Telecom Italia S.p.A., Vodafone, Deutsch Telekom, Orange

Title:	Proposals for Rel-19 TRP TRS requirements working procedure

WI/SI:	TRP_TRS_MIMO_OTA_Ph3

Release:	Rel-19

Document for:	Approval

# 1	Introduction

During RAN4#115 meeting, discussion on Rel-19 TRP TRS requirements related to the WI [1] was held which outcomes and agreements have been captured in the WF [2]. This contribution proposes some changes to the working procedure for TRP TRS Performance Test Campaign defined in [3] and updated according to the WF Issue 4-2-2 [2] to address some of the open and not threated points from the discussion at RAN4#115 meeting.

# 2	1Tx FR1 TRP TRS requirements

Proposal 1: Adopt the following changes to the working procedure to define the TRP TRS requirements:

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: n3, n5, n7, and n8 [2Tx at bands n41;]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand and PDA hand

  - Operation mode: SA with 1Tx as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: UE (width >56mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: second-half 2022 to first-half 2025

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - FFS for the same UE model with supporting different set of bands

  - Logistical aspects for devices provisioning to the labs are TBD

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template [Rel-18 R4-2408104 as starting point, update is needed] can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary as soon as available (i.e., before the measurement activity on such devices starts) using the template defined in R4-2509208

      - The RAN4 Secretary updates the summary of statistical information  and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

      - The complete device information sheet shall be available by RAN4#117 meeting (i.e., not later than the TDoc submission deadline) to evaluate the accomplishment of the defined thresholds and act accordingly in case these are not

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Fulfilment of the following thresholds (for each band and power class) related to the devices pool for Rel-19:

      - Minimum number of devices’ samples: 60

      - Minimum number of device models: 50

      - Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: ≥95%

      - Minimum number of devices' vendors: 6

      - Percentage of devices from second-half 2022 to first-half 2025: 100%

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

  - Laboratories use the same worksheet template R4-2509206 to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is [15] (depends on how many test labs will join the activity) with a minimum of 3 devices.

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - All the measurement results shall be submitted by RAN4#117 meeting (i.e., not later than the TDoc submission deadline) to provide enough time to the companies to analyse the provided results and let RAN4 have an initial discussion on the requirements’ values during the meeting.

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline.

  - Inclusion in the data set of measurement results related to faulty devices shall be avoided:

    - All the measurements results shall be submitted by RAN4#117 meeting

    - Method to identify faulty devices is FFS; some options are reported below:

      - [Define a threshold under which the device is considered a faulty sample

      - Define a delta with respect to the maximum measured value under which the device is considered a faulty sample

      - Analysis of the DCF curve to identify deviations of the lower tail and therefore a faulty sample of the device

      - Other options are not precluded]

    - Each identified faulty device can be replaced with another sample of the same model and measured by the test lab by RAN4#118 meeting

    - If the new measurement results cannot be provided by RAN4#118 meeting, the values related to the faulty device will be removed from the data set

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, for TDD bands 3dB offset, for FDD bands TBD.

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

- Test lab procedures for 2Tx

  - FFS

# 3	Conclusions

In the following, the proposal of the contribution is summarized:

Proposal 1: Adopt the following changes to the working procedure to define the TRP TRS requirements:

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: n3, n5, n7, and n8 [2Tx at bands n41;]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand and PDA hand

  - Operation mode: SA with 1Tx as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: UE (width >56mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: second-half 2022 to first-half 2025

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - FFS for the same UE model with supporting different set of bands

  - Logistical aspects for devices provisioning to the labs are TBD

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template [Rel-18 R4-2408104 as starting point, update is needed] can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary as soon as available (i.e., before the measurement activity on such devices starts) using the template defined in R4-2509208

      - The RAN4 Secretary updates the summary of statistical information  and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

      - The complete device information sheet shall be available by RAN4#117 meeting (i.e., not later than the TDoc submission deadline) to evaluate the accomplishment of the defined thresholds and act accordingly in case these are not

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Fulfilment of the following thresholds (for each band and power class) related to the devices pool for Rel-19:

      - Minimum number of devices’ samples: 60

      - Minimum number of device models: 50

      - Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: ≥95%

      - Minimum number of devices' vendors: 6

      - Percentage of devices from second-half 2022 to first-half 2025: 100%

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

  - Laboratories use the same worksheet template in R4-2509206 to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is [15] (depends on how many test labs will join the activity) with a minimum of 3 devices.

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - All the measurement results shall be submitted by RAN4#117 meeting (i.e., not later than the TDoc submission deadline) to provide enough time to the companies to analyse the provided results and let RAN4 have an initial discussion on the requirements’ values during the meeting.

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline.

  - Inclusion in the data set of measurement results related to faulty devices shall be avoided:

    - All the measurements results shall be submitted by RAN4#117 meeting

    - Method to identify faulty devices is FFS; some options are reported below:

      - [Define a threshold under which the device is considered a faulty sample

      - Define a delta with respect to the maximum measured value under which the device is considered a faulty sample

      - Analysis of the DCF curve to identify deviations of the lower tail and therefore a faulty sample of the device

      - Other options are not precluded]

    - Each identified faulty device can be replaced with another sample of the same model and measured by the test lab by RAN4#118 meeting

    - If the new measurement results cannot be provided by RAN4#118 meeting, the values related to the faulty device will be removed from the data set

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, for TDD bands 3dB offset, for FDD bands TBD.

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

- Test lab procedures for 2Tx

  - FFS

# 4	References

- RP-250197, “Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3”, vivo, CAICT, March 2025

- R4-2508782, “Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3”, vivo, May 2025

- R4-2504763, “WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3”, vivo, April 2025