3GPP TSG-RAN WG4 Meeting #116															 R4-2509218

Bengaluru, 25th - 29th August, 2025

Agenda item:			7.7.7

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for Ku bands for NR NTN

Document for:	Discussion

# Introduction

In this paper, the RRM requirement for Satellite switching with re-sync for Ku band is further discussed.

# Satellite switching with re-sync for Ku band

Issue 1-3: satellite switch requirements

Agreement:

- Introduce RRM requirement for satellite switching with re-sync for Ku band with VSAT UE for FR1 and FR2 numerologies

  - Define requirement for hard satellite switch,

    - Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms

      - Further discuss the details on the definition of each component.

  - FFS whether to define requirements for soft satellite switch

    - FFS only switching delay requirement or both switching delay requirement and scheduling restriction requirement

    - Option for the delay requirement:

      - Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

In last meeting, the Ku band RRM requirements for satellite switching with re-sync was agreed as the following [1].

For both hard and soft satellite switch

Regarding the details on the definition of each component:

- Tsearch , Tprocessing, T∆, Tmargin:  the legacy definition in “6.1C.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization” can be reused.

- Tsat_beam:  the legacy definition in “6.1C.1.3	NR SAN FR2-NTN – NR SAN FR2-NTN Handover” can be reused. Whether to consider both ‘electronic’ and ‘mechanical’ beam steering type can be further discussed.

  - Tsat_beam is additional time for UE to steer the downlink spatial domain reception filter to the target cell.

  - For UE indicating ‘electronic’ via UE capability ntn-VSAT-AntennaType-r18, Tsat_beam is 3*Trs.

  - For UE indicating ‘mechanical’ via UE capability ntn-VSAT-AntennaType-r18, Tsat_beam is Oangle / 22.5 s, where Oangle is the angle offset observed from UE in degree between the satellite for the serving cell and the satellite for the target cell.

Proposal 1: For RRM requirement for satellite switching with re-sync for Ku band:

- Tsearch, Tprocessing, T∆, Tmargin: reuse the legacy definition in “6.1C.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization”.

- Tsat_beam:  reuse the legacy definition in “6.1C.1.3	NR SAN FR2-NTN – NR SAN FR2-NTN Handover”.

Soft satellite switch – scheduling restriction

In soft satellite switch scenario, UE may perform pre-sync toward the target cell (i.e. Sat#2) before t-Service, and the scheduling restriction on SSB symbols is allowed in the current FR1 Satellite switching with re-synchronization.

<From TS38.133 6.1C.3.2.3.>

During the time period from t-seviceStart to t-service, scheduling restriction as defined in clause 9.2C.5.3 is allowed, with the exception that the locations of SSB symbols of target satellite where scheduling restriction applies are determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the SSB-TimeOffset reference point as defined in TS 38.331 [2] to UE.

In Ku band, the additional beam switching period (TRX_switch) during SSB measurements, shown in yellow in Figure 1, should be considered. TRX_switch can be much shorter than the legacy Tsat_beam (3*Trs), which include 3 trials, while TRX_switch only covers switching to the new target cell. Due to the large number of antenna elements on the UE/VSAT, TRX_switch can be a few milliseconds (e.g. 2ms).

Figure 1 Timeline of Soft satellite switch

Proposal 2: Introduce RRM requirement for “soft” satellite switching with re-sync for Ku band.

Proposal 3: For soft satellite switch in Ku band, extend scheduling restriction to TRX_switch [ms] before and after each consecutive SSB symbols. Where TRX_switch is switch period for UE to steer the downlink spatial domain reception filter toward a target cell. TRX_switch = [2] ms.

For the overall soft satellite switch delay, the Tprocessing (after t-Service) should include the beam switching time TRX_switch, since the UE disconnects form the original link and switches to the new target at t-Service.

Proposal 4: For soft satellite switch in Ku band,

- Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + TRX_switch+ Tprocessing  ms.

# Summary

The observations and proposals are summarized as the following:

Proposal 1: For RRM requirement for satellite switching with re-sync for Ku band:

- Tsearch, Tprocessing, T∆, Tmargin: reuse the legacy definition in “6.1C.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization”.

- Tsat_beam:  reuse the legacy definition in “6.1C.1.3	NR SAN FR2-NTN – NR SAN FR2-NTN Handover”.

Proposal 2: Introduce RRM requirement for “soft” satellite switching with re-sync for Ku band.

Proposal 3: For soft satellite switch in Ku band, extend scheduling restriction to TRX_switch [ms] before and after each consecutive SSB symbols. Where TRX_switch is switch period for UE to steer the downlink spatial domain reception filter toward a target cell. TRX_switch = [2] ms.

Proposal 4: For soft satellite switch in Ku band,

- Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + TRX_switch+ Tprocessing  ms.

# Reference

- R4-2508262, WF on RRM requirements for NR_NTN_Ku_bands, May 2025.