3GPP TSG-RAN WG4 Meeting #116										 R4-2509223

Bengaluru, 25th - 29th August, 2025

Agenda Item:	7.27.5.2

Source:	MediaTek Inc.

Title:	Discussion on NTN RRM requirements for SSB periodicity up to 160ms

WID/SID:        NR_NTN_Ph3-Core

Document for:	Discussion

# 1 Introduction

The NTN RRM requirements for SSB periodicity up to 160ms to support Downlink Coverage Enhancement was discussed. In this paper, we further discussed the impact on neighbor cell measurement.

# 2 RRM requirements for SSB periodicity up to 160ms

The UE monitors the serving cell SSB every 160 ms to meet timing accuracy requirements. If the serving cell SSB periodicity is 160 ms and the Measurement Gap overlaps with SSB occasions, the UE may remain on the serving frequency to measure the SSB, and inter-frequency measurements cannot be performed. (see Figure 1). In the other words, if the MG completely fully overlaps with the serving cell’s SSB occasions at a 160 ms periodicity, inter-frequency measurement requirements do not apply.

Figure 1 MG fully overlaps with the serving cell SSB occasions with a 160 ms periodicity

Proposal 1: Inter-frequency measurement requirements do not apply if the MG fully overlaps with the serving cell SSB occasions with a 160 ms periodicity.

If the MG partially overlaps with the serving cell’s SSB occasions with a 160 ms periodicity (see Figure 2), by following the legacy gap sharing scheme (as in Table 9.1.2.1a), the network can set a suitable gap sharing factor X (e.g. 50% for intra-frequency) to guarantee at least 1 serving cell SSB available within 160 ms.

Figure 2 MG partially overlaps with the serving cell SSB occasions with a 160 ms periodicity

Proposal 2: Inter-frequency measurement requirements apply when the MG partially overlaps with the serving cell SSB occasions with a 160 ms periodicity, if the gap sharing scheme applies and at least 1 serving cell SSB available every 160 ms on the serving carrier.