3GPP TSG-RAN WG4 Meeting #116	R4-2509232

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on performance part for Rel-19 LTM

Agenda item:	7.25.4

Document for:	Discussion

Introduction

This contribution discusses about the details of necessary test cases to complete the performance part of Rel-19 mobility enhancements work item.

Discussion

Event-triggered L1 reporting (TBD)

For event-triggered L1 reporting, RAN4 defined and agreed draft CRs for event-triggered reporting delay considering:

- Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

- Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

- Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

RAN4 should define test cases to cover all the listed event-triggered reporting cases. Additionally, RAN4 should define at least one test case to verify that the UE correctly follows event-triggered periodic reporting requirements.

To cover the testing of all these requirements, we propose to introduce the following test cases:

Introduce the following test cases for event-triggered LTM L1 reporting:
- FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)
- FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)
- FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)
- At least 1 test case for event-triggered periodic reporting.

RAN4 agreed the reporting delay to be the maximum measurement period of the cells involved in the event. RAN2 has agree to define the following L1 events for LTM reporting:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In the test cases, we propose to define the test setup so that there is one serving cell and one candidate cell. One of the cells’ (e.g. serving cell) signal quality shall remain constant during the test. The signal level of the other cell (e.g. candidate cell) shall be changed during the test so that the reporting condition becomes fulfilled, and the UE shall send the report within the defined reporting delay. We propose to use LTM3 as the reporting event in the test cases.

On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

Use LTM3 as the reporting event in the test cases.

CSI-RS based L1 measurement

CSI-RS based L1 measurement accuracy

The following agreement was made previously regarding CSI-RS L1-RSRP measurements accuracy:

RAN4#90bis Agreement: The L1-RSRP accuracy requirements are defined based on 48PRB. - Clarify in the spec that the performance with larger bandwidth is equal to or better than the accuracy based on 48PRB. - The requirement for larger or lower bandwidth will be further discussed in the future release.

In release 19 RAN4 agreed to have requirements for intra-frequency and when at least 48 PRBs are within the active BWP. The legacy accuracy requirements have been based already on 48 PRB assumption. Therefore, we think it is ok to reuse them but place them into a new section as they are for LTM candidate cells.

Intra-frequency CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbouring cell. I.e., reuse CSI-RS L1-RSRP accuracy requirements with 48 PRB assumption.

Define CSI-RS accuracy requirements in a new clause (similar approach to Rel-18 LTM)

As the legacy L1-RSRP accuracy requirements are already based on 48 PRB and the current minimum requirement defines that 48 PRBs need to be within the active BWP, the test configuration should have similar assumption. In other words, if UE is configured with 48 PRB CSI-RS resource within active BWP, the UE shall pass the test case.

Test configuration is set to have 48 PRBs within the active BWP.

CSI-RS measurement test cases

As only intra-frequency without gaps test cases can be defined in rel-19 and based on the current agreements.

Define only intra-frequency test cases without gaps in rel-19

From the test coverage point of view, we should make sure that there is test coverage for features specified in the core requirements and those that are supported by RAN1. RAN4 can define test cases according to the following RAN1 table

| CSI-RS L1-RSRP measurements | CSI-RS L1-RSRP reporting | Reporting format | FR1 / FR2 | FR2 SSB / CSI-RS beam sweeping |
| --- | --- | --- | --- | --- |
| Periodic | Periodic | UCI | FR1 and FR2 | SSB, CSI-RS |
| Periodic | Semi-persistent | UCI | FR1 and FR2 | SSB, CSI-RS |
| Periodic | Aperiodic | UCI | FR1 and FR2 | SSB, CSI-RS |
| Semi-persistent | Semi-persistent | UCI | FR1 and FR2 | SSB, CSI-RS |
| Semi-persistent | Aperiodic | UCI | FR1 and FR2 | SSB, CSI-RS |
| Periodic | Event-triggered | MAC CE | FR1 and FR2 | SSB, CSI-RS |

Define at least the following test cases and align with event triggered test cases to avoid overlap

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1

  - Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping

CSI Acquisition test case

Currently CSI acquisition requirements are still being discussed. However, if these are to be defined, at least the following test cases can be considered.

- Test case to verify early CSI acquisition before the cell switch command

- Test case to verify early CSI acquisition after the cell switch command

Define test case(s) to verify early CSI acquisition before the cell switch command, conditionally to core requirement definition.

Define test case(s) to verify early CSI acquisition after the cell switch command, conditionally to core requirement definition.

The frequency location of the CSI-acquisition may also be further discussed. For instance, whether the case where CSI-acquisition RS is outside of the active BWP either fully or partially.

Discuss whether there is need to consider configuration where CSI-acquisition resource(s) are outside of the active BWP

Conditional LTM

RAN4 has defined cell switch delay requirements for CLTM cell switch. The delay requirement is to cover the case of L1- and L3-based conditional cell switch. Furthermore, the evaluation delay Tmeasure may depend on whether gap-based or gap-less intra- or inter-frequency measurements are configured, and whether the cell switch happens towards an FR1 or FR2 target cell. Even further, the cell switch delay may depend on whether the cell switch is RACH-less or RACH-based, whether early ASN.1 decoding is supported, and whether early DL sync is performed. RAN4 has also defined requirements for subsequent LTM cell switch. This will lead to a long test case list, if all the supported scenarios are included.

On high level, we propose that both cell switch based on L1-based condition (in FR1 and FR2) and L3-based condition (in FR1) are to be tested. There should additionally be at least one test case on both FR1 and FR2 to verify cell switch interruption with and without early DL synchronization (TCI state activation). Also, both RACH-less and RACH-based cell switch need to be tested. RAN4 should discuss how many test cases are needed to cover the proper testing of each of these sub-features, considering both initial and subsequent cell switch.

RAN4 to discuss how to ensure proper testing of CLTM cell switch delay taking into account the testing of:
-   L1-based conditional cell switch (intra- and inter-frequency, FR1 to FR1 and FR2 to FR2) and L3-based conditional cell switch (intra- and inter-frequency FR1 to FR1)
-   RACH-less and RACH-based conditional cell switch (i.e., with and without PDCCH ordered RACH)
-   Conditional cell switch with and without early DL synchronization (i.e., with and without early TCI state activation)
-   Initial and subsequent CLTM cell switch

On high level, the setup of event-triggered L1 reporting can be followed in the CLTM test cases. There can be two cells in the test: serving cell and candidate cell. At some point after reception of the conditional LTM configuration the cell switch condition becomes met during the test, and the UE shall switch to the candidate cell within the specified delay. If early DL and UL synchronization are tested in the test case, these shall be timed correctly in the test case so that the UE can be ensured to have the DL synchronization and a valid TA at the time of condition becoming met.

On high level, CLTM test cases to have serving cell and candidate cell, and the UE shall trigger cell switch based on signal level of one of these cells changing so that the cell switch condition becomes met.

Ensure in the test cases with early UL and DL synchronization that the UE has valid DL synchronization for the target TCI state and a valid TA at the time of condition becoming met.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Introduce the following test cases for event-triggered LTM L1 reporting: 
- FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps) 
- FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps) 
- FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps) 
- At least 1 test case for event-triggered periodic reporting.

Proposal 2: On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

Proposal 3: Use LTM3 as the reporting event in the test cases.

Proposal 4: Intra-frequency CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbouring cell. I.e., reuse CSI-RS L1-RSRP accuracy requirements with 48 PRB assumption.

Proposal 5: Define CSI-RS accuracy requirements in a new clause (similar approach to Rel-18 LTM)

Proposal 6: Test configuration is set to have 48 PRBs within the active BWP.

Proposal 7: Define only intra-frequency test cases without gaps in rel-19

Proposal 8: Define at least the following test cases and align with event triggered test cases to avoid overlap

1.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1

2.	Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1

3.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping

4.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping

5.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping

6.	Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping

Proposal 9: Define test case(s) to verify early CSI acquisition before the cell switch command, conditionally to core requirement definition.

Proposal 10: Define test case(s) to verify early CSI acquisition after the cell switch command, conditionally to core requirement definition.

Proposal 11: Discuss whether there is need to consider configuration where CSI-acquisition resource(s) are outside of the active BWP

Proposal 12: RAN4 to discuss how to ensure proper testing of CLTM cell switch delay taking into account the testing of: 
-   L1-based conditional cell switch (intra- and inter-frequency, FR1 to FR1 and FR2 to FR2) and L3-based conditional cell switch (intra- and inter-frequency FR1 to FR1) 
-   RACH-less and RACH-based conditional cell switch (i.e., with and without PDCCH ordered RACH) 
-   Conditional cell switch with and without early DL synchronization (i.e., with and without early TCI state activation) 
-   Initial and subsequent CLTM cell switch

Proposal 13: On high level, CLTM test cases to have serving cell and candidate cell, and the UE shall trigger cell switch based on signal level of one of these cells changing so that the cell switch condition becomes met.

Proposal 14: Ensure in the test cases with early UL and DL synchronization that the UE has valid DL synchronization for the target TCI state and a valid TA at the time of condition becoming met.