3GPP TSG-RAN WG4 Meeting # 116	R4-2509241

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.18.3

Source: 	National Taiwan University

Title: 	Decoder Selection Criterion Based on AI/ML Interoperability from Standardization of Testing and Verification Perspective

Document for:	Approval

Introduction

We present our view on partially specified decoder and reference decoder derivation procedure in this contribution.

Discussion

Discussion for Reference/Test Decoder Selection Criterion and Inter-Operability

In both option 3 and 4 discussions, decoder selection criterion is very arbitrarily decided and no specific conclusion is captured. We provide our view below based on the previous TR agreements.

Most of discussions so far were focusing on the achievable performance as criterion. However, in the agreed procedure, after selected a decoder, the next step is:

Each company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

In order to achieve performance alignment, it is preferred to select a decoder that achieves similar performance when connecting to different encoders with different structures, e.g., the structure options discussed previously for the test decoder, but trained with the same dataset produced by the decoder, i.e., the decoder input and output dataset. Otherwise, even if a decoder can achieve a better performance when connected to the jointly trained encoder, that decoder might fail to achieve performance alignment in the agreed next step, and RAN4 has to reselect the decoder. In addition, we have the following agreements in TR 38.843 for interoperability and test repeatability:

- Test repeatability should be ensured (variation among TE vendor implementations should be bound)

- Other vendors should also be able to develop such a decoder and which can deliver similar performance

Based on the agreement in the TR, we need to check the implementation variation across different decoders from option 3, option 4a and 4b from test repeatability perspective. Performance and complexity determine how relevant the tests are in practice, and we need to take them into consideration. Therefore, we propose the following selection criterion from [1]:

Proposal 1: Consider the following as (test/reference) encoder/decoder selection criterion:

- Achievable performance metrics: SGCS or NMSE

- Complexity: including flops or model storage size, can consider to set an upper bound

- Robustness (based on performance metrics): whether the decoder can achieve satisfactory performance when connecting to encoder with different structures with the decoder, but trained with the decoder’s input and output dataset

The evaluation of performance metrics and complexity is more straightforward. The robustness metric is for guaranteeing test repeatability and interoperability. We propose the following procedure to derive the robustness metric as part of selection criterion:

Proposal 2: For all the candidate decoders, evaluate the robustness metrics based on test repeatability and interoperability by the following procedure

- Companies contribute encoders trained with the candidate decoders (one encoder per candidate decoder)

- Connect a encoder to the corresponding decoder, run the test based on the agree test dataset and record the SGCS

- Repeat step 2 for all the encoder and decoder pairs, record all the SGCS

- For each candidate decoder, capture the variation of the SGCSs when connecting to different encoders, and evaluate the variation together with performance metrics and complexity

In addition, for option 4, none of the options are complete enough for TE vendors to derive the test decoders that meet the test repeatability requirements. For example, in 4a, there are different ways to train the test decoders and the achieved loss functions from the specified dataset can vary a lot, and we still don’t have any test repeatability guarantee across different test decoders. Similarly, the joint training with reference encoder and yield different test decoders without guaranteed test repeatability. Therefore, additional requirements/specifications are needed for dataset based and reference encoder-based approach. We provide the suggestions for further specifications for each option in the following:

Dataset based approach 4a: we can specify a loss/similarity function of the test decoder output and the specified dataset for a given input as follows

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset, and f is the chosen loss/similarity function to evaluate the difference between the decoder output from the dataset corresponding to the latent message z, gref(z), and the test decoder output from z, gtest(z).

The drawback of the approach is that both the test decoder (and implicitly the DUT encoder) are heavily restricted by the dataset due to the specified constraint, and we don’t expect that the DUT encoder can work in practice unless in an environment that is very closely resembled with the specified dataset. In fact, the purpose of the test is, as agreed, “verify whether the minimum performance of AI/ML functionality/feature can be achieved”. But we don’t expect such dataset shared between the UE vendors and infra vendors is available for all the application scenarios, and therefore the performance achieved here is definitely not the minimum performance we can expect in the field. Alternatively, if we consider the approach proposed in the previous meeting (and included also in section 2.2), we can derive a more flexible requirement that can enable both TE vendors and UE vendors to develop decoders and encoders that can be trained by different datasets than the specified one. Such a flexibility can get us closer to the agreed test goal, since we allow the encoders and decoders to be trained by other dataset as long as the specified requirement in the form of latent message/decoder output ordering is fulfilled. To be more specified, the alternative requirement is:

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space.

Similarly, we can add the requirement for option 4b as

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input during test decoder verification procedures. Therefore, we also need to specified the generation procedures of X.

Alternatively, based on the proposal in section 2.2, we can specify the requirements of the test decoder as

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

In summary, we have the following proposal:

Proposal 3: Additional steps for option 4a and 4b feasibility study:

- Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

- RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Observation 1: Option 4a-1 and 4b-1 require both TE vendors and UE vendors to train the test decoder and DUT encoder with the dataset or reference encoder, while option 4a-2 and 4b-2 allow TE vendors and UE vendors to train the test decoder and DUT encoder with arbitrary dataset or encoders/decoders as long as the specified ordering requirements are satisfied.

Conclusion

Proposal 1: Consider the following as (test/reference) encoder/decoder selection criterion:

- Achievable performance metrics: SGCS or NMSE

- Complexity: including flops or model storage size, can consider to set an upper bound

- Robustness (based on performance metrics): whether the decoder can achieve satisfactory performance when connecting to encoder with different structures with the decoder, but trained with the decoder’s input and output dataset

Proposal 2: For all the candidate decoders, evaluate the robustness metrics based on test repeatability and interoperability by the following procedure

- Companies contribute encoders trained with the candidate decoders (one encoder per candidate decoder)

- Connect a encoder to the corresponding decoder, run the test based on the agree test dataset and record the SGCS

- Repeat step 2 for all the encoder and decoder pairs, record all the SGCS

- For each candidate decoder, capture the variation of the SGCSs when connecting to different encoders, and evaluate the variation together with performance metrics and complexity

Proposal 3: Additional steps for option 4a and 4b feasibility study:

        - Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

        - RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Observation 1: Option 4a-1 and 4b-1 require both TE vendors and UE vendors to train the test decoder and DUT encoder with the dataset or reference encoder, while option 4a-2 and 4b-2 allow TE vendors and UE vendors to train the test decoder and DUT encoder with arbitrary dataset or encoders/decoders as long as the specified ordering requirements are satisfied.

Reference

[1] C.-H. Huang, C.-K. Wen, G. Y.  Li, “AI/ML Life Cycle Management for Interoperable AI Native RAN”, submitted to IEEE Communication Standard Magazine, available at http://arxiv.org/abs/2507.18538